3GPP TSG-RAN WG2 Meeting #130	R2-2504466

St.Julians, Malta, May 19th – 23rd, 2025

Agenda Item:	8.2.3

Source:	HONOR

Title: 	Discussion on A-IoT random access

Document for:	Discussion and decision

1	Introduction

During RAN2#129 bis meeting, RAN2 discussed the A-IoT random access and made several agreements as:

Agreements on msg1

1.	In case of CBRA, only 16 bits random ID is included in Msg1.  FFS can be revisited if message type will be needed for other D2R messages purposes

2.	RN16 is not included in the first D2R message in the CFRA procedure.  AS ID is the only ID needed for addressing the device in R2D command message assuming for CFRA no multiple devices are performing the procedures with the given reader.   FFS if we can assume or need to support multiple device scenario.

Agreements on new R2D message

1.	A new R2D message other than the paging message is introduced for A-IoT device determining MSG1 resources unless RAN1 concludes to use L1 signaling.   The R2D message indicates the start of a set of MSG1 resources that were configured in paging message.

2.	Assumption: The R2D message does not include slot number/count down number.

Agreements on msg 2

1.	A-IoT Msg2 contains one or multiple echoed random ID(s) from A-IoT Msg1 of different A-IoT devices.

2.	Same Msg2 format is used for initial transmission and retransmission of Msg2.

Agreements on NACK

1	For CBRA, as a baseline, NACK based mechanism is applied only to the Msg3.   May come back for D2R data, if the NACK feedback indication is needed for the purpose to stop/terminate the “on-going procedure” and release the AS ID accordingly (depending on other later discussion).

2	For msg3, we rely on whether the device receives NACK indication before subsequent R2D message to determine re-access.    No need for a timer.   FFS whether subsequent R2D message is trigger message or paging

3	For CFRA, NACK feedback and re-access is not supported.  FFS how to achieve

4	FFS on end of procedure

In this contribution, we analyze the remaining issues of the A-IoT random access and provide proposals related to A-IoT random access procedure and the failure handling.

- Discussion

Ambient IoT, which is introduced as a new 3GPP IoT technology, aims to rely on ultra-low complexity devices with ultra-low power consumption for the very-low end IoT applications and address some representative use cases. In Release 19 A-IoT WI [1], the contention-based random access is limited to the 3-step only procedure.

As shown in the TR [2], the A-IoT random access procedure consists of Random access type and access occasion/resource determination, (optional) Contention resolution of contention-based random access, and the upper layer data transmission with the reader. Furthermore, the failure handling for the random access is also important. Below, we analyse some issues found in the random access and provide the WF accordingly.

2.1 (Issue 2-4) Failure detection

For 3-step CBRA, the device considers the contention resolution to be a success only when the reader acks the same ID received in Msg1. In another word, the device considers the contention resolution as failed, if the Msg2 including the same ID in Msg1 is not received before the next occasion/slot. This device would join the subsequent paging procedure since it has not completed the RA for the previous paging with the same paging ID.

Observation 1: The device would regard the D2R transmission as failure if it has not received the corresponding mandatory R2D transmission, for example, the Msg2 is not received after sending the Msg1.

For 2 step random access, it is up to the reader implementation whether A-IoT Msg2 is sent in random access as shown in the TR. The device waits for the optional R2D transmission until the reception the Msg2 with its own RN or the next slot. What’s more, the device would not perform the re-access even if it has not received the Msg2 with its RN in Msg1. This issue is more severe for the 2-step CBRA resulted from the contention than CFRA. It is the same situation for the Msg3 in 3-step CBRA since the R2D failure/success feedback indication is optional. The device will not perform re-access in the following rounds if it is not aware of whether the Msg1/Msg3 is not well-received by the reader. That is, it is hard for the device to detect the failure without an implicit indication for Msg3 in 3-step CBRA and 2-step RA.

Observation 2: It is hard for the device to detect D2R data transmission failure if it has not received explicit R2D failure/success feedback indication since the subsequent R2D transmission is optional based on the reader implementation.

However, since RAN1 has studied A-IoT processing time aspects in terms of the following timing relationships between the D2R transmission and the corresponding R2D transmission following it:

TD2R_min: Minimum time between a D2R transmission and the corresponding R2D transmission following it. TD2R_max: Maximum time between the D2R transmission and the corresponding R2D transmission following it, so that the R2D transmission timing is expected to be within [TD2R_min, TD2R_max], when a R2D transmission in response to a D2R transmission is expected for A-IoT Msg2 response to A-IoT Msg1 for the A-IoT device. See clause 6.3 for message descriptions.

As shown in the chart, the device expects the R2D transmission corresponding to the D2R transmission ahead would be sent before the TD2R_max.

Observation 3: Failure could be detected from perspective of device based on the TD2R_max (maximum time between the D2R transmission and the corresponding R2D transmission following it) defined by RAN1.

For example, Msg2 following the Msg1 is mandatory in the 3-step CBRA. Considering the failure occurs shortly before, it is possible that the explicit indication may be missed by the device. Furthermore, the reader could detection the contention without identifying the specific device(s) in this case. Based on the observation before, from the perspective of the device, it could regard the corresponding D2R transmission as failure if the R2D transmission is not received within the certain time from the corresponding D2R transmission. Thus, even though the explicit indication is missed by the device, it could determine the failure based on the gap between the D2R transmission and its following R2D transmission. For example, the TD2R_max could be used to determine the failure when there supposes to be a corresponding R2D transmission after the D2R transmission. The A-IoT processing time TD2R_max seems to be the pre-defined timer based on the UE/NW capability. In addition, the maximum time between the D2R transmission and the corresponding R2D transmission following it could be pre-configured by the reader based on its scheduling strategy or load condition. Thus, based on pre-configured/pre-defined timer, the failure detection from the device side could be the supplementary solution for the agreed explicit indication under the control of the network.

Proposal 1: (Issue 2-4) Support the failure detection based on a timer which could be pre-configured time in the R2D trigger or the pre-defined time in RAN1.

The NACK message following Msg3 could be used to indicate the Msg3 transmission failure and provide some performance improvement. That is, the device would perform the re-access or re-transmission based on the indication from the reader. Furthermore, there are some circumstances that the device could determine the success/failure based on the absent/present of the subsequent R2D transmission. On one hand, the absent of the mandatory subsequent R2D message indicates the failure of the last D2R message. For example, during 3-step CBRA, the Msg2 is mandatory to ack the RN16 in the Msg1. The absent of the Msg2 would indicate the failure of the Msg1. On the other hand, the present of the subsequent R2D message indicates the success of the last D2R message. For example, in the inventory + command use case, the command would be conveyed to the device after the reader receives the device ID in the last D2R message (i.e., Msg3 in CBRA or Msg1 in CFRA). From the perspective of the device, the Msg3 is well received if it receives the command for it and it could skip the failure handling caused by Msg3 failure (e.g., the re-access in the subsequent paging procedure). Thus, the failure detection could be performed based on the signalling in the current procedure as following.

Proposal 2: (Issue 2-4) The subsequent R2D transmission for the device indicates that there is no failure handling for the last D2R message.

Proposal 3: (Issue 2-4) The absent of subsequent R2D transmission (e.g., Msg2 after Msg1 in CBRA) indicates the device should perform failure handling caused by D2R failure.

In addition, RAN2 already agreed that the NACK message is introduced for the device to determine re-access for at least msg3. If the NACK based mechanism as in the proposal 2 could work, the NACK message could be used to confirm the failure of Msg1 and Msg3. Thus, the present of the NACK message indicates the device to perform the failure handling. For Msg3 failure, the reader could identify the specific device and send the NACK message to the specific device. However, for Msg1 failure, the reader could not identify the specific when the failure is caused by contention and the NACK message may not be target to the specific device.

Proposal 4: (Issue 2-4) The present of the NACK indicates the device(s) to perform the failure handling.

2.2 (Issue 2-10/11) NACK feedback

To handling the detected/indicated failure, one solution for the device is to perform re-access during the subsequent paging procedure with the same transaction ID. Another solution is to re-transmit the failed message. Especially for the timer-based failure detection on the device side, the device could trigger the re-transmission. For example, when the device has not received the Msg2 after a certain time from the Msg1, it could trigger the retransmission of Msg1. However, since the retransmission requires some resources, the device could only trigger the retransmission before the next slot (e.g., R2D trigger). Whether to apply the offset to the retransmission for the unsegmented packet is left as FFS when discussing the segment retransmission. We think it is natural and beneficial to reuse the offset to schedule the failed D2R again. In this case, the offset could be set as zero.

Proposal 5: Device could perform re-access or D2R re-transmission upon failure detection by the device if allowed or when it is indicated by the reader.

Proposal 6: (Issue 2-11) One R2D message could be introduced to indicate the NACK for re-access or the scheduling of (re-) transmission of the last D2R message.

In case of the D2R transmission after contention resolution or the failure reception of a specific Msg1, the reader could identify one specific device in this access occasion. It is possible for the reader to provide access occasion to one device and this device could re-send the failed D2R transmission (e.g., Msg1/Msg3). Otherwise, the reader needs to trigger another paging procedure to find the failed device(s) which results in more signalling overhead and longer latency to complete the service request and may cause possible contention in the subsequent paging. In addition, if the failure is informed timely, the failure handling could be performed with less latency. When the reader detects the D2R transmission failure, it is beneficial to perform the re-transmission in the same slot.

Proposal 7: NACK messages could be used by the reader to allocate resource(s) for re-transmission or indicate re-access before the subsequent R2D trigger.

2.3 (Issue 2-5/6/7) Msg2 content/format

Before discussing the content/format of a specific msg in the random-access procedure. Multi-reader scenario is one of the controversial points when designing the mechanism for the AS ID, some companies argues that some ID needs to be included to identify the D2R response is for the specific reader/procedure. For the Msg(s) in the random-access procedure, similar issue about the whether and how to solve the collision when the device receives R2D messages from different reader and even R2D messages from parallel procedures of one reader. Since the format of the transaction ID is still under discussion, we could delay the discussion about whether the transaction ID is enough to solve the multi-reader scenario.

Observation 4: Unless multi-reader scenario could be solved by implementation, more information (e.g., transaction ID) is needed to resolve the collision issue.

For Msg2, RAN1 already come to the agreement about the resource allocation for the Msg3 as following:

Agreement Support the following option(s) for frequency domain resource for Msg3 transmission: · Option 2: The frequency domain resource for Msg3 can be determined based on explicit indication in the PRDCH for Msg2 transmission for one or multiples devices. · FFS: support option 1 as a default if PRDCH does not provide the indication or based on a rule o Option 1: the frequency domain resource for Msg3 reuses the frequency domain resource for Msg1 for the same device

In addition to the RN16 as the one received by Msg1 agreed in the last RAN2 meeting, Msg2 could carry the frequency domain resource for Msg3 based on the RAN1’s agreement. But the detail of the resource allocation is upto RAN1. Furthermore, to support AS ID mechanism, Msg2 could be used to confirm/assign the AS ID to enable the scheduling of the D2R transmission as soon as possible. Thus, for the content of Msg2, we propose that:

Proposal 8: (Issue 2-7) For CBRA, RAN2 to discuss at least the following information carried in Msg2 related to AS ID:

- the information about whether to reuse RN16.

- the information about whether to newly assign and the assigned AS ID.

Furthermore, since there may be some cases that the same RN is transmitted in the same slot or different slot. For the same RN is transmitted in the same slot case, if only the RN is echoed in the Msg2, the two devices would both transmit the Msg3 with the same frequency resource based on explicit indication in the PRDCH for Msg2 transmission.

Observation 5: Two devices would both transmit the Msg3 with the same frequency resource if the same RN is transmitted in the same slot.

Based on the current mechanism, the device would compare every RN in Msg2 with its RN transmitted in Msg1 to check if it needs to transmit the Msg3 with its device ID. This is workable with limited efficiency. Thus, we prefer to have a mechanism to provide information about whether the RN is echoed to the device based on the information related to the Msg1 transmission resource. Based on the information related to the Msg1 transmission resource, the device could jump to the entry for its resource.

Proposal 9: (Issue 2-5/ Issue 2-6) Information related to the resource could be used to indicate if RN16 is echoed the device using the resource to transmit its Msg1.

Proposal 10: (Issue 2-7) Information related to the resource could be used to indication the absent/confirmation/assignment of AS ID to device using the resource to transmit its Msg1.

2.4 (Issue 2-1/2) R2D trigger message and Msg1 related

In the last meeting, it was agreed in RA section that R2D message is used to indicates the start of a set of MSG1 resources that were configured in paging message:

Agreements on new R2D message 1. A new R2D message other than the paging message is introduced for A-IoT device determining MSG1 resources unless RAN1 concludes to use L1 signaling. The R2D message indicates the start of a set of MSG1 resources that were configured in paging message. 2. Assumption: The R2D message does not include slot number/count down number.

The new R2D message is used to indicate the start of a set of MSG1 resources that were configured in paging message. For the first set of access occasions, the paging message can be used to indication the start of MSG1 resources, which can reduce one message sending and receiving. And the device can respond after receiving the paging message but not to wait for the following new R2D message.

Proposal 11: (Issue 2-2) RAN2 to confirm that the paging message can be used to indicate the start of the first set of MSG1 resources.

Since there may be a case to page large number of devices, the resources in the paging message may not be able to allow these devices to access. Therefore, multiple R2D trigger messages can follow a paging message.

Proposal 12: (Issue 2-2) A paging message can be followed by one or multiple R2D trigger message(s) which indicates the start of a set of MSG1 resources.

For the resource selection for Msg1, the procedure may be different from the RFID since there may be X time domain and Y frequency domain after one R2D message based on RAN1’s agreement. However, it is still under discussion whether X and Y is always the same during one paging procedure (i.e., after every R2D trigger in this procedure). Since paging message can include a number of the AOs, to reduce the bits to carry this information, the number is only applied for time domain. In another word, the device could derive which R2D trigger and which time domain after this R2D trigger based on the total number of time domains. Then, the device could randomly select its frequency domain which is also aligned with the previous agreement about FDMA.

Proposal 13: (Issue 2-1) For the resource selection for Msg1, the device:

- randomly selects its time domain number based on the total number of time domain provided in the paging message,

- finds its exact time domain based on the trigger and time domain after the previous R2D triggers,

- randomly selects among FDMA occasions in its time domain.

2.5 Other open issues

Use cases of the NACK feedback

In addition to the data transmission without contention, there may be contention resolution failure from perspectives of both reader and device for the following two cases: 1) more than one device selects the same access occasion; 2) the decoding failure of the received D2R message for one device. Unlike the D2R transmission failure, it is hard to locate only one device for the contention resolution failure for the first case. The TR only defines the default success operation that the device only determines the success of the contention resolution failure if it receives one Msg2 contains the same random number sent in its Msg1.

Proposal 14: Support NACK message could be applied to contention resolution (e.g., NACK for Msg1).

Msg3 carries the device ID in a higher layer container to ack the inventory request. That is, from the perspective of devices, the format of Msg3 message is similar to that of the message containing D2R data since the D2R data and device ID are both included in the higher layer container instead of MAC PDU. Furthermore, there may be some circumstances that the reader would like to terminate the data transmission of one device to enable remaining devices in the following slot to complete their access/data transmission. The NACK to trigger re-access could be used to terminate the on-going procedure after D2R data transmission.

Proposal 15: NACK could also be applied to D2R data (e.g., NACK for command response).

3	Conclusions

In this contribution, we discussed the issues related to the random access procedure and failure handling aspects on Ambient IoT. Based on the  discussion, the following observation and proposals are concluded:

Observation 1: The device would regard the D2R transmission as failure if it has not received the corresponding mandatory R2D transmission, for example, the Msg2 is not received after sending the Msg1.

Observation 2: It is hard for the device to detect D2R data transmission failure if it has not received explicit R2D failure/success feedback indication since the subsequent R2D transmission is optional based on the reader implementation.

Observation 3: Failure could be detected from perspective of device based on the TD2R_max (maximum time between the D2R transmission and the corresponding R2D transmission following it) defined by RAN1.

Proposal 1: (Issue 2-4) Support the failure detection based on a timer which could be pre-configured time in the R2D trigger or the pre-defined time in RAN1.

Proposal 2: (Issue 2-4) The subsequent R2D transmission for the device indicates that there is no failure handling for the last D2R message.

Proposal 3: (Issue 2-4) The absent of subsequent R2D transmission (e.g., Msg2 after Msg1 in CBRA) indicates the device should perform failure handling caused by D2R failure.

Proposal 4: (Issue 2-4) The present of the NACK indicates the device(s) to perform the failure handling.

Proposal 5: Device could perform re-access or D2R re-transmission upon failure detection by the device if allowed or when it is indicated by the reader.

Proposal 6: (Issue 2-11) One R2D message could be introduced to indicate the NACK for re-access or the scheduling of (re-) transmission of the last D2R message.

Proposal 7: NACK messages could be used by the reader to allocate resource(s) for re-transmission or indicate re-access before the subsequent R2D trigger.

Observation 4: Unless multi-reader scenario could be solved by implementation, more information (e.g., transaction ID) is needed to resolve the collision issue.

Proposal 8: (Issue 2-7) For CBRA, RAN2 to discuss at least the following information carried in Msg2 related to AS ID:

- the information about whether to reuse RN16.

- the information about whether to newly assign and the assigned AS ID.

Observation 5: Two devices would both transmit the Msg3 with the same frequency resource if the same RN is transmitted in the same slot.

Proposal 9: (Issue 2-5/ Issue 2-6) Information related to the resource could be used to indicate if RN16 is echoed the device using the resource to transmit its Msg1.

Proposal 10: (Issue 2-7) Information related to the resource could be used to indication the absent/confirmation/assignment of AS ID to device using the resource to transmit its Msg1.

Proposal 11: (Issue 2-2) RAN2 to confirm that the paging message can be used to indicate the start of the first set of MSG1 resources.

Proposal 12: (Issue 2-2) A paging message can be followed by one or multiple R2D trigger message(s) which indicates the start of a set of MSG1 resources.

Proposal 13: (Issue 2-1) For the resource selection for Msg1, the device:

- randomly selects its time domain number based on the total number of time domain provided in the paging message,

- finds its exact time domain based on the trigger and time domain after the previous R2D triggers,

- randomly selects among FDMA occasions in its time domain.

Proposal 14: Support NACK message could be applied to contention resolution (e.g., NACK for Msg1).

Proposal 15: NACK could also be applied to D2R data (e.g., NACK for command response).

4	References

- RP-250796 New Work Item: Solutions for Ambient IoT (Internet of Things) in NR

- TR 38.876 Study on solutions for ambient IoT (Internet of Things) (Release 19).