3GPP RAN WG2 Meeting #130                                                     	R2-2504467

St.Julian, Malta, May. 19th – 23rd, 2025

Agenda Item:	8.2.4

Source:	HONOR

Title:	Discussion on Data Transmission for Ambient IoT

Document for:	Discussion and Decision

1. Introduction

RAN2 has made the below agreements regarding A-IoT data transmission and other general aspects in RAN2#129bis [1]:

AS ID assignment

Agreements

1	AS ID is applied for Inventory + command case;

2	AS ID is not included in D2R message except Msg 1 (RN16 in Msg 1 has been agreed.

3	For both CFRA and CBRA, the AS ID size is same as RN 16, i.e. 16 bits.

4	Do not specify the reader behaviour on how exactly the ASID is generated.

5	The device releases the AS ID upon power off (no stage 3 specification impact);

6	The device only keeps one AS ID at a time.

7	For CFRA, command message is used for AS ID assignment

8	For CBRA, Msg 2 is used for AS ID assignment

9	The device releases the AS ID at least:

- upon receiving Paging with new transaction id for that device, i.e. different session/service

- when it triggers new msg1 transmission as a result of receiving Paging message (i.e. it has to generate a random ID for CBRA)

- FFS other cases for release ASID to avoid keeping it indefinitely.

Segmentation

Agreements on segmentation

- For the retransmission of the first segment/unsegmented D2R message, the reader sends the R2D message by including the upper layer command again.  FFS whether offset zero is always included.

- FFS whether the reader always includes the command for retransmission of segments.

- 1-bit indication is sufficient to indicate whether more D2R data will be sent

- For inventory response, RAN2 assumes that segmentation is not applied.  RAN2 assumes that the reader can avoid segmentation by reader being aware of inventory response size.  Notify SA2 about this assumption.

MAC PDU format

Agreements on MAC PDU format

- The MAC PDU should be byte-aligned, assuming the allocated TBS value is in the unit of byte.  The actual TBS value depends on RAN1.   FFS for R2D trigger message

- RAN2 assumes that the upper layer data SDU is byte-aligned, and an LS can be sent to CT1.

- The D2R MAC PDU size will correspond to the TBS size indicated in the R2D message

- The MAC padding is supported at least for D2R from RAN2 perspective.   The device includes padding bits if there is no more data and there is still space available in the TBS.

- In case where MAC PDU includes both MAC SDU and padding, for D2R a field to indicate how many SDU bits are present is required.  FFS how this is provided (i.e. SDU length field or padding length field).  The size of length field is FFS.

In this contribution, we provide our considerations on remaining open issues on data transmission for A-IoT.

2 Discussion

2.1 (Issue 3-1/Issue 3-2) Segmentation

The device will know the position of the next segment according to starting point and the offset. It is not clear what is the starting point and how to acquire the information. Considering different segment objectives, there could be two options to perform segmentation and acquire the starting point elaborated below:

Segmentation of application layer packets

It is likely that the execution entity of segmentation could be the application layer and the steps include:

Firstly, the execution entity should know the starting point, offset and permitted transmitted length to generate the expected segment.  In this case, the starting point is the address of the storage area to store Command feedback, e.g the address of NVM which may be included in the Command message.

And then, the generated segment will be delivered to NAS layer and it is the NAS layer to construct a NAS PDU.

Finally, the MAC layer will transmit the MAC PDU (including the NAS PDU) to the reader side.

Since starting point is included in the Command message, it is useful to transmit the upper layer command in every R2D message and this could also avoid recoding starting point in device side. However, this option needs to coordinate with application layer and it may be not in the scope of 3GPP. Additionally, there will be several NAS PDUs for one Command feedback and SA2 should be involved to evaluate the solution.

Observation 1: If considering to segment the application layer packet, it is useful to transmit the upper layer command in every R2D message to avoid recoding starting point. However, this needs to coordinate with application layer which is not in the scope of 3GPP and SA2 should be involved to evaluate whether this solution could work.

Segmentation of NAS PDU

The execution entity may be NAS layer or MAC layer and the steps elaborated below:

Firstly, the application layer generates Command feedback and delivery it to the lower layer, i.e. NAS layer.

And then, the NAS layer will construct a NAS PDU including the application layer packets.

The execution entity of segmentation will decide to perform segment or not according to the permitted transmitted length derived from TBS. If it is not the first segment, the entity will know how to segment according to the starting point, offset and permitted length. The starting point will be the storage area address of the execution entity, e.g. the beginning or fixed position of buffer in NAS layer or MAC layer.

Finally, the MAC layer will generate a MAC PDU including the segments and transmit the MAC PDU to the reader side.

In this case, it is natural that the execution entity will record the position to store the NAS PDU which may be the beginning or fixed position of the buffer. The position is the starting point because the device is expected to perform one procedure at a time and there will be only one NAS PDU in the buffer of execution entity.

Observation 2: If considering to segment the NAS PDU, it is not natural to record the starting point which is also the position to store the NAS PDU, because there would be only one NAS PDU in the buffer waiting for transmission. There is no additional requirement introduced by segmentation.

Proposal 1: (Issue 3-1) RAN2 to discuss not include the upper layer command for subsequent segments (re-)transmission.

Except for R2D message with upper layer command, there will be a R2D scheduling message to allocate the resource for subsequent segment transmission. And the content of R2D scheduling message is the same with the R2D message if there is no upper layer command included, which may include AS ID, resource information for scheduling and offset.

For simplicity, it is preferable to define only one type of R2D message for data transmission after MSG3. One possible solution is that the upper layer command could be included optionally. For example, the upper layer command could be present if the R2D message is used to trigger the first segment or unsegmented packets and absent for scheduling the subsequent segment.

Proposal 2: (Issue 3-1) The R2D message could also be used for scheduling subsequent segment and the “upper layer command” should be not present for segment (re-)transmission.

The offset, which means the number of bytes successfully received, is used to indicate the transmission position of the next segment and will be included in the R2D message when the reader sending scheduling information to the device. For the first segment or unsegmented packet, the device could know the position via the information in the Command and it is not necessary to include the offset field. However, it will increase the complexity of the device to deal with the R2D message with/without “offset” field separately. For simplified device implementation, it is proposed to include offset in every R2D message and it could be set to “zero” if the message is to schedule for the first segment and unsegmented message.

Proposal 3: (Issue 3-2) The offset (i.e. Received Data Size) should be included in the R2D message scheduling for the first segment and unsegmented message.

2.2 (Issue 3-3) AS ID Release

For “inventory and command” scenario, the device completes the entire procedure after sending Command Feedback successfully. And the reader will not schedule the device again as there will be no parallel procedure for A-IoT service. If there is new service for the device, a new procedure will be triggered with new paging message. Therefore, there is no need to store the AS ID after completing a procedure successfully.

Observation 3: There is no case for the reader to schedule the device with the assigned AS ID again if the device has already completed current procedure successfully. Therefore, there is no need to store the AS ID after the procedure completed successfully.

Proposal 4: (Issue 3-3) The device will release the AS ID upon completing the service request procedure successfully.

Paging round starts with receiving the paging message and end with receiving the subsequent paging message, i.e. paging message with the stored transaction ID. And there could be several slots in one paging round. After receiving MSG3, the reader should schedule the device as soon as possible to avoid energy depletion. For example, the R2D message with upper layer command should be transmitted to the device in the same slot with receiving MSG3 to reduce the waiting time. Sometimes, the reader could not receive Command message in time which depends on CN. To avoid long time waiting, the R2D message with upper layer command should be transmitted to the device in following slots of the same round.

Observation 4: Considering energy consumption of A-IoT device, the R2D message with upper layer command should be transmitted to the device as soon as possible, e.g. in the same slot or round with sending MSG3.

Upon receiving the R2D message, the device will transmit Command Feedback according to scheduling information sending in the same slot with sending R2D message. Therefore, the device could take the procedure as completed successfully with receiving the subsequent paging message (i.e. paging message with the same transaction ID with the stored one).

Proposal 5: (Issue 3-3) The device will release the AS ID upon receiving the page message with the stored transaction ID.

The NACK based mechanism is supported to determine re-access and the device would perform the random access from the MSG1 transmission after the subsequent paging. In this case, the device could get new AS ID via MSG2 in the re-access procedure and there is no need to store the AS ID after receiving NACK feedback.

Additionally, the reader could not find out whether it is a MSG2 failure or MSG3 failure. In another word, the AS ID indicated by MSG2 may be not received by the device. A common behaviour should be introduced to avoid misalignment between the reader and device about the AS ID. That is, the AS ID is released by the device when the device receives a NACK for re-access. If needed, the reader could re-assign the AS ID for this device in the subsequent paging round.

Proposal 6: (Issue 3-3) The device will release the AS ID upon receiving NACK for re-access.

2.3 (Issue 3-4/Issue 3-5) D2R Message Content

The device includes padding bits if there is no more data and there is still space available in the TBS. And it has been agreed that a field (e.g. Length field) to indicate how many SDU bits are present is required if the D2R MAC PDU includes both MAC SDU and padding.

Observation 5: Length field is necessary to indicate how may SDU bits are present when there is no more data and there is padding in the MAC SDU.

It is hard for the reader to always allocate accurate resource for the D2R transmission since it may not know the size of upper layer data. Sometimes, the TBS is large and the upper layer data is small which means the number of padding bits exceeds that in the Data PDU and vise versa. In other words, the Length field for padding may larger than Length field for Data PDU.

Generally, the length field is used to get the end of Data SDU and it is straightforward to indicate the length of Data SDU.

Proposal 7: (Issue 3-4) Length field indicates the number of bytes in Data SDU field.

The unit of the Length field should be byte because the D2R MAC PDU is byte-aligned while the upper layer data SDU is also byte-aligned. Considering the maximum A-IoT packet size is 1000bits, the Length field should be less than 7bits to indicate maximum 128bytes.

Proposal 8: (Issue 3-4) The size of the Length field is 7bits.

1-bit indication (More Data indication) will be introduced to indicate whether more D2R data will be sent to the reader. It will be set to No more data if there is the last segment or unsegmented packets.

Observation 6: 1-bit indication (More Data indication) is introduced to indicate whether there is more data in the device side.

Thus, the Length field is not necessary if the Data PDU is not the last segment or unsegmented packets as there is no padding. For example, there will be no Length field if 1-bit indication is set to “1” for more data stored and there will be Length field if 1-bit indication is set to “0”.

Proposal 9: (Issue 3-4) Length field is optionally present. The Length field will be present in the D2R MAC PDU if more data indication indicates there is “No” more data stored in the device side.

Sensing and positioning are also target scenarios of A-IoT which could be discussed in the further version. And more D2R MAC PDU formats will be introduced for different scenarios. Considering extensibility, the Message Type should be included in the D2R MAC PDU.

Proposal 10: (Issue 3-5) Message Type should be included in the D2R MAC PDU for further extensible.

2.4 Other open issues

During RA procedure, there could be lots of devices triggered and MSG2 supports multiplexing of information for multiple devices. This is very efficient for uplink FDM and downlink TDM. Similarly, the reader will also need to send Commands to multiple devices if it received multiple MSG3. It is beneficial that one R2D message could multiplex information to schedule multiple devices’ D2R transmission. This could reduce the number of R2D message and maximize the benefits of FDM. Meanwhile, this could also reduce the waiting time after sending MSG3 to avoid energy depleted case.

Proposal 11: The R2D message should be able to multiplex information for multiple devices.

3 Conclusions

In this contribution, we analyze the data transmission procedure for A-IoT device and made the following observations and proposals:

Observations

Observation 1: If considering to segment the application layer packet, it is useful to transmit the upper layer command in every R2D message to avoid recoding starting point. However, this needs to coordinate with application layer which is not in the scope of 3GPP and SA2 should be involved to evaluate whether this solution could work.

Observation 2: If considering to segment the NAS PDU, it is not natural to record the starting point which is also the position to store the NAS PDU, because there would be only one NAS PDU in the buffer waiting for transmission. There is no additional requirement introduced by segmentation.

Observation 3: There is no case for the reader to schedule the device with the assigned AS ID again if the device has already completed current procedure successfully. Therefore, there is no need to store the AS ID after the procedure completed successfully.

Observation 4: Considering energy consumption of A-IoT device, the R2D message with upper layer command should be transmitted to the device as soon as possible, e.g. in the same slot or round with sending MSG3.

Observation 5: Length field is necessary to indicate how may SDU bits are present when there is no more data and there is padding in the MAC SDU.

Observation 6: 1-bit indication (More Data indication) is introduced to indicate whether there is more data in the device side.

Proposals

- Issue 3-1/Issue 3-2: Segmentation

Proposal 1: (Issue 3-1) RAN2 to discuss not include the upper layer command for subsequent segments (re-)transmission.

Proposal 2: (Issue 3-1) The R2D message could also be used for scheduling subsequent segment and the “upper layer command” should be not present for segment (re-)transmission.

Proposal 3: (Issue 3-2) The offset (i.e. Received Data Size) should be included in the R2D message scheduling for the first segment and unsegmented message.

- Issue 3-3: AS ID Release

Proposal 4: (Issue 3-3) The device will release the AS ID upon completing the service request procedure successfully.

Proposal 5: (Issue 3-3) The device will release the AS ID upon receiving the page message with the stored transaction ID.

Proposal 6: (Issue 3-3) The device will release the AS ID upon receiving NACK for re-access.

- Issue 3-4/Issue 3-5: D2R Message Content

Proposal 7: (Issue 3-4) Length field indicates the number of bytes in Data SDU field.

Proposal 8: (Issue 3-4) The size of the Length field is 7bits.

Proposal 9: (Issue 3-4) Length field is optionally present. The Length field will be present in the D2R MAC PDU if more data indication indicates there is “No” more data stored in the device side.

Proposal 10: (Issue 3-5) Message Type should be included in the D2R MAC PDU for further extensible.

Others

Proposal 11: The R2D message should be able to multiplex information for multiple devices.

4. References

[1] RAN2 #129bis meeting report