3GPP RAN WG2 Meeting #130	R2-2504468

# St Julians, Malta, May 19th – 23rd, 2025

Agenda Item:	8.4.2

Source:	HONOR

Title:	Discussion on LP-WUS in RRC_IDLE/INACTIVE

Document for:	Discussion and Decision

1. Introduction

In RAN#103 meeting, the following objectives of Rel-19 LP-WUS work item were agreed [1]:

- For IDLE/INACTIVE modes (RAN1, RAN2, RAN3, RAN4) - Specify procedure and configuration of LP-WUS indicating paging triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI. - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS. - Y will be decided within WI. 320ms is the start point.

The following agreements on LP-WUS monitoring conditions and subgrouping were made [2] in RAN2#129bis meeting:

LP-WUS is supported with eDRX, FFS on exact impact if any Use 5G-S-TMSI to determine the UE_ID in the formula of UE_ID based subgrouping for LP-WUS, i.e., UE_ID = 5G-S-TMSI mod X. X is based on 32 subgrouping number. Details can be discussed in the running CR. Send LS to RAN3 (CC SA2/SA3) to inform our agreements on UE ID based subgrouping. Correct the typo as following for the previous agreed formula of UE_ID based subgrouping for LP-WUS: - Np is the number of subgroupsNumForUEID for PEI, if configured and UE supports PEI; otherwise, Np is 1. - Confirm the principle for determining CN assigned subgrouping or UE_ID based subgrouping for PEI is reused for LP-WUS subgrouping. Details will be discussed in the running CR. All the LP-WUS related configurations except for measurement configurations are provided in SIB1. FFS the details on measurement configurations. Dedicated configuration in RRC signaling is not needed for providing LP-WUS related configuration in RRC_IDLE/INACTIVE modes. Use existing Srxlev/Squal for all MR measurement based entry/exit condition evaluation. Use measured value for all LR measurement based entry/exit condition evaluation.

In this contribution, we focus on the discussion of the following issues for LP-WUS in RRC_IDLE/INACTIVE.

- LP-WUS and emergency PDU session

- LP-WUS capability

- UE paging probability information

2. Discussion

2.1. (Other open issue) LP-WUS and emergency PDU session

RAN2 agreed to FFS whether/how to handle the case for UE-ID based subgrouping when the UE has an emergency PDU session.

In our view, whether UE disables UE-ID based subgrouping when it has an emergency PDU session should depend on the additional paging latency.

For PEI, the additional paging latency can be determined by parameter pei-FrameOffset-r17, which is 0~160ms. The additional paging latency introduced by PEI is deemed tolerable. For LP-WUS, RAN1 has defined three candidate values for the wake-up delay that UE support, which are {[80ms], [500ms] and [900ms]}. Namely, the additional paging latency introduced by usage of LP-WUS are {[80ms], [500ms] and [900ms]}. Compared to additional paging latency introduced by PEI, at least the additional paging latency of 80ms should be deemed tolerable. The other two paging latency are much larger than those introduced by PEI, which are unacceptable.

In our view, when UE satisfies entry conditions and has an emergency PDU session, it can determine whether to disable UE-ID based subgrouping based on UE wake-up delay.

Proposal 1：(Other open issue) UE can determine whether to disable UE-ID based subgrouping based on the wake-up delay it supports when it has an emergency PDU session.

2.2. (Other issue) LP-WUS capability

RAN1 has agreed to report UE wake-up delay to gNB. In our view, UE wake-up delay can also be used to indicate that UE supports LP-WUS feature and UE_ID based subgrouping. Besides, due to UE LP-WUR may not support all the bands that MR supports, another capability that LP-WUS UE should report is the band lists that LP-WUR supports. These two capabilities can be reported to RAN via UE-RadioPagingInfo IE in the RRC UECapabilityInformation message. Based on these two capabilities, gNB can determine whether to send LP-WUS to UE, as well as the time domain location of the LP-WUS.

Proposal 2: (Other issue) UE reports wake-up delay and band lists that LP-WUR supports to RAN via UE-RadioPagingInfo IE in the RRC UECapabilityInformation message.

Some companies believe that LP-WUR type should be reported to gNB so that gNB can send different LP-WUS type based on UE LP-WUR type. According to the analysis in Section 2.1, both OOK-based LP-WUR and OFDM-based LP-WUR can receive both type of LP-WUS correctly. It is not necessary to report LP-WUR type for paging.

Proposal 3: (Other issue) UE does not report LP-WUR type in UE capability message.

2.3. (Other issue) UE paging probability information

Following agreement has been made in RAN2 127bis meeting.

For CN assigned LP-WUS subgrouping, RAN2 assumes similar procedure for PEI will be used for LP-WUS subgrouping. Final design is up to SA2/CT1/RAN3 discussion.

In Rel-17 PEI, it is specified that the UE may include its UE paging probability information in the Requested PEIPS assistance information IE if the UE has set the NR-PSSI bit to "NR paging subgrouping supported" in the 5GMM capability IE. The granularity of the paging probability is 5%.

TS 24.501

Table 9.11.3.80.1: PEIPS assistance information element

(omit)

Bit

5 4 3 2 1                        UE paging probability information value

0 0 0 0 0                                          p00

0 0 0 0 1                                          p05

0 0 0 1 0                                          p10

0 0 0 1 1                                          p15

0 0 1 0 0                                          p20

0 0 1 0 1                                          p25

0 0 1 1 0                                          p30

0 0 1 1 1                                          p35

0 1 0 0 0                                          p40

0 1 0 0 1                                          p45

0 1 0 1 0                                          p50

0 1 0 1 1                                          p55

0 1 1 0 0                                          p60

0 1 1 0 1                                          p65

0 1 1 1 0                                          p70

0 1 1 1 1                                          p75

1 0 0 0 0                                          p80

1 0 0 0 1                                          p85

1 0 0 1 0                                          p90

1 0 0 1 1                                          p95

1 0 1 0 0                                          p100

On the other hand, paging probability is usually less than 5% (e.g., 3% or 1% or <%1) in RAN1 simulation for LP-WUS power consumption. The reason is little power saving gain and even negative power saving gain may be introduced by LP-WUS if the UE paging probability is high due to the extremely high MR transition energy (15000uint ~ 40000uint) [3].

Obviously, the granularity of paging probability defined for PEI is not suitable for LP-WUS. RAN2 consider more finer granularity of paging probability (e.g., 1% or <1%) for reporting to CN, and then inform CT1 our conclusion.

Proposal 4: (Other issue) RAN2 consider a finer granularity of paging probability (e.g. 1% or <1%) for LP-WUS for reporting to CN.

3. Conclusions

The proposals are summarised as below.

Proposal 1：(Other open issue) UE can determine whether to disable UE-ID based subgrouping based on the wake-up delay it supports when it has an emergency PDU session.

Proposal 2: (Other issue) UE reports wake-up delay and band lists that LP-WUR supports to RAN via UE-RadioPagingInfo IE in the RRC UECapabilityInformation message.

Proposal 3: (Other issue) UE does not report LP-WUR type in UE capability message.

Proposal 4: (Other issue) RAN2 consider a finer granularity of paging probability (e.g. 1% or <1%) for LP-WUS for reporting to CN.

4. References

[1] RP-240801 Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)

[2] Report of 3GPP TSG RAN WG2 meeting #129bis

[3] TR 38.869 Study on low-power wake up signal and receiver for NR