3GPP TSG-RAN WG2 Meeting #130	R2-2504472

St.Julians, Malta, May 19th – 23rd, 2025

Agenda Item:	8.6.3

Source:	HONOR

Title: 	Discussion on measurement event evaluation and report

Document for:	Discussion and decision

1	Introduction

The following agreements related to measurement event evaluation and measurement report for purpose of supporting LTM were made during RAN2#129 bis meeting.

Agreements on L1 event-triggered MR

- Introduce new MAC CE to activate/deactivate the semi-persistent CSI-RS resource.

- The new MAC CE includes: A/D indication, Target configuration id or {SP CSI-RS resource set id and/or candidate id}, TCI state id.

- In L1 reporting configuration, the UE can be configured with a list of candidate IDs. For each listed candidate ID, the network can configure an offset, this offset applies to all the RSs belong the candidate ID that associated with the reporting configuration (the offset value also includes 0dB); If a candidate ID is not provided in the reporting configuration, it means the UE is not required to measure/evaluate the RSs that belonging to the candidate ID for this event, even if these are configured within the L1 resource configuration (to which the L1 reporting configuration points to).

- In the MR MAC CE, adopt a uniform format for all the reported beams (i.e., using the same format for beams leaving the entering condition, beams met the entering condition, beams not satisfying the event). Introduce two bits per beam.

- RAN2 will consider only the following options:

- Option1: To introduce four codepoints to represent one of {type1, type2, type3, type4}

- Option3: To introduce three codepoints to represent one of {type1_or_type3, type2, type4}, i.e., type1 beam and type3 beam are not distinguished in reporting.

Type1 beam: a beam that has satisfied the entry condition of the event for TTT and thus triggers the MR MAC CE.

Type2 beam: a beam that has satisfied the leaving condition of the event for TTT and thus triggers the MR MAC CE.

Type3 beam: a beam that is neither type1 nor type2 but has remained in beamsTriggeredList (the beam was reported as type1 beam before).

Type4 beam: a beam that is neither type1 nor type2 and not included in beamsTriggeredList (i.e., just extra beam info)

- Working assumption: Option1: To introduce four codepoints to represent one of {type1, type2, type3, type4}

- Intention is that the UE should be able to report the event-triggered beam(s) that were not included in the truncated MR MAC CE by the following grant. Detailed wording can be further discussed as part of the running CR.

- For co-existence with mTRP, will be revisited in August. If one simple solution is not prepared / agreed until / in August meeting, we will not apply mTRP in Rel-19 event-triggered MR.

In this contribution, we further discuss the L1 measurement event evaluation and report and provide proposals accordingly.

- Discussion

2.1 Signalling burden

For the report of the LTM MR MAC CE, the signalling burden is more severe since the number of the measured SSB/CSI-RS is much bigger than the legacy. Several companies mentioned the signalling burden brought by the SSB/CSI-RS based LTM MR for candidate cells. It is straight forward to introduce a prohibit timer for the report. However, the improper timer configuration may reduce effectiveness by delaying the LTM execution. Considering the motivation of the LTM MR is to switch the cell with early sync, more factors could be considered when design the mechanism to reduce the signalling burden of LTM MR. For example, similar to the L3 measurement, the number of the good beams could be considered.  To improve the possibility of RACH-less and reduce the signalling burden of LTM MR, the early-sync status could also be one of the events to be evaluated.

Proposal 1: RAN2 to discuss how to reduce the signalling burden caused by L1 MR:

- prohibit timer,

- event LTM2~5 combined with other conditions (early-sync status),

- the number of satisfying beams,

- Others.

2.2 Content of the MAC CE MR

Additional information of the candidate cell

For UE-triggered L1 measurement report for the LTM, RAN2 needs to further discuss which beam is included in the report. Beam(s) of neighbour satisfying the event is the basic option since it is useful to help the NW determine the target beam. During the RAN2#129 meeting, it is already agreed that beam(s) not satisfying the condition could be reported within the number limitation configured by the NW. The beam(s) not satisfying the condition are ranked based on the measured quality and the best ones are selected within the limitation. That is, the UE would report some measured beams within the number limitation N. However, if some of the reported beams are with bad quality, the information is useless to the NW and may cause signalling overhead by the useless information. For the L1 measurement report of the serving cell carried by UCI, the NW needs to know the full picture of the serving cell. Since the MAC CE MR is designed for LTM, only beams good enough are useful for the NW to perform the LTM beam selection. Thus, we propose one more selection criterion for the beam not satisfying the condition.

Proposal 2: RAN2 to discuss the additional criterion for the reported beam not satisfying the condition. FFS the criterion is a pre-configured RSRP/RSRQ threshold, not satisfying the leaving condition, satisfying the entering condition, and so on.

2.3 Beam type

During last meeting, four types of beams to be reported are listed and the main controversial point is whether to differentiate the type1 and type3. Compared to L3 measurement report, L1 measurement report is triggered more frequently. Based on our understanding, NW would identify the triggered beam based on these reports. Thus, there is no need to differentiate the type1 and type3.

Proposal 3：There is no need to differentiate the Type1 and Type3 beam.

Based on proposal 3 above, there are two main characteristics for the reported beams that whether the beam is a beam that has satisfied the entry condition of the event for TTT and whether the beam is a beam that has satisfied the leaving condition of the event for TTT. Thus, the most straight-forward information to provide is that the indication about whether the beam is the beam for event triggered or report on leave. Thus, we propose that:

Proposal 4: One bit to indicate the entry triggered beam and one separate bit to indicate the leaving triggered beam. The extra beam (type 4) is neither an entry triggered beam nor a leaving triggered beam then it is a type4 beam.

2.4 (MAC-8) coexistence with mTRP

mTRP is one of the most important features introduced in NR to improve the coverage of the NW. Thus, it is beneficial to support the coexistence of the mTRP and L1 MR.

Proposal 5: (MAC-8) RAN2 to support the coexistence of the mTRP and L1 MR.

To support the coexistence of the mTRP and L1 MR, there are two important issues to be solved:

- the event evaluation with multiple current beams.

- the report content with multiple current beams.

Even though the current beam is not the beam belongs to the serving cell, the NW could communicate with the UE with this beam. Thus, there is no need to treat the current beams differently. To have more up-to-date information about the candidate beams as well as the current beams, it is important to trigger the report as soon as one of the current beams is satisfied. Thus, we propose:

Proposal 6: (MAC-8) If at least one of the current beams met the trigger condition during TTT, then the report could be triggered when the corresponding TTT expires. If all the current beams don’t meet the trigger condition, the TTT stops.

The current beam would be reported without index since there is only one current beam when the mTRP and L1 MR are not configured simultaneously. However, if there is coexistence of the mTRP and L1 MR, the index of the reported current beam should be also included to identified beam for the measurement result.

Proposal 7: (MAC-8) The index of current beam is also included in the MR MAC CE if this is mTRP.

3	Conclusions

In this contribution, we discussed the L1 measurement enhancement for Rel-19 mobility WI. Based on the discussion, the following observation and proposals are concluded:

Proposal 1: RAN2 to discuss how to reduce the signalling burden caused by L1 MR:

- prohibit timer,

- event LTM2~5 combined with other conditions (early-sync status),

- the number of satisfying beams,

- Others.

Proposal 2: RAN2 to discuss the additional criterion for the reported beam not satisfying the condition. FFS the criterion is a pre-configured RSRP/RSRQ threshold, not satisfying the leaving condition, satisfying the entering condition, and so on.

Proposal 3：There is no need to differentiate the Type1 and Type3 beam.

Proposal 4: One bit to indicate the entry triggered beam and one separate bit to indicate the leaving triggered beam. The extra beam (type 4) is neither an entry triggered beam nor a leaving triggered beam then it is a type4 beam.

Proposal 5: (MAC-8) RAN2 to support the coexistence of the mTRP and L1 MR.

Proposal 6: (MAC-8) If at least one of the current beams met the trigger condition during TTT, then the report could be triggered when the corresponding TTT expires. If all the current beams don’t meet the trigger condition, the TTT stops.

Proposal 7: (MAC-8) The index of current beam is also included in the MR MAC CE if this is mTRP.

4	References