3GPP RAN WG2 Meeting #130	R2-2504474

# St.Julians, Malta, May 19th – 23rd, 2025

Agenda Item:	8.7.4.2

Source:	HONOR

Title:	Discussion on DSR enhancements

Document for:	Discussion and Decision

1. Introduction

In RAN2#129bis meeting, the following agreements were agreed [1]:

- To avoid the case that there is no delay-reporting data for DSR, at least one configured reporting threshold should be no lower than the DSR triggering threshold. This means we do not have to address “empty DSR” issue, but it is FFS whether we need to capture this restriction in specifications - During DSR data volume calculation the remaining time of retransmitted is not considered, i.e. it is always put either in the smallest configured or smallest reported threshold. FFS which one. - Clarify RAN2#128 agreement as “the UE may also support including non-delay-reporting data ahead of delay-reporting data for buffer size calculation of Rel-19 DSR, based on the capability indication” (the exact terminology to be discussed as part of CR review) - We will try to find a way to describe how the UE determines non-delay-reporting data ahead of delay-reporting data for delay-reporting data volume calculation. The aim is to have consistent UE behaviour to avoid fairness issues, but also consider different UE implementations.

In this document, we discuss the left issues of DSR.

2 (PDCP Issue 1) Smallest configured threshold vs smallest reported threshold

In the last meeting, it was discussed that which portion the data volume of the control PDU and/or the retransmission data should be belonged to, i.e., the portion associated with smallest configured or smallest reported threshold.

As shown in the last meeting, the option of smallest reported threshold is more straightforward. If there are no delay-reporting data in the PDCP entity, the control PDU and retransmission data cannot be included in the data volume. For the option of smallest configured threshold, if there is no delay-reporting data for the LCH, the control PDU and retransmission data will be considered for the data volume calculation, which may introduce the new issues of whether/how this information should be included in DSR.

Observation 1: If there is no delay-reporting data in the LCG, new issues of whether/how this information should be included in DSR need to be considered for the option of smallest configured threshold.

Control PDU and retransmission data are non-delay reporting data that can be transmitted ahead of delay-reporting data per RLC entity or per PDCP entity. Put them into the smallest configured threshold is more like to consider them in a granularity of LCG which associated with multiple RLC entities. However, even though put these data in the smallest configured threshold, they can’t be scheduling together due to the entire scheduling mechanism is based on each LCH. Therefore, both solutions can work well and has the similar benefit, it is preferring to adopt a simple solution which is not needed the further enhancements and discussion, and the smallest reported threshold can be selected.

Proposal 1: (PDCP Issue 1) Consider Control PDU and retransmission data as the data volume of the smallest reported threshold for the PDCP entity and RLC entity.

3 Conclusions

Proposal 1: (PDCP Issue 1) Consider Control PDU and retransmission data as the data volume of the smallest reported threshold for the PDCP entity and RLC entity.

4. References

[1]	RAN2 #129bis Chairman notes