3GPP RAN WG2 Meeting #130	R2-2504478

St.Julians, Malta, May 19th – 23rd, 2025

Agenda Item:	8.9.2

Source:	HONOR

Title:	Discussion on the Store and Forward satellite operation

Document for:	Discussion and Decision

1. Introduction

In Rel-19 Non-Terrestrial Networks (NTN) for Internet of Things (IoT) WID, the following objectives were achieved:

- Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore: - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2] - At least specify necessary enhancements e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3] Note: Strive to minimise UE impact. Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required

In RAN2#129bis meeting, the following agreements were made:

1. For UEs supporting S&F, the UE AS indicates the information on transition time (if any) from current "S&F operation mode" to "normal mode" (if received) to the upper layers. Whether/how it is used by the upper layers is up to CT1. 2. We introduce an indication in system information for the normal mode to S&F mode transition, at least for NAS use. FFS on the details (e.g. whether we can link this to other existing information). The information on transition time for the normal mode to S&F mode transition is sent from AS to NAS 3. We don’t introduce any new release cause related to S&F operation

In this contribution, we discuss the remaining issues on Store &Forward (S&F) operation.

2. Discussion

According to the WID, the S&F mode is applied in regenerative architecture. For S&F mode, in the previous discussion in RAN2, it is a principle that at least full eNB onboard is required.

Regarding the architecture of store & forward, SA2 has made a conclusion that two kinds of architecture (i.e., split MME onboard and full CN onboard) shall be applied for S&F operation. In TR 23.700-29, the architectures are defined as follows:

MME functionality is split into two parts: MME-onboard - the MME part which is onboard the satellite and MME-ground - the MME part which is on the ground network with an interface out of scope of 3GPP. The whole CN including eNB, MME, SGW, PGW, HSS, E-SMLC, SMSC etc are on board each satellite. Proxies are deployed on the satellite and the ground for application traffic, including support of MT traffic, MO traffic, SMS, etc.

For full CN onboard, we think it has small impacts on RAN2 specification. Therefore, in this contribution, we will focus on the architecture with split MME onboard.

2.1 (issue RRC-1) How to indicate the time information for the transition from normal mode to S&F mode

In RAN2#127bis, the relevant conclusions were drawn:

- When present, the “S&F operation” indication has two possible settings: ‘1’: the cell is operating in S&F mode for all UEs (Rel-19 UEs supporting S&F are allowed to access the cell) ‘0’: the cell is operating in S&F mode for UEs in Connected mode (which are not required to monitor the “S&F indication”), but idle Rel-19 UEs supporting S&F are barred (Rel-19 UEs not supporting S&F will follow legacy barring procedure).

It is not enough to merely provide the information on the operation mode, and the information on the time when the mode transition occurs also needs to be provided. In RAN2#129 meeting, it was agreed that use absolute UTC time to broadcast the (S&F-> normal mode) time information. That is, if this information is present, the UE can assume the NW is currently operating in S&F mode (until the indicated S&F-> normal mode transition time).

But for the normal mode to S&F mode transition, one indication is also needed. In addition, since the existing field t-service is applied for service link switches in NTN quasi-Earth fixed cells and feeder link switches for both NTN quasi-Earth fixed and earth-moving cells, it cannot be linked for mode transition. An additional indication should be introduced.

Proposal 1: (issue RRC-1) An absolute UTC time can be used to indicate the time when the mode will change from normal mode to S&F mode.

2.2 (issue RRC-2) Whether to introduce additional assistance information for the neighbour cells

In current TS36.331, an IE(t-ServiceStartNeigh-r18) has been defined to indicate the earliest time when the area covered by the current serving cell is going to be covered by the neighbour cell(s).

t-ServiceStartNeigh-r18 TimeOffsetUTC-r17 OPTIONAL -- Need OR } t-ServiceStartNeigh Indicates the earliest time when the area covered by the current serving cell is going to be covered by the neighbour cell(s) served by the satellite indicated by satelliteId, see 5.5.3.1, 5.5.8 and 36.304 [4]. This field is only present for the NTN quasi-Earth fixed neighbour cell(s).

In addition, there is a NOTE for time-based measurement initiation, which is described as follows:

NOTE 2A: If timeMeasConfig is configured and t-Service is configured in SystemInformationBlockType3, the exact time to start measurements before t-Service is left up to UE implementation and t-ServiceStartNeigh may be used to decide when to start measurements.

If the legacy t-ServiceStartNeigh is reused for S&F mode, the S&F operation indication is needed. Nevertheless, for the legacy UE and the Rel-19 UE without S&F capability, the UE may start measurement when the network is operating S&F. If this occurs, it is meaningless for these UEs to start the measurement, and it may further lead to an incorrect reselection to one neighbour cell that operated on S&F. Therefore, we think the introduction of an additional IE is necessary.

For the neighbor cell operated on S&F, an additional IE (e.g., t-ServiceStartNeigh_s&f-r19) can be defined. It indicates the earliest time when the area covered by the current serving cell is going to be covered by the neighbour cell(s) served by the satellite that operated on S&F mode currently.

Proposal 2: (issue RRC-2) The S&F timing information of neighbour cell (e.g., t-ServiceStartNeigh_s&f-r19) can be provided as satellite assistance information.

2.3 (issue RRC-4) Whether/how to reduce the paging monitoring for an S&F UE to save power consumption

For CN-initiated paging, the MME-ground sends paging message to the satellites that are going to visit the UE’s tracking areas, when the feeder link becomes available. But for UE, before starting to monitor paging, the UE needs to decide whether there is a need to monitor paging. After monitoring paging from the last time, if the satellite has never connected to MME-ground via feeder link, the UE can skip paging monitoring. Therefore, the UE can decide whether to monitor paging based on the time information about the last time the feeder link was available.

Proposal 3: (issue RRC-4) The UE can decide whether to monitor paging based on the time information about the last time the feeder link was available.

2.4 (Other open issues) Satellite ID list

In RAN2#128, the following agreements were reached:

- RAN2 understands that the UE configured with a satellite ID list by MME is not prevented to camp on a satellite operating in normal IoT NTN mode (i.e. with feeder-link connection), and perform subsequent access and data/signalling communication with that satellite - RAN2 understands the UE configured with a satellite ID list by MME is not prevented to camp on, attempt to access to and communicate with a satellite which is not included in the MME-configured satellite list.

The MME provides UE with the satellite ID list via NAS signalling. That is, the eNB doesn't perceive this information. If the eNB can obtain this information form UE or MME, it will help the eNB make decisions. For instance, for eMTC, the eNB can prepare the candidate cells that are served by the satellite within the satellite ID list in high priority. If so, the UE will access to the satellite within the satellite ID list with a high probability.

Proposal 4: Both the UE and MME can provide the eNB with the configured satellite ID list information.

3. Conclusions

In this contribution, we discussed the issues related to Store & Forward (S&F) satellite operation with split MME onboard. Based on the discussion, the following proposals are concluded:

Proposal 1: (issue RRC-1) An absolute UTC time can be used to indicate the time when the mode will change from normal mode to S&F mode.

Proposal 2: (issue RRC-2) The S&F timing information of neighbour cell (e.g., t-ServiceStartNeigh_s&f-r19) can be provided as satellite assistance information.

Proposal 3: (issue RRC-4) The UE can decide whether to monitor paging based on the time information about the last time the feeder link was available.

Proposal 4: Both the UE and MME can provide the eNB with the configured satellite ID list information.

4. References

[1]