3GPP TSG-RAN WG2 Meeting #130		       R2-2504487

Malta, Malta: 19th May – 23rd May 2025

Agenda Item:	8.1.2.2

Source:	ASUSTeK

Title:	Discussion on Remaining RRC open issues on LCM for UE-side models

Document for:	Discussion and Decision

- Introduction

In RAN2 129bis, RAN2 agreed to focus on the open issues and aim to finalize the RRC spec. In this contribution, we would like to discuss on some of the identified open issues based on the [POST129bis][016][AI PHY] 38.331 Running CR (Ericsson) email discussion.

- Discussion

Open issue RRC-1: Cause of inapplicability

Issue description: It is FFS how to define the simple cause value of inapplicability related to model availability and how to capture it in the spec.

This issue refers to the RAN2#129bis agreement:

Together with inapplicability reporting, UE further indicates a simple cause value of inapplicability FFS how to define this simple cause related to model availability and how we capture it in the spec

Reporting a cause of the inapplicability is to assist the NW to decide how to respond to the inapplicability report. In our opinion, the cause of inapplicability can be based on how urgent the UE is, since the NW could respond to the UE differently based on the urgency level indicated by the cause.

For example, when the UE has no AI/ML model available, the UE is in low urgency level because: (1) UE is not performing inference, and has no immediate impact on performance or reliability. (2) UE needs time to perform data collection, training and so on. Therefore, the UE does not require immediate handling from the NW, and the NW can postpone the LCM for the UE.

On the other hand, when the UE is in low battery, the UE may be in mid urgency level because UE does not want to perform inference as it is power consuming, but can still hold on for a while. If NW has some more urgent task, the UE can wait a bit.

Finally, when the UE is performing inference, but the computational resources are insufficient, the UE may be in high urgency level, which requires the NW to handle immediately. Otherwise, the performance may be degraded.

Proposal 1: The cause of inapplicability can be based on how urgent the UE is (e.g., in urgency levels).

Open issue RRC-3: UE data collection request

Issue description: For UE-side data collection, the details of UAI signaling for the UE start/stop request and especially the following aspects are not yet clarified:

- how to refer in UAI to a preferred radio resource candidate configuration from a list of candidate configurations provided by NW

- where/what the NW provides as candidate configurations

- what the content of otherConfig for enabling UE data collection requests in UAI should be (e.g. just a flag, the list of candidate UE data collection configurations, etc.).

This issue refers to the RAN2#129bis agreements:

The UE can request measurement configuration for data collection of AI/ML based beam management.   The request can contain one or more of the following:

•	An indication on start/stop of data collection

•	Preferred configuration from a list of candidate configurations provided by NW.  Details of signaling are FFS.  It is up to network what it configures at the end.

Introduce UAI message for UE request of data collection measurement configuration. And it is up to UE implementation when to send the request.

In our opinion, the list of candidate configurations provided by NW can reuse the list of inference configurations (e.g., option A) and the list of inference related parameters (e.g., option B). As the inference configurations and the list of inference related parameters are the configurations NW is interested in providing, it makes sense for the UE to only perform data collection and training on those configurations. The configurations also include the target RS resources to measure, this saves some signalling and the selection process at the UE side.

Proposal 2: The list of candidate configurations provided by NW can reuse the list of inference configurations (e.g., option A) and the list of inference related parameters (e.g., option B).

Open issue RRC-4: Activation of a periodic CSI report configuration upon change from inapplicable to applicable

Issue description: RAN2#129 agreed that for periodic CSI reporting, the UE autonomously activates the applicable functionality upon sending the applicability report via RRCReconfigurationComplete in Step 4:

If option A is configured in Step 3, for periodic CSI reporting, the UE autonomously activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 (i.e. without need to wait RRCReconfiguration in Step 5).

Further, RAN2#129bis agreed:

2 Upon receiving one or more full inference configuration(s) via RRCReconfiguration message, UE shall maintain all the full inference configuration(s) no matter the full inference configuration is applicable or inapplicable until the network releases it explicitly.

Thus, the UE will maintain a periodic CSI report configuration that is inapplicable, but it is unclear whether/how a periodic CSI report is activated upon a change of applicability, from inapplicable to applicable, since there is no legacy procedure to activate such a configuration (other than the initial sending of the RRC configuration).

One way to solve this situation is by network implementation, namely the network can de-configure a periodic CSI report configuration after the UE initially reports it as inapplicable via RRCReconfigurationComplete.

In our opinion, a simple solution is for the UE can start the inference after the UE receives ACK from the NW. This solution can also apply to both applicability reporting through RRCReconfigurationComplete and UAI.

Proposal 3: When the UE reports applicability, the UE can start the inference after the UE receives ACK from the NW.

Open Issue RRC-5: Reporting behaviour of inapplicable periodic beam prediction configuration

Issue description: It is ambiguous what the UE should report to the NW when its periodic beam prediction configuration becomes inapplicable and whether the model would output anything at all when if, e.g., the input distribution no longer matches what is expected.

We need to determine whether the configuration should cease reporting, or send the input (measurements) to the NW once it becomes inapplicable, or continue sending (inaccurate) predictions.

In our opinion, when the UE is inapplicable for a periodic beam prediction configuration, there may be several outcomes:

- The UE-side model starts generating irrelevant results (e.g., invalid values)

- The UE-side model is unable to perform (e.g., internal device issues, power issues)

- The UE-side model has bad performance (monitored by the UE itself)

Either cases, the UE does not have any helpful information to report to the NW. Therefore, it is reasonable for the UE to drop the CSI report (and trigger inapplicability report).

Proposal 4: The UE drops the report when its periodic beam prediction configuration becomes inapplicable.

Open issue RRC-8: Coexistence between option A and option B

Issue description: Considering that both Option A and B are to be specified, we need to discuss how the 2 options co-exist, whether there is any interaction between them etc. For instance, it is not clear whether both option A and option B can be configured or not. This can affect signaling design of ApplicabilityReportList.

If further procedures for option B are discussed, it is suggested to taken into account whether these procedures may depend on the list of inference related parameters from RAN1.

In our opinion, the 2 options can both be supported, but it seems more complexity would be introduced if option A and option B are configured at the same time. Therefore, the configuration of option A and option B should not be included in the same configuration/message. We think this simplifies the design and the need to handle interactions.

Proposal 5: Either option A or option B is configured in a RRC message.

Open Issue RRC-14: Repeated reports of applicability for configurations which consistently perform poorly

Issue description: UEs may repeatedly report a configuration as applicable based on the applicability determination information (UE-side and NW-side) but still perform poorly based on unforeseen factors, e.g., scenarios not captured in the dataset which trained one or more models supporting the AI/ML configuration. There is nothing we have agreed to so far to prevent a UE from repeating the same indication of applicability every time it re-enters RRC CONNECTED.

One solution is to let the UE receive feedback sufficient to adjust its applicability determination.

In our opinion, the issue is hard to handle in functionality level because the NW is not aware of the model the UE is using, neither does the NW know if every time the inapplicability origins from the same model under the same conditions. Handling the issue in functionality level may result in a UE being blocked by the NW without knowing the reason.

Proposal 6: For UE that consistently perform poorly, functionality level handling is not pursued.

Open Issue RRC-15: The time duration for an AI functionality to become available for inference when the UE reports applicability

Issue description: The UE needs to synchronize the time duration for an AI functionality to become available for inference with the network when the UE reports applicability for either a full inference configuration or a set of inference parameters.

Unlike conventional non-AI features, where the algorithm size is fixed, AI algorithms can vary significantly in size depending on their generalization performance and specific use cases. These sizes can range from several kilobytes to tens of megabytes. Additionally, UE implementations may use various types of memory to balance cost and performance, and these different memory types and constraints can affect the access speed of stored models.

The UE should indicate the time duration for the AI functionality to become available when reporting applicability to the network. This time duration refers to how long it takes for the UE to load the AI models into RAM or another accessible memory for inference. By providing this information, the network can better understand the timeframe within which the configured radio resources and AI model can be utilized, enabling more efficient and effective resource management, especially for periodic CSI.

In our opinion, we also think that including time information in the applicability/inapplicability report can be helpful. In various cases. The NW can have a more accurate understanding on when to send the RS, the UE can also be in less strict in time limitations for processing the CSI measurement and reports.

Proposal 7: Time information can be included in the applicability/inapplicability report.

- Conclusion

Proposal 1: The cause of inapplicability can be based on how urgent the UE is (e.g., in urgency levels).

Proposal 2: The list of candidate configurations provided by NW can reuse the list of inference configurations (e.g., option A) and the list of inference related parameters (e.g., option B).

Proposal 3: When the UE reports applicability, the UE can start the inference after the UE receives ACK from the NW.

Proposal 4: The UE drops the report when its periodic beam prediction configuration becomes inapplicable.

Proposal 5: Either option A or option B is configured in a RRC message.

Proposal 6: For UE that consistently perform poorly, functionality level handling is not pursued.

Proposal 7: Time information can be included in the applicability/inapplicability report.

References

[1] [POST129bis][016][AI PHY] 38.331 Running CR (Ericsson)

[2] TS 38.214

Annex