3GPP TSG-RAN WG2 Meeting #130		       R2-2504488

Malta, Malta: 19th May – 23rd May 2025

Agenda Item:	8.1.3

Source:	ASUSTeK

Title:	Discussion on Remaining RRC open issues for NW-sided data collection

Document for:	Discussion and Decision

- Introduction

In RAN2 129bis, RAN2 agreed to focus on the open issues and aim to finalize the RRC spec. In this contribution, we would like to discuss on some of the identified open issues based on the [POST129bis][016][AI PHY] 38.331 Running CR (Ericsson) email discussion.

- Discussion

Open issue RRC-24: Where to include the logging configuration from NW to UE

Issue description: It is FFS whether the logging configuration is included in the CSI framework (whether in CSI-ReportConfig or directly under CSI-MeasConfig) or at L3.

This issue was discussed in RAN2#129bis and the following outcome was captured:

Next meeting proponents should work together and bring complete proposals to show specification impact and consider future use cases.

In our opinion, the only part that can be unified is the logging related parameters (excluding the measurement targets). For UE-side data collection, RAN1 already decided to use CSI-ReportConfig. For NW-side data collection based on immediate MDT (i.e., without logging), CSI-ReportConfig is also reused. It does not seem to make much sense to have measurement targets (e.g., CSI-RS) included in MeasObjects when CSI-ReportConfig is already available for use. For the logging related parameters, we think a new IE or CSI-MeasConfig may be a good option.

Proposal 1: The logging configuration only includes logging related parameters (i.e., excluding the measurement targets).

Proposal 2: The logging configuration can be placed in a new IE or CSI-MeasConfig.

- Conclusion

Proposal 1: The logging configuration only includes logging related parameters (i.e., excluding the measurement targets).

Proposal 2: The logging configuration can be placed in a new IE or CSI-MeasConfig.

References

[1] [POST129bis][016][AI PHY] 38.331 Running CR (Ericsson)

Annex