3GPP TSG-RAN WG2 Meeting #130		       R2-2504492

Malta, Malta: 19th May – 23rd May 2025

Agenda Item:	8.3.2

Source:	ASUSTeK

Title:	Discussion on functionality management for AIML Mobility

Document for:	Discussion and Decision

- Introduction

The objectives of SI FS_NR_AIML_Mob agreed to work on the specification impact of AI/ML mobility, where enhancements should be based on the Rel19 AI/ML-air interface WID general framework. In this document, we would like to discuss the potential impacts and whether/how the Rel19 AI/ML-air interface WID general framework can be reused.

Study and evaluate potential benefits and gains of AI/ML aided mobility for network triggered L3-based handover, considering the following aspects: - AI/ML based RRM measurement and event prediction, - Cell-level measurement prediction including intra and inter-frequency (UE sided and NW sided model) [RAN2] - Inter-cell Beam-level measurement prediction for L3 Mobility (UE sided and NW sided model) [RAN2] - HO failure/RLF prediction (UE sided model) [RAN2] - Measurement events prediction (UE sided model) [RAN2] - Study the need/benefits of any other UE assistance information for the network side model [RAN2] - The evaluation of the AI/ML aided mobility benefits should consider HO performance KPIs (e.g., Ping-pong HO, HOF/RLF, Time of stay, Handover interruption, prediction accuracy, and measurement reduction) etc.) and complexity tradeoffs [RAN2] - NOTE: Simulation assumption and methodology can leverage TR 38.901, 38.843 and 36.839. And leave the detail discussion to RAN2 - Potential AI mobility specific enhancement should be based on the Rel19 AI/ML-air interface WID general framework (e.g. LCM, performance monitoring etc) [RAN2] - NOTE: This would only be treated after sufficient progress is made in the Rel-19 AI/ML air interface WID - Potential specification impacts of AI/ML aided mobility [RAN2] - Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4] - Study the impacts on requirements based on RAN2 assumptions, and coordinate with RAN2 if needed - Study the testability and interoperability based on RAN2 framework (e.g., number of cells to measure, beams etc.) - NOTE 4: Leverage the work from “AI/ML for NR air interface” led by RAN1 and avoid the duplicate study for testability and interoperability. - NOTE 5: Avoid the overlaps with RAN2 work for evaluation - NOTE 1: RAN1/3 work can be triggered via LS NOTE 2: To avoid duplicate study with “AI/ML for NG-RAN” led by RAN3 NOTE 3: Two-sided model is not included

- Discussion

AI/ML BM-Case 2 have some similarities to RRM prediction for AI/ML mobility. Therefore, we think the parameters for inference configuration for BM-Case 2 can be considered as a starting point for configuration.

Proposal 1: Use the concept of BM-Case 2 in AI/ML PHY for inference configuration for RRM prediction.

Based on the AI/ML PHY, the configuration for AI/ML mobility may consist of the following aspects:

- Measurement targets related information (e.g., set B in AI/ML PHY)

- Prediction targets related information (e.g., set A in AI/ML PHY)

- NW-side additional information (e.g., associated ID)

- Report content related information

- Time instance related information

For AI/ML beam management, set A and set B are configured using CSI-ResourceConfig. For AI/ML mobility, we think the configuration of measurement object (e.g., MeasObjectNR) is a suitable replacement. And for report content related information (e.g., event prediction), it is natural to use the configuration of measurement report (e.g., ReportConfigNR). FFS on the details.

Proposal 2: MeasObjectNR is used for configuring measurement targets and prediction targets.

Proposal 3: ReportConfigNR is used for configuring report content related information. FFS on details.

Currently, the measurement targets for cell level measurements are SSBs and CSI-RS. This is the same for every subcase as cell level measurements are derived from L1 beam measurements.

For the prediction targets,

- For subcase 1, the intermediate outcome is a set of L1 RSRP of one or more future time instances, and the final outcome is the cell level L3 RSRP of one or more future time instance.

- For subcase 2 and 3, the final outcome is the cell level L3 RSRP of one or more future time instances.

The only difference is that subcase 1 has the intermediate outcome of L1 RSRP of one or more future time instances. However, in our opinion, the intermediate outcome does not need to be specified. Additionally, if we consider the configuration for cell level measurements, the configuration and measurement is also based on RS resources (e.g., SSBs and CSI-RS).

Consider the report content and time instance related information should have minimal impact on the subcase, we think that subcase 1/2/3 has minimal difference in specification impact.

Proposal 4: For UE-side model, agree that subcase 1/2/3 has minimal difference in specification impact.

In our opinion, for AI/ML mobility, applicability reporting based on inference configuration can be the baseline to study, which is similar to AI/ML Beam Management. Further enhancements can be made if some aspects are identified to be redundant.

Proposal 5: For AI/ML mobility, applicability reporting based on inference configuration can be the baseline to study.

- Conclusion

Proposal 1: Reuse the concept of BM-Case 2 in AI/ML PHY for inference configuration for RRM prediction.

Proposal 2: MeasObjectNR is used for configuring the measurement targets and prediction targets.

Proposal 3: ReportConfigNR is used for configuring report content related information. FFS on details.

Proposal 4: For UE-side model, agree that subcase 1/2/3 has minimal difference in specification impact.

Proposal 5: For AI/ML mobility, applicability reporting based on inference configuration can be the baseline to study.

References

[1] RP-242393, “Revised SID on AIML for mobility in NR”

Annex