3GPP TSG-RAN WG2 Meeting #130	R2-2504495

St. Julians, Malta, May 19th – 23rd, 2025

Agenda Item:	8.6.3

Source: 	ASUSTeK

Title:  	Discussion on event-triggered L1 MR MAC CE

Document for:	Discussion and Decision

- Introduction

In the previous meeting RAN2#129bis, L1 event triggered report for LTM was discussed and has made several agreements including details of Measurement report (MR) MAC CE:

Introducing a new R19 LTM cell switch command MAC CE with indicating the NCC value for R19 inter-CU LTM cell switch. Details will be further discussed during CR phase. For differential L1-RSRP reporting, the best quality beam among the beams included in L1 MR MAC CE is taken as the reference beam as the first one. The differential L1-RSRP value is derived based on the absolute L1-RSRP of the reference beam. FFS for truncated MAC CE. When UL grant is enough to accommodate only limited number of beams in MR MAC CE, the triggered beam should be included in the truncated MR MAC CE. When current beam is configured to be included in the L1 MR MAC CE, 7 bits of RSRP is included and no RSRI is needed. The current beam information of SpCell (if configured to be included) is always placed at the end of the normal (not truncated) MAC CE. Same as Rel-18: When the C-LTM execution is triggered, if the TA of the target cell acquired by early RACH is valid, UE applies the TA acquired by early RACH, otherwise if UE has successfully measured the TA of the target cell, UE applies the measured TA. FFS For MR triggered by LTM2, whether only include the current beam information in the MR MAC CE or the MR can include measurements for LTM candidates.

Agreements on L1 event-triggered MR

- Introduce new MAC CE to activate/deactivate the semi-persistent CSI-RS resource.

- The new MAC CE includes: A/D indication, Target configuration id or {SP CSI-RS resource set id and/or candidate id}, TCI state id.

- In L1 reporting configuration, the UE can be configured with a list of candidate IDs. For each listed candidate ID, the network can configure an offset, this offset applies to all the RSs belong the candidate ID that associated with the reporting configuration (the offset value also includes 0dB); If a candidate ID is not provided in the reporting configuration, it means the UE is not required to measure/evaluate the RSs that belonging to the candidate ID for this event, even if these are configured within the L1 resource configuration (to which the L1 reporting configuration points to).

- In the MR MAC CE, adopt a uniform format for all the reported beams (i.e., using the same format for beams leaving the entering condition, beams met the entering condition, beams not satisfying the event). Introduce two bits per beam.

- RAN2 will consider only the following options:

- Option1: To introduce four codepoints to represent one of {type1, type2, type3, type4}

- Option3: To introduce three codepoints to represent one of {type1_or_type3, type2, type4}, i.e., type1 beam and type3 beam are not distinguished in reporting.

Type1 beam: a beam that has satisfied the entry condition of the event for TTT and thus triggers the MR MAC CE.

Type2 beam: a beam that has satisfied the leaving condition of the event for TTT and thus triggers the MR MAC CE.

Type3 beam: a beam that is neither type1 nor type2 but has remained in beamsTriggeredList (the beam was reported as type1 beam before).

Type4 beam: a beam that is neither type1 nor type2 and not included in beamsTriggeredList (i.e., just extra beam info)

- Working assumption: Option1: To introduce four codepoints to represent one of {type1, type2, type3, type4}

- Intention is that the UE should be able to report the event-triggered beam(s) that were not included in the truncated MR MAC CE by the following grant. Detailed wording can be further discussed as part of the running CR.

- For co-existence with mTRP, will be revisited in August. If one simple solution is not prepared / agreed until / in August meeting, we will not apply mTRP in Rel-19 event-triggered MR.

In this contribution, we discuss further details for L1 measurement report for LTM.

- Discussion

Beam selection for MR MAC CE

According to the current agreement, the first beam in a MR MAC CE is the beam with the best measured quality among beams included in this MR MAC CE, which is used as a reference beam for reporting of other beams. However, how to select the beams (within maximum number of reported beams, maxNumberOfReportedBeams) to be included in the MR MAC CE is not yet determined, and the order of the rest of the beams in the MR MAC CE is also not yet decided. If selection of the reported beams for MR MAC CEs is completely up to UE implementation, the MR MAC CE may not be able to carry information needed by the network (e.g., triggered beams or highest quality beams) and the UE may prematurely cancel the triggered L1 measurement report after generating the MR MAC CE that does not provide enough information. In addition, the generated (truncated) MR MAC CE may not meet requirements by the agreements if no rules of beam selection are defined. To ensure MR MAC CE is correctly generated, we discuss how to select and include beams in the MR MAC CE below.

Firstly, according to the previous agreements, the UE should include triggered beam in truncated MR MAC CE:

- (RAN2#129) Select the beam not satisfying the event condition, based on the measured quality (i.e. according to the best beam(s)) … Truncated MR MAC CE includes the triggered beam information: - One report config ID (event config ID) - At least one triggered beam id (SSBRI or CRI) + L1 RSRP Common format for normal MR MAC CE and truncated MR MAC CE is used. Details are handled by MAC running CR rapporteur. (RAN2#129bis) When UL grant is enough to accommodate only limited number of beams in MR MAC CE, the triggered beam should be included in the truncated MR MAC CE.

One approach to select the beams to report in a MR MAC CE is to select based on measured quality (e.g., based on best beams), which was already agreed for selection of beams not satisfying the conditions. The advantage of this approach is that the network can always know the best quality beams for further mobility procedures. However, a possible issue to apply beam selection solely based on beam quality is that there may be a case where the UE does not include any of the triggered beams, which will not fulfil the previous agreements (e.g., when there are n high-quality beams that does not satisfy the event conditions that have quality higher than triggered beams, n = maximum number of beams to report). For example, triggered beams that fulfil leaving condition will often have a lower quality than other beams. In addition, if triggered beams are not reported and instead other beams are included, the network will not be able to know the current event condition and the UE will prematurely cancel the measurement report.

Another approach to ensure at least one triggered beam will be included in a (truncated or non-truncated) MR MAC CE, and to have the reported beams being included as much as possible, is to prioritize selecting triggered beams that meet entering condition first, then triggered beams that meet leaving condition, and finally other beams that does not fulfil any conditions. If there are multiple triggered beams that meet entering condition, selection prioritization between the multiple triggered beams can be based on measured beam quality.

Proposal 1:	When generating a truncated or non-truncated Event Triggered L1 Measurement Report MAC CE, the UE decides which beams to be included in the MAC CE, up to the maximum number of reported beams (based on size of UL grant or maxNumberOfReportedBeams), based on the one of the two approaches:

Approach 1: The UE selects the reported beams in decreasing order of beam quality.

Approach 2: the UE selects the reported beams in the following order:

- Triggered beam(s) that meets the entering condition, in decreasing order of beam quality.

- Triggered beam(s) that meets the leaving condition, in decreasing order of beam quality.

- Beam(s) that does not satisfy any event condition, in decreasing order of beam quality.

Order of reported beams in MR MAC CE

Currently, the first beam is the best quality beam among reported beams in the MR MAC CE, and the order for the rest of reported beams were not discussed. In our view, it is more straightforward to apply the same rule to all reported beams in addition to the first beam, that is, the reported beams are included in decreasing order of beam quality. This is also beneficial for the network to obtain high-quality beam info for early sync and LTM execution first, when truncated MR MAC CE is generated.

Proposal 2:	When generating a truncated or non-truncated Event Triggered L1 Measurement Report MAC CE, the reported beams are included in decreasing order of beam quality.

prioritization between multiple MR MAC CEs

According to last meeting agreement, the Measurement Report MAC CE is transmitted with the same LCH priority as MAC CE for (Enhanced) BFR. As one MR MAC CE can only report information of a single event, it is possible that when multiple events are fulfilled for different report configurations (LTM-CSI-ReportConfig), the UE needs to generate and report multiple MR MAC CEs that are associated with different triggered events.

Observation:	When there are multiple triggered L1 measurement reports for different events, the UE needs to generate and report multiple MR MAC CEs.

It is possible that an UL grant cannot accommodate all MR MAC CEs to be reported. Then, the order between separate MR MAC CEs need to be decided during assembly and multiplexing. One simple way is to prioritize the MR MAC CE in an ascending order of ltm-CSI-ReportConfigId (e.g., prioritize MR MAC CE associated with smaller ltm-CSI-ReportConfigId first). Another way is to prioritize MR MAC CEs with smaller report configuration type first (e.g., the UE prioritizes MR MAC CE with LTM2 over MR MAC CE with LTM5).

Another direction is to have priority for each triggered event. The priority can be a fixed order according to timing when the pending events was triggered (e.g., measurement report that was triggered first in time can be included first), or the priority can be configured by network (e.g., priority of each event or measurement configuration can be configured. For example, report with triggering event LTM3 can be prioritized over LTM2).

In our view, it’d be simpler to prioritize MR MAC CE with smaller report ID. While it may be beneficial to prioritize MR MAC CE associated with more mobility-related events (e.g., events that considers candidate quality, LTM 3 or LTM5), it can be up to NW to assign the more important measurement configurations with smaller report IDs so that the UE can report the corresponding MR MAC CEs first.

Proposal 3:	When multiple MR MAC CEs are to be reported, the UE prioritizes MR MAC CEs containing smaller report ID (ltm-CSI-ReportConfigId).

- Conclusion

We have the following observation and proposals for MR MAC CE for LTM:

Proposal 1:	When generating a truncated or non-truncated Event Triggered L1 Measurement Report MAC CE, the UE decides which beams to be included in the MAC CE, up to the maximum number of reported beams (based on size of UL grant or maxNumberOfReportedBeams), based on the one of the two approaches:

Approach 1: The UE selects the reported beams in decreasing order of beam quality.

Approach 2: the UE selects the reported beams in the following order:

- Triggered beam(s) that meets the entering condition, in decreasing order of beam quality.

- Triggered beam(s) that meets the leaving condition, in decreasing order of beam quality.

- Beam(s) that does not satisfy any event condition, in decreasing order of beam quality.

Proposal 2:	When generating a truncated or non-truncated Event Triggered L1 Measurement Report MAC CE, the reported beams are included in decreasing order of beam quality.

Observation:	When there are multiple triggered L1 measurement reports for different events, the UE needs to generate and report multiple MR MAC CEs.

Proposal 3:	When multiple MR MAC CEs are to be reported, the UE prioritizes MR MAC CEs containing smaller report ID (ltm-CSI-ReportConfigId).

- Reference