3GPP TSG-RAN WG2 Meeting #130	R2-2504496

St. Julians, Malta, May 19th – 23rd, 2025

Agenda Item:	8.6.4

Source: 	ASUSTeK

Title:  	Discussion on TA handling for C-LTM

Document for:	Discussion and Decision

- Introduction

In the previous meeting RAN2#129bis, TA handling and 2TA support for C-LTM are discussed and following agreements have been made:

Agreements on C-LTM

- Upon C-LTM execution, UE performs the following for received TA values and TA timers for other candidate cells than the target candidate cell: Maintain the TA value and Keep the early TA timer running.

- C-LTM TAT for target cell is not stopped upon C-LTM cell switch execution to the target cell if it is running, and starts the PTAG using the remaining time from the C-LTM timer.

- RAN2 confirm CLTM with Rel-18 MIMO 2TA is supported, introducing TAG ID field in LTM candidate TAC MAC CE.

- For CLTM with Rel-18 MIMO 2TA, two TA timer length (i.e. ltm-TimeAlignmentTimer) can be configured via RRC.

In this contribution, we discuss further details for handling TA for C-LTM.

- Discussion

Storing TA for LTM source Cell

In the previous meeting, it was agreed that TAT of the PTAG of the target cell is started using the remaining time from C-LTM timer upon C-LTM cell switch. On the other hand, TA handling of the source cell were not discussed. In our view, source cell is possible to be included in one of the C-LTM candidates (For example, the UE switched to this source cell via a previous C-LTM procedure with this source cell as the target cell.). When a LTM is performed to a target candidate cell, the TA of the source cell may still be valid as the TAT is still running.

Observation:	When performing LTM to a target cell, the TA associated with a source cell of the LTM, which could be a C-LTM candidate Cell, can still be valid.

If the source cell is associated with a C-LTM candidate, then it would be beneficial to store the valid TA of the source cell for possible RACH-less C-LTM back to the source cell. The UE can then start the corresponding C-LTM timer (ltm-Candidate-TimeAlignmentTimer) associated with the C-LTM candidate of the source cell.

Figure 1

As illustrated in Figure 1, the UE determines to perform LTM on a candidate cell at timing t1, where the TAT of the source cell has been running for n ms. The UE then stores the TA of the source cell for the corresponding C-LTM candidate that includes the source cell as the SpCell (e.g., the C-LTM-Candidate of which ltm-CandidatePCI is set to the PCI of the source cell). The UE then (re)starts the ltm-Candidate-TimeAlignmentTimer associated with the corresponding C-LTM candidate, with the length of the remaining time of the timeAlignmentTimer of the source cell (i.e., m-n ms).

Proposal 1:	Upon execution of a C-LTM to a candidate cell, if the source cell is a SpCell of a C-LTM candidate configuration, the UE stores the TA of the source cell for the C-LTM candidate configuration, and starts the ltm-Candidate-TimeAlignmentTimer associated with the C-LTM candidate with the length of the remaining time of the timeAlignmentTimer of the source cell.

Candidate beam selection for performing C-LTM on a 2-TA candidate cell

According to the agreement, C-LTM with Rel-18 MIMO 2TA is supported. That is, a target candidate cell can be configured with 2 TAGs and the UE can perform C-LTM via a selected beam associated with one of the 2 TAGs. A scenario could occur when the UE is provided with a TA associated with one of the TAG of a target candidate cell, when multiple candidate beams satisfy event for C-LTM, a part of the candidate beams are associated with the valid TA while the other part of the candidate beams are not. In this case, we think that it is beneficial to leverage the valid TA and prioritize performing RACH-less C-LTM towards the candidate cell. Therefore, we propose the UE to prioritize selecting candidate beams that are associated with a valid TA when performing C-LTM to a target candidate Cell.

Proposal 2:	When multiple candidate beams satisfy the event for C-LTM on a 2TA candidate cell, the UE prioritizes selecting candidate beams that are associated with a valid TA.

- Conclusion

We have the following observation and proposal for TA handling on C-LTM:

Observation:	When performing LTM to a target cell, the TA associated with a source cell of the LTM, which could be a C-LTM candidate Cell, can still be valid.

Proposal 1:	Upon execution of a C-LTM to a candidate cell, if the source cell is a SpCell of a C-LTM candidate configuration, the UE stores the TA of the source cell for the C-LTM candidate configuration, and starts the ltm-Candidate-TimeAlignmentTimer associated with the C-LTM candidate with the length of the remaining time of the timeAlignmentTimer of the source cell.

Proposal 2:	When multiple candidate beams satisfy the event for C-LTM on a 2TA candidate cell, the UE prioritizes selecting candidate beams that are associated with a valid TA.

- Reference