3GPP TSG-RAN WG2 Meeting #130	R2-2504497

St. Julian’s, Malta, 19th-23rd May, 2025

Agenda Item:	8.9.2

Source:			ASUSTeK

Title:	Discussion on time information for S&F

Document for: 	Discussion and Decision

- Introduction

For time information for Store & Forward operation, following agreements are made in previous meetings.

In RAN2 #128 meeting:

Agreements:

1.	Time information can be broadcasted to indicate when the current satellite operation mode will transit from “S&F operation” mode to real-time/normal mode. RAN2 assumes that a R19 UE could use this information at least to delay some NAS procedures until the feeder link is resumed (FFS what we specify in RAN2 specs for this and if we differentiate the behaviour for R19 UEs supporting and not supporting S&F mode, also considering the relation to the Wait Timer). FFS which SIB is used (SIB1 or SIB31). FFS if absolute or relative time is used. FFS on the opposite transition (i.e. whether to indicate when the “S&F operation” will start)

2.	It is FFS whether we need to introduce a new release cause when the NW wants to release a R19 UE to RRC idle because the feeder link becomes unavailable

3.	“S&F operation” indication is provided in SIB1

In RAN2 #129 meeting:

Agreements:

- RAN2 currently assumes that a S&F mode changes does not necessarily imply a MME change

- The already agreed time information on transition from S&F operation mode to normal mode is provided in SIB31 (can work on signalling optimizations in case the new time information and T-service would carry the same information)

- Use absolute UTC time to broadcast the (S&F-> normal mode) time information. When this information is broadcast and indicates a time in the future the UE can assume the NW is currently operating in S&F mode (until the indicated S&F-> normal mode transition time).

- A change to the (S&F-> normal mode) time indication shall not result in system information change notification. Legacy rules for acquiring SIB31 apply

In RAN2 #129bis meeting:

Agreements:

1.	For UEs supporting S&F, the UE AS indicates the information on transition time (if any) from current "S&F operation mode" to "normal mode" (if received) to the upper layers. Whether/how it is used by the upper layers is up to CT1.

2.	We introduce an indication in system information for the normal mode to S&F mode transition, at least for NAS use. FFS on the details (e.g. whether we can link this to other existing information). The information on transition time for the normal mode to S&F mode transition is sent from AS to NAS

3.	We don’t introduce any new release cause related to S&F operation

In this contribution, we would like to further discuss the time information for S&F mode.

- Discussion

The Store and Forward (S&F) operation in IoT NTN is intended to provide some level of communication service for UEs under satellite coverage with intermittent/temporary satellite connectivity, e.g., for delay-tolerant communication service. It has been agreed that the network broadcasts a “S&F operation indication” via SIB1 to indicate whether an NTN cell is operating in S&F operation mode. If the S&F operation indication is provided, the UE considers that the NTN cell is in S&F operation mode; otherwise, the UE considers that the NTN cell is operating in normal mode. And the network broadcasts a time information on transition from S&F operation mode to normal mode via SIB31. The two parameters are currently implemented in the running RRC CR [1] as sf-OperationIdication and t-ModeSwitching separately.

For the time information on opposite transition (i.e., transition from normal mode to S&F operation mode), it was agreed in the last meeting to introduce an indication in a system information. Although some companies propose to reuse t-Service to indicate the time information, an additional indication is needed for the S&F capable UE to know whether the t-Service is cell stop serving time or S&F operation mode transition time. For simplification, the t-ModeSwitching is SIB31 could be reused for this purpose. If sf-OperationIdication is present in SIB1, t-ModeSwitching in SIB31 indicates the time when the NTN cell is going to switch from S&F operation mode to normal mode. And if sf-OperationIdication is absent in SIB1, t-ModeSwitching in SIB31 indicates the time when the NTN cell is going to switch from normal mode to S&F operation mode.

Proposal 1: Use t-ModeSwitching to indicate both transition time on S&F operation mode to normal mode and transition time on normal mode to S&F mode.

As agreed in the last meeting, for UEs supporting S&F, the information on transition time from S&F operation mode to normal mode is indicated to NAS. And the information on transition time from normal mode to S&F operation mode is indicated to NAS. A related LS was sent to CT1. For the UE supporting S&F receiving t-ModeSwitching, it indicates the t-ModeSwitching to the upper layers. For the UE not supporting S&F receiving t-ModeSwitching, in addition to indicating the time information to the upper layers, the UE could not continue its operation after the NTN cell switches to S&F mode. For example, for a UE not supporting S&F, the UE may camp on or connect to a cell supporting S&F while the cell is operating in normal mode. The UE may perform data transmission in the cell. However, the UE may not be able to continue the data transmission when the cell is operating in S&F mode.

Therefore, the UE not supporting S&F should perform some actions when t-ModeSwitching indicating transition time on normal mode to S&F operation mode, similar to the case of t-Service. For example, the UE performs measurements of neighbouring cells before t-ModeSwitching, and considers radio link failure upon t-ModeSwitching. Then the UE could select a new cell operating in normal mode without other NW signalling.

Proposal 2: When t-ModeSwitching indicates transition time on normal mode to S&F mode, for a UE not supporting S&F, the UE performs measurements of neighbouring cells before t-ModeSwitching and considers radio link failure upon t-ModeSwitching.

- Conclusion

We have the following proposals for S&F transition time information:

Proposal 1: Use t-ModeSwitching to indicate both transition time on S&F operation mode to normal mode and transition time on normal mode to S&F mode.

Proposal 2: When t-ModeSwitching indicates transition time on normal mode to S&F mode, for a UE not supporting S&F, the UE performs measurements of neighbouring cells before t-ModeSwitching and considers radio link failure upon t-ModeSwitching.

- Reference

- R2-2504068, “RRC Running CR for IoT NTN”