3GPP TSG-RAN WG2 Meeting #130	R2-2504511

Saint Julian’s, Malta, 19 – 23 May 2025

Agenda item:	8.8.3

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on UL Capacity and Throughput Enhancement

WID/SID:	NR_NTN_Ph3-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

In RAN#106 the work item on NR NTN (Non-Terrestrial Networks) enhancements was updated [1]. Among the objectives of the work item, there is uplink capacity/throughput enhancement for FR1-NTN, using orthogonal cover codes (OCC). The objective description is as follows:

Uplink Capacity/Cell Throughput Enhancement for FR1-NTN [RAN1, RAN2, RAN4] - Specify Orthogonal Cover Codes (OCC) for DFT-s-OFDM PUSCH at least for multiplexing 2 or 4 UEs when PUSCH repetitions are used - Specify necessary signalling - Update RF requirements accordingly, if needed - Notes for this objective: - The enhancement is not targeting improvements/impacts of MU-MIMO capability - The enhancement is not targeted to PUSCH DMRS - No enhancement for initial access - Enhancements to PRACH are not in scope. - This feature may be applicable for UEs operating in terrestrial networks based on a common design

In RAN2-129bis meeting, following agreement was reached.

Agreements. 1. There is no need to introduce dedicated access control mechanism for OCC capable UEs

In this contribution, we provide further views on the possible RAN2 impacts.

# 2	Discussion

In RAN1 #119, an agreement was reached for the OCC scheme to support for Rel-19:

Agreement RAN1 to confirm the working assumption of RAN1#118bis with revisions as follows: - Support OCC length 2 with inter-slot OCC to multiplex up to 2 UEs. - Support Option 1: inter-slot OCC with OCC length 4 to multiplex up to 4 UEs using Hadamard sequences - RAN1 does not pursue Option 2: Intra-symbol pre-DFT OCC with OCC length 4 to multiplex up to 4 UEs. - RAN1 does not pursue Option 3: Combination of Inter-slot OCC with OCC length 2 and intra-symbol pre-DFT OCC with OCC length 2 to multiplex up to 4 UEs. Note 1: there will be separate UE capabilities for OCC length 2 and OCC length 4, where UE capability for OCC length 2 is a prerequisite for UE capability for OCC length 4.

Based on the RAN1 agreement, both 2 and 4 UEs multiplexing are supported by using inter-slot OCC.  As the OCC is an optional feature for NTN UEs, RAN1 agreed there will be separate UE capabilities for OCC length 2 and OCC length 4, where UE capability for OCC length 2 is a prerequisite for UE capability for OCC length 4. The exact UE feature requirement to support the inter-slot OCC is still in discussion in RAN1.

Proposal 1: Introduce separate UE capabilities for OCC with length 2 and OCC with length 4. The UE capability for OCC length 2 is a prerequisite for UE capability for OCC length 4. The exact UE capability description can be finalized based on the RAN1 LS on feature list.

To enable orthogonal transmission (and reception) of multiple UEs over the same time/frequency resources using OCC, methods for indication of OCC operation to corresponding UEs need to be introduced. It is indeed fundamental that UEs scheduled in the same time frequency resources use different and orthogonal codes for their transmissions, and this can be guaranteed via gNB indication and management of UEs OCC code selection. Therefore, the code characteristics including OCC length, actual code index should be clearly known at the UE.

In RAN1-120bis meeting, below agreement were reached for how to indicate the OCC sequence length and OCC sequence index for Configured Grant Type 1, Configured Grant Type 2 and Dynamic Grant.

Agreement OCC length and OCC sequence for OCC with CG PUSCH Type 1 is configured by RRC higher-layers. - Up to RAN2 whether to signal this with one or two RRC parameters - Note: OCC lengths and sequences to be provided in the table of RRC parameters to be prepared by the WI rapporteur Agreement For the OCC length applied for OCC DG PUSCH and CG PUSCH Type 2, consider the following options: - Option 1: Configured by RRC higher-layer parameter. - Note: this does not preclude jointly configuring OCC sequence and OCC length with the same RRC configuration - Option 2: Max value is configured by RRC higher-layer parameter, and the applied value is implicitly determined from the configured repetition number. - Option 3: Candidates values are configured by RRC higher-layer parameter, and the applied value is indicated by scheduling DCI for DG PUSCH or by activation DCI for CG PUSCH Type 2. - Note: this does not preclude jointly configuring OCC sequence and OCC length with the same RRC configuration - Option 4: Single value is indicated by scheduling DCI for DG PUSCH and by activation DCI for CG PUSCH Type 2 (no RRC configuration of candidate values). - Option 5: Max value is configured by RRC higher-layer parameter, and the applied value is indicated by scheduling DCI for DG PUSCH and by activation DCI for CG PUSCH Type 2. Note: it is not precluded to select a different option for DG PUSCH and CG PUSCH Type 2. Agreement For the OCC sequence applied for OCC DG PUSCH and CG PUSCH Type 2, the sequence is indicated dynamically in DCI. FFS: with a new field or reusing an existing field

For CG PUSCH Type1 where the CG resource periodicity and PRBs/MCS are configured via RRC but without DCI activation/deactivation, RAN1 agreed the OCC sequence length and OCC sequence index are configured by RRC layer. It is up to RAN2 to decide whether to signal the information to UE with one or two RRC parameters.

From R1-2502985 (Feature lead summary #3 of AI 9.11.3 on NR-NTN uplink capacity and throughput), RAN1 has discussed below RRC parameters for CG PUSCH Type1. Since the UE may support OCC with length 2 or OCC with length 4, the sequence set and the available sequence index range are different in two cases. In our understanding, one parameter for OCC sequence length and the other parameter for OCC sequence index is simple and clear. Otherwise (if only one RRC parameter is defined), the network may need to indicate the actual Hadamard OCC sequence to UE which will cost more bits.

| Parameter name in the spec | Description | Value range | UE-specific or Cell-specific |
| --- | --- | --- | --- |
| occ-Index-r19 | Length of OCC for inter-slot OCC with CG-PUSCH Type 1 | 2 or 4 | UE-specific |
| occ-SequesnceIndex-r19 | Sequence index for inter-slot OCC with CG-PUSCH Type 1 in Hadamard sequence set [+1, +1; +1, -1] for occ-Length =2; or Hadamard sequences [+1, +1, +1, +1; +1, -1, +1, -1; +1, +1, -1, -1;+1, -1, -1, +1] for occ-Length = 4 | 0,1 for occ-Length =2 0,1,2,3 for occ-Length = 4 | UE-specific |

Proposal 2: For CG PUSCH Type1, RAN2 defines two RRC parameters to indicate OCC sequence length and OCC sequence index to UE.

For DG PUSCH and CG PUSCH Type 2, it is still in RAN1 discussion on how to indicate the OCC sequence length and index to UE. From RAN2 point of view, we can wait for more progress in RAN1.

Proposal 3: For DG PUSCH and CG PUSCH Type 2, wait for RAN1’s conclusion on how to indicate the OCC sequence length and OCC sequence index to UE.

# 3	Conclusion

This document has made the following proposals:

Proposal 1: Introduce separate UE capabilities for OCC with length 2 and OCC with length 4. The UE capability for OCC length 2 is a prerequisite for UE capability for OCC length 4. The exact UE capability description can be finalized based on the RAN1 LS on feature list.

Proposal 2: For CG PUSCH Type1, RAN2 defines two RRC parameters to indicate OCC sequence length and OCC sequence index to UE.

Proposal 3: For DG PUSCH and CG PUSCH Type 2, wait for RAN1’s conclusion on how to indicate the OCC sequence length and OCC sequence index to UE.

# References

- RP-243300, ”Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3”