3GPP TSG-RAN WG2 Meeting #130	R2-2504542

St.Julians, Malta, May 19th-23th, 2025

Agenda item:	8.6.2

Source:	Samsung

Title:	Further Considerations to Support Inter-CU LTM

WID/SID:	NR_Mob_Ph4-Core

Document for:	Discussion and Decision

Introduction

In RAN2#129bis meeting, RAN2 reached the following agreements for fast recovery:

At RLF and reconfiguration with sync failure:

- If RLF: If the selected candidate cell has the same Rel-19 ID as source (no security key change), the UE performs fast RLF recovery (same as in Rel-18).

- if reconfiguration failure (inter-CU LTM): If the selected candidate cell has the same Rel-19 ID as target, the UE performs fast failure recovery. FFS if fast failure recovery with different Rel-19 IDs is allowed.

In this contribution, we will further address fast recovery for inter-CU LTM.

Discussion

- Clarification on the concept of fast recovery

The basic concept is that after the handover/cell switch failure, the UE selects one of configured candidate cell so that it can perform the accessing to the new selected cell and send the RRCReconfigurationComplete message, instead of sending RRCReestablishmentRequest message. Before Rel-19, we have the following two types of fast recovery:

- Type 1: fast recovery with security key update

For CHO case, such fast recovery will result in the security key change after accessing to the new selected cell. Then, the UE will use the new security key for RRCReconfigurationComplete message transfer.

- Type 2: fast recovery without security key update

For intra-CU LTM case, such fast recovery will NOT result in the security key change after accessing to the new selected cell. The reason is that Rel-18 allows no security key change during intra-CU LTM. Then, the UE will use the security key in the source cell for RRCReconfigurationComplete message transfer.

Compared to Type 1, Type 2 realizes the faster recovery due to no security key change. Moreover, in Rel-18, the LTM fast recovery is referring to Type 2. So, we would like to clarify that the fast recovery in Rel19 is referring to Type 2.

Proposal 1: RAN2 is kindly asked to agree to support the fast recovery without security key change.

- Cases to support fast recovery after inter-CU LTM failure

Let’s consider one example: an UE executes an inter-CU cell switch from Cell A (source cell) to Cell B (target cell); after failure, the UE selects a cell, i.e., Cell C, which may result in the following cases:

- Case 1: Cell C is an LTM candidate cell under the same CU as the Cell A (source cell)

- Case 2: Cell C is an LTM candidate cell under same CU as the Cell B (target cell)

- Case 3: Cell C is an LTM candidate cell under CU different from the one serving Cell A and Cell B

- Case 4: Cell C is a cell not being configured as LTM candidate cell.

Among those cases, whether Case 1/Case 3 supports fast recovery or not is unclear.

For Case 1, during the inter-CU LTM, the UE performs the security key update along with PDCP/RLC re-establishment. After the failure, the legacy behaviour results in that the UE reverts back to the source configuration, including the security key. However, in Rel-18, we have the following agreement:

LTM recovery is not supported for T304 expiry after HO (L3 HO) and CHO, but supported for T304 expiry after LTM cell switch (and RLF as earlier agreed). This agreement supersedes earlier agreements.

The reason explained in R2-2402235 is that HO/CHO execution may incur PDCP and RLC re-establishment, which prohibits UE from reverting back to configuration and L2 state used in the source cell before HO/CHO attempt. Thus, some companies think that Case 1 is similar as Rel-18 discussion so that the fast recovery for Case 1 cannot be supported. Instead, the UE can only perform RRC re-establishment. However, Rel18 agreements are unclear to us. After T304 expiry, the “revert back” operation is the default behaviour at the UE side. This is needed for the UE to perform RRC re-establishment procedure. Thus, the UE can revert back to the source cell configuration (including security key) after inter-CU LTM cell switch failure. Such “revert back” operation does not contain L2 state due to the PDCP/RLC re-establishment during the inter-CU LTM cell switch. However, this cannot prohibit UE from fast recovery using the LTM candidate configuration, i.e., the UE can use the security key of source cell to perform the fast recovery (Type2: fast recovery without security key update).

Proposal 2: RAN2 is kindly asked to clarify the correct behaviour when the UE selects the cell under the same CU as source cell after inter-CU LTM failure (e.g., performing re-establishment vs. fast recovery).

For Case 3, RAN3 has made the following agreements:

The new source gNB needs to provide the new KgNB* to candidate gNBs via LTM configuration procedure. For the initial inter-CU LTM preparation, the source gNB computes the KgNB*(s) (per candidate cell) for the candidate gNB, and forwards the {KgNB*, NCC} pair(s) to the candidate gNB via a Handover Request message as legacy. The new source gNB needs to provide the new KgNB*(s) to the corresponding candidate gNB via a LTM configuration update procedure. FFS on the procedure design either per cell or per gNB.

It can be observed that, different cells under different gNBs have different security keys. Thus, the fast recovery without security key update cannot be supported.

Proposal 3: the fast recovery without security key update cannot be supported for the case of selecting a cell under different CU from source/target cell.

- Timing of “reverting back” operation

After the failure, the UE has to perform the “revert back” operation. The original intention is to allow the UE perform the RRC re-establishment procedure by using the configuration in source cell. In last meeting, RAN2 agreed that the UE can perform the fast recovery for the above Case 2. The intention is to support fast recovery without security key change, and the UE needn’t perform “revert back” if the selected cell has the same CU as the target cell. Thus, the UE can determine the “revert back” operation when a cell is selected. In this sense, we can have two options for the timing of “revert back” operation:

  - Option 1: “revert back” right after T304 expiry (legacy)

  - Option 2: determine “revert back” when a cell is selected

In order to support the above Case 2, i.e., Cell C is an LTM candidate cell under same CU as the Cell B (target cell). The UE needn’t perform “revert back” right after T304 expiry; then, if the Cell C is under the same CU as Cell B, the UE can perform the fast recovery without security key update. However, if the Cell C is under different CU as Cell B or the Cell is not a LTM candidate cell, the UE has to perform the re-establishment procedure, which requires the UE revert back to the source configuration. Thus, in order to cover the case that the selected Cell C is under different CU as cell B or it is not an LTM candidate cell, the UE has to perform the revert back operation after selecting a cell, which is option 2.

Proposal 4: RAN2 is kindly asked to discuss the timing of “reverting back” operation after inter-CU LTM failure by considering the following two options:

- Option 1: “revert back” after T304 expiry (legacy)

- Option 2: determine “revert back” when a cell is selected, i.e., skip “revert back” if the selected cell is under the same CU as the target cell, otherwise, “revert back” to the source cell.

- Behaviour after the LTM recovery failure

During the LTM recovery procedure, the UE may encounter the failure due to T304 expiry. In legacy, the UE has to revert back to source cell configuration. Such source cell should be the one triggering the LTM cell switch. The reason is that the UE needs to use the configuration of such source cell for RRC reestablishment.

Proposal 5: RAN2 is kindly asked to agree that after the fast recovery failure, the UE should revert back to the configuration of the source cell triggering the LTM cell switch.

- Whether to apply new LTM cell switch MAC CE for intra CU LTM (or case where key refresh is not needed)

RAN2 agreed to introducing a new R19 LTM cell switch command MAC CE with indicating the NCC value for R19 inter-CU LTM cell switch. This means that this new MAC CE is used for R19 inter-CU LTM cell switch and NCC is always included. Legacy MAC CE is used for the case security key refresh is not needed.

However, the draft CR defines a new MAC CE where the NCC is optional. In order to determine whether NCC is present or not in this new MAC CE:

- UE first checks the Target Configuration ID. UE identifies the target based on this field. If the R19 ID of this target is same as R19 ID of source cell, UE assumes that NCC is not present in this new MAC CE.

Having NCC as optional in new MAC CE makes processing of new MAC CE more complex at UE. MAC needs to look into RRC configuration (for R19 set ID check) after processing Target Configuration ID to further parse the MAC CE.

In our view, it is simple to use legacy MAC CE for case where security key refresh is not needed (e.g. intra CU case) and use new MAC CE for case where security key refresh is needed (e.g. inter CU case).

Proposal 6: Legacy cell switch command MAC CE is used for case where security key refresh is not needed (e.g. intra CU case). New cell switch command MAC CE is used for case where security key refresh is needed (e.g. inter CU case).

Conclusion

In this contribution, we address the fast recovery, and have the following proposals:

Proposal 1: RAN2 is kindly asked to agree to support the fast recovery without security key change.

Proposal 2: RAN2 is kindly asked to clarify the correct behaviour when the UE selects the cell under the same CU as source cell after inter-CU LTM failure (e.g., performing re-establishment vs. fast recovery).

Proposal 3: the fast recovery without security key update cannot be supported for the case of selecting a cell under different CU from source/target cell.

Proposal 4: RAN2 is kindly asked to discuss the timing of “reverting back” operation after inter-CU LTM failure by considering the following two options:

- Option 1: “revert back” after T304 expiry (legacy)

- Option 2: determine “revert back” when a cell is selected, i.e., skip “revert back” if the selected cell is under the same CU as the target cell, otherwise, “revert back” to the source cell.

Proposal 5: RAN2 is kindly asked to agree that after the fast recovery failure, the UE should revert back to the configuration of the source cell triggering the LTM cell switch.

Proposal 6: Legacy cell switch command MAC CE is used for case where security key refresh is not needed (e.g. intra CU case). New cell switch command MAC CE is used for case where security key refresh is needed (e.g. inter CU case).

References