3GPP TSG-RAN WG2 Meeting #130                                 R2-2504545

St.Julians, Malta, May 19th- 23th, 2025

Agenda item:		8.13.1

Source:		Samsung

Title:			Discussion on Capabilities of Multi-hop SL relay

Document for:	Discussion & Decision

- 1 Introduction

In this contribution, we will address the UE capabilities for multi-hop SL relay.

- 2 Discussion

Issue 1: basic operation of Rel-19 remote UE

In Rel-17, we already have the capability of L2 remote UE operation. The question is whether we need define a new capability for remote UE in multi-hop SL relay. So far, the remote UE reuses the same Rel-17 functions. It seems that there is no need to define a separate capability for Rel-19 remote UE.

Proposal 1: RAN2 is kindly asked to discuss whether to define a new capability for Rel-19 remote UE, e.g., remoteUE-MH-Operation-L2.

Issue 2: basic operation of first/intermediate(non-first)/last relay UE

In multi-hop SL relay, three types of relay UEs are introduced, i.e., first relay UE, intermediate (non-first) relay UE, last relay UE, where first relay UE can be considered as a special type of intermediate relay UE. However, from capability point of view, those relay UEs may have different capabilities:

- First relay UE: it can only allow the access of remote UE. If we allow the UE being act as first relay UE only, its child UE cannot be accessed by other UEs.

- Intermediate (non-first) relay UE: it can access to the parent relay UE via PC5; meanwhile, it can be accessed by other relay UEs.

- Last relay UE: it can be accessed by other relay UEs and remote UE; meanwhile, it can access gNB via Uu interface.

Considering the difference among three types of relay UEs, it is beneficial to allow separate capabilities.

Proposal 2: RAN2 is kindly asked to agree three separate capability for first/intermediate (non-first)/last relay UEs, e.g., firstRelay-MH-Operation-L2, nonfirstRelay-MH-Operation-L2, and lastRelay-MH-Operation-L2.

Meanwhile, the first/intermediate relay UE should be act as remote UE first before being accessed by other UEs. Thus, the remote UE capability should be prerequisite. Similarly, the first relay UE capability should be the prerequisite of the capability of intermediate (non-first) relay UE.

Proposal 3: RAN2 is kindly asked to agree that the remote UE capability is the prerequisite of the capability of first/intermediate (non-first) relay UE.

Proposal 4: RAN2 is kindly asked to agree that the first relay UE capability is the prerequisite of the capability of intermediate (non-first) relay UE.

Issue 3: band combination

In Rel-17, we have the supportedbandcombListPerBC-SL-RelayDiscovery which indicates, for a particular Uu band combination, the PC5 Relay discovery on which the UE supports simultaneous transmission/reception of PC5 data and Uu uplink/downlink respectively. Rel-19 may need a separate capability for multi-hop SL relay discovery.

Proposal 5: RAN2 is kindly asked to agree the new band combination capability for multi-hop SL relay discovery, e.g., supportedBandCombListPerBS-MH-SL-RelayDiscovery.

Issue 4: path switch capability

In Rel-19, the following two cases are supported for service continuity:

- Intra-gNB multi-hop indirect to direct path switching using existing framework

- Intra-gNB multi-hop indirect to single-hop indirect path switching using existing framework

The above two cases are different from Rel-17 path switch. It is better to define new capability for Rel-19.

Proposal 6: RAN2 is kindly asked to agree that capability for Rel-19 path switch, e.g., remoteUE-MH-PathSwitchOperation-L2-r18, which indicates the support of intra-gNB multi-hop indirect to direct path switching and intra-gNB multi-hop indirect to single-hop indirect path switching.

Issue 5: L3 relay capability

Although this WI is related to L2 relay, we also have discussion on L3 relay. So, we need decide whether to have the corresponding capabilities for L3 remote/first relay/intermediate relay(non-first)/last relay UE.

Proposal 7: RAN2 is kindly asked to discuss whether to define new capabilities for multi-hop L3 relay w.r.t L3 remote/first relay/intermediate relay(non-first)/last relay UE.

Issue 6: additional capability

RAN2 is still working on the scheme of fast RRC setup. Such scheme can be considered as the optional feature. If such feature is finally agreed, a capability w.r.t. this feature may be needed.

Proposal 8: RAN2 is kindly asked to discuss if the fast RRC setup scheme is agreed, whether a new capability w.r.t such optional feature is needed or not.

- 3 Conclusion

Based on the discussion above, we have the following proposals:

Proposal 1: RAN2 is kindly asked to discuss whether to define a new capability for Rel-19 remote UE, e.g., remoteUE-MH-Operation-L2.

Proposal 2: RAN2 is kindly asked to agree three separate capability for first/intermediate (non-first)/last relay UEs, e.g., firstRelay-MH-Operation-L2, nonfirstRelay-MH-Operation-L2, and lastRelay-MH-Operation-L2.

Proposal 3: RAN2 is kindly asked to agree that the remote UE capability is the prerequisite of the capability of first/intermediate (non-first) relay UE.

Proposal 4: RAN2 is kindly asked to agree that the first relay UE capability is the prerequisite of the capability of intermediate (non-first) relay UE.

Proposal 5: RAN2 is kindly asked to agree the new band combination capability for multi-hop SL relay discovery, e.g., supportedBandCombListPerBS-MH-SL-RelayDiscovery.

Proposal 6: RAN2 is kindly asked to agree that capability for Rel-19 path switch, e.g., remoteUE-MH-PathSwitchOperation-L2-r18, which indicates the support of intra-gNB multi-hop indirect to direct path switching and intra-gNB multi-hop indirect to single-hop indirect path switching.

Proposal 7: RAN2 is kindly asked to discuss whether to define new capabilities for multi-hop L3 relay w.r.t L3 remote/first relay/intermediate relay(non-first)/last relay UE.

Proposal 8: RAN2 is kindly asked to discuss if the fast RRC setup scheme is agreed, whether a new capability w.r.t such optional feature is needed or not.