3GPP TSG RAN WG2 Meeting #130	R2-2504550

St. Julian’s, Malta, May. 19th - 23rd, 2025

Agenda Item:	8.9.2

Source:	ETRI, Korea University

Title:		Remaining issues on Store and Forward satellite operation

Document for:	Discussion and Decision

1	Introduction

In this paper, we would like to discuss remaining issues on Store and Forward focused on possible impacts to the radio interface. In the previous meeting and email discussions after the meeting, the transition time from normal mode to S&F mode is agreed to introduce as an indication in system information. However, the details such as the reuse of t-Service is still one of remaining issues. In addition, whether to introduce new assistance information of the neighbour cells for S&F operation is another remaining issue. We would like to discuss these issues and provide our views on them.

2	Discussion

2.1	Indication of Time information of transition from normal mode to S&F mode

In the previous meeting, RAN2 #129bis, RAN2 discussed the time information of transition from normal mode to S&F mode as follows.

Proposal 5: Do not introduce transition time to indicate when the current satellite operation mode will transit from normal operation to S&F operation. - Google thinks we should have such an indication redefining the time information we already agreed. Oppo agrees. - Lenovo also agrees we need such an indication - Samsung thinks that when we move to S&F there will be a system information update - Samsung thinks we should not reuse T-service for this. Nokia agrees. ZTE also - Honor thinks we need a new indication, different from T-service. - Samsung wonders what we use this for. ⇒ We introduce an indication in system information for the normal mode to S&F mode transition, at least for NAS use. FFS on the details (e.g. whether we can link this to other existing information). The information on transition time for the normal mode to S&F mode transition is sent from AS to NAS, and we inform CT1 about this in the LS

As shown above, even though RAN2 agreed to introduce the transition time from normal mode to S&F mode the details remained as FFS. In addition, there have been more discussions on the details in the post email discussion on the running RRC CR [1], which is listed as one of the open issues and three options are summarized to resolve the issue as follows.

Open issue RRC-1: How to indicate the time information for the transition from normal mode to S&F mode. ... Since the details are still FFS, the time information for the transition from normal mode to S&F mode hasn’t been captured in the RRC running CR. Based on the contributions, the following options are proposed on how the time information is indicated: Option 1: It is up to NW implementation to set the legacy t-Service as the transition time from normal mode to S&F mode. Option 2: Introduce a new indication for the transition time from normal mode to S&F mode for the S&F UEs. Option 3: Using the agreed time information in SIB31 for both directions of transition. UE determines which direction it is based on whether the S&F indication is present.

To discuss this issue, we consider the following example scenario of S&F satellite operation in a quasi-Earth-fixed cell.

Figure 1. Example S&F operation scenario in a quasi-Earth-fixed cell

The scenario presented in the figure is based on a well-known feeder link switch scenario, with only an S&F operation period is added, which is described as follows:

initially, the cell is served by satellite #1,

t1: feeder link with gateway #1 becomes unavailable,

t2: feeder link with gateway #2 becomes available, and

t3: the serving satellite stops serving the area and is changed from satellite #1 to satellite #2.

For the scenario in the presented figure, we understand that the three options can be used as shown in the table below.

Table 1. Indication of t1, t2, and t3 in the presented figure by using each option.

|  | t1 (normal S&F) | t2 (S&F normal) | t3 (stop serving the area) |
| --- | --- | --- | --- |
| Option 1 | t-Service (inidcaiton by reusing t-Service in the current spec.) | t-ModeSwitching (t-ModeSwitching appears In the running RRC CR [1]) | t-Service |
| Option 2 | a new indication | t-ModeSwitching | t-Service |
| Option 3 | t-ModeSwitching that represents normal S&F | t-ModeSwitching that represents S&F normal | t-Service |

Even though Option 1 does not need any new information, it requires modifying the description of t-Service in the current specifications.

With regenerative payload, the serving cell is not changed after the feeder link switch either in a quasi-Earth-fixed cell or an Earth-moving cell. After the feeder link switch in Figure 1, the serving cell is not changed because S&F operation is defined for regenerative payload, specifically, payload with split MME or full CN.

In the current spec, t-Service is used to indicate the time information on when a cell provided via NTN is going to stop serving the area it is currently covering.

If t-Service set to t1 in the figure according to Option 1, the usage of t-Service does not agree with the current spec. because the serving cell is not changed and does not stop serving the area at t1. Note that the cell stops serving the area and changed to another cell served by Satellite 2 at t3.

Therefore, to adopt Option 1, the description of t-Service in the current specifications need to be modified to accommodate indication of the transition time from normal mode to S&F mode.

Option 2 is the clearest choice we can make if we accept the burden of introducing addition system information.

Option 3 does not require any new information. The new parameter of t-ModeSwitching appears in the running RRC CR [1] and it is an implementation of the already agreed time information on transition from S&F mode to normal mode. If we utilize the already agreed S&F mode indication, we agree that t-ModeSwitching can be used to indicate of S&F mode change of either direction. For example, if S&F mode indication is absent and t-ModeSwitching is present, then t-ModeSwitching indicates the time for the transition from normal mode to S&F mode.

To indicate the time information of transition from normal mode to S&F mode, RAN2 to choose either Option 2 (a new parameter) or Option 3 (t-ModeSwitching + absence of S&F mode indication).

2.2	Additional assistance information for the neighbour cells

Another remaining issue relevant to S&F satellite operation is whether to introduce new assistance information of the neighbour cells for S&F operation. The issue has been discussed in the post email discussion on the running RRC CR [1] and described as follows.

RRC-2: Whether to introduce additional assistance information for the neighbour cells, e.g., the operation mode and/or mode transmission time. Issue description: Some companies believe it is beneficial to let the UE know about the operation mode/ mode transmission time of the upcoming neighbour cells.

We believe introduction of additional information for the neighbour cells served by a satellite operating in S&F mode is beneficial for S&F-capable UEs.

For example, an S&F-capable UE has no reason to (re-)select a cell served by a satellite operating in S&F mode in most of cases. However, if there is no available satellite operating in normal mode, (re-)selection of a cell cells served by a satellite operating in S&F mode can be a better choice rather than keep trying to find a cell served by a satellite in normal mode.

Consequently, additional information that may enable S&F-capable UEs to (re-)select a neighbour cells served by a satellite operating in S&F mode needs to be introduced.

Additional assistance information for the neighbour cells served by a satellite operating in S&F mode is introduced.

If the addition Proposal 2	Additional assistance information for the neighbour cells operating in S&F mode is introduced, then the neighbour cell information having the additional assistance information need to be invisible for the UEs that do not support S&F operation. That is, the UEs that do not support S&F operation do not need to have information of the neighbour cells operating in S&F mode. Otherwise, those UEs may perform measurement for the neighbour cells operating in S&F mode, which is useless measurement and results in unnecessary power dissipation.

Information of the neighbour cells served by a satellite operating in S&F mode need to be delivered to only S&F-capable UEs.

Currently, t-ServiceStartNeigh can be optionally broadcast as neighbour satellite information in SIB33. For the viewpoint of S&F operation context, t-ServiceStartNeigh can be regarded as the time information related to the satellite always operating in normal mode.

For neighbour cell served by a satellite operating in S&F mode, a new time indication that plays a similar role to t-ServiceStartNeigh is required. That is, the new time indication is the earliest time when the area covered by the current serving cell is going to be covered by the neighbour cell served by the satellite operating in S&F mode. The new time indication is not decodable for the UEs other than S&F-capable UEs, which can avoid unnecessary measurement mentioned earlier.

A new time indication of the earliest time when the area covered by the current serving cell is going to be covered by the neighbour cell served by the satellite operating in S&F mode is introduced.

3	Conclusion

In this paper, the following observations are made:

Observation 1	Information of the neighbour cells served by a satellite operating in S&F mode need to be delivered to only S&F-capable UEs.

In this paper, we propose the followings:

Proposal 1	To indicate the time information of transition from normal mode to S&F mode, RAN2 to choose either Option 2 (a new parameter) or Option 3 (t-ModeSwitching + absence of S&F mode indication).

Proposal 2	Additional assistance information for the neighbour cells served by a satellite operating in S&F mode is introduced.

Proposal 3	A new time indication of the earliest time when the area covered by the current serving cell is going to be covered by the neighbour cell served by the satellite operating in S&F mode is introduced.

References

[Post129bis][310][R19 IoT NTN] RRC CR (Huawei)