3GPP TSG-RAN WG2 Meeting #130	R2-2504551

St.Julians, Malta,  May 19th – 23rd , 2025

Agenda Item:	7.0.2.1

Source:	NEC

Title:	Possible RAN1 impact on RACH-less HO in TN

Document for:  Discussion and Decision

1	Introduction

RACH-less HO is supported in NTN, mobile IAB and generalized TN cases. Some agreements on RACH-less HO have been informed to RAN1, while some information seemed missing, which may impact on RAN1 specification. In this contribution, we discuss the possible RAN1 specification impact on supporting the RACH-less HO and propose to send an LS to RAN1.

2	Discussion

First of all, RACH-less handover is supported in NTN and TN (including mobile IAB) in Rel-18, and the CR R2-2503167 is principle agreed in RAN2#129bis meeting to clarify the use case for RACH-less HO.

RAN2#129bis: R2-2503167 Correction on RACH-less HO Use Case vivo, Nokia, Nokia Shanghai Bell, Ericsson, Samsung CR Rel-18 38.300 18.5.0 0904 3 F NR_NTN_enh-Core, NR_mobile_IAB-Core, TEI18 - The CR is in principle agreed #### 9.2.3.6 RACH-less handover RACH-less handover is supported for an intra-gNB handover procedure in TN and NTN. RACH-less handover is also supported for an inter-gNB handover procedure in the case of NTN or mobile IAB-DU migration procedure. The RACH-less handover procedure applies the following functionality:.

Observation 1: RACH-less HO is supported for an intra-gNB HO procedure in TN and NTN. RACH-less HO is also supported for an inter-gNB HO procedure in the case on NTN or mobile IAB.

During the previous RAN2 meeting, RAN2 sent the LS (R2-2304271) to RAN1 for RACH-less HO in NTN, which includes the CG-based RACH-less HO and the DG-based RACH-less HO for NTN. Then, RAN2 received the reply LS (R2-2307011) from RAN1.

In addition, RAN2 also sent the LS (R2-2403996) to RAN1 on parameters used for CG-based RACH-less HO in general (i.e., NTN, mobile IAB and TN generic). Then, RAN2 received the reply LS (R2-2405997).

After checking the history of information exchanges between RAN2 and RAN1 for RACH-less HO, there was no LS on DG-based RACH-less HO for TN (including mobile IAB).

Observation 2: There were two LS to RAN1 for RACH-less HO, i.e., LS on RACH-less HO for NTN and LS on parameters used for CG-based RACH-less HO, but there was no LS on DG-based RACH-less HO in TN.

For DG-based RACH-less HO, the ssb-index may be configured in NTN cell or cell is not a mobile IAB cell, and the tci-stateID may be configured in case the cell is not a NTN cell, which is used to indicate a beam that the UE should use in the target cell to monitor PDCCH for initial uplink transmission, which is specified in TS 38.331.

CellGroupConfig information element -- ASN1START -- TAG-CELLGROUPCONFIG-START -- Configuration of one Cell-Group: CellGroupConfig ::= SEQUENCE { … … RACH-LessHO-r18 ::= SEQUENCE { targetNTA-r18 ENUMERATED {zero, source} OPTIONAL, -- Need N beamIndication-r18 CHOICE { tci-StateID-r18 TCI-StateId, ssb-Index-r18 SSB-Index } OPTIONAL, -- Need N ... } RACH-LessHO field descriptions. ssb-Index This field indicates a beam that the UE should use in the target cell to monitor PDCCH for initial uplink transmission, see TS 38.321 [3]. The network configures this field when cg-RRC-Configuration is not configured for the initial uplink transmission in RACH-less handover in NTN or in case this cell is not a mobile IAB cell.. … …. tci-StateID This field indicates a beam that the UE should use in the target cell to monitor PDCCH for initial uplink transmission and also indicates the TCI state information to be used in the target cell. The network configures this field in case this cell is not a NTN cell..

In RAN1 specification (TS 38.213), DG-based RACH-less HO is specified as following. However, only the ssb-index related description used for RACH-less HO in NTN is included, DG-based RACH-less HO for TN (e.g., tci-StateID in RACH-lessHO) is not specified in RAN1 specification.

TS 38.213 22.2 Dynamic-grant PUSCH transmission If ssb-Index is provided in RACH-LessHO, the UE may assume that the DM-RS antenna port associated with the PDCCH receptions for scheduling initial PUSCH transmission and the SS/PBCH block indicated by ssb-Index are quasi co-located with respect to average gain and quasi co-location 'typeA' or 'typeD' properties.

Observation 3: Only DG-based RACH-less HO for NTN is specified in RAN1 specification. DG-based RACH-less HO for TN may need some changes in RAN1 specification.

Based on the above observations, RAN1 captures RACH-less HO in NTN based on RAN2's LS. But RAN2 did not send LS to RAN1 on supporting RACH-less HO in TN, so RAN1 is unaware of the need to capture anything for DG-based RACH-less HO in TN, resulting RAN1 and RAN2 specifications are not consistent to support RACH-less HO in TN.

Actually, we also raised this issue in RAN1 at this meeting, but we are not sure if RAN1 needs to wait for RAN2 LS to trigger the discussion. If RAN1 thinks the LS is needed, then RAN2 should send LS to RAN1 to trigger the discussion.

Therefore, we propose to send an LS to RAN1 on RACH-less HO for TN to trigger corresponding discussion in RAN1 if RAN1 did not capture anything for RACH-less HO in TN (including mobile IAB).

Proposal 1: RAN2 send LS to RAN1 on RACH-less HO for TN to trigger corresponding discussion in RAN1, if RAN1 did not capture anything for RACH-less HO in TN during the May meeting.

# 4	Conclusion

In this paper, there are the following observations and proposals.

Observation 1: RACH-less HO is supported for an intra-gNB HO procedure in TN and NTN. RACH-less HO is also supported for an inter-gNB HO procedure in the case on NTN or mobile IAB.

Observation 2: There were two LS to RAN1 for RACH-less HO, i.e., LS on RACH-less HO for NTN and LS on parameters used for CG-based RACH-less HO, but there was no LS on DG-based RACH-less HO in TN.

Observation 3: Only DG-based RACH-less HO for NTN is specified in RAN1 specification. DG-based RACH-less HO for TN may need some changes in RAN1 specification.

Proposal 1: RAN2 send LS to RAN1 on RACH-less HO for TN to trigger corresponding discussion in RAN1, if RAN1 did not capture anything for RACH-less HO in TN during the May meeting.

# 5	Reference

[1] 3GPP TS 38.300: "NR; Overall description; Stage 2".

[2] 3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[3] R2-2403167, Correction on RACH-less HO Use Case

[4] R2-2304271, LS on RACH-less Handover, RAN2

[5] R2-2307011, Reply LS on RACH-less Handover, RAN1

[6] R2-2403996, LS on parameters used for CG RACH-less Handover, RAN2

[7] R2-2405997, Reply LS on parameters used for CG RACH-less Handover, RAN1

# 6. Annex

3GPP TSG RAN WG2 #130			R2-25xxxxx

St.Julians, Malta,  May 19th – 23rd , 2025

Title:	(draft) LS on RACH-less handover in TN

Response to:	-

Release:	Rel-18

Work Items:	NR_mobile_IAB -Core, TEI18

Source:	RAN WG2

To:	RAN WG1

CC:	-

Contact Person:

Name:	  TBD

E-mail Address:	TBD

Attachment:

1. Overall Description:

In Rel-18, RACH-less HO is supported for an intra-gNB HO procedure in TN and NTN. RACH-less HO is also supported for an inter-gNB HO procedure in the case on NTN or mobile IAB. RAN2 sent two LS to RAN1 for RACH-less HO (LS on RACH-less HO for NTN in R2-2304271 and LS on parameters used for CG-based RACH-less HO in R2-2403996) and received the corresponding reply LS in R2-2307011 and R2-2405997, respectively. However, RAN2 has found that there was no LS on DG-based RACH-less HO in TN.

For DG-based RACH-less HO in TN, the tci-StateID in RACH-LessHO may be configured in case the cell is TN cell, which is used to indicate a beam that the UE should use in the target cell to monitor PDCCH for initial uplink transmission, which is specified in TS 38.331.

As RAN1 may be unaware of the need to capture anything for DG-based RACH-less HO in TN, RAN2 decides to send an LS to RAN1 to ask necessary specification work on it, if any.

2. Actions:

To RAN1

ACTION:	RAN2 respectfully asks RAN1 to take the above information related to RACH-less HO into consideration for the specification work.

3. Date of Next RAN2 Meetings:

TSG-RAN WG2#131                      2025-08-25 to 2025-08-29		India, IN

TSG-RAN WG2#131bis                   2025-10-13 to 2025-10-17		Prague, CZ