3GPP TSG-RAN WG2 Meeting #130	R2-2504554

St.Julians, Malta,  May 19th – 23rd , 2025

Source:	NTT DOCOMO, INC.

Title:	Discussion on LCM for UE-sided model  for Beam Management

Agenda Item:	8.1.2.2

Document for: 	Discussion and Decisions

- Introduction

The application of AI/ML techniques to NR air interface has been studied in FS_NR_AIML_Air. The new WI has been approved in RAN #106 meeting and the objective of the WI is to provide the normative support for the general framework for AI/ML for air interface, as well as, enable the recommended use cases in the preceding studies, where following aspects are regarding beam management use case[1]:

Provide specification support for the following aspects: - AI/ML general framework for one-sided AI/ML models within the realm of what has been studied in the FS_NR_AIML_Air project [RAN2]: - Signalling and protocol aspects of Life Cycle Management (LCM) enabling functionality and model (if justified) selection, activation, deactivation, switching, fallback - Identification related signalling is part of the above objective - Necessary signalling/mechanism(s) for LCM to facilitate model training, inference, performance monitoring, data collection (except for the purpose of CN/OAM/OTT collection of UE-sided model training data) for both UE-sided and NW-sided models - Signalling mechanism of applicable functionalities/models - Beam management - DL Tx beam prediction for both UE-sided model and NW-sided model, encompassing [RAN1/RAN2]: - Spatial-domain DL Tx beam prediction for Set A of beams based on measurement results of Set B of beams (“BM-Case1”) - Temporal DL Tx beam prediction for Set A of beams based on the historic measurement results of Set B of beams (“BM-Case2”) - Specify necessary signalling/mechanism(s) to facilitate LCM operations specific to the Beam Management use cases, if any - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE NOTE: Strive for common framework design to support both BM-Case1 and BM-Case2

In this contribution, the issues regarding the LCM for UE-side model for beam management use case will be discussed.

- Overall procedure of applicability reporting and inference configuration

According to the agreements made in RAN1 and RAN2, the overall procedure of applicability reporting and inference configuration could be summarized in Fig.1.

Fig.1 Overall procedure of applicability reporting and inference configuration

This contribution focuses on the applicability determination and reporting, functionality activation/deactivation by considering following agreements made in RAN1 #119 meeting and RAN2 #128,#129,129bis meetings [2]-[5].

Agreement in RAN1 #119 - In Step 3, following configurations are provided from NW to UE: - UE is allowed to do UAI reporting via OtherConfig, - The applicability report is based on A) and/or B) - It is up to RAN 2 to design the container - A) one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied) - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3 - B) One set or multiple sets of inference related parameters for applicability report only (not for inference) - It is up to RAN2 to design the container. - The set of inference related parameters selected from the IEs in/or the IEs referred by CSI-ReportConfig as a starting point, e.g., - the associated ID - Note: this doesn’t imply the associated ID is mandatory - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction Conclusion For the CSI-ReportConfig for inference configuration provided in Step 5, - aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after RRCReconfigurationComplete. - periodic CSI Report is considered as activated after RRCReconfigurationComplete. - Note: UE is not expected to be configured with a CSI-ReportConfig for inference configuration for a non-applicable set of inference parameters or a non-applicable CSI-ReportConfig - Any specification impact is a separate discussion

Agreements on option B in RAN2 #129bis 1 RAN2 assumes UE receives RRCReconfiguration message including one set or multiple sets of inference related parameters via OtherConfig for option B. This assumption can be confirmed (i.e., whether to reconsider CSI-ReportConfig) after receiving Option B inference related parameters (e.g., in RAN1 RRC parameters list). Potential aspects to consider if RAN2 revisit: - To reconsider CSI-ReportConfig for option B, for example, if the list of inference related parameters is fully contained within existing CSI-ReportConfig. - to take into accounts UE behaviour when confirming the assumption e.g., whether option A and option B result in different UE behavior

Agreements on deactivation in RAN2 #128: - When a functionality becomes non-applicable the UE doesn’t autonomously deactivate. NW is expected to deactivate active functionality when it receives report from UE that it is non-applicable. Agreements on activation in RAN2 #129: - If option A is configured in Step 3, for periodic CSI reporting, the UE autonomously activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 (i.e. without need to wait RRCReconfiguration in Step 5). 3. Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability. Agreements on Option A in RAN2 #129b: - Upon receiving one or more full inference configuration(s) via RRCReconfiguration message, UE shall maintain all the full inference configuration(s) no matter the full inference configuration is applicable or non-applicable until the network releases it explicitly.

- Applicability determination and reporting

According to the RAN1 agreement and LS reply to RAN2, the associated ID/NW-side additional condition may not be mandatory in Step 3. Therefore,  how the UE decides the applicability of the functionality without associated ID needs to be solved. Although this issue is still under discussion in RAN 1, RAN 2 could make the progress from signaling and procedure point of view. In addition, since both Option A and Option B are supported, the corresponding UE behavior for each option should also be clarified.

  - Determination and reporting without NW-side additional condition in Step 3

As shown in Fig.1, as long as UE gets the inference configuration(Option A) or inference related parameters(Option B), it may conduct the applicability determination based on following information:

- Model availability in device.

- UE-side additional condition(internally known by UE)

- NW-side additional condition(if provided)

If the NW-side additional condition is not available, there are following two alternatives to choose:

Alt.1: UE assumes the previous/historical configured associated ID is supported.

Alt.2: UE assumes all the candidate associated IDs are supported.

The benefit of Alt.1 is UE could get a unique decision for each of the inference configurations/inference related parameters. However, there is possibility that NW changes its configuration, e.g. beam shaping or beam pattern, which makes the assumed NW-side additional condition outdated and applicability reporting invalid. Alt.2 could avoid this problem but it cannot provide the unique applicability decision since the NW-side additional condition in practice is not available. For both of the alternatives, the safer solution is to make UE report all the candidate associated IDs associated with each of the applicable or non-applicable inference configurations/inference related parameters and NW makes the final decision. As shown in Fig.2, using Alt.2 as an example, the UE assumes all the candidate associated IDs are supported, and identifies inference configuration #1/inference related parameter #1 is applicable while inference configuration #2/inference related parameter #2 is non-applicable. In Step 4, besides the explicit reporting of applicability/inapplicability, the UE may report the supported associated IDs for each configuration, i.e. associated ID#1and #3 for inference configuration #1/inference related parameter #1, and associated ID #2 and #3 for inference configuration #2/inference related parameter #2.

Fig.2 Reporting supported associated IDs in Step 4

Proposal 1: If the associated ID is unavailable in Step 3, the UE report all the candidate associated IDs associated with each of the applicable or non-applicable inference configurations/inference related parameters in Step 4.

  - UE behavior on applicability determination and reporting for Option A and Option B

As explained in Section 3.1, the UE may determine the applicability of the inference configurations(i.e. CSI-ReportConfig) for Option A or determine the applicability of one or multiple sets of inference related parameters(i.e. OtherConfig) for Option B.

According to the agreements, “the set of inference related parameters selected from the IEs in/or the IEs referred by CSI-ReportConfig”for Option B and the detailed RRC parameter list are still under discussion. Therefore, whether the UE behavior on determining the applicability for Option A and Option B depends on the designing of RRC parameters.

In addition, after the applicability determination, the UE behavior on reporting applicability in Step 4 may also defer for Option A and Option B. For Option A, UE may report the applicable/non-applicable for all of the configured one or more CSI-ReportConfigID(s). For Option B, a new container/ID may possibly need to be specified for UE to report the applicable/non-applicable for all of the configured one or more set(s) of inference related parameters, depending on the RRC parameter design as well.

However, from operator point of view, it is preferred that the UE behavior on determining the applicability is the same for both Option A and Option B since it is easier to trace back the reason of the possible problem when happened, e.g. RLF or HOF.

Proposal 2: The identical UE behavior on applicability determination and reporting is preferred for both Option A and Option B.

- Functionality activation/deactivation

For both Option A and Option B, after receiving the one or more full inference configurations in Step 3(Option A) or Step 5(Option B), the following UE behaviors are the same:

- Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete.

- UE shall maintain all the full inference configuration(s) no matter the full inference configuration is applicable or non-applicable until the network releases it explicitly.

- UE activates the applicable functionalities autonomously for periodic CSI reporting or by legacy CSI framework for semi-persistent reporting or aperiodic CSI reporting.

- When a functionality becomes non-applicable the UE doesn’t autonomously deactivate. NW is expected to deactivate active functionality when it receives report from UE that it is non-applicable.

- UAI can be sent to update applicability.

Observation 1: The UE behaviors on activation/deactivation for applicable functionalities configured in Step 3 for Option A and in Step 5 for Option B are the same.

However, for Option A, since the one or more full inference configurations configured in Step 3 could also be non-applicable, the UE behaviors need to be clarified especially when a functionality becomes applicable from non-applicable afterwards. There are two optional UE behaviors when one or multiple full inference configuration(s) become applicable:

Option 1(UE-based):

- UE shall maintain all the full inference configuration(s) no matter the full inference configuration is applicable or non-applicable until the network releases it explicitly.

- For periodic CSI reporting, the UE autonomously activate the applicable functionalities.

- For semi-persistent and aperiodic CSI reporting,

  - UAI can be sent to update applicability.

  - UE activates the applicable functionalities by legacy CSI framework.

Option 2(NW-based):

- UE shall maintain all the full inference configuration(s) no matter the full inference configuration is applicable or non-applicable until the network releases it explicitly.

- UAI can be sent to update applicability.

- UE may receive the confirmation/(re-)configuration corresponding to the updated functionality.

- Upon receiving the confirmation/(re-)configuration,

  - For periodic CSI reporting, the UE autonomously activate the applicable functionalities.

  - For semi-persistent and aperiodic CSI reporting, UE activates the applicable functionalities by legacy CSI framework.

Both Option 1 and Option 2 have their own pros and cons. Option 1 does not need extra signaling and has less latency for functionality activation. However, NW needs to keep the CSI resources valid even if the corresponding inference configuration(s) will not become applicable or will become applicable after a long period. On the contrary, Option 2 provides NW more freedom to release the CSI resources for the inference configuration(s) which are reported non-applicable, but it needs extra signalling and latency to synchronize the applicability even if NW has not released the CSI resources for the non-applicable inference configuration(s).

Observation 2: The UE behaviors on activation/deactivation for non-applicable functionalities configured in Step 3 for Option A need to be clarified.

Observation 3: Both Option 1(UE-based) and Option 2(NW-based) have their own pros and cons considering the NW freedom on CSI resources maintenance, signaling overhead and activation latency.

In order to exploit the benefits of Option 1 and Option 2, a timer, namely Holding Timer could be introduced in Step 3 for each of the configured inference configurations. Upon receiving the inference configurations in Step 3, UE may decide the behavior based on the expiry status of Holding Timer for each of the inference configurations, as shown in Fig.3.

- If the Holding Timer(s) do not expire, UE may follow the behaviors in Option 1(UE-based).

- If the Holding Timer(s) expire, UE may follow the behaviors in Option 2(NW-based).

Fig.3 Holding Timer for non-applicable functionalities

Proposal 2 A timer(e.g. Holding Timer) is introduced in RRCReconfiguration in Step 3 for Option A, and UE may decide the behavior based on the expiry status of Holding Timer for each of the inference configurations.

-	If the Holding Timer(s) do not expire, UE may follow the behaviors in Option 1(UE-based).

-	If the Holding Timer(s) expire, UE may follow the behaviors in Option 2(NW-based).

- Conclusion

In this contribution, the applicability determination and reporting, functionality activation/deactivation were discussed. The observations and proposals are summarized as follows:

- For determination and reporting without NW-side additional condition in Step 3:

Proposal 1: If the associated ID is unavailable in Step 3, the UE report all the candidate associated IDs associated with each of the applicable or non-applicable inference configurations/inference related parameters in Step 4.

- For UE behavior on determination and reporting for Option A and Option B:

Proposal 2: The identical UE behavior on applicability determination and reporting is preferred for both Option A and Option B.

- For functionality activation/deactivation:

Observation 1: The UE behaviors on activation/deactivation for applicable functionalities configured in Step 3 for Option A and in Step 5 for Option B are the same.

Observation 2: The UE behaviors on activation/deactivation for non-applicable functionalities configured in Step 3 for Option A need to be clarified.

Observation 3: Both Option 1(UE-based) and Option 2(NW-based) have their own pros and cons considering the NW freedom on CSI resources maintenance, signaling overhead and activation latency.

Proposal 2 A timer(e.g. Holding Timer) is introduced in RRCReconfiguration in Step 3 for Option A, and UE may decide the behavior based on the expiry status of Holding Timer for each of the inference configurations.

-	If the Holding Timer(s) do not expire, UE may follow the behaviors in Option 1(UE-based).

- UE shall maintain all the full inference configuration(s) no matter the full inference configuration is applicable or non-applicable until the network releases it explicitly.

- For periodic CSI reporting, the UE autonomously activate the applicable functionalities.

- For semi-persistent and aperiodic CSI reporting,

  - UAI can be sent to update applicability.

  - UE activates the applicable functionalities by legacy CSI framework.

-	If the Holding Timer(s) expire, UE may follow the behaviors in Option 2(NW-based).

- UE shall maintain all the full inference configuration(s) no matter the full inference configuration is applicable or non-applicable until the network releases it explicitly.

- UAI can be sent to update applicability.

- UE may receive the confirmation/(re-)configuration corresponding to the updated functionality.

- Upon receiving the confirmation/(re-)configuration,

  - For periodic CSI reporting, the UE autonomously activate the applicable functionalities.

  - For semi-persistent and aperiodic CSI reporting, UE activates the applicable functionalities by legacy CSI framework.

References

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, RAN #102, Dec. 2023

- RAN1 Chair (Samsung), “RAN1 #119 Chair Notes”, Nov, 2024.

- RAN2 Chair (InterDigital), “RAN2 #128 Chair Notes”, Nov, 2024.

- RAN2 Chair (InterDigital), “RAN2 #129 Chair Notes”, Feb, 2025.

- RAN2 Chair (InterDigital), “RAN2 #129bis Chair Notes”, Apr, 2025.