3GPP TSG-RAN WG2 Meeting #130	R2-2504560

St.Julians, Malta,  May 19th – 23rd , 2025

Source:	Qualcomm Incorporated

Title:	Discussion on notification when the intermediate relay UE performs relay reselection

Agenda Item:	8.13.2

Document for:	Discussion and Decision

# Introduction & Background

In the WID [1], multi-hop U2N relay discovery and Relay (re)selection needs to discuss. This contribution discusses the related aspects.

- Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay. The work starts with one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB). A necessary criterion for the specified mechanisms is easy extensibility to support two additional hop relays for the work done until RAN#107 and to be forward compatible for future extensions for additional relays.

  - Relay discovery and (re)selection [RAN2]

  - Signalling support for relay UEs and remote UE authorization if SA2 concludes it is needed [RAN3]

  - Impact on SRAP and QoS handling for multi-hop [RAN2]

  - Control plane procedures [RAN2, RAN3]

# Discussion

## 2.4 Relay (Re)selection

In the last RAN2 meeting, there are FFS issues on intermediate relay UE reselection. The main issue is whether the intermediate relay UE needs to notify the child UE when performing relay reselection.

Agreements:

When the parent UE in idle/inactive performs reselection, RAN2 assume that an indication of the reselection is needed for the child UE.

FFS detailed information in the indication and child UE handling.

FFS if there are exceptional cases where the indication can be suppressed, e.g., reselection under the same serving cell without changing the hop count.

FFS if the notification message is used or we rely on upper layer signalling (e.g., discovery).

In SA2, there are some discussions on how the intermediate relay UE behaves when performing relay reselection.

--------------------------------------------------------TS 23.304----------------------------------------------------------------

6.4.3.8.3	Relay reselection and mobility support  (Model A discovery)

The relay reselection procedure can be used by a 5G ProSe Intermediate UE-to-Network Relay, taking the role of the 5G ProSe Remote UE in the procedure, when its connection with a parent Relay (either a 5G ProSe Intermediate UE-to-Network Relay or a 5G ProSe UE-to-Network Relay) is lost or the link quality drops below a configured threshold.

If the 5G ProSe Intermediate UE-to-Network Relay can not complete the reselection, e.g. it is not able to discover the 5G ProSe UE-to-Network Relay, or the 5G ProSe Intermediate UE-to-Network Relay determines to not perform the reselection procedure, it may send re-selection indication and optionally with a cause value to 5G ProSe Remote UE via the established multi-hop relay path. After receving the re-selection indication the 5G ProSe Remote UE may trigger the reselection of the relays as described above.

During the transmition of re-selection indication to 5G ProSe Remote UE, after receving the re-selection indication a 5G ProSe Intermediate UE-to-Network Relay in the path based on configuration may determine to relay the re-selection indication to 5G ProSe Remote UE, or perform relay reselection by itself as described above.

When the 5G ProSe Intermediate Relay-1 has lost the connection or has an imminent connection loss/deteriorated connection with its parent 5G ProSe Intermediate Relay or the 5G ProSe UE-to-Network Relay, the 5G ProSe Intermediate Relay-1 performs relay reselection by using the relay discovery procedure in clasue 6.3.2.5. For Layer-3 UE-to-Network relay, if the 5G ProSe Intermediate Relay-1 decides a new 5G ProSe UE-to-Network Relay is used after the relay reselection, IP address/prefix changes may happen. In this case, the Intermediate UE-to-Network Relay-1 notifies its child relay(s) and/or Remote UE(s) about the change of Root 5G ProSe UE-to-Network Relay before sending Remote User ID and Remote UE Info to its parent Relay. The 5G ProSe Intermediate Relay-1 sends Link Modification Request message to its child relay(s) and Remote UE(s) including an indication that the Root UE-to-Network Relay is changed. The nofitication triggers the Remote UE to request for a new IP address/prefix from the new Root 5G ProSe UE-to-Network Relay.

##### 6.4.3.9.3	Relay reselection (Model B discovery)

When a 5G ProSe Intermediate UE-to-Network Relay detects the link failure or link quality degradation, the 5G ProSe Intermediate UE-to-Network Relay sends to the 5G ProSe Remote UE a Relay re-selection indication with the cause (e.g. the link failure or link quality degradation of the PC5 link) so that the 5G ProSe Remote UE may initiate multihop UE-to-Network Relay reselection procedure.

From SA2 specification, it can be observed that SA2 already discussed and agreed how to handle the case that intermediate relay UE perform relay reselection for both of model A and model B discovery. For model A discovery, the intermediate relay UE can perform relay reselection when the PC5 link is lost of link quality is below a threshold, and if it cannot complete the relay reselection, it may send re-selection indication and optionally with a cause value to 5G ProSe Remote UE via the established multi-hop relay path. For model B discovery, when the intermediate relay UE detects link failure or link quality degradation, the intermediate relay UE sends reselection indication to the child UE.

Observation 1: For Model A discovery, SA2 agreed that the intermediate relay UE can perform relay reselection when the PC5 link is lost or link quality is below a threshold, and if it cannot complete the relay reselection, it may send re-selection indication and optionally with a cause value to the Remote UE via the established multi-hop relay path.

Observation 2: For Model B discovery, SA2 agreed that the intermediate relay UE sends reselection indication to the child UE when detecting link failure or link quality degradation.

Considering there is already upper layer indication when the intermediate Relay UE performs relay reselection, no AS layer notification to child UE is needed.

- For both Model A discovery and Model B discovery, no AS layer notification is needed to child UE when the intermediate relay UE performs relay reselection.

From the last relay UE perspective, HO, RLF or cell reselection is AS event and transparent to upper layer, then the last relay UE can notify the child UE.

When the intermediate relay UE receives notification from the parent relay UE, the intermediate relay UE could perform relay reselection acting as a Remote UE.

- When the last relay UE performs HO, RLF, cell reselection, the last relay UE can notify the child UE.

- When the intermediate relay UE receives notification from the parent relay UE, the intermediate relay UE could perform relay reselection acting as a Remote UE.

- Conclusion

Relay UE (re)selection

Observation 1: For Model A discovery, SA2 agreed that the intermediate relay UE can perform relay reselection when the PC5 link is lost or link quality is below a threshold, and if it cannot complete the relay reselection, it may send re-selection indication and optionally with a cause value to the Remote UE via the established multi-hop relay path.

Observation 2: For Model B discovery, SA2 agreed that the intermediate relay UE sends reselection indication to the child UE when detecting link failure or link quality degradation.

- For both Model A discovery and Model B discovery, no AS layer notification is needed to child UE when the intermediate relay UE performs relay reselection.

- When the last relay UE performs HO, RLF, cell reselection, the last relay UE can notify the child UE.

- When the intermediate relay UE receives notification from the parent relay UE, the intermediate relay UE could perform relay reselection acting as a Remote UE.

# Reference

[1]. RP-241609    New WID on NR sidelink multi-hop relay    LG Electronics, InterDigital, FirstNet