3GPP TSG-RAN WG2 Meeting #129bis	R2-2502456

China, Wuhan, April 7th – 11th, 2025

Source:	Qualcomm Incorporated

Title:	Discovery and Relay (re)selection for multi-hop U2N relay

Agenda Item:	8.13.2

Document for:	Discussion and Decision

# Introduction & Background

This contribution discusses service continuity in multi-hop U2N.

# Discussion

## 2.1 Remote UE performs path switching in Scenario C and Scenario D

For Scenario C and Scenario D, the important issue is what Remote UE should report to the gNB. Scenario C and Scenario D can be shown as follows.

Figure 1

Figure 2

Different from existing single-hop relay, in multi-hop relay, the Remote UE cannot evaluate PC5 link quality between different relay UEs, so it is not suitable to use an event to evaluate whether the new relay path is a candidate path or not. One possible way is to reuse relay (re)selection criteria to evaluate whether a new path is suitable path, because in both Model A and Model B discovery procedure, the link quality on each PC5 hop should be above a threshold, then the relay path can be considered as candidate relay UE for service continuity.

When the Remote UE detects a candidate new relay path using relay (re)selection criteria, and the source link quality is below a threshold e.g. existing event A2 and Z2, the Remote UE can report measurement report to the gNB.

- Remote UE sends measurement report to the gNB when the following conditions are satisfied,

- The source link quality is below a threshold, e.g. existing event A2 and Z2.

- Remote UE detects candidate multi-hop relay path(s) using relay (re)selection criteria.

In last RAN2 meeting, RAN2 confirms that the relay UEs on the candidate target paths has to be in CONNECTED state. There could be two options to achieve this.

Option 1: Include relay UE’s RRC state in discovery message, the Remote UE only selects the candidate relay UEs in CONNECTED state.

Option 2: Remote UE reports all the intermediate relay UE IDs to the gNB in measurement report, and gNB selects the target path on which all the relay UEs are in CONNECTED state.

In last RAN2 meeting, RAN2 confirms that the relay UEs on the candidate target paths has to be in CONNECTED state. There could be two options to achieve this.

Option 1: Include relay UE’s RRC state in discovery message, the Remote UE only selects the candidate relay UEs in CONNECTED state.

Option 2: Remote UE reports all the intermediate relay UE IDs to the gNB in measurement report, and gNB selects the target path on which all the relay UEs are in CONNECTED state.

Option 1 will impace upper layer relay (re)selection procedure.

Option 2, currently, for Model A discovery, the following information can be included in Discovery Announcement message: RSC, (optional) Root Relay Info, Announcer Info, Hop-Count, (optional) Accumulated QoS for PC5 link, (optional) NCGI, (optional) RRC Container, (optional) Hop-Limit. For Model B discovery, Discovery Response message includes: Type of Discovery Message, Discoveree Info and RSC, path information(a list of User Info IDs of 5G ProSe Intermediate UE-to-Network Relay(s) and the remote UE in the order). It can be observed for Model A discovery, the Remote UE cannot get the second intermediate relay info, so cannot report to the gNB.

For simplicity perspective, option 2 is preferred. But for Model A discovery, service continuity scenario C and D only support switching to two relay UEs (i.e. one intermediate relay UE and last relay UE) path.

- Remote UE includes all the relay UE info on the candidate relay path to the gNB in measurement report.

- For Model A discovery, Remote UE only treats the relay path on which there is only one intermediate Relay UE as candidate relay path to be reported to the gNB.

For both Model A and Model B discovery, the hop number should be reported to the gNB in measurement report for gNB to provide PC5 RLC channel configuration to the Remote UE during path switching.

- Remote UE include hop number of the candidate relay path into measurement report.

The Remote UE could detect multiple candidate relay paths, then Remote UE can report all of the candidate relay paths to the gNB.

Additionally, in SA2 one parameter “Accumulated QoS for PC5 link” in both Modal A and Modal B discovery message for multi-hop U2N relay, and it can reflect the supported QoS over all the PC5 links to the last Relay UE as defined in the following. Then the Remote UE can report this parameter to the gNB to assist gNB to select the right target relay path.

-	(optional) Accumulated QoS for PC5 link: this reflects the QoS supported over all the PC5 links to the Root Relay, i.e. the 5G ProSe UE-to-Network Relay, for this RSC.

- Remote UE can report multiple candidate relay paths to the gNB in measurement report.

- Remote UE can report Accumulated QoS for PC5 link on each candidate relay path to gNB in measurement report.

## 2.2 Intermediate Relay UE performs path switching

In RAN2 #129 meeting, RAN2 agreed that for multi-hop i2i, first evaluates adjacent upstream relay link and candidate relay link, where the sidelink relay measurement report shall include at least serving intermediate/ last relay UE's source L2 ID, serving cell ID and sidelink measurement quantity result.  FFS if intermediate relay UE reports.

Agreements:

For multi-hop i2d, Remote UE evaluates both relay link (for the link between remote UE and serving first relay UE) and Uu link, where the sidelink relay measurement report shall include at least serving first relay UE's source L2 ID, serving cell ID and sidelink measurement quantity result.

For multi-hop i2d, first relay UE evaluates adjacent upstream relay link and Uu link, where the sidelink relay measurement report shall include at least measured relay UE's source L2 ID, serving cell ID and sidelink measurement quantity result.  FFS if intermediate relay UE reports.

For multi-hop i2i, the remote UE reports serving first relay UE and candidate single-hop relay UE(s), including at least a source L2 ID, serving cell ID, and a sidelink measurement quantity information.

For multi-hop i2i, first evaluates adjacent upstream relay link and candidate relay link, where the sidelink relay measurement report shall include at least serving intermediate/ last relay UE's source L2 ID, serving cell ID and sidelink measurement quantity result.  FFS if intermediate relay UE reports.

If intermediate Relay UE sends measurement report to the gNB, then intermediate Relay UE could perform path switching. There could be cases that the intermediate relay UE performs path switching, and the following figures show the scenarios where service continuity may or may not be supported from Remote UE perspective.

Case 1: Intermediate relay UE performs path switching to direct path from multi-hop U2N path (example that path switching from multi-hop indirect path to single-hop indirect/direct path from Remote UE perspective)

Case 2: Intermediate relay UE performs path switching to multiple-hop U2N path from direct path

(example that hop number of the target path is larger than the source path from Remote UE perspective)

Case 3: Intermediate relay UE performs path switching to single-hop U2N path from multi-hop U2N path

(example that the Remote UE service continuity scenario is out of WID scope)

Similar with existing single-hop U2N relay, when the relay UE performs HO, the relay UE notifies the remote UE, and Remote UE goes back to IDLE. Here, in multi-hop relay, the similar principle can be reused, i.e. the intermediate Relay UE can send measurement report to the gNB for path switching, and notifies the Remote UE. When the Remote UE receives path switching notification from the relay UE, the Remote UE should go back to IDLE.

- The intermediate Relay UE can report measurement report UE to gNB for path switching to direct path or indirect path acting as a Remote.

- When the intermediate Relay UE performs path switching to direct path or indirect path, the intermediate Relay UE informs the Remote UE via the child intermediate relay UE with release cause 'RRC connection failure'.

- When the Remote UE receives notification from the relay UE, the Remote UE should go back to IDLE.

- Conclusion

Remote UE performs path switching in Scenario C and Scenario D

- Remote UE sends measurement report to the gNB when the following conditions are satisfied,

- The source link quality is below a threshold, e.g. existing event A2 and Z2.

- Remote UE detects candidate multi-hop relay path(s) using relay (re)selection criteria.

- Remote UE includes all the relay UE info on the candidate relay path to the gNB in measurement report.

- For Model A discovery, Remote UE only treats the relay path on which there is only one intermediate Relay UE as candidate relay path to be reported to the gNB.

- Remote UE includes hop number of the candidate relay path into measurement report.

- Remote UE can report multiple candidate relay paths to the gNB in measurement report.

- Remote UE can report Accumulated QoS for PC5 link on each candidate relay path to gNB in measurement report.

Intermediate Relay UE performs path switching

- The intermediate Relay UE can report measurement report UE to gNB for path switching to direct path or indirect path acting as a Remote.

- When the intermediate Relay UE performs path switching to direct path or indirect path, the intermediate Relay UE informs the Remote UE via the child intermediate relay UE with release cause 'RRC connection failure'.

- When the Remote UE receives notification from the relay UE, the Remote UE should go back to IDLE.

# Reference

[1]. RP-241609    New WID on NR sidelink multi-hop relay    LG Electronics, InterDigital, FirstNet