3GPP TSG-RAN WG2 Meeting #130	R2-2504565

Malta, May. 19th-23th, 2025

Source:	Panasonic

Title:	RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE for LP-WUS/WUR

Agenda item:	8.4.3

Document for:	Discussion and Decision

- Introduction:

For RRM measurement of LP-WUS/WUR, there are too many thresholds including entry/exit condition for LP-WUS monitoring, serving cell measurement relaxation/offloading, and neighboring cell measurement relaxation, which results in challenges for its implementation. Therefore, RAN2 discussed whether/how to reduce the number of thresholds in RAN2#129-bis. This contribution describes our view on this.

- Discussion:

In RAN2#129-bis, RAN2 had an offline discussion of whether/how to reduce the number of thresholds based on following Direction1-4 [1][2].

Direction 1: Remove the LR based thresholds for all entry conditions in the above 4 RRM relaxation/offloading and LP-WUS monitoring procedures

Direction 2: Merge the entry/exit condition for Serving Cell RRM measurement relaxation and Neighboring Cell RRM measurement relaxation

Direction 3: Merge the entry/exit conditions for Serving Cell RRM measurement offloading or Serving Cell RRM measurement relaxation or Neighboring Cell RRM measurement relaxation and LP-WUS monitoring

Direction 4: Merge the entry/exit conditions for Serving Cell RRM measurement offloading and the current threshold for stopping neighboring cell measurement

"Above 4 RRM relaxation/offloading and LP-WUS monitoring procedures" in Direction 1 means the following.

- LP-WUS monitoring

- Serving cell RRM measurement offloading

- Serving cell RRM measurement relaxation

- Neighboring cell RRM measurement relaxation

Based on the Direction 2, the following agreement was agreed in the last RAN2 meeting.

Agreements Merge the entry/exit condition for Serving Cell RRM measurement relaxation and Rel-19 Neighboring Cell RRM measurement relaxation (higher priority frequency is separate discussion).

The thresholds for entry/exit conditions for LP-WUS/WUR of current agreement can be summarized in the following table.

Table 1: Thresholds for entry/exit conditions for LP-WUS/WUR WI

| Number of thresholds UE procedure | entry conditions | exit conditions |
| --- | --- | --- |
| LP-WUS monitoring | 8, in which - 4 configured for LR measures SSB - 4 configured for LR only measures LP-SS | 4 include only LR thresholds - 2 configured for LR measures SSB - 2 configured for LR only measures LP-SS |
| Serving Cell RRM measurement offloading | 8, in which - 4 configured for LR measures SSB - 4 configured for LR only measures LP-SS | 4 include only LR thresholds - 2 configured for LR measures SSB - 2 configured for LR only measures LP-SS |
| Serving Cell RRM measurement relaxation and Neighboring Cell RRM measurement relaxation | 8, in which - 4 configured for LR measures SSB - 4 configured for LR only measures LP-SS | - If only ‘not fulfilling the entry condition’: 0 - Otherwise: 8 |

We further discuss other Directions i.e. Direction 1, 3 and 4.

2.1 Direction 1

Direction 1 is to remove the LR based thresholds for all entry conditions.

Once the UE transitions to the RRM relaxation/offloading and LP-WUS monitoring procedures, LR-based measurements mainly determine RRM operation. Therefore, to use LP based thresholds is important in order not to use LP based measurement in non-suitable conditions. Specifically, even if the same channel condition, MR-based measurements and LR-based measurements could generate different measurement output because of the measurement method difference and other factors. Therefore, we would like to have the LR based thresholds for all entry conditions in order to control when LR based measurement is carried out by NW.

Proposal 1: Not to remove the LR based thresholds for all entry conditions.

2.2 Direction 3

Direction 3 was not agreed in the last meeting, but following chair's observation was minuted. Therefore, we would like to discuss following Direction 3a and 3b separately.

Chair observation: only one company have strong concern on the following discussion point
Can further discuss Direction 3:
- Direction 3a. merge LPWUS monitoring and serving cell offloading
- Direction 3b. merge LPWUS monitoring and Rel-19 RRM relaxation

The UE behaviour regarding Direction 3a and 3b can be illustrated as the following figures. Direction 3a means LP-WUS monitoring is always performed without MR measurement. Direction 3b means to support the case of LP-WUS monitoring with Rel.19 RRM relaxation in addition to the case of LP-WUS monitoring without MR measurement.

Fig 1. Direction 3a/3b

Our view is to support the case of LP-WUS monitoring with Rel.19 RRM relaxation is important as the relation between LR based measurement and MR measurement should be carefully handled in the network. The original cell coverage and planning are based on MR measurement. Then to use MR measurement for controlling the operating condition of LP-WUS monitoring can be aligned with original cell dimension even if/when LR measurement results are different than MR measurement. This also allows for increased opportunity to utilize LP-WUS monitoring as the system. The power saving gain of this case is reduced compared with LR measurement, but the scaling factor 16 agreed by RAN4 would still contribute to reduce the power consumption.

LP-WUS monitoring with Rel.19 RRM relaxation is removed by 3a but supported by direction 3b. Therefore, we propose the following.

Proposal 2: Not to merge LPWUS monitoring and serving cell offloading and to merge LPWUS monitoring and Rel-19 RRM relaxation.

2.3 Direction 4

Direction 4 is to merge the entry/exit conditions for Serving Cell RRM measurement offloading and the threshold for stopping neighboring cell measurement. The entry/exit conditions for Serving Cell RRM measurement offloading are usually higher than the threshold for stopping neighboring cell measurement. If these are merged, neighboring cell measurement would be required to be carried out even if it is not necessary. This increases the UE power consumption. Therefore, we propose not to take this merge.

Proposal 3: Not to merge the entry/exit conditions for Serving Cell RRM measurement offloading and the threshold for stopping neighboring cell measurement.

- Conclusion:

Proposal 1: Not to remove the LR based thresholds for all entry conditions.

Proposal 2: Not to merge LPWUS monitoring and serving cell offloading, and to merge LPWUS monitoring and Rel-19 RRM relaxation.

Proposal 3: Not to merge the entry/exit conditions for Serving Cell RRM measurement offloading and the threshold for stopping neighboring cell measurement.

- Reference:

- R2-2502154, Whether/How to reduce the number of thresholds for LP-WUS monitoring and RRM relaxation/offloading, vivo

- R2-2503032, Discussion report on [AT129bis][202][LPWUS]