3GPP TSG RAN WG2 #130	                                           R2-2504581

St Julian’s, Malta, May 19th – 23th, 2025

Source:	CEWiT

Title:	A-IoT Random Access

Agenda Item:	8.2.3

Document for:	Discussion and Decision

- Introduction

In RAN2 #129-bis [1] meeting, the following agreements have been made on random access aspects related to A-IoT.

- Agreements on msg1 In case of CBRA, only 16 bits random ID is included in Msg1. FFS can be revisited if message type will be needed for other D2R messages purposes RN16 is not included in the first D2R message in the CFRA procedure. AS ID is the only ID needed for addressing the device in R2D command message assuming for CFRA no multiple devices are performing the procedures with the given reader. FFS if we can assume or need to support multiple device scenario. - Agreements on new R2D message A new R2D message other than the paging message is introduced for A-IoT device determining MSG1 resources unless RAN1 concludes to use L1 signaling. The R2D message indicates the start of a set of MSG1 resources that were configured in paging message. Assumption: The R2D message does not include slot number/count down number. - Agreements on msg 2 A-IoT Msg2 contains one or multiple echoed random ID(s) from A-IoT Msg1 of different A-IoT devices. Same Msg2 format is used for initial transmission and retransmission of Msg2. - Agreements on NACK For CBRA, as a baseline, NACK based mechanism is applied only to the Msg3. May come back for D2R data, if the NACK feedback indication is needed for the purpose to stop/terminate the “on-going procedure” and release the AS ID accordingly (depending on other later discussion). For msg3, we rely on whether the device receives NACK indication before subsequent R2D message to determine re-access. No need for a timer. FFS whether subsequent R2D message is trigger message or paging For CFRA, NACK feedback and re-access is not supported. FFS how to achieve FFS on end of procedure

In this contribution, we discuss our views on random access procedure related aspects for A-IoT.

- Random access

2.1 Determination of resources for random access

2.2.1 For MSG1

RAN2 has agreed that slotted-ALOHA is the baseline for A-IoT random access procedure. In this process, each A-IoT device randomly selects one of the time-frequency resources to transmit its A-IoT MSG1. In the RAN2 #129 [3] meeting, it is agreed that paging message can include the total number of resources for MSG1. For A-IoT random access, to further select the resources for the MSG1 transmission, a separate R2D trigger message is agreed to be transmitting after the paging message. This ensures that the A-IoT device is aware of the resources available for transmitting its initial message. As shown in fig. 1, a predefined time window, referred to as the RACH time window can be used for A-IoT device to select the time resources randomly from this window. This RACH window allows multiple devices to backscatter their responses. The RACH window represents the duration allocated for Msg1 transmissions, during which multiple devices can transmit at different time instances. In the RACH window different occasions for MSG1 transmission are available. The R2D trigger message can be used to indicate the start of the RACH window. Similar to RFID, the A-IoT device can use similar approach i.e. random number based on Q value approach to further chose one time resource form the RACH window for transmitting the MSG1. In this case, the A-IoT device generates a random number based on Q value, loads it to a timer, start decrementing the timer and transmits the MSG1 at the expiry of the timer. The Q value can be configured through the paging message.

Fig.1 TDMA of Msg1 Transmission

Proposal 1: For TDMA of multiple Msg1 transmissions in response to a R2D transmission triggering random access:

- Based on the slotted aloha in a predefined RACH time window.

Proposal 2: R2D trigger used to determine the start of the RACH time window for transmission of MSG1 is supported.

Proposal 3: Support to generate the random number based on Q value configured in paging message by the A-IoT device for transmission of MSG1 in the predefine RACH time window.

When multiple devices backscatter their MSG1, the reader responds by transmitting MSG2 to these devices. In RAN1#120 [2] meeting, RAN1 has agreed that MSG2 supports both options which are given below:

Option 1: A PRDCH for Msg2 transmission corresponds to an A-IoT Msg1 received from one device

Option 2: A PRDCH for Msg2 transmission corresponds to multiple A-IoT Msg1 received from different devices

To ensure proper reception, the monitoring of Msg2 transmission is scheduled to take place after the RACH window. The specific time window during which devices are expected to listen for Msg2 is referred to as the RAR time window, as illustrated in Fig. 2. The RAR time window can start after a predefined time gap ( TD2R_min) from the end of RACH time window. The RAR time window is divided into multiple RAR occasions similar to RACH occasions within RACH window.

Fig.2 Random access procedure

Proposal 4. Monitoring for Msg2 transmission using a RAR time window after the end of the RACH time window.

- The time duration between the end of RACH time window and start of RAR time window is indicated by the TD2R_min.

For Msg3’s transmission resource configuration, the resources can be communicated in two possible ways i.e. the reader can indicate the resources in MSG2 based on the received MSG1 which provides a more efficient resource allocation process or the A-IoT device can reuse the same resources that were previously used for transmitting Msg1. The first option of explicit indication of resources is agreed in RAN1. Further in the case, when the explicit indication is missing in the Msg2 then option 2 can be considered.

Proposal 5: Support to reuse the same resources of MSG1 transmission in absence of explicit indication in Msg 2 to determine the transmission resources for MSG3.

2.2 NACK as feedback

According to the RAN2#129 [3] meeting agreement, if the reader fails to receive Msg3 from the device, it sends a NACK to indicate that the device should attempt retransmission. However, if Msg3 is successfully received, the reader does not send an explicit ACK to minimize unnecessary acknowledgments.

After transmitting Msg3, the device remains uncertain about whether it should continue monitoring for an R2D message. If no further information is sent by the reader, the device will continue monitoring transmissions, leading to potential inefficiencies. To address this, a timer based approach should be adopted. In the timer-based approach, the device will start a timer after transmitting Msg3. During this timer, the reader can send a follow-up message if needed. If no message is received before the timer expires, the device will assume that no further communication is required. Additionally, if the device does not receive a NACK within the expected timeframe, it will assume Msg3 was successfully received.

Proposal 6: Support the timer based approach for monitoring the NACK feedback by the A-IoT device in case of MSG3 failure.

When the device does not receive any R2D message after transmitting a D2R, or receives a NACK in response to the D2R, its behavior must be clearly defined. In such cases, the device is left uncertain about how to proceed, whether it should begin monitoring for the next incoming service request or initiate a retransmission of the previous D2R. To ensure consistent and predictable system behavior, the protocol should explicitly specify the expected device actions under these failure conditions. In these cases, either device declare the process failure and start monitoring the service request or it can retransmit the D2R again and still it won’t receive the subsequent R2D message then it can declare the procedure failure.

Proposal 7: Support the following device behavior in case of failure of D2R (i.e., not receiving subsequent R2D or receiving NACK in subsequent R2D)

•Retransmission of D2R

•Declare procedure failure and start monitoring the service request

To enable the device to detect a Msg2 failure, a predefined time window similar to the RAR window described above can be used as a reference. If the device does not receive a Msg2 (i.e., R2D) within this specified time window after transmitting the initial D2R, it should treat this as a potential failure of the ongoing procedure. Based on system design, two possible behaviors can be defined: (1) the device initiates a retransmission of the D2R to attempt recovery, (2) the device declares the procedure as failed and begins monitoring for a new service request.

Proposal 8: Support the time based monitoring window for R2D to enable the device to detect D2R failure and respond accordingly.

- Conclusion

In this contribution, we provide our views on random access for A-IoT. The proposals are summarized as follows.

Proposal 1: For TDMA of multiple Msg1 transmissions in response to a R2D transmission triggering random access:

- Based on the slotted aloha in a predefined RACH time window.

Proposal 2: R2D trigger used to determine the start of the RACH time window for transmission of MSG1 is supported.

Proposal 3: Support to generate the random number based on Q value configured in paging message by the A-IoT device for transmission of MSG1 in the predefine RACH time window.

Proposal 4. Monitoring for Msg2 transmission using a RAR time window after the end of the RACH time window.

- The time duration between the end of RACH time window and start of RAR time window is indicated by the TD2R_min.

Proposal 5: Support to reuse the same resources of MSG1 transmission in absence of explicit indication in Msg 2 to determine the transmission resources for MSG3.

Proposal 6: Support the timer based approach for monitoring the NACK feedback by the A-IoT device in case of MSG3 failure.

Proposal 7: Support the following device behavior in case of failure of D2R (i.e., not receiving subsequent R2D or receiving NACK in subsequent R2D)

- Retransmission of D2R

- Declare procedure failure and start monitoring the service request

Proposal 8: Support the time based monitoring window for R2D to enable the device to detect D2R failure and respond accordingly.

# References

[1]   RAN2#129-bis Chair Notes.

[2]  RAN1#120 Chair Notes.

[2]  RAN1#129 Chair Notes.