3GPP TSG-RAN WG2 Meeting#130	R2-2504603

St Julian, Malta, May 19th-23rd, 2025

Agenda item:		8.4.2

Source:		Lenovo

Title:		 Procedure and Configuration of LP-WUS in RRC Idle/ Inactive

Document for:		Discussion and Decision

- Introduction

In RAN#103 meeting, WID of Rel-19 Low-power wake-up signal and receiver for NR (LP-WUS/ WUR) [1] was discussed and confirmed. Currently, the UE’s Main Radio (MR) needs to periodically monitor PDCCH once per DRX cycle, which dominates the power consumption in periods with no signaling or data traffic. If the UE’s MR can wake up only when they are triggered (e.g. paging), power consumption could be dramatically reduced. This can be achieved by using a wake-up signal to trigger the main radio and a separate receiver which has the ability to monitor wake-up signal with ultra-low power consumption. Main radio works for data transmission and reception, which can be turned off or set to deep sleep unless it needs to monitor PDCCH [2].

Furthermore, the following agreements were made during previous RAN2 meetings:

Baseline for entry condition definition: If the serving cell quality, e.g. RSRP, RSRQ from MR, is above threshold(s) (if configured), UE may start to monitor LP-WUS, if UE monitors LP-WUS, it may stop monitoring the legacy PO. FFS if any measurement from LR is needed. Baseline for exit condition definition: If the serving cell measurement result based on LR is below a threshold (if configured), UE monitors PO as in legacy and it may stop monitoring the LP-WUS. From RAN2 perspective, no new procedure is introduced for SI reception/updates. If NW configure thresholds for both MR and LR measurements, then the entry condition is met when all the measured results are above the configured threshold(s). The LPWUS monitoring exit condition does not include MR measurements. FFS whether/how to handle the case for UE-ID based subgrouping when the UE has an emergency PDU session. RAN2 understands that UE can report which band(s) is supported by LR to NW. RAN2 understands that any potential overload issues could be addressed by current mechanism in spec. Send LS to RAN1 and RAN4 to inform the agreements. For UE_ID based subgrouping, similar formula defined for PEI subgrouping is reused for LP-WUS subgrouping, i.e., SubgroupID = (floor (UE_ID/(N*Ns*Np)) mod subgroupsNumForUEID) + (subgroupsNumPerPO – subgroupsNumForUEID), where - - UE_ID is related to 5G-S-TMSI, - - N is the number of total paging frames in one DRX cycle, - - Ns is the number of the PO for a PF, - - Np is the number of subgroupNumForUEID for PEI, if configured and UE supports PEI; otherwise, Np is 1, - - subgroupsNumForUEID and subgroupsNumPerPO are the subgroup number for UE_ID based subgrouping for LP-WUS and the total subgroup number for LP-WUS, respectively. LP-WUS is supported with eDRX, FFS on exact impact if any Use 5G-S-TMSI to determine the UE_ID in the formula of UE_ID based subgrouping for LP-WUS, i.e., UE_ID = 5G-S-TMSI mod X. X is based on 32 subgrouping number. Details can be discussed in the running CR. Send LS to RAN3 (CC SA2/SA3) to inform our agreements on UE ID based subgrouping. Confirm the principle for determining CN assigned subgrouping or UE_ID based subgrouping for PEI is reused for LP-WUS subgrouping. Details will be discussed in the running CR. All the LP-WUS related configurations except for measurement configurations are provided in SIB1. FFS the details on measurement configurations. Dedicated configuration in RRC signaling is not needed for providing LP-WUS related configuration in RRC_IDLE/INACTIVE modes. Use existing Srxlev/Squal for all MR measurement based entry/exit condition evaluation. Use measured value for all LR measurement based entry/exit condition evaluation.

This contribution looks at the possible RRC Idle/ Inactive procedures for an LP-WUR enabled UE including entry and exit conditions for LR activation, subgrouping and possible paging enhancements.

- Discussion

  - Remaining Issues for Idle/ Inactive Modes

One open issue identified during the CR review was regarding whether the LP-WUS feature can be enabled/disabled using additional signaling such as RRC or NAS signaling. In our view, additional signaling to enable/ disable the LP-WUS feature is not necessary for the Idle/ Inactive modes since the cell-specific configuration is sufficient for a LP-WUS UE to enable/ disable the feature when the entry or exit conditions are met. Additionally, RAN2 currently does not define any additional enable/ disable signaling for features such as PEI, so we think that LP-WUS feature can follow the same principle.

(RRC-3) RAN2 does not additionally specify any signaling for enable/ disable of LP-WUS feature in Idle/ Inactive modes.

Since it was previously agreed to use PEI as a baseline for LP-WUS subgrouping, we think that the same principle can be applied with respect to the reception of MBS group notification, i.e., UEs expecting MBS group notification shall monitor its PO to receive the MBS group notification regardless of whether LP-WUS is received or not.

Similarly, we think that if lastUsedCellOnly is configured in system information of a cell, the UE monitors LP-WUS in this cell only if the UE most recently received RRCRelease without noLastCellUpdate in this cell, then a LP-WUS UE needs to store its last used cell information. Otherwise (i.e., if lastUsedCellOnly is not configured in system information of a cell), the UE monitors LP-WUS in the camped cell.

(38304-5) UEs expecting MBS group notification shall monitor its PO to receive the MBS group notification regardless of whether LP-WUS is received or not.

(38304-8) LP-WUS is only used in the last used cell when lastUsedCellOnly  is configured in SIB.

  - Paging monitoring triggered by LP-WUS

. In some exceptional cases, UE may fail to receive the paging message after receiving the LP-WUS indicating wake-up. For example, there was no paging transmitted during the target PO(s), or UE misses the target PO/PEI(s) due to e.g., potential transition time, or if the UE has been falsely alarmed by the LP-WUS targeted to other UEs.  RAN2 is suggested to discuss the UE behaviors for the cases that UE fails to receive paging after being woken up by LP-WUS. From our understanding, if UE keeps monitoring paging, then the energy consumption can be aggravated. UE may re-enter LP-WUS monitoring mode in case no paging is received in target PO(s) after being woken up by LP-WUS provided that at least the entry condition for LP-WUS monitoring is satisfied.

UE may re-enter LP-WUS monitoring mode in case no paging is received in target PO(s) after being woken up by LP-WUS provided that at least the entry condition for LP-WUS monitoring is satisfied.

- Additionally, in the case that UE determines to monitor PEI (if configured) after being woken up by LP-WUS and fails to detect the PEI occasion due to some exceptional cases as mentioned above, to avoid UE missing potential paging during the subsequent PO, it is proposed that the UE may keep monitoring associated PO of this PEI.

In the case that UE fails to monitor PEI occasion (if configured) after being woken up by LP-WUS, UE continues to monitor for the associated PO.

# Conclusion

This document discusses some procedures for a LR capable UE in the RRC_IDLE/ RRC_INACTIVE Mode. Following are the proposals and observations made in this document:

- (RRC-3) RAN2 does not additionally specify any signaling for enable/ disable of LP-WUS feature in Idle/ Inactive modes.

- (38304-5) UEs expecting MBS group notification shall monitor its PO to receive the MBS group notification regardless of whether LP-WUS is received or not.

- (38304-8) LP-WUS is only used in the last used cell when lastUsedCellOnly  is configured in SIB.

- UE may re-enter LP-WUS monitoring mode in case no paging is received in target PO(s) after being woken up by LP-WUS provided that at least the entry condition for LP-WUS monitoring is satisfied.

In the case that UE fails to monitor PEI occasion (if configured) after being woken up by LP-WUS, UE continues to monitor for the associated PO.

# References

[1] 3GPP RAN#103, RP-240801, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)

[2] 3GPP TR 38.869 v18.0.0: Study on low-power wake up signal and receiver for NR (Release 18)

[3] 3GPP TSG RAN WG1#118bis, Chair’s Notes

[4] 3GPP RAN2 #129, R2-2500050

[5] 3GPP TS 23.501