3GPP TSG-RAN WG2 Meeting #130	R2-2504608

# St.Julians, Malta, May 19th – 23rd , 2025

Agenda Item:	8.7.4.1

Source:	Ericsson

Title:	Discussion on LCP enhancement

Document for:	Discussion, Decision

# 1	Introduction

RAN2 has been working on scheduling enhancements to the LCP mechanism according to the objective in the XR WID [1] for Rel-19:

-	Specify Enhancements for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs, as follows [RAN2]:

-	Specify additional Logical Channel priority handling using delay/deadline information of packets;

RAN2 has agreed on a new LCP delay enhancement solution. However details on the solution are still to be settled. Several open issues appeared during CR review and this paper discuss some of these issues.

# 2	Discussion of LCP enhancement

Two of the open issues that were identified on the LCP enhancement during CR reviews was the “[MAC-13] Priority fallback with consideration of PDU Set integrated handling relation” and “[MAC-01] Impact of priority adjustment on SR priority determination” which will be discussed in following sections.

## 2.1 Priority fallback with PDU Set integrated handling

An open issue during CR review is the handling of PDU Set when doing the priority adjusting in the enhanced LCP procedure. Since the priority adjustment is based on the running PDPC discard timer it is a question what happen when the timers running for a PDU Set has different values and not all resides inside the threshold. As a starting point, it should be remembered that the intention of PDU Set was always to treat the set as one entity with the PDU Set discarding is activated. There were two modelling options on the table, either keep multiple running timers (one for each SDU as in legacy) or have only one single timer running for each PDU Set. In the end it was decided that multiple timers should be used. That this is just a modelling detail is evident when looking at the features impacted by the PDU Set integrated handling. In the case of PDU Set discarding, as soon as any timer value is expired all the other timers belonging to the PDU Set are also expired. In the case of the DSR feature, when PDU Set discarding is configured whenever any timer for an SDU belonging to a PDU Set is below the configured remaining time threshold all SDUs belonging to the PDU Set will be reported in the data volume (despite their timer values).

Logically similar handling as for the DSR should be introduced for the LCP enhancement when PDU Set discarding is activated.

Enhanced LCP feature should have similar handling of PDU Sets as the DSR feature with respect to PDU Set integrated handling.

This can be specified using similar formulations in MAC specification chapter 5.4.3.1.3 Allocation of resources as has been used for the DSR related to PDU Set discarding:

2>If pdu-SetDiscard is configured, if any of the PDCP SDU belonging to a PDU Set has a running PDCP discardTimer whose remaining value, evaluated at the time of the first symbol of this transmission, is below or at its priorityAdjustmentThreshold:

3>  apply additionalPriority of this logical channel;

2> Else…

## 2.2 Impact of priority adjustment on SR priority determination

The agreement in RAN2#129bis was to analyse first the impact of SR priority on the specification before taking a final decision:

- We will check the impact of using additional LCP priority for SR priority determination during intra-UE prioritization in specifications and make a final decision on whether to have it based on that, i.e. it should be simple enough.

During CR review a TP from rapporteur was provided on this topic. We think there are some issues with the TP, for example unclarities related to the reference of “priority of PUCCH transmission”. However more importantly our previous raised concerns related to using the additional LCP priority for SR priority determination remains. The priority of a LCID is changed during the logical channel prioritization. According to the MAC specifications, this procedure is executed whenever a new transmission is performed. Triggering of a BSR and transmission of the SR does not result in a Logical Channel Prioritization. Ultimately, changing the priority of the logical channel had the intent to emulate a simple delay-aware scheduler, allowing the UE to choose data which remaining latency was below a threshold. The purpose was never to impact other procedures and processes at the UE. Thus, UL grant priority determination and SR priority determination should simply use the legacy value.

Intra-UE prioritization shall not use the additional LCP priority for SR priority determination.

However, if RAN2 still want to introduce this dependency between the features we think it is of importance that the legacy behaviour can still be applied. These features are independent features and there is not necessary that a coupled behaviour is wanted. Thus, it should be up to network control what behaviour is applied when both features are activated, i.e. based on network configuration.

If priority adjustment is agreed to impact the SR priority determination, then applying legacy or new behaviour should be controlled by network configuration.

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Enhanced LCP feature should have similar handling of PDU Sets as the DSR feature with respect to PDU Set integrated handling.

Proposal 2	Intra-UE prioritization shall not use the additional LCP priority for SR priority determination.

Proposal 3	If priority adjustment is agreed to impact the SR priority determination, then applying legacy or new behaviour should be controlled by network configuration.

# References

RP-241771 Revised WID: XR (eXtended Reality) for NR Phase 3, September 2024