3GPP TSG-RAN WG2 Meeting #130	R2-2504618

St Julian’s, Malta, May 19th – 23rd, 2025

Source:	vivo

Title:	Discussion on open issues about CHO with candidate SCGs

Agenda Item:	8.10.2

Document for:	Discussion and Decision

- Introduction

In the post-meeting email [Post129bis][603][SONMDT], the following open issue was concluded regarding the CHO with candidate SCGs as follows.[1]

Proposal 1. Companies are invited to discuss the Open issue RRC-1 (i.e., the correlation of SHR and SPR and avoiding duplication of the information) in their contribution for the meeting RAN2#130.

The above open issue involves sub-optimal execution of both CHO and CPC procedures, where one or more SHR/SPR triggering conditions are met for each conditional cell change procedure, including PCell and PSCell. In such scenario according to the offline discussions in RAN2#129bis, the UE may have to log redundant measurements (e.g., neighbour cell measurements) in SHR and SPR. Such duplicated measurements may increase the UE reports overhead to the network, particularly if such scenarios happen frequently.

In this contribution, we will provide an analysis of the above open issue.

- Discussion

The new field ChoWithCandidateSCGInfo in SHR/SPR for the CHO with candidate SCGs is copied as follows:

ChoWithCandidateSCGInfo information element

ChoWithCandidateSCGInfo::=                     SEQUENCE{

firstFulfilledConfig-r19                       ENUMERATED {cho, cpc}                             OPTIONAL,

timeBetweenFulfillment-r19                     TimeBetweenEvent-r17                              OPTIONAL,

timeBetweenLastFulfillmentAndEvent-r19         TimeBetweenEvent-r17                              OPTIONAL,

PcellId-r19              CHOICE {

cellGlobalId-r19         CGI-Info-Logging-r16,

pci-arfcn-r19            PCI-ARFCN-NR-r16

}                                                                                                OPTIONAL,

PScellId-r19             CHOICE {

cellGlobalId-r19         CGI-Info-Logging-r16,

pci-arfcn-r19            PCI-ARFCN-NR-r16

}                                                                                                OPTIONAL,

It can be observed that the UE will indicate the candidate PCell and PSCell in both SHR and SPR, if any. In addition, the UE will record

- Source PCell ID and target PCell ID in the SHR;

- Source PSCell ID and target PSCell ID in the SHR;

Figure 2-1: Example of potential issue

In the above example, if the UE generates both SHR and SPR, the SHR will include the following information:

- Source PCell 1 and Target PCell 2

- Candidate PCell 2 and Candidate PSCell b

- Conditions are fulfilled based on timeBetweenFulfillment

The SPR will include the following information:

- Source PSCell a and Target PSCell b

- Candidate PCell 2 and Candidate PSCell b

- Conditions are fulfilled based on timeBetweenFulfillment

Based on the information highlighted in red, if the NW successfully retrieves both the SHR and SPR, the NW can already correlate the SHR and SPR based on the existing source/target cells information and the paring information in ChoWithCandidateSCGInfo. Therefore, no additional correlation information is needed.

Proposal 1: For CHO with candidate SCGs, the NW can correlate the SHR and SPR based on the existing source/target cells information and the new paring information (in ChoWithCandidateSCGInfo). Thus no additional correlation information is needed.

However, we think there is no need to further enhancement SHR/SPR procedures to avoid duplication of record information, for the following reasons:

- Such an event (fulfillment of both SHR and SPR triggering conditions in CHO with Candidate SCG) is not expected to happen frequently in a real deployment.

- Whether/when to retrieve the SHR and/or SPR is up to NW implementation. The NW may request the SHR and SPR at different times. From our understanding, no need for UE to monitor/check whether some information has already been sent to the NW.

Proposal 2: For CHO with candidate SCGs, no need to consider avoiding duplicated information in SHR and SPR.

- Conclusion

Based on the above analysis, we make the following proposals:

Proposal 1: For CHO with candidate SCGs, the NW can correlate the SHR and SPR based on the existing source/target cells information and the new paring information (in ChoWithCandidateSCGInfo). Thus no additional correlation information is needed.

Proposal 2: For CHO with candidate SCGs, no need to consider avoiding duplicated information in SHR and SPR.

- Reference

[1]	R2-2504296	Open Issues for NR RRC SONMDT features	Ericsson