3GPP TSG-RAN WG2 Meeting #130	R2-2504625

St. Julian’s, Malta, May 19th – 23rd, 2025

Source:        ZTE Corporation

Title:           Consideration on the Remaining Issues of Asymmetric TRP

Agenda item:   8.12.2

Document for:  Discussion and Decision

- Introduction

According to the post email discussion [Post129bis][215][MIMO_Ph5] Running CR for TS 38.321 (Samsung), the following issues on the asymmetric DL sTRP/UL mTRP need to be further discussed.

Issue 1: discuss the scenario asymmetric DL sTRL/UL mTRP with no PL offset (i.e. the UL TRP may transmit SSB) based on RAN1/RAN4 LS (R1-2503091) and spec. impact regarding 2TA for sDCI mTRP. Issue 1a: discuss to support sDCI mTRP 2TA for intra-DU LTM, in the same way as Rel-18 mDCI mTRP 2TA, i.e., the TA included in the LTM cell switch command is applied to the TAG configured for the indicated TCI state in LTM cell switch command. Issue 2a: Regarding PL offset in LTM, discuss - how to apply the PL offset in intra-DU LTM when PL offset is provided in LTM configuration (e.g., the PL offset for the indicated TCI state in the LTM cell switch command is applied for the first PUSCH transmission in RACH-less LTM cell switch); - whether to support PL offset in intra-DU LTM when PL offset is not provided in LTM configuration for the target cell. Issue 3: Discuss whether/how Rel-18 TCI state activation MAC CEs for sDCI mTRP is applied for asymmetric DL sTRP/UL mTRP.

Meanwhile, there are also some remaining issues/EDs on this topic in the post email discussion “[Post129bis][214][ MIMO_Ph5] Running CR for 38.331 (Ericsson)” as below:

| Asymmetric DL sTRP/UL mTRP | How to capture PL offset behaviour in RRC for handover and resume case. | To be discussed in contributions |
| --- | --- | --- |
|  | Editor’s note: this description may require further updates to capture the agreement “remove the restriction that RRC field tag2 is configured only if coresetPoolIndex is configured with more than one value;”. | It could be good to have stage-3 suggestions if/how to update these either by contribution to next meeting or as input directly to a next e-mail discussion to update the running CR. |
|  | Editor’s note: this description may require further updates to capture the agreement “a single n-TimingAdvanceoffset is configured, i.e., n-TimingAdvanceOffset2 is not configured for 2TA in asymmetric DL sTRP/UL mTRP scenario”. | It could be good to have stage-3 suggestions if/how to update these or as input directly to a next e-mail discussion to update the running CR. |

In this paper we would like to share our views on these remaining issues.

- Discussion

Based the above remaining issues and EDs, there are 4 main topics:

- Topic 1: 2TA spec impact on the RRC spec (e.g. issue 1 and EDs in the RRC spec)

- Topic 2: Combination of R19 2TA feature and the LTM feature (e.g. issue 1/1a/2a)

- Topic 3: How to capture PL offset behaviour in RRC for handover and resume case (RRC spec remaining issue)

- Topic 4: Whether/how Rel-18 TCI state activation MAC CEs for sDCI mTRP is applied for asymmetric DL sTRP/UL mTRP (e.g. Issue 3)

Furthermore, for the asymmetric DL sTRL/UL mTRP, there are 2 scenarios:

- Scenario 1: UE is configured with PL offset in joint/UL TCI state(s)

- Scenario 2: UE is not configured with PL offset in joint/UL TCI state(s) and UE may expect to receive SSB from UL TRP based on RAN1/RAN4 LS (R1-2503091)

In the next chapters we will discuss these topics one by one by taking the above 2 scenarios into consideration.

2.1 2TA spec impact on the RRC spec

As summarized in the RRC open issue list, there are 2 EDs left on 2TA as below:

| Editor’s note: this description may require further updates to capture the agreement “remove the restriction that RRC field tag2 is configured only if coresetPoolIndex is configured with more than one value;”. | It could be good to have stage-3 suggestions if/how to update these either by contribution to next meeting or as input directly to a next e-mail discussion to update the running CR. |
| --- | --- |
| Editor’s note: this description may require further updates to capture the agreement “a single n-TimingAdvanceoffset is configured, i.e., n-TimingAdvanceOffset2 is not configured for 2TA in asymmetric DL sTRP/UL mTRP scenario”. | It could be good to have stage-3 suggestions if/how to update these or as input directly to a next e-mail discussion to update the running CR. |

Obviously, both the above scenario 1 and 2 need to be considered. For the Scenario 1, the below agreements have been made in the last RAN2 meeting:

For 2TA in asymmetric DL sTRP/UL mTRP scenario with pathloss offset configured Rel-18 2TA operation is applied with the following RRC changes: • remove the restriction that RRC field tag2 is configured only if coresetPoolIndex is configured with more than one value; • a single n-TimingAdvanceoffset is configured, i.e., n-TimingAdvanceOffset2 is not configured for 2TA in asymmetric DL sTRP/UL mTRP scenario.

For the scenario 2, according to the RAN1 agreements, it’s clear that the rel-19 2TAs can still be supported for this scenario, and thus the restriction that “RRC field tag2 is configured only if coresetPoolIndex is configured with more than one value” should also be removed.

For the scenario that the UE is not configured with PL offset in joint/UL TCI state(s) and UE may expect to receive SSB from UL TRP(s), when the UE is configured with rel-19 2 TAs: • The UE maintains two downlink reference timings; …

Proposal 1: For the scenario 2 (i.e. UE is not configured with PL offset in joint/UL TCI state(s) and UE may expect to receive SSB from UL TRP), remove the restriction that RRC field tag2 is configured only if coresetPoolIndex is configured with more than one value.

Furthermore, if the UE may expect to receive SSB from UL TRP and the UE maintains two downlink reference timings, it should be allowed to configure the n-TimingAdvanceOffset2 for the 2TA.

Proposal 2: For the scenario 2, it should be allowed to configure the n-TimingAdvanceOffset2 for the 2TA.

If the above proposals are agreed, the detail wording in the RRC spec can be left to the next round Running CR review.

2.2 Combination with LTM Feature

On this topic, it’s also needed to discuss the above 2 different scenarios.

For the scenario 1, it would depends on both the 2TA and the PL offset. The 2TA can be supported with the current signalling. However, the difficult part is about the PL offset, e.g. whether it’s possible to get the PL offset in advance. For the FR1 intra-DU, maybe it’s possible, but for the FR2, it’s seems impossible to get the PL offset in advance.

Furthermore, if the PL offset feature can’t be supported for the LTM feature, even the UE can be configured with TA in advance, UE still can’t transmit the UL signalling with the suitable power. Thus for the scenario 1, it’s better to send an LS to RAN1 first to confirm whether it’s possible to get the PL offset in advance for intra-DU case.

Proposal 3: Regarding support R19 MIMO with scenario 1 (UE is configured with PL offset in joint/UL TCI state(s)) for intra-DU LTM feature, send an LS to RAN1 to confirm whether it’s possible to get the PL offset in advance for intra-DU case.

For the scenario 2, it has no PL offset, and in the last RAN2 meeting, it has been agreed that

MIMO 2TA (R18) can be configured with inter-CU LTM using PDCCH-order based TA measurement.

It’s quite nature to also support R19 MIMO 2TA with scenario 2 for the LTM feature.

Proposal 4: Support R19 MIMO 2TA with scenario 2 (i.e. UE is not configured with PL offset in joint/UL TCI state(s) and UE may expect to receive SSB from UL TRP) for the LTM feature.

2.3 PL offset Behaviour in RRC for Handover and Resume

In the last meeting, it has confirmed the below working assumption.

From RAN2 point of view, UE applies the latest PL offset value received in RRC or MAC CE.

Then companies also ask how to use the PL offset value during the handover or resume case, if the PL offset value has been updated by the MAC CE, how the CU gets the latest value.

To solve this issue, the simple way is to define the PL offset as a need R field. Then there would be no mismatch between the UE and the network side, and the UE only needs to store the latest value configured either by the RRC or the MAC. Furthermore, in the running CR, the need R has been used for the PL offset configuration.

Proposal 5: Use need R for the PL offset configuration.

2.4 Rel-18 TCI state activation MAC CEs

This remaining issue is about whether/how Rel-18 TCI state activation MAC CEs for sDCI mTRP is applied for asymmetric DL sTRP/UL mTRP. For that there is only one DL TCI state for each DCI code point, if the R18 MACE CE is reused for the asymmetric case, how to indicate the DL TCI state.

Based on the contributions in the last meeting, companies have different understanding on this issue, e.g.

- Rel-18 TCI state activation MAC CEs for sDCI mTRP cannot work for asymmetric DL sTRP/UL mTRP. Send an LS to RAN1 to ask whether the Rel-18 MAC CEs can be applied or enhancement is needed.

- The Rel-18 MAC CEs can work for asymmetric DL sTRP/UL mTRP with no enhancement, e.g., NW always activates one joint/DL TCI state for each codepoint (by setting a Fi,j field as 0), or provides same QCL information for the two indicated DL TCI states so that from UE perspective it is DL sTRP operation.

- Reuse the Rel-18 MAC CEs, for separate mode the Fi,j field for the DL TCI state for the UL-only TRP is set to 0 and the UE ignores this Fi,j field.

For the separate TCI state mode, it can be easily solved by the network configuration, e.g. configure the same DL TCI state value, or set one value to 0. For the joint mode, it seems a little different. However, if we check the RAN1 discussion history, we can find that this issue has been discussed in the RAN1. During RAN1 discussion, there is even a Note for the joint TCI state case as below [1]:

Updated Conclusion 1.7a: For the asymmetric DL sTRP/UL mTRP deployment scenario, - When rel-17 unified TCI/ICBM is configured: - For FR1: one joint TCI state or one DL TCI state + one UL TCI state can be applied to the system - For FR2: one DL TCI state + one UL TCI state can be applied to the system. - When rel-18 unified TCI is configured: - For FR1: up to two joint TCI states or one DL TCI state + up to two UL TCI state can be applied to the system. - Note: When two joint TCI states are applied, the 1st joint TCI state is applied on DL transmission and both joint TCI states can be applied on UL transmissions - For FR2: one DL TCI state + up to two UL TCI states can be applied to the system.

At last companies in RAN1 think this can be left to network implementation [2], e.g., network configure same TCI state for two DL/joint TCI states, and thus the “note” is not needed. Based on the above RAN1 discussion history, we don’t think there is a need to reopen the same discussion in RAN2, or RAN2 can confirm the RAN1’s understanding in the chairman Note.

Proposal 6: RAN2 confirm that Rel-18 TCI state activation MAC CEs for sDCI mTRP is applied for asymmetric DL sTRP/UL mTRP with no spec impact (or no enhancement). It’s left to the NW implementation on how Rel-18 TCI state activation MAC is applied for asymmetric DL sTRP/UL mTRP.

- Conclusion and proposals

With the above analysis, we have the following proposals:

Proposal 1: For the scenario 2 (i.e. UE is not configured with PL offset in joint/UL TCI state(s) and UE may expect to receive SSB from UL TRP), remove the restriction that RRC field tag2 is configured only if coresetPoolIndex is configured with more than one value.

Proposal 2: For the scenario 2, it should be allowed to configure the n-TimingAdvanceOffset2 for the 2TA.

Proposal 3: Regarding support R19 MIMO with scenario 1(UE is configured with PL offset in joint/UL TCI state(s)) for intra-DU LTM feature, send an LS to RAN1 to confirm whether it’s possible to get the PL offset in advance for intra-DU case.

Proposal 4: Support R19 MIMO 2TA with scenario 2 (i.e. UE is not configured with PL offset in joint/UL TCI state(s) and UE may expect to receive SSB from UL TRP) for the LTM feature.

Proposal 5: Use need R for the PL offset configuration.

Proposal 6: RAN2 confirm that Rel-18 TCI state activation MAC CEs for sDCI mTRP is applied for asymmetric DL sTRP/UL mTRP with no spec impact (or no enhancement). It’s left to the NW implementation on how Rel-18 TCI state activation MAC is applied for asymmetric DL sTRP/UL mTRP.

- Reference

- R1-2405346	Summary #1 on Rel-19 asymmetric DL sTRP/UL mTRP	Moderator (OPPO)

- R1-2405347	Summary #2 on Rel-19 asymmetric DL sTRP/UL mTRP	Moderator (OPPO)