3GPP TSG-RAN WG2 Meeting #130	R2-2504640

St. Julian’s, Malta, 19th – 23rd May, 2025

Agenda Item:	8.11.3

Source:	Huawei, HiSilicon

Title:	Other aspects about the evolution of duplex operation

Document for: Discussion and Decision

# 1	Introduction

In the RAN2#129-bis meeting, RAN2 achieved the following agreements about the impacts on random access operation by the evolution of duplex operation:

Output of email discussion [Post129][217] - On RO type signaling for CFRA 1. For CFRA triggered by BFR, the RO type is indicated in BeamFailureRecoveryConfig. 2. For CFRA triggered by ReconfigurationwithSync, the RO type is indicated in RACH-ConfigDedicated. - When both NW indication on RO type and RSRP threshold are absent, it is up to UE implementation to select the RO type. - Random access procedure in SBFD symbols is supported for all the existing RACH trigger events except for SI request. FFS for LTM. On RSRP threshold for RO type selection (e.g., configuration, related procedure, etc.) - NW indicate via explicit signaling whether the SBFD RO is selected when SSB RSRP are 'below' or 'above' the configured threshold. RO type selection before or after RA type selection - Working assumption: For SBFD-aware UE, the selection of RO type is suggested to be performed before the selection of the set of Random Access resources. On RO type fallback - RO-Type change procedure on RO type selection from legacy RO to additional RO in SBFD symbols is supported when the number of PRACH transmission attempts exceed a threshold (we assume it is the same threshold with the fallback from additional RO to legacy RO). If fallback from legacy RO to additional RO occurs, no further fallback to legacy RO is supported.

In this contribution, we will further analyse the remaining issues for SBFD.

# 2	Discussion

Based on the RRC running CR and the open issue discussion, we would like to discuss the following issues:

Issue 1: the location of sbfd-RACH-SingleConfig-r19

In the current RRC running CR, the field sbfd-RACH-SingleConfig-r19 is used to indicate whether RACH configuration Option 1 for SBFD random access operation is enabled or not, which is included in RACH-ConfigCommonSBFD-r19. We understand this indicator is feature-independent, which can be only provided once in BWP-UplinkCommon IE. Considering that RAN2 is responsible for RRC signaling design, we think the field sbfd-RACH-SingleConfig-r19 can be included in RACH-ConfigCommonSBFD-r19 in BWP-UplinkCommon, as in current running CR. Thus, the corresponding Editor’s Note can be removed.

Proposal 1: RAN2 to confirm that the field sbfd-RACH-SingleConfig-r19 is included in RACH-ConfigCommonSBFD-r19 in BWP-UplinkCommon.

Issue 2: Whether early UL sync with an LTM candidate cell is supported in SBFD symbols

In the last meeting, there is an FFS about whether “Early UL synchronization with an LTM candidate cell” and/or ”RACH-based LTM” can be supported in SBFD symbols. In order to support this, the RO type and the SBFD RACH resource need to be added in the EarlyUL-SyncConfig IE.

For this feature, early synchronization for LTM is only used to send a preamble in advance to determine TA for a candidate cell, and the actual RACH procedure is not immediately triggered. After this preamble transmission, UE keeps performing L1 measurement and waiting for handover indication. Therefore, it is understood that the eventual RACH latency is not significantly impacted by using SBFD for early synchronization for LTM hence the legacy resources can be used for this purpose while sparing the SBFD resources for other features.

Further, in inter-DU case, the candidate DU sends the TA value to the source DU via the CU, along with the preamble index, candidate cell ID as well as RO information (i.e. RA-RNTI), so that the source DU would know which UE the sent TA is targeting for. If the early UL sync with LTM candidate cell using SBFD resource is supported, the F1 procedure may need to be enhanced to clarify which type of PRACH resource is used and needs to involve RAN3. Besides, we are not certain about the other potential impact for different working groups, which needs further evaluation. Considering the time needed to complete this WI, we think it is better to de-prioritize supporting this feature combination in Rel-19.

Proposal 2: The combination of LTM and SBFD is not supported in Rel-19.

# 3	Conclusion

Based on the above discussion we have the following proposals:

Proposal 1: RAN2 to confirm that the field sbfd-RACH-SingleConfig-r19 is included in RACH-ConfigCommonSBFD-r19 in BWP-UplinkCommon.

Proposal 2: The combination of LTM and SBFD is not supported in Rel-19.

# 4	Reference

- RP‑241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)