3GPP TSG-RAN WG2 Meeting #130	R2-2504651

St. Julians, Malta, May 19th – 23rd, 2025

Agenda item:	8.1.3

Source:	Rakuten Mobile

Title:	Further Discussion on NW-Side AI/ML Data Logging Framework

Document for:	 Discussion and Decision

# 1. Introduction

This contribution discusses Rakuten’s proposals on Enhancements related to NW Side Data collection for AI/ML Air Interface WI.

# 2. Discussion

## 2.1 Logging Configuration & Triggering

Rel-19 permits two trigger modes:

- periodic logging and

- L3 radio-condition events derived from A1/A2 other L3 events.

The draft CR still have these parameters across several IEs and lacks a container that also lets the network set an independent loggingInterval to save UE power on short-period RS.

This was agreed in RAN2#127bis and confirmed in RAN2#129
However, the logging interval parameter remains an FFS item in multiple places

Each active logging session is already identified by logConfigID; a boolean flag per ID fulfils granularity. A timer (similar to Logged-MDT’s 48 h rule) fulfils lifetime control with negligible extra signalling.

Observation 1
No single IE configures an A1/A2-based trigger together with an optional logging interval.

Proposal 1
Introduce LoggingTriggerConfig under AIML-LoggingConfig in TS 38.331

| Field | Type / Range | Notes |
| --- | --- | --- |
| triggerType | ENUM {l3Condition(0)} | Only L3 events in Rel-19 scope |
| eventId | INTEGER(0..31) | Maps to A1/A2 derivatives (e.g., X1, X2) |
| timeToTrigger | INTEGER(0..500) ms | Same scale as RRM TTT |
| loggingInterval | OPTIONAL INTEGER(1..1024) ms | If absent → log per RS period |

## 2.2 Data-Availability & Constraint Reporting

Agreed causes for availability signalling are bufferThresholdReached, bufferFull, and lowPower. A single Boolean flag cannot represent them.

Observation 2-1
The CR draft offers only one flag; the gNB cannot prioritise urgent low-power cases over routine buffer-threshold events.

Proposal 2-1
Add structured element in UEAssistanceInformation / UAI:

DataAvailabilityIndication ::= SEQUENCE {

cause ENUMERATED {

bufferThresholdReached (0),

bufferFull            (1),

lowPower              (2)

},

remainingBytes INTEGER (0..262143) OPTIONAL

}

Discussion 2-2
RAN-2 bis captured diverging operator requirements:

- Network-controlled thresholds—deterministic.

- UE-autonomous heuristics.
The current draft CR lacks a mechanism to toggle between these, forcing vendors to choose one or other.

Observation 2-2
No IE allows the network to select between nw-controlled and UE-autonomous availability-trigger logic.

Proposal 2-2
Add field availabilityTriggerMode to AIML-LoggingConfig in TS 38.331:

| Field | Type | Default | Semantics |
| --- | --- | --- | --- |
| availabilityTriggerMode | ENUM { nwControlled(0), ueAutonomous(1) } | nwControlled | Decides which entity evaluates buffer & power thresholds |

Discussion 2-3
If a UE silently stops logging due to low memory or critical battery, the network sees unexplained data holes that bias ML training. Chair notes (128 §6) requested an explicit “reason code”.

Observation 2-3
The spec provides only an on/off availability flag; it cannot convey why logging paused.

Proposal 2-3
Extend UEAssistanceInformation and UAI with:

| Field | Type / Size | Bit Mapping |
| --- | --- | --- |
| resourceConstraintCause | BIT STRING (SIZE 3) | b0 memLow, b1 powerLow, b2 cpuBusy |

## 2.3 Retention & State / Mobility Handling

NW-side training data collection may run for hours or days, so partially collected logs may span multiple cell changes, DRX cycles, or even RAT transitions. Without a deterministic rule set we can see three different behaviours:

- Discard logs whenever a handover command arrives, guaranteeing fresh context for the target cell but losing any unsent data.

- Keep logs by default, if the target gNB is from a different vendor, those logs may never be fetched because the new node lacks decoding knowledge.

- Indeterminate devices—particularly dual-mode chipsets may retain data in some cases (intra-Freq HO) and flush in others (Idle, Inactive, RLF), hence missing data in the time series.

RAN-2 resolved the principle by agreeing on a one-bit directive set by the source gNB. Yet two critical gaps remain:

- Signalling location -Placing the bit inside the HO-Command (i.e. RRCReconfiguration from the target cell) is difficult : if the source and target are from different vendors, the proprietary container may be ignored. Pre-HO delivery from the source gNB will enhance interoperability.

- Lifetime management - If retained logs are not fetched for long intervals (e.g., UE soon enters Idle or executes inter-RAT HO), they must self-expire to satisfy privacy policies and free memory. MDT use upper bound timer similar to MDT.

A uniform rule set guarantees that:
UEs do not hoard stale data indefinitely, limiting privacy exposure and memory pressure.

Observation 3
Without a deterministic location and expiry timer, vendors differ on when logs are purged.

Proposal 3

- 3-1 Add Boolean retainLoggedData in source-cell RRCReconfiguration sent before HO execution. Include retentionTimer (24–168 h, default 48 h) inside AIML-LoggingConfig. UE discards retained logs when the timer expires or after HO/RRC_IDLE/RLF.

With concurrent configs (beam-AI + CSI-AI), gNB cannot separate datasets unless each entry declares its origin. Several companies flagged this in RRC

Observation 3-2
With concurrent configs (beam-AI + CSI-AI), gNB cannot separate datasets unless each entry declares its origin. Several companies flagged this in RRC

logMeasEntry and aiLogMeasEntry lack a session tag.

Proposal 3-2
Prepend 5-bit logConfigID to both entry structures:

aiLogMeasEntry-v19a0 ::= SEQUENCE {

logConfigID   INTEGER (0..31),

...           -- existing fields

}

## 2.4 Data Format & Metadata

Network may aggregate logs from millions of UEs, and data may  arrive out of order due to HARQ retransmissions, DRX sleep, or multi-SIM split UL. Current MDT messages rely on context in the enclosing RRC/PDU; if segmentation occurs, that context can be lost. For NW-side AI training data collection is important.

Observation 4
Message-level context alone cannot reassemble interleaved segments.

Proposal 4
Add fixed header to every UEInformationResponse segment:

AIMLLogHeader ::= SEQUENCE {

logConfigID  INTEGER (0..15),

segmentNo    INTEGER (0..255),

lastSegment  BOOLEAN,

logPriority  ENUM {normal, high},

moreSegmentsPending BOOLEAN

}

Add cellId, gapInd, and optional entryPriority to each LoggedMeasurementEntry.

## 2.5 Transport Bearer & SRB Use

AIML  logs and MDT must share SRB-4 without starving each other, assigning a new LCG to AIML traffic lets MAC allocate fewer bytes per TTI, throttle bursts during congestion, and still rely on SRB-4’s security context. Coupled with a moreSegmentsPending flag, the scheduler gains explicit knowledge of burst boundaries, enabling fair-queue or weighted deficit round-robin without redefining another SRB.

Observation 5
Single LCG on SRB-4 can create head-of-line blocking.

Proposal 5
Map AI/ML log PDUs on SRB-4 to new logicalChannelGroup, (aiModelData); MDT remains on its existing LCG. Scheduler may throttle using moreSegmentsPending.

When moreSegmentsPending=1 UE continues using SRB-4 / aiModelData-LCG until all segments sent

## 2.4 Cross-layer RS Referencing

RAN-1 defines CSI-ReportConfig IDs and their associated RS-ResourceSet lists. Some companies want logging to follow exactly the same RS pool used for inference (to guarantee feature alignment in model training). Others, custom RS sets (e.g., occasional SS/PBCH blocks) that are not part of CSI inference. Embedding RS details in RRC every time duplicates RAN-1 ASN.1 and risks version skew; but forcing all logging to tie to a CSI-ReportConfig ID will not work with those custom cases.

Observation 6
Spec provides no mechanism to reference or inherit RS sets.

Proposal 6
Add optional linkedCSI-ReportConfigId (INTEGER(0..255)) inside AIML-LoggingConfig. If present, UE inherits RS definitions from that CSI-ReportConfig; else it uses explicit RS IDs.

Liaison RAN-1 request: expose stable rsSetId in CSI-ReportConfig ASN.1.

## 2.6 Memory / Buffer Limits

Operators configuring a fixed bufferThreshold (e.g., 50 kB) may experience opposite issues.

- Threshold exceeds total buffer on small devices → UE hits bufferFull constantly, causing signalling storms and data truncation.

- Threshold never reached on large devices → network delays fetching; old logs occupy memory and are overwritten when new high-value events occur.

An MDT-style minimum baseline (32 kB) guarantees that any threshold ≤32 kB is always supportable. Exposing the actual bufferSizeCap in UE capabilities lets OAM or self-optimising SON adjust thresholds dynamically (e.g., 60 % cap). Constraining bufferThreshold to 50–90 % of bufferSizeCap prevents pathological settings, maintains uniform trigger cadence, and simplifies cross-vendor configuration templates.

Observation 7
Without these, low-cap UEs over-trigger and high-cap UEs under-trigger.

Proposal 7

- 7-1- Require ≥ 32 kB dedicated AS-layer memory for AI logs; UE declares bufferSizeCap in UECapabilityInformation-AI.

- 7-2 Signal bufferThreshold in absolute bytes; UE clamps to 50–90 % of declared capacity.

## 2.7 Combined Trigger-Mask Field

Some deployments need periodic + event (X1/X2) logging simultaneously; others want one to suppress the other.

Observation 8
No unified mask lets the network enable/disable multiple triggers per session.

Proposal 8
Inside LoggingTriggerConfig, add triggerMask BIT STRING (SIZE (4)) — bits {periodic, X1, X2, lowPower}.
UE logs when any enabled bit condition is satisfied.

# 3. Conclusion

In this contribution, we provide our views for Rel-19 NW-side AI/ML Data Logging Framework, and the observations and proposal are summarized as follows:

Logging Configuration & Triggering

Observation 1
No single IE configures an A1/A2-based trigger together with an optional logging interval.

Proposal 1
Introduce LoggingTriggerConfig under AIML-LoggingConfig in TS 38.331

| Field | Type / Range | Notes |
| --- | --- | --- |
| triggerType | ENUM {l3Condition(0)} | Only L3 events in Rel-19 scope |
| eventId | INTEGER(0..31) | Maps to A1/A2 derivatives (e.g., X1, X2) |
| timeToTrigger | INTEGER(0..500) ms | Same scale as RRM TTT |
| loggingInterval | OPTIONAL INTEGER(1..1024) ms | If absent → log per RS period |

Data-Availability & Constraint Reporting

Observation 2-1
The draft offers only one flag; the gNB cannot prioritise urgent low-power cases over routine buffer-threshold events.

Proposal 2-1
Add structured element in UEAssistanceInformation / UAI:

DataAvailabilityIndication ::= SEQUENCE {

cause ENUMERATED {

bufferThresholdReached (0),

bufferFull            (1),

lowPower              (2)

},

remainingBytes INTEGER (0..262143) OPTIONAL

}

Observation 2-2
No IE allows the network to select between nw-controlled and UE-autonomous availability-trigger logic.

Proposal 2-2
Add field availabilityTriggerMode to AIML-LoggingConfig in TS 38.331:

| Field | Type | Default |  |
| --- | --- | --- | --- |
| availabilityTriggerMode | ENUM { nwControlled(0), ueAutonomous(1) } | nwControlled | Decides which entity evaluates buffer & power thresholds |

Observation 2-3
The spec provides only an on/off availability flag; it cannot convey why logging paused.

Proposal 2-3
Extend UEAssistanceInformation and UAI with:

| Field | Type / Size | Bit Mapping |
| --- | --- | --- |
| resourceConstraintCause | BIT STRING (SIZE 3) | b0 memLow, b1 powerLow, b2 cpuBusy |

Retention & State / Mobility Handling

Observation 3
Without a deterministic location and expiry timer, vendors differ on when logs are purged.

Proposal 3

- 3-1 Add Boolean retainLoggedData in source-cell RRCReconfiguration sent before HO execution. Include retentionTimer (24–168 h, default 48 h) inside AIML-LoggingConfig. UE discards retained logs when the timer expires or after HO/RRC_IDLE/RLF.

Observation 3-a
logMeasEntry and aiLogMeasEntry lack a session tag.

Proposal 3-a
Prepend 5-bit logConfigID to both entry structures:

aiLogMeasEntry-v19a0 ::= SEQUENCE {

logConfigID   INTEGER (0..31),

...           -- existing fields

}

Data Format & Metadata

Observation 4
Message-level context alone cannot reassemble interleaved segments.

Proposal 4
Add fixed header to every UEInformationResponse segment:

AIMLLogHeader ::= SEQUENCE {

logConfigID  INTEGER (0..15),

segmentNo    INTEGER (0..255),

lastSegment  BOOLEAN,

logPriority  ENUM {normal, high},

moreSegmentsPending BOOLEAN

}

Add cellId, gapInd, and optional entryPriority to each LoggedMeasurementEntry.

Transport Bearer & SRB Use

Observation 5
Single LCG on SRB-4 can create head-of-line blocking.

Proposal 5
Map AI/ML log PDUs on SRB-4 to new logicalChannelGroup, (aiModelData); MDT remains on its existing LCG. Scheduler may throttle using moreSegmentsPending.

When moreSegmentsPending=1 UE continues using SRB-4 / aiModelData-LCG until all segments sent

Cross-layer RS Referencing

Observation 6
Spec provides no mechanism to reference or inherit RS sets.

Proposal 6
Add optional linkedCSI-ReportConfigId (INTEGER(0..255)) inside AIML-LoggingConfig. If present, UE inherits RS definitions from that CSI-ReportConfig; else it uses explicit RS IDs.

Proposal 6-a

Liaison RAN-1 request: expose stable rsSetId in CSI-ReportConfig ASN.1.

Memory / Buffer Limits

Observation 7
A method needed to Distinguish, low-cap UEs over-trigger and high-cap UEs under-trigger.

Proposal 7

- 7-1- Require ≥ 32 kB dedicated AS-layer memory for AI logs; UE declares bufferSizeCap in UECapabilityInformation-AI.

- 7-2 Signal bufferThreshold in absolute bytes; UE clamps to 50–90 % of declared capacity.

Combined Trigger-Mask Field

Observation 8
No unified mask lets the network enable/disable multiple triggers per session.

Proposal 8
Inside LoggingTriggerConfig, add triggerMask BIT STRING (SIZE (4)) — bits {periodic, X1, X2, lowPower}.
UE logs when any enabled bit condition is satisfied.

# 4 References