3GPP TSG-RAN WG2 #130	R2-2504655

# St. Julian’s, Malta, May 19th – 23rd, 2025

Agenda Item:	8.9.4

Source:	Ericsson

Title:	Enhancements to support PWS in NB-IoT NTN

Document for:	Discussion, Decision

# 1	Introduction

The Rel-19 WID on IoT-NTN [1] was updated in RAN#105 with the following additional objective:

- Support broadcast of PWS messages for NB-IoT re-using the LTE mechanisms [RAN2]

Note: The solutions will be developed for NTN but can be applicable to TN (if possible). Specific NB-IoT TN optimizations will not be considered as part of this WI.

Note: Support for indication of intended service area for ETWS will be considered based on NR NTN discussions/outcome.

In this contribution we focus on some of the open issues listed by the IoT NTN RRC rapporteur.

# 2	Discussion

## 2.1	Enhancements to eDRX

Clarify in the spec (and at least in 36.300) that for NB-IoT the ETWS, CMAS, PWS requirement may not be met in some scenarios, e.g. when the UE is in eDRX (can come back to further clarify the specific cases).

During RAN2#127bis, it was agreed to include a disclaimer on PWS performance in eDRX as follows:

While such a disclaimer is reasonable for specific scenarios, it is important to emphasize that PWS is a safety-critical mechanism and should not be treated as a best-effort service. This could lead to dangerous situations where users expect to receive warning messages, but the UE is not actively monitoring these notifications or changes in broadcast signaling during emergencies. In the initial commercial deployments of NTN, there is a clear preference for using NB-IoT to provide messaging and emergency services via satellite [2]. Although regular 5G NR UEs will continue to operate in terrestrial cells, they may also be equipped with NB-IoT NTN modems to cover areas lacking terrestrial network connections, such as remote regions. In this context, one of the foundational paradigms of NB-IoT—battery-constrained operation—may not be as critical as in previous releases. These UEs might prioritize better service delivery over energy-saving configurations.

- First commercial NB-IoT NTN UEs may not be battery constrained and may prioritize better service delivery.

An example of improved service delivery is the timely reception of PWS emergency notifications. As discussed earlier, this is challenging or even impossible when an NB-IoT UE employs certain power-saving mechanisms like eDRX. Therefore, it may be advantageous for such NB-IoT UEs to inform the network that they are not battery-constrained and are interested in specific services, such as PWS. For instance, the UE may send a UE Assistance Information message to the network to inform about its preference to receive and prioritize PWS reception over power-saving optimizations in the cell. Based on the UE's preferences, the network can adjust the assigned power-saving configurations to align with the UE's specific interests, such as receiving timely PWS notifications.

A UE interested in receiving PWS may indicate its interest to the network to adjust the assigned power saving configuration.

## 2.2	SIB1-NB acquisition

During RAN2#127bis, the acquisition of SIB1-NB upon receiving an emergency message was briefly discussed and identified as an area requiring further study. In emergency situations, acquiring SIB1-NB is essential to understand the new System Information scheduling configuration, enabling the UE to determine how and when to acquire the emergency SIBs that contain the emergency notification details. Given the lengthy time required for this process (see Table 1 in the Annex), RAN2 address this issue, particularly for time-sensitive cases like the ETWS primary notification. This is because for the ETWS primary notification, the requirements for delivering the ETWS primary notification are stringent, necessitating its arrival at the UE within 4 seconds. Therefore, efforts should focus on optimizing this system. For instance, SIB10-NB for ETWS primary notification could be broadcasted with predefined periodicity and SI window position, or SIB1-NB includes the scheduling information of SIB10-NB even when it is not broadcast. By this the UE can skip SIB1-NB acquisition when receiving the indication about ETWS primary notification. Other notifications, such as the ETWS secondary notification, CMAS, or EU-alert, do not have such stringent requirements and can continue to use existing procedures for SIB acquisition.

RAN2 enables a NB-IoT UE to skip SIB1-NB acquisition when receiving the ETWS primary notification.

## 2.3	Reception of PWS segments when switching serving cell

Segmentation may be applied for the delivery of ETWS secondary notification and CMAS notification. Upon entering a cell during RRC_IDLE, following successful handover or upon connection re-establishment, or if either the received value of messageIdentifier or of serialNumber or of both are different from the current stored values, the UE discards any previously buffered warningMessageSegment. In NTN, the seving cell/satellite changes frequently due to the cell/satellite movement and a UE may not be able to receive all warningMessageSegments before switching serving cell/satellite. With the legacy warningMessageSegment discarding mechanism, the UE may fail to receive ETWS secondary notification and CMAS notification.

- Frequent cell changes in NTN may lead to the UE discarding warningMessageSegment and potentially a failure in receiving an emergency notification.

In line with the proposals from other companies last meeting, we think that the UE should be allowed to assemble warningMessageSegments received from different cells/satellites. In case of the transparent architecture, it is likely that satellites serving the same area would be connected to the same gNB. Therefore, it is possible for the network to deliver the warningMessageSegments with the same messageIdentifier, serialNumber and segment size. Provided it is the same gNB upon cell or satellite switch, there is no need for inter-gNB coordination nor impact in other working groups. Finally, the UE should be informed to decide whether it should discard (legacy behavior) or keep previously buffered warningMessageSegments.

Provided it is served by the same gNB, the UE should be allowed to assemble warningMessageSegments received from different cells/satellites.

The network can indicate whether the UE should discard or continue using the previously buffered warningMessageSegments upon cell/satellite switch.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	First commercial NB-IoT NTN UEs may not be battery constrained and may prioritize better service delivery.

Observation 2	Frequent cell changes in NTN may lead to the UE discarding warningMessageSegment and potentially a failure in receiving an emergency notification.

Based on the discussion in the previous sections we propose the following:

Proposal 1	A UE interested in receiving PWS may indicate its interest to the network to adjust the assigned power saving configuration.

Proposal 2	RAN2 enables a NB-IoT UE to skip SIB1-NB acquisition when receiving the ETWS primary notification.

Proposal 3	Provided it is served by the same gNB, the UE should be allowed to assemble warningMessageSegments received from different cells/satellites.

Proposal 4	The network can indicate whether the UE should discard or continue using the previously buffered warningMessageSegments upon cell/satellite switch.

# 4	References

RP-242397, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, RAN#105, Sep. 2024.

R2-1910428, Ericsson, ETWS/CMAS notification delivery in connected mode for non-BL UEs in CE, RAN2#107, Czech Republic, May 2019.

R4-1609680, Ericsson, Simulation result of MIB-NB/SIB1-NB/SIB2-NB acquisition time, RAN4#81, USA, November, 2016.

https://samsungmagazine.eu/en/2024/04/19/samsung-oficialne-predstavil-svuj-prvni-5g-modem-s-obousmernym-satelitnim-pripojenim/

# Annex

## Time budget for the delivery of emergency notification

The prompt and reliable delivery of emergency notifications is critical for their effectiveness. Within 3GPP, both SA1 and RAN2 have recognized the challenges associated with meeting the stringent delivery requirements of PWS. A notable example is the ETWS primary notification, which is mandated to be delivered within 4 seconds. However, this requirement presents significant challenges in the context of NB-IoT devices, where the extended DRX periods could be as long as 3 hours. This discrepancy highlights the need for a thorough evaluation of current protocols and potential enhancements to ensure timely emergency notifications, even in constrained scenarios. Following a previous analysis in [4], we provide a detailed time budget for the delivery of the ETWS primary notification, comprising the following steps:

- Initial actions based on UE’s RRC mode:

  - RRC_CONNECTED:

    - Time to wait during C-DRX sleep mode depending upon its configuration.

    - Acquiring the Direct Indication message with up to 256 repetitions on NPDCCH.

  - RRC_IDLE:

    - Wait for at least a paging cycle in DRX. For simplicity, eDRX is not considered in this analysis.

- Acquisition of SIB1-NB to obtain the scheduling information of ETWS related SIBs (e.g., SIB10-NB).

- Acquisition of SIB10-NB which contains the ETWS primary notification.

For the first step, if the UE is in RRC_CONNECTED, the minimum and maximum values of the long DRX cycle are 256 and 10240 subframes, respectively. The longer values pose a challenge to the 4-second requirement. The acquisition of the Direct Indication may involve up to 32 NPDCCH repetitions in CE mode A and 2048 NPDCCH repetitions in CE mode B.

If the UE is in RRC_IDLE, the minimum and maximum values of the paging cycle are 128 and 1024 radio frames, respectively. The exact number of paging occasions within a paging frame depend on the specific network configuration. In any case, the longer values pose a challenge to the 4-second requirement. The scheduling of the paging message may involve up to 32 NPDCCH repetitions in CE mode A and 2048 NPDCCH repetitions in CE mode B. The acquisition of the paging message may involve up to 32 NPDSCH repetitions in CE mode A and 2048 NPDSCH repetitions in CE mode B.

In the second step, the acquisition of SIB1-NB with 16 repetitions may take less than 30.72 seconds in CE Mode A, and longer than 40.96 seconds in CE Mode B, while in normal coverage up to around 25 seconds [3]. We assume that the UE may start immediately the acquisition of SIB1-NB.

For the final step, the acquisition of SIB10-NB depends on the scheduling periodicity (64, 128, 256, or 512 frames) and the number of repetitions (every 2nd, 4th, 8th, 16th radio frames). Considering PWS SIBs are only scheduled during an emergency, we can assume that network configures a short periodicity in order to deliver them quickly. The contents of SIB10 would be less than 500 bits. Based on the simulations for SIB2 in [2], it can be estimated that this step may take less than 1.28s in CE Mode A, and less than 2.56s in CE Mode B.

Table 1. Time budget estimation for ETWS primary notification delivery time for RRC_CONNECTED mode.

|  | Long DRX | NPDCCH | SIB1-NB | SIB10-NB |
| --- | --- | --- | --- | --- |
| CE Mode A | 2.56 s | 0.032s | 30.72 s | 1.28s |
| CE Mode B | 2.56 s | 2.048s | 40.96 s | 2.56s |

The time budget estimation for the best-case scenario (RRC_CONNECTED) is summarized in Table 1. The results reveal two significant bottlenecks: DRX and SIB1-NB acquisition. Without enhancements, it may be impossible to meet the existing requirements for PWS notification delivery. In the following sections, we will discuss alternatives to address these bottlenecks within the context of both NTN and TN.