3GPP TSG-RAN WG2 #130	R2-2504657

# St. Julian’s, Malta, May 19th – 23rd, 2025

Agenda Item:	7.0.2.18

Source:	Ericsson

Title:	In-band operation for NB-IoT

Document for:	Discussion, Decision

# 1	Introduction

RAN2 has received an LS [1] from RAN4 about the introduction of inband operation for NB-IoT NTN and the potential RAN2 impact.

1. Overall Description:

RAN4 introduced the inband operation for NTN IoT in NR NTN in TEI18. RAN4 has assumed there is no RAN2 impact. RAN4 has made below agreements

- In Rel-18, UE is only required to support the same operation mode for anchor and non-anchor carriers

- For in-band operation, RRC signalling should indicate guardband-r13 in operationModeInfo IE in MasterInformationBlock-NB during UE conformance tests.

- in Rel-18 for in-band operation, UE is not expected to access a cell indicating inband-SamePCI or inband-DifferentPCI in operationMode IE in MasterInformationBlock-NB.

RAN4 would like to respectfully ask RAN2 to confirm above assumptions aligning with RAN2 specification.

2. Actions:

To RAN WG1/WG2 group.

ACTION: RAN4 would respectfully ask RAN2 to provide answer on the above.

# 2	Discussion

RAN4 has recently introduced support for inband operation mode in Release 18. The other operation mode supported in NTN is standalone mode. More specifically, this means that there were no RAN4 requirements for inband operation in Rel-17 and these are now added in Rel-18. RAN2 initially discussed RAN4’s incoming LS during RAN2#129bis without reaching a consensus on the potential specification impact. As a result, an initial reply LS was sent to RAN4 confirming the feasibility of all RAN4 assumptions [2].

- RAN2 has already confirmed the technical feasibility of RAN4 assumptions.

In their LS, RAN4's initial assumption specifies that “the UE is only required to support the same operation mode for anchor and non-anchor carriers.” During a related offline discussion [3], the majority of RAN2 companies agreed that a clarification in Stage 2 would be beneficial to clearly capture which deployment modes are currently supported in NTN. Two alternative text proposals were suggested:

- A new table (e.g., Table 5.5a-1) which includes the specific deployment combinations that are supported for each Release for NB-IoT NTN.

- A general informative note at the end of the existing Table 5.5a-1 stating that only the same operation mode for anchor and non-anchor carrier is supported in NTN for this release of the specification.

Provided the intention with the specification change is clarificatory, we consider the note to be sufficient. In addition, a note offers better forward compatibility. For example, if guardband operation mode is supported in future releases for NB-IoT NTN by RAN4, the note can be updated or removed. In contrast, if a new table for NTN is introduced, it needs to be also subsequently revised and, when all operation modes are supported, it would be identical as the existing table for TN.

Clarify in Stage 2 that only the same operation mode for anchor and non-anchor carrier is supported in NTN.

Following option a), introduce a note in Table 5.5a-1 to clarify operation mode in NB-IoT NTN.

RAN4's second assumption specifies that “for in-band operation, RRC signaling should indicate guardband-r13 in operationModeInfo IE in MasterInformationBlock-NB during UE conformance tests.” Conformance tests are designed to ensure that devices meet the technical requirements specified by the standard and are defined in RAN5 specifications (e.g., TS 36.521-1), along with the (RRC) parameters necessary for each case. RAN4 has a section about conformance tests. In addition, in the related offline [3], it was also the majority view that this assumption does not have any RAN2 impact.

- RAN4's assumption that RRC should indicate guardband-r13 in operationModeInfo IE in MIB-NB does not have stage 3 impact in RAN2 specifications.

There are no stage 3 issues related to the introduction of in-band support for NB-IoT.

# 4	Conclusion

In the previous sections we made the following observations:

Observation 1	RAN2 has already confirmed the technical feasibility of RAN4 assumptions.

Observation 2	RAN4's assumption that RRC should indicate guardband-r13 in operationModeInfo IE in MIB-NB does not have stage 3 impact in RAN2 specifications.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Clarify in Stage 2 that only the same operation mode for anchor and non-anchor carrier is supported in NTN.

Proposal 2	Following option a), introduce a note in Table 5.5a-1 to clarify operation mode in NB-IoT NTN.

Proposal 3	There are no stage 3 issues related to the introduction of in-band support for NB-IoT.

# 4	References

[1]	R4-2503035, LS on NR NB-IoT in-band operation, Ericsson, RAN4#114, February 2025.

[2]	R2-2503064, Reply LS on NR NB-IoT in-band operation, Ericsson, RAN2#129bis, April 2025.

[3]	R2-2503053, Report of [AT129bis][302][TEI18] In-band operation for NB-IoT (Ericsson), Ericsson, RAN#129bis, April 2025.