3GPP TSG-RAN WG2 #130	R2-2504660

# St. Julian’s, Malta, May 19th – 23rd, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.304 | CR | 0437 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Removal of PWS support in NR NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_enh-Core |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As per the CT1 LS in R2-2501704, the support for PWS over satellite would only start from Rel-19 due to the UE impacts that would create backward compatibility issues. However, in section 4.5, it is specified that an acceptable cell is such where the UE can obtain limited service (emergency calls and ETWS and CMAS notifications). Both emergency calls and PWS are requirements for the definition of an acceptable cell. Provided there is no support for PWS in NTN prior to Release 19, the NTN UE should consider a cell acceptable when it can receive emergency calls, i.e. limited service does not refer to PWS messages. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce a clarification that PWS is not supported for NR NTN in this release of the specification and that limited service within an acceptable cell definition refers only to emergency calls. Impact analysis Impacted functionality: PWS support and idle mode operation in NR NTN. Inter-operability: There is no inter-operability issues foreseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If the UE does not implement this new definition of acceptable cell, it may misinterpret that an NTN cell cannot classify as an acceptable cell since there is not support for PWS prior to Release 19. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

## 4.5	Cell Categories

The cells are categorised according to which services they offer:

The cells are categorised according to which services they offer:

acceptable cell:

An "acceptable cell" is a cell on which the UE may camp to obtain limited service (originate emergency calls and receive ETWS and CMAS notifications). Such a cell shall fulfil the following requirements, which is the minimum set of requirements to initiate an emergency call and to receive ETWS and CMAS notification in an NR network:

-	The cell is not barred, see clause 5.3.1;

-	The cell selection criteria are fulfilled, see clause 5.2.3.2.

The reception of ETWS and CMAS notifications is not required by the UE in NTN in this version of the specification and limited service only refers to the initiation of emergency calls.

suitable cell:

For UE not operating in SNPN Access Mode, a cell is considered as suitable if the following conditions are fulfilled:

-	The cell is part of either the selected PLMN or the registered PLMN or PLMN of the Equivalent PLMN list, and for that PLMN either:

-	The PLMN-ID of that PLMN is broadcast by the cell with no associated CAG-IDs and CAG-only indication in the UE for that PLMN (TS 23.501 [10]) is absent or false;

-	Allowed CAG list in the UE for that PLMN (TS 23.501 [10]) includes a CAG-ID broadcast by the cell for that PLMN;

-	The cell selection criteria are fulfilled, see clause 5.2.3.2.

According to the latest information provided by NAS:

-	The cell is not barred, see clause 5.3.1;

-	The cell is part of at least one TA that is not part of the list of "Forbidden Tracking Areas for Roaming" (TS 22.011 [18]), which belongs to a PLMN that fulfils the first bullet above.

For UE operating in SNPN Access Mode, a cell is considered as suitable if the following conditions are fulfilled:

-	The cell is part of the selected SNPN or the registered SNPN or SNPN of the Equivalent SNPN list of the UE;

-	The cell selection criteria are fulfilled, see clause 5.2.3.2;

According to the latest information provided by NAS:

-	The cell is not barred, see clause 5.3.1;

-	The cell is part of at least one TA that is not part of the list of "Forbidden Tracking Areas for Roaming" which belongs to the selected SNPN or the registered SNPN or SNPN of the Equivalent SNPN list of the UE.

barred cell:

A cell is barred if it is so indicated in the system information, as specified in TS 38.331 [3].

reserved cell:

A cell is reserved if it is so indicated in system information, as specified in TS 38.331 [3].

Following exception to these definitions are applicable for UEs:

-	if a UE has an ongoing emergency call, all acceptable cells of that PLMN/SNPN are treated as suitable for the duration of the emergency call.

-	camped on a cell that belongs to a tracking area that is forbidden for regional provision of service; a cell that belongs to a tracking area that is forbidden for regional provision service (TS 23.122 [9], TS 24.501 [14]) is suitable but provides only limited service.

-	if the UE in RRC_IDLE fulfils the conditions to support NR sidelink communication/discovery or V2X sidelink communication in limited service state as specified in TS23.287 [16] clause 5.7, the UE may perform NR sidelink communication/discovery or V2X sidelink communication.

NOTE:	UE is not required to support manual search and selection of PLMN or CAG or SNPN while in RRC CONNECTED state. The UE may use local release of RRC connection to perform manual search if it is not possible to perform the search while RRC connected.

END OF CHANGE