3GPP TSG RAN WG2 Meeting #130

St Julian's, Malta, 19 - 23 May, 2025

R2-2504663

Agenda Item:	8.1.4

Source:	Rakuten Mobile

Title:	Discussion on UE Side Data collection

Document for:	Discussion and Decision

# Introduction

This contribution discusses additional aspects of UE side data collection especially focusing on how to control the UE Side data collection for AIML Training.

# Applicability & Session Control.

Current RAN2 AIML CR Text list number of FFS and also have several functional gaps in the UE-side data-collection control framework:

- UEs report that AI functionality is inapplicable, but the cause is not standardized yet. Hence,  NG-RAN have certainty that UAI was received before UE activates inference, leading to timing mismatches.

- UEs may choose between network-offered candidate configs. Current signaling lacks a mechanism to express this selection or the preferred time window for collection.

- No mechanism for a UE to reference one preferred configuration (or time-window) among multiple candidates presented by the network;

- There is absence of a network-configurable low-power threshold, that may result in different Vendor UE’s behave differently.

- After HO or RRCResume, UE-side logging state may be different from gNB expectations. Per-session applicability reporting is required for better understanding.

- UEs currently erase logs based on timer expiry even if they were never uploaded, it may be removed before delivery.

- After RLF or restart of UE the gNB receives no autonomous indication of any residual log buffer retained by the UE.

Adressing these items will help to have predictable radio-resource usage, consistency in data availability aswell as  interoperable behaviour in multi-vendor deployments.

Observation 1: it is still FFS in RAN2 whether the UE should report explicit causes for inapplicability also UE activation is not related to network receipt of UAI.

Proposal 1:
Add optional inappCause ENUM { modelNotFound, envMismatch, resourceConflict, UEpolicy } in applicability report.

Introduce T_wait-UAI (FFS, e.g., 20 ms). UE activates inference only after:

- Receiving ACK for UAI; or

- T_wait-UAI expires.

Observation 2: The UE cannot indicate a preferred configuration index, intent or collection window when sending a data-collection start/stop request.

Proposal 2:
Extend AIDataCollectionRequest to include

- preferredConfigIndex (INTEGER 0..15)

- collectionIntent (ENUM {training, monitoring})

- startStopFlag (BOOLEAN) OPTIONAL

- preferredWindow { startOffset, duration } OPTIONAL

Observation 3: No standard low-power trigger or clearance mechanism exists.

Proposal 3:
RAN2 introduce optional lowPowerThreshold (percentage) in TrainingDataCollectionConfiguration after confirmation with RAN1.
The UE shall raise lowPower = TRUE when SOC ≤ threshold and clear it only after SOC ≥ threshold + 5 pp.

Observation 4: Applicability is not re-established after mobility events.

Proposal 4:UE includes per-logConfigID applicability in UAI after Resume/Re-establishment,
When sending UAI after RRC ResumeComplete or RRC Re-establishmentComplete, the UE shall include one ApplicabilityReportConfig per active logConfigID.

Observation 5: Log-retention expiry is independent of delivery success.

Proposal 5:
Add field logDeliveryStatus (ENUM {success, failed, notAttempted}) to UEInformationResponse.
Logged data will be kept until both retentionTimer has expired and/or logDeliveryStatus = success.

Observation 6: Following RLF or power-cycle the gNB is unaware of any retained data unless it re-configures the UE.

Proposal 6:
Introduce UL RRC message AIDataCollectionStatusReport carrying logConfigID, bufferSizeBytes, lastSampleTime, optional inappCause.
The UE may transmit this message autonomously immediately after RRC ResumeComplete or HO Complete when no context for the active log exists in the serving gNB.

## Data-Collection Configuration & Logging Schema

UE-side data logging in Rel-19 is a generic framework applicable to all use cases including beam-management, CSI and future AI/ML use-cases. In under discussion CRs several foundational items remain “FFS”:

- How to have explicit indication of measurement gaps, Interruptions (e.g., due to power save or radio loss) can cause timestamp gaps in logs. These must be flagged to avoid misleading AI models. (Chair Notes #129bis);

- Discussion on payload efficiency (header aggregation / compression);

- Configuration of sub-sampling factor independent of RS periodicity;

- AS-layer memory requirements to guarantee at hand-over data retention;

- Cause code for availability indication

- Observation 7: LoggedMeasurementReport provides no mechanism to indicate discontinuity in the measurement sequence.

- Proposal 7:
Add optional fields gapMarkerPresent (BOOLEAN) and maxGapDuration (INTEGER in ms) to aiLogMeasEntry
When the UE resumes logging after any pause ≥ maxGapDuration, it sets gapMarkerPresent = TRUE in the first sample after the gap.

- Observation 8: Current framing conveys one measurement per PDCP SDU, producing excessive header overhead; no aggregation container is defined for AI/ML logs.

- Proposal 8:
RAN2 to introduce AI-Log Aggregation Container (ALAC) in TS 38.323.

  - Header elements: sampleCount (4 bits, value 1-16) and deltaEncoding (1 bit).

  - UEs advertise capability pdcp-AIlog-aggregation; NG-RAN enables the mode via ALAC-Allowed in DRB-Config. Aggregation is transparent to RLC/MAC.

- Observation 9: Logging periodicity is implicitly locked to RS transmission periodicity, there is no parameter exists to change that.

- Proposal 9:
Add in  TrainingDataCollectionConfiguration loggingInterval element.
Value 1 → UE logs every RS occasion (legacy behaviour).
Value N → UE logs every N-th RS occasion,

- Observation 10: UAI reporting behavior doesn’t have semantic clarity, making it difficult for NG-RAN to distinguish between cases when buffer is exhausted or configured buffer thresholds, and power-based issues. Standardized cause coding ensures interoperable handling and supports policy based scheduling of AIML data.

- Proposal 10:
UAI triggering based on buffer thresholds and cause reporting

Extend TrainingDataCollectionConfiguration with the following optional parameters:

bufferThresholdBytes INTEGER OPTIONAL
— Defines the logging buffer level (in bytes) at which the UE shall consider availability reporting.

availabilityCauseCoding ENUM { fullBuffer, thresholdReached, lowPower } OPTIONAL
— Indicates the condition under which the UE triggered availabilityIndication = TRUE.

Capture behaviour in Specifications:
The UE shall trigger UEAssistanceInformation with availabilityIndication = TRUE when any of the following apply:

- The configured bufferThresholdBytes is reached or exceeded;

- The logging buffer is full (if no threshold is configured);

- The UE enters a defined low-power state.

- The availabilityCauseCoding field in the report shall reflect the reason for triggering the indication. This enables the network to differentiate between resource-based and energy-based constraints and respond appropriately.

## Data-Transfer & Traffic-Management Framework

AI/ML logging introduces sizeable user-plane bursts, RAN2 129Bis  “whether/how NG-RAN can be aware of AI/ML dataset transfer” and “how partial or opaque payloads should be treated” . At the same time, several companies propose that NG-RAN reuse 5GC admission parameters to control training traffic , while others insist on a full-visibility policy .

Based on the discussion, Rel-19 therefore requires a unified framework providing:

- an operator-selectable visibility policy for standardised data vs opaque data;

- an admission gate to avoid cell from being overloaded,

- A mechanism for NG-RAN to throttle ongoing uploads during congestion.

- A method to place bulk AI/ML data on the user plane rather than on SRB1/2.

- Observation 11 There is signalling element for MNO to mandate “standard-only” versus “mixed/opaque” dataset content.

- Proposal 11 (Visibility policy)
Introduce visibilityLevel (2 bits) in DRB-Config:
00 standard-only; 01 mixed; 10 opaque-permitted; 11 reserved.
UE behaviour shall comply with the configured level; NG-RAN may reject establishment if the requested level conflicts with local policy.

- Observation 12 There is no AI/ML-specific admission criteria for PDU-admission procedure; training bursts may be admitted regardless of cell load which is unacceptable for operators.

- Proposal 12(Admission control)
When AIML-TrafficPurpose = training, the gNB shall apply PDU-admission parameters conveyed by 5GC (AIML-AdmissionPolicy).

- If cell capacity for AI traffic is exceeded, the gNB rejects the DRB with cause cellCapacityAIexceeded (new cause value, TS 38.331).

- Observation 13 Control-plane carrying of large AI/ML datasets risks saturating SRB1/2; Rel-19 scope for CP payload still not defined.

- Proposal 13 (Control-plane boundary)
Add normative Note in 38.331 that 
“UE-side data-collection payload exceeding 2048 octets shall be transferred by user-plane options (1a / 2 / 3). Control-plane carriage is restricted to supervision or status messages only; no new SRB type for bulk AI/ML data is introduced in Rel-19.”

- Observation 14NG-RAN has no explicit mechanism to request that the UE suspend or throttle an ongoing training upload during transient congestion.

- Proposal 14 (Upload-throttling indicator)
Define AIUploadThrottlingIndicator (MAC CE or RRC IE, FFS) with values {pauseImmediately, reduceRate, resume}. NG-RAN may signal this indicator to UEs that have an active AIML-TrafficPurpose = training DRB.

- Proposal 14-b UEs need to comply within one HARQ RTT for MAC-level signalling or one RRC RTT for RRC-level signalling.

## Mobility, Context Transfer

RAN2 has previously agreed that UE-side data-collection shall survive RRC state transitions and intra-frequency handovers, as recorded in the retention flag included in RRCReconfiguration messages. However, a number of remaining gaps are identified by the open-issue list and CR notes:

- the HO Preparation message (sent from source to target gNB) contains no indication of ongoing AI logging activity;

- the target gNB lacks a mechanism to query UE logging state for example buffer length, active configuration.

- if two logging sessions are configured independently such as  CSI-logging and beam-prediction, there is no field to identify which session remains applicable after handover;

Observations

Observation 15 HO Preparation signalling contains no indication that UE-side logging is active.

Proposal 15 (HO context flag)
Introduce BOOLEAN field AIML-LoggingContextActive in HandoverPreparationInformation (TS 38.331).
If TRUE, the target gNB shall expect that UE-side data logging is ongoing.

Observation 16 The target gNB cannot obtain the current logging configuration, applicability status, or buffer state following handover.

Proposal 16 (Post-HO status exchange)
Define DL RRC message AIMLDataCollectionStatusRequest and UL AIMLDataCollectionStatus.
The UE shall respond with:

- logConfigID

- applicability (BOOLEAN)

- bufferSizeBytes

- lastSampleTime

- inappCause (if applicable)

This procedure is initiated by the target gNB after HO execution and may be used upon RRC ResumeComplete as well.

Observation 17 No per-session identifier exists for distinguishing which of several logConfigIDs is active or applicable after mobility.

Proposal 17 (Session identification in post-HO UAI)
In UAI triggered after HO or Resume, the UE shall include logConfigID and applicability, for clarity during multiple sessions.

# Conclusion

This contribution discusses the issues in terms of UE Side Data collection for AIML training  RAN2 is kindly asked to take into account the proposals below:

Observation 1: it is still FFS in RAN2 whether the UE should report explicit causes for inapplicability also UE activation is not related to network receipt of UAI.

Proposal 1:
Add optional inappCause ENUM { modelNotFound, envMismatch, resourceConflict, UEpolicy } in applicability report.

Introduce T_wait-UAI (FFS, e.g., 20 ms). UE activates inference only after:

- Receiving ACK for UAI; or

- T_wait-UAI expires.

Observation 2: The UE cannot indicate a preferred configuration index, intent or collection window when sending a data-collection start/stop request.

Proposal 2:
Extend AIDataCollectionRequest to include

- preferredConfigIndex (INTEGER 0..15)

- collectionIntent (ENUM {training, monitoring})

- startStopFlag (BOOLEAN) OPTIONAL

- preferredWindow { startOffset, duration } OPTIONAL

Observation 3: No standard low-power trigger or clearance mechanism exists.

Proposal 3:
RAN2 introduce optional lowPowerThreshold (percentage) in TrainingDataCollectionConfiguration after confirmation with RAN1.
The UE shall raise lowPower = TRUE when SOC ≤ threshold and clear it only after SOC ≥ threshold + 5 pp.

Observation 4: Applicability is not re-established after mobility events.

Proposal 4:UE includes per-logConfigID applicability in UAI after Resume/Re-establishment,
When sending UAI after RRC ResumeComplete or RRC Re-establishmentComplete, the UE shall include one ApplicabilityReportConfig per active logConfigID.

Observation 5: Log-retention expiry is independent of delivery success.

Proposal 5:
Add field logDeliveryStatus (ENUM {success, failed, notAttempted}) to UEInformationResponse.
Logged data will be kept until both retentionTimer has expired and/or logDeliveryStatus = success.

Observation 6: Following RLF or power-cycle the gNB is unaware of any retained data unless it re-configures the UE.

Proposal 6:
Introduce UL RRC message AIDataCollectionStatusReport carrying logConfigID, bufferSizeBytes, lastSampleTime, optional inappCause.
The UE may transmit this message autonomously immediately after RRC ResumeComplete or HO Complete when no context for the active log exists in the serving gNB.

Observation 7: LoggedMeasurementReport provides no mechanism to indicate discontinuity in the measurement sequence.

Proposal 7:
Add optional fields gapMarkerPresent (BOOLEAN) and maxGapDuration (INTEGER in ms) to aiLogMeasEntry
When the UE resumes logging after any pause ≥ maxGapDuration, it sets gapMarkerPresent = TRUE in the first sample after the gap.

Observation 8: Current framing conveys one measurement per PDCP SDU, producing excessive header overhead; no aggregation container is defined for AI/ML logs.

Proposal 8:
RAN2 to introduce AI-Log Aggregation Container (ALAC) in TS 38.323.

Header elements: sampleCount (4 bits, value 1-16) and deltaEncoding (1 bit).

UEs advertise capability pdcp-AIlog-aggregation; NG-RAN enables the mode via ALAC-Allowed in DRB-Config. Aggregation is transparent to RLC/MAC.

Observation 9: Logging periodicity is implicitly locked to RS transmission periodicity, there is no parameter exists to change that.

Proposal 9:
Add in  TrainingDataCollectionConfiguration loggingInterval element.
Value 1 → UE logs every RS occasion (legacy behaviour).
Value N → UE logs every N-th RS occasion,

Observation 10: UAI reporting behavior doesn’t have semantic clarity, making it difficult for NG-RAN to distinguish between cases when buffer is exhausted or configured buffer thresholds, and power-based issues. Standardized cause coding ensures interoperable handling and supports policy based scheduling of AIML data.

Proposal 10:
UAI triggering based on buffer thresholds and cause reporting

Extend TrainingDataCollectionConfiguration with the following optional parameters:

bufferThresholdBytes INTEGER OPTIONAL
— Defines the logging buffer level (in bytes) at which the UE shall consider availability reporting.

availabilityCauseCoding ENUM { fullBuffer, thresholdReached, lowPower } OPTIONAL
— Indicates the condition under which the UE triggered availabilityIndication = TRUE.

Capture behaviour in Specifications:
The UE shall trigger UEAssistanceInformation with availabilityIndication = TRUE when any of the following apply:

The configured bufferThresholdBytes is reached or exceeded;

The logging buffer is full (if no threshold is configured);

The UE enters a defined low-power state.

The availabilityCauseCoding field in the report shall reflect the reason for triggering the indication. This enables the network to differentiate between resource-based and energy-based constraints and respond appropriately.

Observation 11 There is signalling element for MNO to mandate “standard-only” versus “mixed/opaque” dataset content.

Proposal 11 
Introduce visibilityLevel (2 bits) in DRB-Config:
00 standard-only; 01 mixed; 10 opaque-permitted; 11 reserved.
UE behaviour shall comply with the configured level; NG-RAN may reject establishment if the requested level conflicts with local policy.

Observation 12 There is no AI/ML-specific admission criteria for PDU-admission procedure; training bursts may be admitted regardless of cell load which is unacceptable for operators.

Proposal 12
When AIML-TrafficPurpose = training, the gNB shall apply PDU-admission parameters conveyed by 5GC (AIML-AdmissionPolicy).

If cell capacity for AI traffic is exceeded, the gNB rejects the DRB with cause cellCapacityAIexceeded (new cause value, TS 38.331).

Observation 13 Control-plane carrying of large AI/ML datasets risks saturating SRB1/2; Rel-19 scope for CP payload still not defined.

Proposal 13 
Add normative Note in 38.331 that 
“UE-side data-collection payload exceeding 2048 octets shall be transferred by user-plane options (1a / 2 / 3). Control-plane carriage is restricted to supervision or status messages only; no new SRB type for bulk AI/ML data is introduced in Rel-19.”

Observation 14 NG-RAN has no explicit mechanism to request that the UE suspend or throttle an ongoing training upload during transient congestion.

Proposal 14 
Define AIUploadThrottlingIndicator (MAC CE or RRC IE, FFS) with values {pauseImmediately, reduceRate, resume}. NG-RAN may signal this indicator to UEs that have an active AIML-TrafficPurpose = training DRB.

Proposal 14-b UEs need to comply within one HARQ RTT for MAC-level signalling or one RRC RTT for RRC-level signalling.

Observation 15 HO Preparation signalling contains no indication that UE-side logging is active.

Proposal 15
Introduce BOOLEAN field AIML-LoggingContextActive in HandoverPreparationInformation (TS 38.331).
If TRUE, the target gNB shall expect that UE-side data logging is ongoing.

Observation 16 The target gNB cannot obtain the current logging configuration, applicability status, or buffer state following handover.

Proposal 16 
Define DL RRC message AIMLDataCollectionStatusRequest and UL AIMLDataCollectionStatus.
The UE shall respond with:

- logConfigID

- applicability (BOOLEAN)

- bufferSizeBytes

- lastSampleTime

- inappCause (if applicable)

This procedure is initiated by the target gNB after HO execution and may be used upon RRC ResumeComplete as well.

Observation 17 No per-session identifier exists for distinguishing which of several logConfigIDs is active or applicable after mobility.

Proposal 17 
In UAI triggered after HO or Resume, the UE shall include logConfigID and applicability, for clarity during multiple sessions.

# Reference

- RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, TSG RAN, RAN#94-e, Dec. 2021

- TR 38.843 v18.0.0, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18),” Dec. 2023

- RAN2#128 meeting report

- R1-2310681, Reply LS on Data Collection Requirements and Assumptions, RAN1