3GPP TSG-RAN WG2 Meeting #130	R2-2504667

St.Julians, Malta, May. 19th – 23rd, 2025

Agenda Item	: 8.13.3

Source	: LG Electronics Inc.

Title	: Discussion on Control Plane Procedure for multi-hop U2N relay

Document for	: Discussion and Decision

1.	Introduction

The work item on NR sidelink multi-hop relay was approved for Rel-19 and the WID is shown in RP-242349 including the following objectives.

The objective of this work item is to specify solutions that are needed to support multi-hop Layer-2 UE-to-Network relay for a single indirect path via SL relay UEs based on Rel-17/18 SL relay functionalities [RAN2, RAN3] - Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay. The work starts with one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB). A necessary criterion for the specified mechanisms is easy extensibility to support two additional hop relays for the work done until RAN#107 and to be forward compatible for future extensions for additional relays. - Relay discovery and (re)selection [RAN2] - Signalling support for relay UEs and remote UE authorization if SA2 concludes it is needed [RAN3] - Impact on SRAP and QoS handling for multi-hop [RAN2] - Control plane procedures [RAN2, RAN3] - Specify the following intra-gNB service continuity scenarios for multi-hop U2N relay based on Rel-17/18 procedures (for remote UE): First Priority: - Intra-gNB multi-hop indirect to direct path switching using existing framework - Intra-gNB multi-hop indirect to single-hop indirect path switching using existing framework Second Priority in order of importance: - Intra-gNB direct to multi-hop indirect path switching - Intra-gNB single-hop indirect to multi-hop indirect path switching The scenarios C and D are limited to path switching to a target indirect path consisting of the last relay UE in “direct” RRC Connected mode and all the other intermediate relay(s) in “indirect” RRC Connected mode to the same cell. NOTE: The current existing measurement framework and existing data forwarding mechanism should be reused.

In this document, we mainly investigate the control procedure for fast/parallel setup procedure based on the last meeting (RAN2#129bis). Also, we propose the timer setting method for RRC establishment or stop conditions at the multi-hop relay.

2.	Discussion

2.1 Fast/parallel setup procedure considering RAN3 impact

In the legacy U2N relay, the gNB naturally knows which Remote UE is connected with which Relay UE before configuring SRB1. The CONNECTED Relay UE reports the L2 ID of the Remote UE and receives the local ID. Then the Relay UE sends the received UL SRB0 message receiving from the Remote UE to the gNB with the SRAP header including the local ID. Upon receiving the message with the SRAP header, the gNB knows that the UL SRB0 is coming from the Remote UE, which is connected with the Relay UE. Because it is a single-hop. When gNB configures SRB1, the gNB knows that the PC5 RLC channel configuration of the SRB1 will be used between the Remote UE and Relay UE.

In the multi-hop, if we use the fast/parallel setup procedure we discussed at the last meeting, the gNB does not know whether the Remote UE is connected with the last Relay UE by a single-hop or indirectly connected with the last Relay UE by a multi-hop. All the initial UL SRB0 messages from Remote UE(s) in the multi-hop are delivered via the last Relay UE. gNB cannot differentiate whether the SRB0 messages arrive via a single-hop or multi-hop. When gNB configures SRB1 for the Remote UE as legacy, gNB may interpret that the Remote UE is connected with the last Relay UE directly, such as a single-hop. The gNB considers that the PC5 RLC channel configuration for the SRB1 is used between the Remote UE and the last Relay UE. However, the PC5 RLC channel configuration of the SRB1 may be used between the Remote UE and the first Relay UE. So, if gNB does not know the path information, gNB may not manage the PC5 RCL channel configuration for SRB1 properly.

According to the RAN3 spec (TS 38.473), we can find the following description:

9.3.1.264 Sidelink Relay Configuration This IE provides information of a U2N Remote UE when accessing the network via a U2N Relay UE. IE/Group Name; Presence; Range; IE type and reference; Semantics description. gNB-DU UE F1AP ID of Relay UE; M; gNB-DU UE F1AP ID 9.3.1.5. Remote UE Local ID; M; 9.3.1.267. Sidelink Configuration Container; O; OCTET STRING; Includes the SL-ConfigDedicatedNR IE as defined in subclause 6.3.5 in TS 38.331 [8] to carry PC5 Relay RLC channel configuration for Remote UE’s SRB1..

The gNB uses the “SL-ConfigDedicatedNR” IE, which is included in the DL SRB0 message. If we use approach 1, the gNB can confirm that the IE is used between the Remote UE and the first Rely UE. However, in multi-hop with fast/parallel procedure, there is no way for the gNB to know that the “SL-ConfigDedicatedNR” IE will be used between the Remote UE and the first Relay UE. If the gNB doesn’t receive the information, the gNB will misunderstand that the RLC channel configuration for SRB1 is used between the Remote UE and the last Relay UE, since the initial SRB0 message is delivered only by the last Relay UE. This is not the correct understanding of the gNB. So, the gNB has to know that the SRB1 configuration is for the PC5 link between the Remote UE and the first Relay UE.

Also, RAN2 need to decides that SRB1 bearers (SL-RLC1) from multiple Remote UEs can be multiplexed at the PC5 link between intermediate Relay UE and first Relay UE. It’s the similar thing, the SRB1 bearers from multiple Remote UEs are multiplexed at Uu link between the U2N Relay UE and gNB.

Observation 1: In the legacy, gNB naturally knows which Remote UE has a connection with which Relay UE.

Observation 2: The gNB has to know the configured SL RLC channel configuration for SRB1 is used for which PC5 link to manage the PC5 RLC channel configuration.

Observation 3: In the multi-hop with fast/parallel setup procedure, gNB cannot know which PC5 link is associated with which Remote UE or intermediate Relay UE.

Observation 4: Considering RAN3 operation, the gNB configures SRB1 for the Remote UE upon receiving the initial SRB0 message of the Remote UE. To do this, gNB has to know that the SRB1 configuration is for the PC5 link between the Remote UE and the first Relay UE.

Proposal 1: Considering RAN3 operation, gNB has to know which PC5 link is used for the SRB1 when configuring SRB1.

Proposal 2: If the last Relay UE reports path information, gNB can configure SRB1 parallelly for each intermediate Relay UE and Remote UE.

Proposal 3: The SRB1 of the Remote UEs can be multiplexed over the egress PC5 RLC channel of the parent UE, as the SRB1 of the Remote UE can be multiplexed at the Uu link of the U2N relay UE in the legacy.

Then, how can the last Relay UE know the path information of each Remote UE? We think about the following fast/parallel setup procedure:

Figure 1. Multi-hop Relay scenario

Figure 1 shows an example of the multi-hop relay scenario. The last Relay UE is in RRC_CONNECTED, and the Remote UE, Relay UE1, and Relay UE2 are in RRC_IDLE/INACTIVE. Each Remote UE and Relay UEs are connected with a PC5 link. The source and destination L2 ID pair show the same alphabet, for example, the L2 IDs (i.e., source and destination) of the PC5 link between Remote UE and Relay UE1 are “L2 ID(A)” and “L2 ID(A’)”.

Figure 2. fast/parallel setup procedure for the last Relay UE to make infer the path information

Figure 2 shows the fast/parallel setup procedure for the upstream. It’s a very similar way we discussed at the last meeting. It has changed just the SRAP header for the initial UL SRB0 message to inform the path information for the last Relay UE.

Step 1) Remote UE sends the initial UL SRB0 message (e.g., RRCSetupRequest(a)) to the Relay UE1 as legacy.

Step 2) If the Relay UE1 is in RRC_INACTIVE/IDLE, it triggers sending the initial UL SRB0 message of itself (e.g., RRCSetupRequest(b)) to the Relay UE2 (step 2-2 in Figure 2).

Parallelly, the initial UL SRB0 message from the Remote UE is delivered with an SRAP header. The header includes the source L2 ID of the Remote UE and the source L2 ID of Relay UE1 (step 2-1 in Figure 2). The source means the upstream direction.

If the Relay UE1 is in RRC_CONNECTED, it acts the same as the legacy U2N Relay UE.

Step 3) If the Relay UE2 is in RRC_INACTIVE/IDLE, it triggers sending the initial UL SRB0 message of itself (e.g., RRCSetupRequest(c)) to the Relay UE2 (step 3-3 in Figure 2).

If the receiving message is just the initial UL SRB0 message (e.g., RRCSetupRequest(b)), the initial UL SRB0 message from the Relay UE1 is delivered with an SRAP header to the Relay UE2. If the Relay UE2 is in RRC_CONNECTED, the header is the same as the legacy, i.e., including bearer ID and local ID. If the Relay UE2 is in RRC_IDLE/INACTIVE, the header includes the source L2 IDs of both the Relay UE1 and the Relay UE2 (step 3-2 in Figure 2).

If the SRAP header already exists in the received SRB0 message, the Relay UE2 forwards it transparently to the last Relay UE, if the Relay UE2 is in RRC_IDLE/INACTIVE (step 3-1 in Figure 2). However, if the Relay UE2 is in RRC_CONNECTED, the Relay UE2 can report path information based on the received SRAP header and request the local ID for the Remote UE, such as the last Relay UE.

Step 4) When the last Relay UE receives from each relay UE and Remote UE, the last Relay UE can infer the path information. For example,

- the RRCSetupRequest(a) is related to the L2 ID(A), L2 ID(B) and L2 ID (C),

- the RRCSetupRequest(b) is related to the L2 ID(B) and L2 ID(C),

- the RRCSetupRequest(c) is related to the L2 ID(C), since it is directly connected with the last Relay UE.

Now, the last Relay UE can make the path information. The RRCSetupRequest(a) comes from the path A-B-C. The RRCSetupRequest(b) comes from the path B-C.

If the packet in step 3-1 arrives at the last Relay UE before arriving at the packet in step 3-2, the last UE cannot infer the path information. In this case, the last Relay UE waits until receiving the packet in step 3-2. In other words, if the last Relay UE cannot infer the path information, the last Relay UE waits for the reporting until to infer the path information.

The last Relay UE requests local ID allocation for each L2 ID to the gNB. gNB can make the mapping between which L2 ID is mapped to which local ID.

Step 5) gNB allocates the local ID according to the L2 ID. Each L2 ID belongs to each Remote UE or Relay UE. Also, gNB sends RRCReconfiguration for the last Relay UE to configure the mapping local ID and SL_RLC.

Step 6) The last Relay UE generates the SRAP header for the received initial UL SRB0 message of each Remote UE and Relay UE. And then it is delivered to the gNB. The SRAP header structure is the same as the legacy SRAP header.

Proposal 4: For the initial UL SRB0 message delivery, the following new SRAP header structure can be used for the CONNECTED intermediate or last Relay UE to infer the path information.

-  The L2 ID (A) is the source L2 ID of the Remote UE (the source means the uplink direction).

- The L2 ID (B) is the source L2 ID of the first Relay UE (the source means the uplink direction).

Figure 3. SRAP header structure for delivering the initial UL SRB0 message when the first Relay UE is in RRC_IDLE/INACTIVE

Proposal 5: The new SRAP header can be attached when the first Relay UE is in RRC_IDLE/INACTIVE.

Proposal 6: If the IDLE/INACTIVE intermediate Relay UE receives the new SRAP header, the intermediate Relay UE forwards it transparently.

Proposal 7: If the CONNECTED intermediate Relay UE or last Relay UE receives the initial UL SRB0 message with the new SRAP header, the intermediate Relay UE or last Relay UE reports the path information and requests local ID allocation to the gNB.

For the DL, gNB will configure the initial DL SRB0 message, which includes the SRB1 configuration for the Remote UE or intermediate Relay UE, and then send the message sequentially based on the path information. Even though the gNB sends the initial DL SRB0 message sequentially, but the RRCSetup message can be delivered almost parallelly. The message can be delivered in disorder. It can be a very rare case. However, it may not be a big problem. When the Remote UE or intermediate Relay UE receives the RRCSetup message, the configuration is set internally even though not yet sent the RRCSetupComplete message.

Figure 4. fast/parallel setup procedure for the last Relay UE to make infer the path information

Figure 4 shows the downstream procedure for the fast/parallel setup. It seems that the SRAP header structure is the same as the legacy. The RRCSetup message can include the configuration of the egress RLC channel for SRB0. Including DL SRB0 configuration in the RRCSetup message will impact the RAN3 spec. We estimate that it has a small impact. However, if we want to avoid the RAN3 impact, the DL SRAP header has to be changed. For example, the last Relay UE can replace the DL SRAP header, including local ID and the mapped L2 ID. If we use this type SRAP header structure, the intermediate Relay UE can decide the DL egress RLC channel by using the reflective method.

The legacy RRCSetup message for the Remote UE includes the RLC channel configuration for SRB1 of the Remote UE. But this case the RRCSetup message can include the RLC channel configuration for SRB0 and SRB1 since the gNB knows the path information. The Relay UE is already in RRC_CONNECTED state in the legacy. So, the gNB can configure egress RLC channel for DL SRB0 via RRCReconfiguration message. However, if the last Relay UE reports the path information, the gNB includes the egress RLC channel configuration for the DL SRB0 message in the RRCSetup message for the intermediate Relay UE.

Step 7/8/9) The gNB sends the SRB0 message for each intermediate Relay UE and Remote UE by using the same SRAP header structure as the legacy. i.e., it includes local ID and bearer ID. When the last Relay UE receives the SRB0 message with the SRAP header, the last Relay UE is already configured about which RLC channel is for the local ID and bearer ID via the RRCReconfiguration message. The last Relay UE forwards the SRB0 message for the intermediate Relay UE or Remote UE via the configured RLC channel. The gNB can send a RRCSetup message orderly based on the path information. Each RRCSetup message includes the RLC channel configuration for SRB0. When the intermediate Relay UE in RRC_IDLE/INACTIVE receives the RRCSetup message, the configuration is set even though the intermediate Relay UE does not yet send the RRCSetupComplete message. So, as long as the RRCSetup message is delivered orderly, the intermediate Relay UE can forward the RRCSetup message toward the Remote UE or the other intermediate Relay UE without latency. Even though the gNB sends the RRCSetup message orderly, the delivery order can be a little bit different. However, we believe that it’s a very rare case, and it’s not a dominant effect on the latency. For example, if Relay UE 2 receives RRCSetup message for Relay UE 1 before receiving the RRCSetup message for itself, the Relay UE 2 may have to wait for the RRCsetup message for itself. We expect it will be a very rare case because gNB sends a RRCsetup message in order. Even though this disorder delivery is happened, it may be a reasonable operation in terms of the parent Relay UE have to be in RRC_CONNECTED to support the Remote UE.

When an intermediate Relay UE in RRC_IDLE/INACTIVE receives the DL SRB0 message with an SRAP header including local ID, the intermediate Relay UE checks whether it belongs to itself. If it does not belong to itself, the intermediate Relay UE checks whether it belongs to its child UE. If it is for its child UE, the intermediate Relay UE removes the SRAP header and sends it to the child UE via the configured RLC channel for SRB0. Otherwise, the intermediate Relay UE forwards it transparently to a proper child UE based on the configuration, which configuration is included in the RRCSetup message for itself.

Proposal 8: The gNB sends the SRB0 message with the legacy SRAP header:

Figure 5 SRAP header structure for delivering the initial UL SRB0 message when the first Relay UE is in RRC_IDLE/INACTIVE

Proposal 9: The gNB includes the egress RLC channel configuration for SRB0 in the RRCSetup message for delivering the DL SRB0 message of the Remote UE or intermediate Relay UE.

Proposal 10: The last Relay UE or intermediate Relay UE transparently forwards the DL SRB0 message via the configured egress link for SRB0, if the DL SRB0 message does not belong to the child UE.

Proposal 11: The last Relay UE or intermediate Relay UE delivers the DL SRB0 message via the configured egress link for the SRB0 without the SRAP header, if the DL SRB0 message belong to the child UE.

2.2 local ID pre-allocation for the INACTIVE relay UE

In the last L2 meeting (RAN2#129bis), we discussed the way to pre-allocate a local ID for the RRC_INACTIVE Remote UE. If the INACTIVE Remote UE has a pre-allocated local ID, it can use it for the fast resume procedure. If the INACTIVE Relay UE uses the pre-allocated local ID, the parent UE of the INACTIVE Relay UE should be configured for the local ID not to be discarded in the received packet. So, we wonder about the following cases:

1) If the INACTIVE Remote UE performs cell reselection, the new parent Relay UE in RRC_INACTIVE may not be configured for the pre-allocated local ID of the Remote UE. In this case, the new parent Relay UE will discard the received RRCResumeRequest message. But the Remote UE will not know whether its RRCResumeRequest message is discarded. We need to discuss how to handle this case.

2) The pre-allocated local ID should be a unique value for each Remote UE. If a local ID is pre-allocated to a Remote UE in RRC_INACTIVE, the same local ID value should not be allocated to the other Remote UE in RRC_INACTIVE or RRC_CONNECTED. If the Remote UE moves to the other cell or gNB, the allocated local ID should be recalled to the original gNB. Because the number of local IDs is a limited resource, gNB should manage them. For example, before detaching the parent Relay UE, the intermediate Relay UE in RRC_INACTIVE informs the parent Relay UE. Or the gNB can use a timer. If the timer expires, the local ID can be recalled automatically. Anyway, we need to discuss how to recall the re-allocated local ID when the Remote UE in RRC_INACTIVE moves to the other cell/gNB.

Proposal 12: When the Remote UE in RRC_INACTIVE uses the pre-allocated local ID to become RRC_CONNECTED, if its parent Relay UE doesn’t have the related configuration of the local ID, the parent Relay UE will discard the received packet. In this case, RAN2 has to discuss how the Remote UE or the parent Relay UE in RRC_INACTIVE performs.

Proposal 13: When the RRC_INACTIVE Remote UE having the pre-allocated local ID moves to the other cell/gNB, the original gNB should recall the pre-allocated local ID from the Remote UE. RAN2 has to discuss how the gNB can recall the pre-allocated local ID.

2.3 Timers for multi-hop

In the multi-hop, the timers for RRC connection establishment, such as T300, T301, or T319, have to be set for a much longer timer value to support multi-hop U2N relay operation. The timer value may have a dependency on the hop count to reach the gNB. And the timer value should be set to the same value in the gNB and the Remote UE. The Remote UE may know the hop count by receiving the value from its upper layer. However, gNB will not know the hop count information of the Remote UE, especially if the Remote UE uses the fast/parallel setup procedure. If the last Relay UE reports path information of the Remote UE gNB can know the hop count of the remote UE implicitly.

Proposal 14: The timer (e.g., T300, T301, or T319) value for RRC connection establishment has a dependency on the hop count.

Proposal 15:  The gNB and Remote UE have to be set to the same timer value for RRC connection establishment.

Proposal 16: If the path information is reported to the gNB, the gNB can implicitly know the hop count to reach the Remote UE in the fast/parallel setup procedure.

If the intermediate Relay UE receives a notification message when the timer, such as T300, T301, or T319, is running, the intermediate Relay UE stops the behaviour for making the connection with the gNB. Because the intermediate Relay UE notices some problems occur on the path while trying to attach. So, the intermediate Relay UE stops its behaviour and forwards the received notification message to the child UE. In this case, the intermediate Relay UE doesn’t need to generate another notification message for the child UE.

Proposal 17: If the intermediate Relay UE receives a notification message while the timer (e.g., T300, T301, or T319) for connection establishment is running, the intermediate Relay UE stops the behaviour and forwards the received notification message to the child UE.

3.	Conclusion

Observation 1: In the legacy, gNB naturally knows that which Remote UE has a connection with which Relay UE.

Observation 2: The gNB has to know the configured SL RLC channel configuration for SRB1 is used for which PC5 link to manage the PC5 RLC channel configuration.

Observation 3: In the multi-hop with fast/parallel setup procedure, gNB cannot know which PC5 link is associated with which Remote UE or intermediate Relay UE.

Observation 4: Considering RAN3 operation, the gNB configures SRB1 for the Remote UE upon receiving the initial SRB0 message of the Remote UE. To do this, gNB has to know that the SRB1 configuration is for the PC5 link between the Remote UE and the first Relay UE.

Proposal 1: Considering RAN3 operation, gNB has to know which PC5 link is used for the SRB1 when configuring SRB1.

Proposal 2: If the last Relay UE reports path information, gNB can configure SRB1 parallelly for each intermediate Relay UEs and Remote UEs.

Proposal 3: The SRB1 of the Remote UEs can be multiplexed over the egress PC5 RLC channel of the parent UE as the SRB1 of the Remote UE can be multiplexed at the Uu link of the U2N relay UE in the legacy.

Proposal 4: For the initial UL SRB0 message delivery, the following new SRAP header structure can be used for the CONNECTED intermediate or last Relay UE to infer the path information.

-  The L2 ID (A) is the source L2 ID of the Remote UE (the source means the uplink direction).

- The L2 ID (B) is the source L2 ID of the first Relay UE (the source means the uplink direction).

Figure 6. SRAP header structure for delivering the initial UL SRB0 message when the first Relay UE is in RRC_IDLE/INACTIVE

Proposal 5: The new SRAP header can be attached when the first Relay UE is in RRC_IDLE/INACTIVE.

Proposal 6: If the IDLE/INACTIVE intermediate Relay UE receives the new SRAP header, the intermediate Relay UE forwards it transparently.

Proposal 7: If the CONNECTED intermediate Relay UE or last Relay UE receives the initial UL SRB0 message with the new SRAP header, the intermediate Relay UE or last Relay UE reports the path information and request local ID allocation to the gNB.

Proposal 8: The gNB sends the SRB0 message with the legacy SRAP header:

Figure 5 SRAP header structure for delivering the initial UL SRB0 message when the first Relay UE is in RRC_IDLE/INACTIVE

Proposal 9: The gNB includes the egress RLC channel configuration for SRB0 in the RRCSetup message for delivering the DL SRB0 message of the Remote UE or intermediate Relay UE.

Proposal 10: The last Relay UE or intermediate Relay UE transparently forwards the DL SRB0 message via the configured egress link for SRB0, if the DL SRB0 message does not belong to the child UE.

Proposal 11: The last Relay UE or intermediate Relay UE delivers the DL SRB0 message via the configured egress link for the SRB0 without the SRAP header, if the DL SRB0 message belong to the child UE.

Proposal 12: When the Remote UE in RRC_INACTIVE uses the pre-allocated local ID to become RRC_CONNECTED, if its parent Relay UE doesn’t have the related configuration of the local ID, the parent Relay UE will discard the received packet. In this case, RAN2 has to discuss how the Remote UE or the parent Relay UE in RRC_INACTIVE performs.

Proposal 13: When the RRC_INACTIVE Remote UE having the pre-allocated local ID moves to the other cell/gNB, the original gNB should recall the pre-allocated local ID from the Remote UE. RAN2 has to discuss how the gNB can recall the pre-allocated local ID.

Proposal 14: The timer (e.g., T300, T301 or T319) value for RRC connection establishment has a dependency on the hop count.

Proposal 15:  The gNB and Remote UE has to be set the same timer value for RRC connection establishment.

Proposal 16: If the path information is reported to the gNB, the gNB can know implicitly the hop count to reach the Remote UE in the fast/parallel setup procedure.

Proposal 17: If the intermediate Relay UE receives a notification message while the timer (e.g., T300, T301 or T319) for connection establishment is running, the intermediate Relay UE stops the behaviour and forwards the received notification message to the child UE.

4.	Reference

- RP-241609, Approved WID: NR sidelink multi-hop relay, LG Electronics Inc., InterDigital, FirstNet..