3GPP TSG-RAN WG2 Meeting #130	R2-2504698

St.Julians, Malta, May. 19th – 23rd, 2025

Agenda Item	: 8.13.3

Source	: LG Electronics Inc., FirstNet

Title	: Discussion on path information reporting

Document for	: Discussion and Decision

1.	Introduction

In the last meeting, we discussed the fast/parallel connection setup procedure. In this case, it is not clear how much it has RAN2/3 spec impact.

In this document, we investigate the RAN2/3 impact of the fast/parallel connection setup procedure and search for how to reduce the expected RAN2/RAN3 impact.

2.	Discussion

2.1 The way to reduce RAN2/RAN3 spec impact

In Rel-17 U2N Relay operation, Remote UE sends the initial UL SRB0 message via a specified relay RLC channel (i.e., SL-RLC0). The Relay UE in RRC_CONNECTED that received the Remote UE’s initial UL SRB0 message reports the L2 ID of the Remote UE to the gNB to solicit the local ID of the Remote UE. After receiving the local ID, the Relay UE generates an SRAP header for the initial UL SRB0 message of the Remote UE and then sends it to the gNB. The SRAP header includes the L2 ID of the Remote UE and bearer ID. By this procedure, the gNB can know that the Remote UE is connected with which Relay UE when receiving the Remote UE’s initial UL SRB0 message with the SRAP header. The gNB configures the SL-RLC channel to the Relay UE, which is for the DL SRB0 message delivery. The gNB configures the SL-RLC channel to the Remote UE, which is for the UL SRB1 message delivery. The SL-RLC channel configuration for the UL SRB1 message delivery of the Remote UE is enclosed in the initial SRB0 message for the Remote UE.

To summarize, the initial UL SRB0 message (i.e., RRCSetupRequest) of the Remote UE is delivered via the specified SL_RLC at the Remote UE. The initial DL SRB0 message (i.e., RRCSetup) of the Remote UE is delivered via a dedicated SL_RLC configuration at the Relay UE. The UL SRB1 message (i.e., RRCSetupRequest) of the Remote UE is delivered via a dedicated SL_RLC configuration at the Remote UE. The SL_RLC configuration for the UL SRB1 message is enclosed in the initial DL SRB0 message.

Observation 1: In the perspective of RAN2 spec, the initial DL SRB0 message (i.e., RRCSetup) for the Remote UE is delivered via a dedicated SL-RLC configuration at the Relay UE.

Observation 2: In the perspective of RAN2 spec, the initial UL SRB1 message (i.e., RRCSetupComplete) for the Remote UE is delivered via a dedicated configuration at the Remote UE.

In the RAN3 spec in Rel-17 U2N Relay operation, the initial SRB1 configuration for the Remote UE is generated at the gNB-DU. When the gNB-DU generates the initial SRB1 configuration for the Remote UE, the gNB-DU can know the SRB1 is for which PC5 link by using the local ID in the SRAP header. The gNB can make the linkage for itself between the local ID and the PC5 link since the gNB remembers the L2 ID for the local ID and which Relay UE has solicited the local ID. The RAN3 operation can be shown simply in the following Figure 1.

Figure 1. RRC Setup Procedure

In Figure 1, DU generates the SRB1 configuration for the Remote UE and then sends the RRCSetupRequest message to the CU, which includes the SRB1 configuration (UL RRC MESSAGE TRANSFER in Figure 1). The CU generates the RRC message (i.e., RRCSetup message), which includes the SRB1 configuration received from DU. The RRCSetup message is delivered to the Remote UE via the gNB-DU (DL RRC_MESSAGE TRANSFER in Figure 1). When the DU configures the SRB1, the DU has to know which SRB1 is for which PC5 link.

Usually, the dedicated RRC configuration is configured for the TX side, however, the gNB required that the configuration for the TX UE is for which PC5 link. In the Rel-17 U2N, there wasn’t an agreement for this since the gNB-DU can always know that the SRB1 configuration for the Remote UE is for which PC5 link naturally based on the procedure. But if we apply the fast/parallel connection setup, the gNB-DU cannot know the SRB1 configuration for the Remote UE is for which PC5 link.

In RAN3, they made the following agreement in Figure 2. According to their agreement, gNB-CU provides the counterpart information of a configured PC5 Relay RLC channel to the gNB-DU. This agreement may not be an exception for the SRB1 configuration of the Remote UE. So, if the CONNECTED Relay UE (intermediate Relay UE or last Relay UE) can provide the path information via SUI, the gNB can configure the SRB1 for the Remote UE when receiving the initial SRB0 message of the Remote UE.

<RAN3#127bis agreement> The gNB-CU provides the counterpart information of a configured PC5 Relay RLC channel to the gNB-DU. WA: Reuse the Peer UE ID IE as counter part information. >>Peer UE ID; O; BIT STRING (SIZE(24)); Corresponds to information provided in the sl-DestinationIdentityL2-U2U contained in the SL-TxResourceReqL2-U2U IE, defined in TS 38.331 [8]. This IE is included if the gNB-CU UE F1AP ID and/or gNB-DU UE F1AP ID are associated with a L2 U2U Remote UE or L2 U2U Relay UE.; YES; reject.

Figure 2. RAN3 agreement at 127-bis meeting

In the perspective RAN3 spec, the gNB has to identify that the PC5 Relay RLC channel configuration is for which PC5 link. The way to identify the PC5 link is reporting peer L2 ID of a UE. In the same principle, if the gNB configures PC5 Relay RLC for SRB1 of the Remote UE, the gNB has to know the L2 ID of the first Relay UE.

Observation 3: In the perspective of the RAN3 spec, the gNB has to know the L2 ID of the first Relay UE to configure PC5 Relay RLC for the SRB1 of the Remote UE.

In the fast/parallel connection setup, the gNB receives the initial UL SRB0 messages from the Remote UE, the first Relay UE, or the intermediate Relay UEs parallelly if they are in RRC_IDLE/INACTIVE. To configure SRB1 for each UE, the gNB has to know which PC5 link is for the Remote UE, the first Relay UE, and which intermediate Relay UEs. If the RRC_CONNECTED intermediate/last Relay UE receiving the fast/parallel connection setup request message reports the path information from the Remote UE to itself, the gNB can configure UL SRB1 for each IDLE/INACTIVE Remote UE or intermediate Relay UE. Also, the gNB can configure the DL configuration for the SRB0 message for each IDLE/INACTIVE intermediate Relay UE. The DL configuration for SRB0 message delivery includes the initial SRB0 message.

If the gNB can configure SL-RLC channel for UL SRB1 message to the Remote UE and configure SL-RLC channel for DL SRB0 message to the intermediate Relay UE when receiving the initial UL SRB0 message of the Remote UE, the RAN2 spec impact for the DL SRB0 and UL SRB1 will be almost none. However, the SUI needs to be changed since the path information is reported. In the RAN3 perspective, the fast/parallel connection setup procedure we discussed last meeting cannot inform the Remote UE has a PC5 connection with which Relay UE. So, the gNB cannot configure the SL-RLC for the UL SRB1 message of the Remote UE. How to solve this issue can be a big spec imact.

Proposal 1: To reduce RAN2/3 impact, the RRC_CONNECTED Relay UE reports the path information of the Remote UE when receiving the initial UL SRB0 messages for the fast/parallel connection.

3.	Conclusion

Proposal 1: To reduce RAN2/3 impact, the RRC_CONNECTED Relay UE reports the path information of the Remote UE when receiving the initial UL SRB0 messages for the fast/parallel connection.

4.	Reference

- R2-2502189, Fast & Parallel RRC Establishment/Configuration for Multi-hop U2N relaying, Apple, FirstNet, Ericsson, AT&T, Kyocera