3GPP TSG-RAN2 Meeting #130	R2-2504726

# St.Julians, Malta, 19th May 2025 – 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.306 | CR | 1274 | rev | 1 | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on PDCCH ordered RACH capabilities for LTM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc.,Ericsson, Nokia, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-Core |  |  |  |  |  | Date: |  |  | 2025-05-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The UE capability pdcch-RACH-AffectedBandsList-r18 is defined as whether the UE may cause an interruption on DL slot(s) on serving cells due to PDCCH-ordered RACH transmission, with the granularity of feature set. The UE reports a list with the length of the bands indicated in appliedFreqBandListFilter. Each element of the list has an ENUMERATED with two values {noInterruption, interruption}. In RAN2 #129 meeting, an LS (R2-2501392) was sent to RAN4 for the definition of the source band in source-target band pair for the granularity in pdcch-RACH-AffectedBandsList-r18. In the reply LS R2-2503325 (R4-2504940), RAN4 clairfies that the understanding 2 (The source band of the band pair represents the specific source band that have interruption or not) is the correct understanding. Therefore, the description of the capability should be updated accordingly for clarification. The description has been updated with the wording of feature set structure for clarity. The name of pdcch-RACH-AffectedBandsList is misleading as the list is made of target bands rather than affected bands. The name of this capability has been updated. Additionally, if the UE does not support PDCCH-ordered RACH transmission for LTM towards certain target bands indicated in the appliedFreqBandListFilter, it is unclear whether the UE should report noInterruption or interruption for those elements. The related change was agreed in RAN2#129 meeting but was put on hold due to the above definition issue. Therefore, this change is also reintroduced to this CR. For the UE capabilities pdcch-RACH-PrepTimeList-r18 and pdcch-RACH-SwitchingTimeList-r18, the description has also been updated with the wording of feature set structure for clarity. The name of these two capabilities are also updated for a unified name style with pdcch-RACH-AffectedBandsList-r18. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update the name of pdcch-RACH-AffectedBandsList-r18, pdcch-RACH-PrepTimeList-r18 and pdcch-RACH-SwitchingTimeList-r18 to pdcch-RACH-AffectedBand-TargetBandList-r18, pdcch-RACH-PrepTime-TargetBandList-r18 and pdcch-RACH-SwitchingTime-TargetBandList-r18 respectively. - Update the description of the three capabilities (pdcch-RACH-AffectedBand-TargetBandList-r18, pdcch-RACH-PrepTime-TargetBandList-r18 and pdcch-RACH-SwitchingTime-TargetBandList-r18) with the wording of the feature set structure. - For pdcch-RACH-AffectedBand-TargetBandList-r18, clarify the definition of source band in the source-target band pair represent the specific source band that have interruption or not (depends on the value “interruption” or “noInterruption”) in the current BC when UE performs PDCCH-ordered early RACH toward a target cell on target band. - For pdcch-RACH-AffectedBands-TargetBandList-r18, clarify that for those target bands indicated in appliedFreqBandListFilter where the UE does not support PDCCH-ordered RACH towards target bands for LTM, it is up to UE implementation to select noInterruption or interruption for that element and it means nothing. UE should not report support for the corrsponding band in the capability rach-EarlyTA-Measurement-r18. Impact Analysis Impacted 5G architecture options: NR SA and NR-DC Impacted functionality: LTM Inter-operability analysis: - If the network is implemented according to the CR and the UE is not, UE may not know how to indicate the serving band which UE may cause an interruption on DL slot when performing PDCCH-ordered early RACH. UE does not know how to select the values for those elements indicated in appliedFreqBandListFilter while UE does not support as target band for PDCCH-ordered RACH for LTM. - If the UE is implemented according to the CR and the network is not, network may misunderstand UE capability for the defination of the reported source band and may misunderstand those unsupported target bands of PDCCH-ordered RACH. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Network may misunderstand the UE capability for the defination of the reported band entry of the feature set and may misunderstand those unsupported target bands of PDCCH-ordered RACH. UE may not know how to indicate the affected serving band which UE may cause an interruption on DL slot when performing PDCCH-ordered early RACH to a target band. UE does not know how to select the values for those elements indicated in appliedFreqBandListFilter while UE does not support as target band for PDCCH-ordered RACH for LTM. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.7.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

4.2.7.5	FeatureSetDownlink parameters

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| pdcch-RACH-AffectedBands-TargetBandList-r18 Indicates whether interruption may occur on DL slot(s) on serving cells due to PDCCH-ordered RACH transmission towards target bands, as specified in TS 38.133, clause 8.2.2.2.20 [5]. The band entry of this feature set corresponds to the band of the serving cell(s) that may be affected during PDCCH-ordered RACH transmission toward target band. Each entry in the list corresponds to a target band for RACH transmission. The target bands only consist of the bands indicated in appliedFreqBandListFilter. They are listed in the same order as in appliedFreqBandListFilter and the first entry correspond to the first entry on appliedFreqBandListFilter and so on. For those bands indicated in appliedFreqBandListFilter where the UE does not support PDCCH-ordered RACH towards target bands for LTM, it is up to UE implementation to select noInterruption or interruption for that element and this value is ignored, as UE does not report the support for the corresponding band in the capability rach-EarlyTA-Measurement-r18. A UE supporting this feature shall also indicate support of rach-EarlyTA-Measurement-r18. | FS | No | N/A | N/A |
| pdcch-RACH-PrepTime-TargetBandList-r18 Indicates the RF/BB preparation time for PDCCH ordered RACH of which the resources are not fully contained in any of UE's configured UL BWP(s) of active serving cells. If absent, the UE does not support PDCCH ordered RACH if the PRACH bandwidth is outside of any configured UL BWP, as specified in TS 38.133, clause 6.2.2C.2 [5]. Each entry in the list corresponds to a target band for RACH transmission. If an entry is set to notSupported, the UE does not support PDCCH ordered RACH if the PRACH bandwidth is outside of any configured UL BWP in that target band. The target bands only consist of the bands indicated in appliedFreqBandListFilter. They are listed in the same order as in appliedFreqBandListFilter and the first entry correspond to the first entry on appliedFreqBandListFilter and so on. A UE supporting this feature shall also indicate support of rach-EarlyTA-Measurement-r18. A UE that sets pdcch-RACH-SwitchingTime-TargetBandList to a value different from notSupported for a target band also sets pdcch-RACH-PrepTime-TargetBandList to a value different from notSupported for that target band. | FS | No | N/A | N/A |
| pdcch-RACH-SwitchingTime-TargetBandList-r18 Indicates the interruption length (Y ms) due to RF re-tuning for PDCCH ordered RACH of which the resources are not fully contained in any of UE's configured UL BWP(s) of active serving cells, if absent, the UE does not support PDCCH ordered RACH if the PRACH bandwidth is outside of any configured UL BWP, as specified in TS 38.133, clause 8.2.2.2.20 [5]. Each entry in the list corresponds to a target band for RACH transmission. If an entry is set to notSupported, the UE does not support PDCCH ordered RACH if the PRACH bandwidth is outside of any configured UL BWP in that target band. The target bands only consist of the bands indicated in appliedFreqBandListFilter. They are listed in the same order as in appliedFreqBandListFilter and the first entry correspond to the first entry on appliedFreqBandListFilter and so on. A UE supporting this feature shall also indicate support of rach-EarlyTA-Measurement-r18. A UE that sets pdcch-RACH-PrepTime-TargetBandList to a value different from notSupported for a target band also sets pdcch-RACH-SwitchingTime-TargetBandList to a value different from notSupported for that target band. | FS | No | N/A | N/A |