3GPP TSG-RAN WG2 Meeting #130                                                                    R2-2504745

St. Julian, Malta, 19th – 23rd May, 2025

Source:	CATT

Title:	Way forward for Rx BSF optimization

Agenda Item:	8.20.1

Document for:	Discussion and Decision

Introduction

This is to summarize some unofficial offline discussions for Rx BSF optimization.

Discussion

Regarding the following BSF CB on Friday,

Rx BSF optimization R2-2503326 LS on Rx BSF optimization for NR RRM Ph5 (R4-2504962; contact: CICT Mobile) RAN4 LS in Rel-19 NR_RRM_Ph5-Core To:RAN2 Noted R2-2503580 Discussion on Rx BSF optimization for NR RRM Ph5 CATT discussion Rel-19 NR_RRM_Ph5-Core Noted Proposal 1: RAN2 waits for RAN4 further progress for Rx BSF optimization. [CB] Friday, check potential progress from R4 during the week R2-2504030 Discussion on Rx BSF optimization ZTE Corporation discussion Rel-19 NR_RRM_Ph5-Core Noted Proposal 1: Multi-Rx L3 measurement is only applicable when the UE is configured with FR2-1 PCell only, and there is only one SSB-based MO configured, the MO is FR2-1 carrier and can be different from PCell’s frequency. Proposal 2: Separate RSRP threshold and RSRQ threshold are introduced. If both are configured, it means the UE activates multi-Rx L3 measurement when both of the thresholds are satisfied. Proposal 3: Do not consider TimeToTrigger and Hysteresis in activation condition. Proposal 4: RAN2 assumes the UE at least deactivates multi-Rx L3 measurement when the threshold becomes not satisfied. FFS on other conditions, pending RAN4. Discussion - OPPO think we can wait for more info from R4. R2-2503897 Reduced Beam Sweeping Factor Nokia CR Rel-19 38.331 18.5.1 5354 - B NR_RRM_Ph5-Core

We have checked with our RAN4 colleagues that RAN4 didn’t discuss the RRC signaling details of activation condition in this meeting. So RAN2 could start RRC CR work without further input from RAN4.

Based on the proposals in ZTE paper R2-2504030, we further discuss the following aspects:

1.       In our RRC draft CR, we don’t implement “TimeToTrigger and Hysteresis in activation condition” for now. (RAN4 also didn’t mention this in their LS)

2.       Separate RSRP threshold and RSRQ threshold are introduced. RAN2 assumes If both are configured, it means the UE activates multi-Rx L3 measurement when both of the thresholds are satisfied. (RAN4 mentioned “based on RSRP and/or RSRQ threshold(s)”, RAN2 can make this assumption to proceed RRC CR)

3.       Rx BSF optimization is applicable to only one carrier in the single FR2-1 band which is configured for L3 SSB measurement, while UE is configured with single carrier on FR2-1 band, i.e. FR2-1 PCell without CA/DC. (this is to clarify the target scenario)

Another RRC detail I’d like to check with you is under what IE the new threshold parameters are added:

Option 1: MeasConfig IE

Option 2: ServingCellConfig IE

Option 3: MeasObjectNR IE

After some offline discussions, the following way forward can be further discussed in CB session:

1.       In RRC draft CR, we don’t implement “TimeToTrigger and Hysteresis in activation condition” for now.

2.       Separate RSRP threshold and RSRQ threshold are introduced. RAN2 assumes If both are configured, it means the UE activates multi-Rx L3 measurement when both of the thresholds are satisfied.

3.       Rx BSF optimization is applicable to only one carrier in the single FR2-1 band which is configured for L3 SSB measurement, while UE is configured with single carrier on FR2-1 band, i.e. FR2-1 PCell without CA/DC. FFS whether this needs to be captured in RRC spec.

4.     The new threshold parameters are added in MeasConfig IE

Conclusion

Proposal 1: RAN2 to further discuss the following RRC details for RX BSF optimization:

1.       In RRC draft CR, we don’t implement “TimeToTrigger and Hysteresis in activation condition” for now.

2.       Separate RSRP threshold and RSRQ threshold are introduced. RAN2 assumes If both are configured, it means the UE activates multi-Rx L3 measurement when both of the thresholds are satisfied.

3.       Rx BSF optimization is applicable to only one carrier in the single FR2-1 band which is configured for L3 SSB measurement, while UE is configured with single carrier on FR2-1 band, i.e. FR2-1 PCell without CA/DC. FFS whether this needs to be captured in RRC spec.

4.     The new threshold parameters are added in MeasConfig IE.