3GPP TSG-RAN WG2 Meeting #130	R2-2504852

St. Julians, Malta, May 19th – 23rd, 2025

Source:	Session Chair (Ericsson)

Title:	Report from session on maintenance and SON/MDT

Agenda item:	9.6

[AT130][600] Organizational – Maintenance and SON/MDT (Ericsson)

Scope:

    - Share plans for the meeting and list of ongoing email discussions

    - Share meetings notes and agreements for review and endorsement

    - Flag LSs and agreed CRs for discussion

Intended outcome:

    - General information sharing about the sessions

# 4	EUTRA Rel-17 and earlier

Only essential corrections. No documents should be submitted to 4. Please submit to 4.x

## 4.1	EUTRA corrections Rel-17 and earlier

(NB_IOTenh4_LTE_eMTC6-Core; leading WG: RAN1; REL-17; WID: RP-211340)

(UPIP_EN-DC_UE; leading WG: RAN3; REL-17; WID: RP-213669)

(LTE TEI17)

Essential corrections to LTE Rel-17 topics not covered by other agenda items.

(NB_IOTenh3-Core; leading WG: RAN1; REL-16; started: Jun 18; Completed: June 20; WID: RP-200293); REL-15 and Earlier NB-IoT WIs are in scope but not listed explicitly (long list).

(LTE_eMTC5-Core; LTE_eMTC5-Core; leading WG: RAN1; REL-16; started: Jun 18; Completed:  June 20; WID: RP-192875;), REL-15 and Earlier eMTC WIs are in scope but not listed explicitly (long list).

(LTE_feMob-Core; leading WG: RAN2; REL-16; started: Jun 18; Completed: June 20; WID: RP-190921);

(LTE_terr_bcast-Core, LTE_DL_MIMO_EE-Core, LTE_high_speed_enh2-Core; LTE TEI16 Non-positioning);

(LTE_NBIOT_eMTC_NTN; leading WG: RAN1; REL-17; WID: RP-211601)

REL-16 and Earlier EUTRA WIs are in scope but not listed explicitly (long list), Except Positioning WI, which is addressed by AIs below.

NOTE that LTE corrections related to NR WIs or Joint NR LTE WIs should be submitted to NR AIs below.

NOTE that LTE corrections which are the same as an NR correction should be submitted to the respective NR AI (so the NR CR and LTE CR can be treated together).

This Agenda Item is treated in the Maintenance Breakout session (Corrections for LTE_NBIOT_eMTC_NTN might be treated in the NTN breakout session)

Inter-RAT cell selection

R2-2503472	Clarification on Inter-RAT Cell Selection for NB-IoT	vivo	CR	Rel-16	36.304	16.9.0	0879	-	F	NB_IOTenh3-Core

R2-2503473	Clarification on Inter-RAT Cell Selection for NB-IoT	vivo	CR	Rel-17	36.304	17.5.0	0880	-	A	NB_IOTenh3-Core

R2-2503474	Clarification on Inter-RAT Cell Selection for NB-IoT	vivo	CR	Rel-18	36.304	18.3.0	0881	-	A	NB_IOTenh3-Core

The other specs affected boxes should be ticked (with No), with this all 3 are agreed in R2-2504832, R2-2504833, and R2-2504834

Need for gap capabilities

R2-2503939	Correction to NeedForGap capability	Huawei, HiSilicon	CR	Rel-16	36.331	16.19.0	5114	-	F	NR_newRAT-Core, TEI16

R2-2503940	Correction to NeedForGap capability	Huawei, HiSilicon	CR	Rel-17	36.331	17.12.0	5115	-	A	NR_newRAT-Core, TEI16

R2-2503982	Correction to NeedForGap capability	Huawei, HiSilicon	CR	Rel-18	36.331	18.5.0	5116	1	A	NR_newRAT-Core, TEI16	R2-2503941

-	Qualcomm wonders why the -r16 version of the field is not mentioned in the field description.

-	Qualcomm think it should be “UE is operating on an E-UTRA cell the an E-UTRAN band”

-	Ericsson think we should spell out “single carrier”

Continue polish offline

[AT130][601][Maint] Correction to NeedForGap capability (Huawei)

Scope:

    - Polish the CRs offline

Intended outcome:

    - Agreed CR(s) in R2-2504835, R2-2504836, R2-2504837

Deadline:

    - Wednesday 17:00. The intention is to agree over email.

R2-2504835	Correction to NeedForGap capability	Huawei, HiSilicon	CR	Rel-16	36.331	16.19.0	5114	1	F	NR_newRAT-Core, TEI16

R2-2504836	Correction to NeedForGap capability	Huawei, HiSilicon	CR	Rel-17	36.331	17.12.0	5115	1	A	NR_newRAT-Core, TEI16

R2-2504837	Correction to NeedForGap capability	Huawei, HiSilicon	CR	Rel-18	36.331	18.5.0	5116	2	A	NR_newRAT-Core, TEI16

All 3 are agreed

Report on leave

R2-2504017	Introducing UE capability for A4 A5 ReportOnLeave	ZTE Corporation, Sanechips, Qualcomm Incorporated, Samsung, MediaTek Inc, Xiaomi, Huawei, HiSilicon	CR	Rel-15	36.331	15.23.0	5117	-	F	LTE_Aerial-Core

R2-2504018	Introducing UE capability for A4 A5 ReportOnLeave	ZTE Corporation, Sanechips, Qualcomm Incorporated, Samsung, MediaTek Inc, Xiaomi, Huawei, HiSilicon	CR	Rel-16	36.331	16.19.0	5118	-	A	LTE_Aerial-Core

R2-2504019	Introducing UE capability for A4 A5 ReportOnLeave	ZTE Corporation, Sanechips, Qualcomm Incorporated, Samsung, MediaTek Inc, Xiaomi, Huawei, HiSilicon	CR	Rel-17	36.331	17.12.0	5119	-	A	LTE_Aerial-Core

R2-2504020	Introducing UE capability for A4 A5 ReportOnLeave	ZTE Corporation, Sanechips, Qualcomm Incorporated, Samsung, MediaTek Inc, Xiaomi, Huawei, HiSilicon	CR	Rel-18	36.331	18.5.0	5120	-	A	LTE_Aerial-Core

R2-2504021	Introducing UE capability for A4 A5 ReportOnLeave	ZTE Corporation, Sanechips, Qualcomm Incorporated, Samsung, MediaTek Inc, Xiaomi, Huawei, HiSilicon	CR	Rel-15	36.306	15.12.0	1907	1	F	LTE_Aerial-Core	R2-2502331

R2-2504022	Introducing UE capability for A4 A5 ReportOnLeave	ZTE Corporation, Sanechips, Qualcomm Incorporated, Samsung, MediaTek Inc, Xiaomi, Huawei, HiSilicon	CR	Rel-16	36.306	16.13.0	1908	1	A	LTE_Aerial-Core	R2-2502332

R2-2504023	Introducing UE capability for A4 A5 ReportOnLeave	ZTE Corporation, Sanechips, Qualcomm Incorporated, Samsung, MediaTek Inc, Xiaomi, Huawei, HiSilicon	CR	Rel-17	36.306	17.8.0	1909	1	A	LTE_Aerial-Core	R2-2502333

R2-2504024	Introducing UE capability for A4 A5 ReportOnLeave	ZTE Corporation, Sanechips, Qualcomm Incorporated, Samsung, MediaTek Inc, Xiaomi, Huawei, HiSilicon	CR	Rel-18	36.306	18.4.0	1910	1	A	LTE_Aerial-Core	R2-2502334

-	Lenovo wonders if the new capa only applies to UAV UEs? ZTE think this should not be such restriction. Samsung think there is another capability for UAV UEs. Lenovo want to clarify this on the cover page. ZTE and Qualcomm thinks its clear.

-	Nokia wonders if this applies to non-aerial UEs then the WI code can be e.g. TE15 also

All are agreed

MBMS

R2-2504322	Correction on MBMS Interest Indication	Samsung	CR	Rel-15	36.331	15.23.0	5122	-	F	TEI15

R2-2504323	Correction on MBMS Interest Indication	Samsung	CR	Rel-16	36.331	16.19.0	5123	-	A	TEI15

R2-2504324	Correction on MBMS Interest Indication	Samsung	CR	Rel-17	36.331	17.12.0	5124	-	A	TEI15

R2-2504325	Correction on MBMS Interest Indication	Samsung	CR	Rel-18	36.331	18.5.0	5125	-	A	TEI15

-	Ericsson think this is good to clarify this but think this should not be from Rel-15, instead it should be from Rel-18.

-	Ericsson think there is a similar issue in NR.

-	Xiaomi think we should postpone this to see if this is NBC.

-	Qualcomm are OK with these CRs.

Postponed

Misc

R2-2504553	Miscellaneous non-controversial corrections	Samsung	CR	Rel-16	36.331	16.19.0	5131	1	F	NR_newRAT-Core	R2-2504451

R2-2504452	Miscellaneous non-controversial corrections	Samsung	CR	Rel-17	36.331	17.12.0	5132	-	A	NR_newRAT-Core

R2-2504453	Miscellaneous non-controversial corrections	Samsung	CR	Rel-18	36.331	18.5.0	5133	-	F	NR_newRAT-Core

-	Lenovo think only (NG)EN-DC is impacted

-	Lenovo think that R18 should be Cat A.

-	Lenovo want to add TEI16 to the WI codes.

All agreed, but updated so only (NG)EN-DC is impacted, the R18 CR should be Cat A and TEI16 should be added as WI-code. With this they are agreed unseen in R2-2504838, R2-2504839, R2-2504840

R2-2504838	Miscellaneous non-controversial corrections	Samsung

R2-2504839	Miscellaneous non-controversial corrections	Samsung

R2-2504840	Miscellaneous non-controversial corrections	Samsung

All 3 agreed

NTN – To be handled in Sergio’s session

R2-2504332	Correction to explicit indication of epoch in SIB31	Nordic Semiconductor ASA, Qualcomm, Mediatek Inc., Ericsson	CR	Rel-17	36.331	17.12.0	5127	-	F	LTE_NBIOT_eMTC_NTN

R2-2504333	Correction to explicit indication of epoch in SIB31	Nordic Semiconductor ASA, Qualcomm, Mediatek Inc., Ericsson	CR	Rel-18	36.331	18.5.0	5128	-	A	LTE_NBIOT_eMTC_NTN

R2-2504335	Clarification to NTN SI message accumulation over SI-windows	Nordic Semiconductor ASA	discussion	Rel-17

R2-2504336	Clarification to NTN SI message accumulation over SI-windows	Nordic Semiconductor ASA	CR	Rel-17	36.331	17.12.0	5129	-	F	LTE_NBIOT_eMTC_NTN

R2-2504337	Clarification to NTN SI message accumulation over SI-windows	Nordic Semiconductor ASA	CR	Rel-18	36.331	18.5.0	5130	-	A	LTE_NBIOT_eMTC_NTN

Withdrawn/Old revisisions

R2-2503941	Correction to NeedForGap capability (R18)	Huawei, HiSilicon	CR	Rel-18	36.331	18.5.0	5116	-	A	NR_newRAT-Core, TEI16

=> Revised in R2-2503982

R2-2504451	Miscellaneous non-controversial corrections	Samsung	CR	Rel-16	36.331	16.19.0	5131	-	F	NR_newRAT-Core

=> Revised in R2-2504553

R2-2504326	Correction on MBMS Interest Indication	Samsung	CR	Rel-18	36.331	18.5.0	5126	-	A	TEI15	Withdrawn

# 5	NR Rel-15 and Rel-16

Essential corrections only.

Tdoc Limitation: 2 tdocs in total for all sub agenda items

In case a correction need to be reflected in both NR TS and LTE TS, the corrections should be submitted under one single AI (so the NR and LTE correction can be treated together), the sub-Ais below this

## 5.1	Common

Includes the following WIs and input that doesn’t fit elsewhere.

(NR_newRAT-Core; leading WG: RAN1; REL-15; started: Mar. 17; closed: Jun. 19: WID: RP-191971)

(NR_IAB-Core; leading WG: RAN2; REL-16; started: Dec 18; target Aug 20; WID: RP-200840)

(NR_unlic-Core; leading WG: RAN1; REL-16; started: Dec 18; Closed June 20; WID: RP-192926).

(NR_IIOT-Core; leading WG: RAN2; REL-16; started: Mar 19; Completed: Jun 20; WID: RP-200797)

(NR_UE_pow_sav-Core; leading WG: RAN1; REL-16; started: Mar 19; Completed Jun 20; WID: RP-200494).

(NR_2step_RACH-Core; leading WG: RAN1; REL-16; started: Dec 18; Completed: June 20; WID: RP-200085).

(SRVCC_NR_to_UMTS-Core; leading WG: RAN2; REL-16; started: Dec 18; Completed; Mar 20; WID: RP-190713)

(RACS-RAN-Core, leading WG: RAN2; REL-16; started: Mar 19; completed: Jun 20; WID: RP-191088)

(NG_RAN_PRN-Core; leading WG: RAN3; REL-16; started: Mar 19; completed: June 20; WID: RP-200122)

(NR_eMIMO-Core, leading WG: RAN1; REL-16; started: Jun 18; target; Aug 20; WID: RP-200474)

(NR_CLI_RIM; leading WG: RAN1; REL-16; started: Dec 18; Completed: Jun 20; WID: RP-191997)

(NR_L1enh_URLLC-Core, leading WG: RAN1; REL-16; Completed: June 20; WID: RP-191584)

(LTE_NR_DC_CA_enh-Core; leading WG: RAN2; REL-16; started: Jun 18; Target Aug 20; WI RP-200791)

(NR_Mob_enh-Core; leading WG: RAN2; REL-16; started: Jun 18; Completed June 20; WID: RP-192277).

(NR_SON_MDT-Core; leading WG: RAN3; REL-16; started: Jun 19; Completed June 20; WID: RP-191776)

(5G_V2X_NRSL-Core; leading WG: RAN1; REL-16; started: Mar 19; completed; Aug 20; WID: RP-200129)

(NR_HST, NR_RRM_enh-Core, NR_RF_FR1, NR_RF_FR2_req_enh, NR_n66_BW, LTE_NR_B41_Bn41_PC29dBm-Core, NR_CSIRS_L3meas,)

(NR TEI16)

LTE mob enh corrections that are common with NR mobility enhancements should be submitted to this AI.

### 5.1.1	Stage 2 and Organisational

Submit to one of the agenda items below.

R2-2503307	Reply LS on segmented UE capabilities (C4-251474; contact: Ericsson)	CT4	LS in	Rel-16	RACS-RAN-Core, TEI17	To:RAN2

-	Lenovo understands that the source node should ensure that the size limitation is not exceeded by implementation or should we update our specs? Nokia think that with the new feature allowing NW to control nrof segments this is handled. Qualcomm agrees with Nokia.

Noted

R2-2503315	LS on Handling of UE Radio Capability for Paging (R3-252276; contact: Ericsson)	RAN3	LS in	Rel-16	NR_newRAT-Core	To:SA2	Cc:RAN2

Noted

#### 5.1.1.0	In-principle agreed CRs

R2-2503946	Corrections on DCP	Huawei, HiSilicon, Nokia (Rapporteur)	CR	Rel-16	38.300	16.19.0	0983	1	F	NR_UE_pow_sav-Core	R2-2502473

R2-2503947	Corrections on DCP	Huawei, HiSilicon, Nokia (Rapporteur)	CR	Rel-17	38.300	17.12.0	0984	1	A	NR_UE_pow_sav-Core	R2-2502474

R2-2503948	Corrections on DCP	Huawei, HiSilicon, Nokia (Rapporteur)	CR	Rel-18	38.300	18.5.0	0985	1	A	NR_UE_pow_sav-Core	R2-2502475

All 3 agreed

#### 5.1.1.1	Other

Incoming LSs, etc. You should discuss your stage 2 CRs with the specification rapporteurs before submission. Includes impact to 38.300, 36.300, 37.340

### 5.1.3	Control Plane corrections

Submit to one of the agenda items below.

#### 5.1.3.0	In-principle agreed CRs

R2-2503630	Correction to Immediate MDT measurements	Nokia	CR	Rel-16	37.320	16.8.0	0137	1	F	NR_SON_MDT-Core	R2-2502404

R2-2503631	Correction to Immediate MDT measurements	Nokia	CR	Rel-17	37.320	17.5.0	0138	1	A	NR_SON_MDT-Core	R2-2502405

R2-2503632	Correction to Immediate MDT measurements	Nokia	CR	Rel-18	37.320	18.3.0	0139	1	A	NR_SON_MDT-Core	R2-2502406

R2-2503993	Correction to nrofCandidates-CI for DCI format 2_4	Huawei, HiSilicon	CR	Rel-16	38.331	16.19.0	5295	1	F	NR_L1enh_URLLC-Core	R2-2502271

R2-2503994	Correction to nrofCandidates-CI for DCI format 2_4	Huawei, HiSilicon	CR	Rel-17	38.331	17.12.0	5296	1	A	NR_L1enh_URLLC-Core	R2-2502272

R2-2503995	Correction to nrofCandidates-CI for DCI format 2_4	Huawei, HiSilicon	CR	Rel-18	38.331	18.5.1	5297	1	A	NR_L1enh_URLLC-Core	R2-2502273

R2-2504086	Correction on timeUntilReconnection in RLF report	ZTE Corporation, Xiaomi, Sanechips	CR	Rel-16	36.331	16.19.0	5113	1	F	NR_SON_MDT-Core	R2-2503116

R2-2504198	Correction to pdsch-256QAM-FR1 for IAB-MT	Nokia	CR	Rel-16	38.306	16.20.0	1259	1	F	NR_IAB-Core	R2-2502631

R2-2504199	Correction to pdsch-256QAM-FR1 for IAB-MT	Nokia	CR	Rel-17	38.306	17.12.0	1260	1	A	NR_IAB-Core	R2-2502632

R2-2504200	Correction to pdsch-256QAM-FR1 for IAB-MT	Nokia	CR	Rel-18	38.306	18.5.0	1261	1	A	NR_IAB-Core	R2-2502633

R2-2504680	Correction on number of NZP CSI-RS resources	Ericsson	CR	Rel-15	38.331	15.28.0	5298	2	F	NR_newRAT-Core

R2-2504681	Correction on number of NZP CSI-RS resources	Ericsson	CR	Rel-16	38.331	16.19.0	5299	2	A	NR_newRAT-Core, TEI16

R2-2504682	Correction on number of NZP CSI-RS resources	Ericsson	CR	Rel-17	38.331	17.12.0	5300	2	A	NR_newRAT-Core

R2-2504683	Correction on number of NZP CSI-RS resources	Ericsson	CR	Rel-18	38.331	18.5.1	5301	2	A	NR_newRAT-Core

R2-2504582	Correction on SRS capability reporting	Huawei, HiSilicon, Ericsson, OPPO	CR	Rel-15	38.331	15.28.0	5324	3	F

R2-2504687	Correction on SRS capability reporting	Huawei, HiSilicon, Ericsson, OPPO	CR	Rel-16	38.331	16.19.0	5325	3	F

R2-2504688	Correction on SRS capability reporting	Huawei, HiSilicon, Ericsson, OPPO	CR	Rel-17	38.331	17.12.0	5326	3	F

R2-2504689	Correction on SRS capability reporting	Huawei, HiSilicon, Ericsson, OPPO	CR	Rel-18	38.331	18.5.1	5327	3	F

R2-2504586	Correction on SRS capability reporting	Huawei, HiSilicon, Ericsson, OPPO	CR	Rel-15	38.306	15.27.0	1268	3	F

R2-2504690	Correction on SRS capability reporting	Huawei, HiSilicon, Ericsson, OPPO	CR	Rel-16	38.306	16.20.0	1269	3	F

R2-2504691	Correction on SRS capability reporting	Huawei, HiSilicon, Ericsson, OPPO	CR	Rel-17	38.306	17.12.0	1270	3	F

R2-2504692	Correction on SRS capability reporting	Huawei, HiSilicon, Ericsson, OPPO	CR	Rel-18	38.306	18.5.0	1271	3	F

All above are agreed

R2-2504163	Type clarification for interBandMRDC-WithOverlapDL-Bands-r16	Apple, Huawei, HiSilicon, xiaomi, Samsung	CR	Rel-16	38.306	16.20.0	1251	1	F	NR_newRAT-Core, TEI16	R2-2502505

R2-2504164	Type clarification for interBandMRDC-WithOverlapDL-Bands-r16	Apple, Huawei, HiSilicon, xiaomi, Samsung	CR	Rel-17	38.306	17.12.0	1252	1	A	NR_newRAT-Core, TEI16	R2-2502506

R2-2504165	Type clarification for interBandMRDC-WithOverlapDL-Bands-r16	Apple, Huawei, HiSilicon, xiaomi, Samsung	CR	Rel-18	38.306	18.5.0	1253	1	A	NR_newRAT-Core, TEI16	R2-2502507

-	Lenovo wonders if we need further clarifications (regarding type1/type 2)? Huawei think there is no need for the capability part. Qualcomm want to clarify. Ericsson agrees that some clarification is needed.

To be handled in offline 016

Withdrawn/Old revisions

R2-2504195	Correction to pdsch-256QAM-FR1 for IAB-MT	Nokia	CR	Rel-16	38.306	16.20.0	1291	-	F	NR_IAB-Core	Withdrawn

R2-2504196	Correction to pdsch-256QAM-FR1 for IAB-MT	Nokia	CR	Rel-17	38.306	17.12.0	1292	-	A	NR_IAB-Core	Withdrawn

R2-2504197	Correction to pdsch-256QAM-FR1 for IAB-MT	Nokia	CR	Rel-18	38.306	18.5.0	1293	-	A	NR_IAB-Core	Withdrawn

R2-2504583	Correction on SRS capability reporting	Huawei, HiSilicon, Ericsson, OPPO	CR	Rel-16	38.331	16.19.0	5325	2	F	NR_newRAT-Core	R2-2503105

R2-2504584	Correction on SRS capability reporting	Huawei, HiSilicon, Ericsson, OPPO	CR	Rel-17	38.331	17.12.0	5326	2	F	NR_newRAT-Core	R2-2503106

R2-2504585	Correction on SRS capability reporting	Huawei, HiSilicon, Ericsson, OPPO	CR	Rel-18	38.331	18.5.1	5327	2	F	NR_newRAT-Core	R2-2503107

R2-2504587	Correction on SRS capability reporting	Huawei, HiSilicon, Ericsson, OPPO	CR	Rel-16	38.306	16.20.0	1269	2	F	NR_newRAT-Core	R2-2503109

R2-2504588	Correction on SRS capability reporting	Huawei, HiSilicon, Ericsson, OPPO	CR	Rel-17	38.306	17.12.0	1270	2	F	NR_newRAT-Core	R2-2503110

R2-2504589	Correction on SRS capability reporting	Huawei, HiSilicon, Ericsson, OPPO	CR	Rel-18	38.306	18.5.0	1271	2	F	NR_newRAT-Core	R2-2503111

R2-2504250	Correction on number of NZP CSI-RS resources	Ericsson	CR	Rel-15	38.331	15.28.0	5298	1	F	NR_newRAT-Core	R2-2502344

R2-2504251	Correction on number of NZP CSI-RS resources	Ericsson	CR	Rel-16	38.331	16.19.0	5299	1	A	NR_newRAT-Core, TEI16	R2-2502389

R2-2504252	Correction on number of NZP CSI-RS resources	Ericsson	CR	Rel-17	38.331	17.12.0	5300	1	A	NR_newRAT-Core, TEI17	R2-2502390

R2-2504253	Correction on number of NZP CSI-RS resources	Ericsson	CR	Rel-18	38.331	18.5.1	5301	1	A	NR_newRAT-Core, TEI18	R2-2502391

#### 5.1.3.1	NR RRC

Corrections to 38331, and related change to other TS if applicable, e.g. 36331, Stage-2 etc.

On-demand PosSIBs

R2-2504105	Correction to delivery of SIB segments by dedicated signalling in RRC_CONNECTED	Huawei, HiSilicon	CR	Rel-16	38.331	16.19.0	5360	-	F	TEI16

Moved from 5.1.3

R2-2504106	Correction to delivery of SIB segments by dedicated signalling in RRC_CONNECTED	Huawei, HiSilicon	CR	Rel-17	38.331	17.12.0	5361	-	A	TEI16

Moved from 5.1.3

R2-2504107	Correction to delivery of SIB segments by dedicated signalling in RRC_CONNECTED	Huawei, HiSilicon	CR	Rel-18	38.331	18.5.1	5362	-	A	TEI16

Moved from 5.1.3

-	Xiaomi think this is an optimization and don’t want to change, but if all companies want to do this then only the second change is acceptable. Nokia think this is not essential. Ericsson has some sympathy for the change but don’t think this CR in the current form is not acceptable since the change should be done to the on-demand part and not the broadcast part. Qualcomm understands that there is an issue but think the proposed solution does not cover all cases e.g. the pos-sibs can have 64 segments but there can at most be 32 SI messages. Also, Qualcomm agrees that the on-demand provisioning section is the place to change instead.

-	Huawei think this is not an optimization. Huawei think it is OK to change this part of the spec rather than the on-demand.

-	ZTE doesn’t think the CR is necessary.

Postponed

Spatial Relation Info list extension

R2-2503854	Corrections on Spatial Relation Info list and sizeExt list for PUCCH	ZTE, Samsung, Qualcomm, CATT, MediaTek	CR	Rel-16	38.331	16.19.0	5350	-	F	NR_eMIMO-Core

R2-2503855	Corrections on Spatial Relation Info list and sizeExt list for PUCCH	ZTE, Samsung, Qualcomm, CATT, MediaTek	CR	Rel-17	38.331	17.12.0	5351	-	A	NR_eMIMO-Core

R2-2503856	Corrections on Spatial Relation Info list and sizeExt list for PUCCH	ZTE, Samsung, Qualcomm, CATT, MediaTek	CR	Rel-18	38.331	18.5.1	5352	-	A	NR_eMIMO-Core

R2-2504257	PUCCH SpatialRelationInfo Lists	Ericsson	discussion	Rel-16	NR_eMIMO-Core

Proposal 1	Any of the spatialRelationInfToReleaseLists can be used to remove list entries per identity value. No need for spec change.

Proposal 2	No spec changes to spatialRelationInfoToAddModList field description to clarify “single” and “concatenated”

Proposal 3	Consider whether the field description for spatialRelationInfoToAddModList should be clarified by adding “...as concatenated into a single list...” (in 38331 Rapp CR).

Proposal 4	In the field description for spatialRelationInfoToAddModList, delete text “(or deleted using spatialRelationInfoToReleaseListSizeExt)” in 38331 Rapp CR.

-	ZTE think there are critical issues which need to be addressed. CATT think the issue is valid but think we need further discussion. Nokia think the Ericsson approach is the way to go. Huawei think the current spec is clear enough and works so no change is needed at all. Huawei wonders what scenarios are problematic and want companies to clarify this. Samsung think there are different interpretations in RAN2 and some clarification is needed.

-	Ericsson think there is no issue with current spec and want companies who have seen issue to clarify what those are.

-	ZTE want to postponed. Ericsson can consider clarifying the guidelines in RRC.

[Post130][602][Maint] Spatial Relation Info list extension (ZTE)

Scope:

    - Discuss if/how to update the spec to clarify handling of extension lists.

Intended outcome:

    - Agreeable CRs submitted to the next meeting, if needed

Deadline:

    - Long

#### 5.1.3.2	UE capabilities

UE cap corrections 38306, 38331

Band combination specific power class

R2-2503595	Clarifications on band combination specific power class	Qualcomm Incorporated	CR	Rel-16	38.306	16.20.0	1275	-	F	NR_newRAT-Core, TEI16

R2-2503596	Clarifications on band combination specific power class	Qualcomm Incorporated	CR	Rel-17	38.306	17.12.0	1276	-	A	NR_newRAT-Core, TEI16

R2-2503597	Clarifications on band combination specific power class	Qualcomm Incorporated	CR	Rel-18	38.306	18.5.0	1277	-	A	NR_newRAT-Core, TEI16

The 3 above are noted

-	Huawei think the CR is wrong and there is still ongoing discussion in RAN4 and RAN2 cannot open this discussion. OPPO think there is an issue about UL being 1 CC and DL is 2 CC and this is discussed in RAN4. Nokia think the CR is just pointing to RAN4 specs and think this shouldn’t be controversial. ZTE agrees with the CR. Ericsson think we should spell out that this capability is only applicable for UL CA.

-	CATT think we should just refer to the RAN4 spec not the section numbers. Qualcomm think this is not feasible.

-	Huawei has strong concerns due to relation to a very long RAN4 discussion. Qualcomm does not understand why this CR is not agreeable. Vivo think the change is OK to do and there is no issue with RAN4. Huawei has an issue with the cover page.

Attempt to produce agreeable CRs offline, if possible

[AT130][603][Maint] Band combination specific power class (Qualcomm)

Scope:

    - Produce agreeable CRs if needed

Intended outcome:

    - Agreed CR(s) in R2-2504841, R2-2504842, R2-2504843, if needed

Deadline:

    - Wednesday 17:00. The intention is to agree over email.

-	After the offline discussion, Qualcomm reports that this better be discussed in RAN4 and their CRs can be noted.

scellWithoutSSB

R2-2504190	Correction to scellWithoutSSB	Ericsson	CR	Rel-15	38.306	15.27.0	1287	-	F	NR_newRAT-Core

R2-2504191	Correction to scellWithoutSSB	Ericsson	CR	Rel-16	38.306	16.20.0	1288	-	A	NR_newRAT-Core

R2-2504192	Correction to scellWithoutSSB	Ericsson	CR	Rel-17	38.306	17.12.0	1289	-	A	NR_newRAT-Core

R2-2504193	Correction to scellWithoutSSB	Ericsson	CR	Rel-18	38.306	18.5.0	1290	-	A	NR_newRAT-Core

-	Xiaomi, OPPO and Huawei think the capability is Conditionally Mandatory. Xiaomi points out there is another related capability. Nokia think that we could change from Rel-18, perhaps a note. Ericsson is OK to change from Rel-18.

Postponed, but if we do changes, only do them from Rel-18.

PDCP duplication

R2-2504590	Correction on pdcp-DuplicationSRB for NR-DC	Huawei, HiSilicon	CR	Rel-15	38.306	15.27.0	1300	-	F	NR_newRAT-Core

R2-2504591	Correction on pdcp-DuplicationSRB for NR-DC	Huawei, HiSilicon	CR	Rel-16	38.306	16.20.0	1301	-	A	NR_newRAT-Core

R2-2504592	Correction on pdcp-DuplicationSRB for NR-DC	Huawei, HiSilicon	CR	Rel-17	38.306	17.12.0	1302	-	A	NR_newRAT-Core

R2-2504593	Correction on pdcp-DuplicationSRB for NR-DC	Huawei, HiSilicon	CR	Rel-18	38.306	18.5.0	1303	-	A	NR_newRAT-Core

All 4 are agreed

MIMO

R2-2504624	Clarification on the maxMIMO-LayersForMulti-DCI-mTRP-r16	ZTE Corporation	discussion	Rel-16	NR_eMIMO

Proposal 1: RAN2 to clarify that for the mDCI mTRP:

If maxMIMO-LayersForMulti-DCI-mTRP-r16 is not included, maxNumberMIMO-LayersPDSCH is interpreted as the maximum number of layers per PDSCH if no RE overlapped.

Proposal 1a: If the proposal 1 was confirmed, agree the TP in the Annex.

Proposal 2: RAN2 to confirm that for the case 3 (maxMIMO-LayersForMulti-DCI-mTRP-r16 is absent and no RE overlappedHo), each multi-DCI based multi-TRP CC is counted 1 time toward J and thus no spec change is needed for the maximum data rate calculation.

Proposal 3: If companies can’t achieve the consensus, an LS can be sent to RAN1 to confirm the above proposal 1 and proposal 2.

-	Xiaomi think no change is needed. OPPO agrees with Xiaomi. Qualcomm would be OK with this change but a clarification in minutes may be good enough. Huawei agrees with Xiaomi and OPPO and would be OK to capture something in the minutes.

RAN2 understands that if maxMIMO-LayersForMulti-DCI-mTRP-r16 is not included, maxNumberMIMO-LayersPDSCH is interpreted as the maximum number of layers per PDSCH if no RE overlapped.

#### 5.1.3.3	Other

This agenda item addresses the idle and inactive behaviour specified in 38.304 or 36.304, LTE-specific changes for the applicable WIs, Other parts not covered elsewhere.

# 6	NR Rel-17

Essential corrections only.  Editorial/clarifications should be sent to be reviewed and approved by spec rapporteurs prior to submission.  Editorials should only be submitted by spec rapporteurs.

Tdoc limitation: 3 Tdocs

## 6.1	Common

(NR_MG_enh-Core; leading WG: RAN4; REL-17; WID: RP-211591)

(NR_UDC_enh-Core; leading WG: RAN2; REL-17; WID: RP-211203)

(NG_RAN_PRN_enh-Core; leading WG: RAN3; REL-17; WID: RP-202363)

(NR_IAB_enh-Core; leading WG: RAN2; REL-17; WID: RP-211548)

(NR_UE_pow_sav_enh-Core; leading WG: RAN2; REL-17; WID: RP-212630)

(LTE_NR_DC_enh2-Core; leading WG: RAN2; REL-17; WID: RP-201040)

(LTE_NR_MUSIM-Core; leading WG: RAN2; REL-17; WID: RP-212610)

(NR_Slice-Core; leading WG: RAN2; REL-17; WID: RP-212534)

(NR_QoE-Core; leading WG: RAN3; REL-17; WID: RP-211406)

(NR_ext_to_71GHz-Core; leading WG: RAN1; REL-17; WID: RP-212637)

(NR_cov_enh-Core; leading WG: RAN1; REL-17; WID: RP-211566): non-RACH-indication parts

(NR_redcap-Core; leading WG: RAN1; REL-17; WID: RP-211574)

(NR_feMIMO-Core; leading WG: RAN1; REL-17; WID: RP-212535)

(NR_SmallData_INACTIVE-Core, leading WG: RAN2; REL-17; WID: RP-212594)

(NR_IIOT_URLLC_enh-Core; leading WG: RAN2; REL-17; WID: RP-210854)

(NR_MBS-Core; leading WG: RAN2; REL-17; WID: RP-201038)

(NR_ENDC_SON_MDT_enh-Core; leading WG: RAN3; REL-17; WID: RP-201281)

(NR_NTN_solutions-Core; leading WG: RAN2; REL-17; WID: RP-211557)

(NR_SL_enh-Core; leading WG: RAN1; REL-17; WID: RP-202846)

(NR_SL_Relay-Core; leading WG: RAN2; REL-17; WID: RP-212601)

PRACH partitioning items

(NR TEI17)

Includes Rel-17 Work Items without specific R2 Agenda Item, e.g. RAN1 and RAN4 led items, SA2 and CT1 led items (was previously “Rel-17 Other”)

Includes aspects that does not fit under the more specific AIs, e.g. multi-WI aspects.

Corrections for NR_NTN_solutions-Core might be treated in the NTN breakout session.

NTN – To be handled in Sergio’s session

R2-2504661	Removal of PWS support in NR NTN	Ericsson	CR	Rel-17	38.304	17.10.0	0438	-	F	NR_NTN_solutions-Core

### 6.1.1	Stage 2 and Organisational

Submit to one of the agenda items below.

R2-2503318	Reply LS on emergency call back and paging (R3-252402; contact: ZTE)	RAN3	LS in	Rel-17	NR_newRAT-Core, NR_redcap-Core	To:SA2, RAN2	Cc:CT1

Noted

#### 6.1.1.0	In-principle agreed CRs

R2-2504508	Correction on cell type specific cell reselection prioritisation [NR_HSDN]	Huawei, HiSilicon, Nokia, Qualcomm, CMCC, Xiaomi, CATT, Apple, NTT DOCOMO, INC., Ericsson	CR	Rel-17	38.300

R2-2504509	Correction on cell type specific cell reselection prioritisation [NR_HSDN]	Huawei, HiSilicon, Nokia, Qualcomm, CMCC, Xiaomi, CATT, Apple, NTT DOCOMO, INC., Ericsson	CR	Rel-18	38.300	18.5.0	0993	-	F	TEI17, NR_mobile_IAB-Core

Both agreed

#### 6.1.1.1	Other

Incoming LSs, etc. You should discuss your stage 2 CRs with the specification rapporteurs before submission. Includes impact to 38.300, 37.340, (36.300 if applicable)

Delay measurements

R2-2504228	Correction on M6 definition for NR MDT	Huawei, HiSilicon	CR	Rel-17	37.320	17.5.0	0141	-	F	NR_ENDC_SON_MDT_enh-Core

R2-2504229	Correction on M6 definition for NR MDT	Huawei, HiSilicon	CR	Rel-18	37.320	18.3.0	0142	-	A	NR_ENDC_SON_MDT_enh-Core

Both agreed

NTN – To be handled in Sergio’s session

R2-2504520	Clarification on redirection and cell reselection to NR NTN	CATT, Nokia, Nokia Shanghai Bell, Qualcomm Incorporated	CR	Rel-17	36.300	17.9.0	1418	-	F	NR_NTN_solutions-Core

R2-2504521	Clarification on redirection and cell reselection to NR NTN	CATT, Nokia, Nokia Shanghai Bell, Qualcomm Incorporated	CR	Rel-18	36.300	18.4.0	1419	-	A	NR_NTN_solutions-Core

### 6.1.3	Control Plane corrections

Submit to one of the agenda items below.

#### 6.1.3.0	In-principle agreed CRs

R2-2503456	Usage of msg3-transmitPrecoder	CATT, Huawei, HiSilicon	CR	Rel-17	38.331	17.12.0	5286	2	F	NR_SmallData_INACTIVE-Core, NR_cov_enh-Core, NR_redcap-Core, NR_slice-Core	R2-2503112

R2-2503457	Usage of msg3-transmitPrecoder	CATT, Huawei, HiSilicon	CR	Rel-18	38.331	18.5.1	5287	2	A	NR_SmallData_INACTIVE-Core, NR_cov_enh-Core, NR_redcap-Core, NR_slice-Core	R2-2503113

R2-2504696	Correction to ul-GapFR2-r17	Huawei, HiSilicon, Apple, Nokia, Nokia Shanghai Bell, Ericsson, MediaTek Inc.	CR	Rel-17	38.306	17.12.0	1265	2	F	NR_RF_FR2_req_enh2	R2-2502837

R2-2504697	Correction to ul-GapFR2-r17	Huawei, HiSilicon, Apple, Nokia, Nokia Shanghai Bell, Ericsson, MediaTek Inc.	CR	Rel-18	38.306	18.5.0	1266	2	A	NR_RF_FR2_req_enh2	R2-2502838

R2-2504648	Correction on SCS and CP configuration in RedCap-specific initial BWP	ZTE Corporation, Sanechips, Vivo, Ericsson, MediaTek Inc, Qualcomm Incorporated, OPPO, Samsung	CR	Rel-17	38.331	17.12.0	5212	3	F	NR_redcap-Core	R2-2502335

R2-2504016	Correction on SCS and CP configuration in RedCap-specific initial BWP	ZTE Corporation, Sanechips, Vivo, Ericsson, MediaTek Inc, Qualcomm Incorporated, OPPO, Samsung	CR	Rel-18	38.331	18.5.1	5213	2	A	NR_redcap-Core	R2-2502336

R2-2504083	Corrections to location-based measurement	ZTE Corporation, Ericsson, CATT, Sanechips	CR	Rel-17	38.331	17.12.0	5303	1	F	NR_NTN_solutions-Core	R2-2502519

R2-2504084	Corrections to location-based measurement initiation	ZTE Corporation, Ericsson, CATT, Sanechips	CR	Rel-17	38.306	17.12.0	1255	1	F	NR_NTN_solutions-Core	R2-2502520

R2-2504085	Corrections to location-based measurement initiation	ZTE Corporation, Ericsson, CATT, Sanechips	CR	Rel-18	38.306	18.5.0	1256	1	A	NR_NTN_solutions-Core	R2-2502521

R2-2504087	Correction on timeUntilReconnection in RLF report	ZTE Corporation, Xiaomi, Sanechips	CR	Rel-17	36.331	17.12.0	5108	2	F	NR_ENDC_SON_MDT_enh-Core	R2-2503102

R2-2504088	Correction on timeUntilReconnection in RLF report	ZTE Corporation, Xiaomi, Sanechips	CR	Rel-18	36.331	18.5.0	5109	2	A	NR_ENDC_SON_MDT_enh-Core	R2-2503103

All above are agreed

R2-2503887	Slot aggregation configuration with multi-PUSCH	Nokia	CR	Rel-17	38.331	17.12.0	5328	1	F	NR_ext_to_71GHz-Core	R2-2502920

Agreed

R2-2503888	Slot aggregation configuration with multi-PUSCH	Nokia	CR	Rel-18	38.331	18.5.1	5329	1	A	NR_ext_to_71GHz-Core	R2-2502921

Agreed unseen in R2-2504844 but revision number should be 2.

R2-2504844	Slot aggregation configuration with multi-PUSCH	Nokia	CR	Rel-18	38.331	18.5.1	5329	2	A	NR_ext_to_71GHz-Core

Agreed

Withdrawn/Old revisions

R2-2504015	Correction on SCS and CP configuration in RedCap-specific initial BWP	ZTE Corporation, Sanechips, Vivo, Ericsson, MediaTek Inc, Qualcomm Incorporated, OPPO, Samsung	CR	Rel-17	38.331	17.12.0	5212	2	F	NR_redcap-Core	R2-2502335	Withdrawn

R2-2503933	Correction to ul-GapFR2-r17	Huawei, HiSilicon, Apple, Nokia, Nokia Shanghai Bell, Ericsson, MediaTek Inc.	CR	Rel-17	38.306	17.12.0	1265	1	F	NR_RF_FR2_req_enh2	R2-2502837

R2-2503934	Correction to ul-GapFR2-r17	Huawei, HiSilicon, Apple, Nokia, Nokia Shanghai Bell, Ericsson, MediaTek Inc.	CR	Rel-18	38.306	18.5.0	1266	1	A	NR_RF_FR2_req_enh2	R2-2502838

#### 6.1.3.1	NR RRC

Corrections to 38331, and related change to other TS if applicable, except UE caps.

SHR

R2-2503576	Correction on SHR determination procedure	CATT	CR	Rel-17	38.331	17.12.0	5340	-	F	NR_ENDC_SON_MDT_enh-Core

Agreed

R2-2503577	Correction on SHR determination procedure	CATT	CR	Rel-18	38.331	18.5.1	5341	-	A	NR_ENDC_SON_MDT_enh-Core

-	Xiaomi think the R18 CR has errors on the cover page, and want to remove the sentence on the cover page starting with “By adding a restriction”

Remove the sentence starting with “By adding a restriction” on the cover page and with this it is agreed unseen in R2-2504849.

R2-2504515	Correction for setting choCandidate in SHR	Sharp	CR	Rel-17	38.331	17.12.0	5378	-	F	NR_ENDC_SON_MDT_enh-Core

R2-2504517	Correction for setting choCandidate in SHR	Sharp	CR	Rel-18	38.331	18.5.1	5379	-	A	NR_ENDC_SON_MDT_enh2-Core

Moved from 7.0.2.11

-	CATT think this is obsolete since the CATT CR was agreed above.

Noted

Preamble partitioning

R2-2504456	Discussion on rsrp-ThresholdSSB-r17 in TS 38.331	Huawei, HiSilicon, Qualcomm, MediaTek, ZTE, Xiaomi, Vivo, Ericsson	discussion	Rel-17	NR_redcap-Core, NR_cov_enh-Core, NR_slice-Core, NR_SmallData_INACTIVE-Core

Proposal 1: RAN2 to discuss for 4-step Case 1 (Rel-15):

•	If the rsrp-ThresholdSSB (without suffix) is absent in rach-ConfigCommon (without suffix)；

	Option 1a: The UE shall not apply any threshold for SSB selection in the 4-step RA procedure (majority).

	Option 1b: The network should always include rsrp-ThresholdSSB in rach-ConfigCommon.

Proposal 2: RAN2 to agree for 4-step Case 2 and case 3 (Rel-17):

•	If the rsrp-ThresholdSSB-r17 is absent in FeatureCombinationPreambles included in rach-ConfigCommon(without suffix) or rach-ConfigCommon-r17, the UE shall apply the corresponding parameter (i.e. rsrp-ThresholdSSB without suffix) included in the RACH-ConfigCommon which includes the FeatureCombinationPreambles.

•	Furthermore, if the rsrp-ThresholdSSB-r17 is absent in FeatureCombinationPreambles and the rsrp-ThresholdSSB (without suffix) is absent in the RACH-ConfigCommon which includes the FeatureCombinationPreambles, the UE shall not apply any threshold for SSB selection in the 4-step RA procedure, i.e. NO parameter selection fallback from the rach-ConfigCommon-r17 in AdditionalRACH-Config-r17 to rach-ConfigCommon (without suffix).

Proposal 3: RAN2 to discuss for 2-step Case 4 (Rel-16):

•	If the msgA-RSRP-ThresholdSSB-r16 is absent in msgA-ConfigCommon-r16；

	Option 4a: The UE shall not apply any threshold for SSB selection in the 2-step RA procedure (majority).

	Option 4b: The network should always include msgA-RSRP-ThresholdSSB-r16 in msgA-ConfigCommon-r16.

Proposal 4: RAN2 to discuss for 2-step Case 5 (Rel-17):

-	If the rsrp-ThresholdSSB-r17 (i.e. msgA-RSRP-ThresholdSSB for 2-step RA procedure) is absent in FeatureCombinationPreambles-r17 included in msgA-ConfigCommon-r16 or msgA-ConfigCommon-r17.

-	Option 5a: The UE shall apply the rsrp-ThresholdSSB-r17 if included in rach-ConfigCommon for the same feature combination. If there is no rsrp-ThresholdSSB-r17 in rach-ConfigCommon for the same feature combination, the UE shall apply the msgA-RSRP-ThresholdSSB-r16 if included in msgA-ConfigCommon-r16 or msgA-ConfigCommon-r17 including the FeatureCombinationPreambles-r17.

-	Option 5b: The UE shall apply the msgA-RSRP-ThresholdSSB-r16 if included in the msgA-ConfigCommon-r16 or msgA-ConfigCommon-r17 including the FeatureCombinationPreambles-r17. (slight Majority)

Proposal 5: RAN2 to discuss for 2-step Case 6 (Rel-17):

•	If the msgA-RSRP-ThresholdSSB-r16 is absent in msgA-ConfigCommon-r17 and rsrp-ThresholdSSB-r17 (i.e. msgA-RSRP-ThresholdSSB for 2-step RA procedure) is absent in FeatureCombinationPreambles-r17 included in the msgA-ConfigCommon-r17；

	Option 6a: The UE shall not apply any threshold for SSB selection in the 2-step RA procedure, i.e. NO parameter selection fallback from the msgA-ConfigCommon-r17 in AdditionalRACH-Config-r17 to msgA-ConfigCommon-r16 (Majority).

	Option 6b: The network should always include either msgA-RSRP-ThresholdSSB-r17 in msgA-ConfigCommon-r16 or rsrp-ThresholdSSB-r17 (i.e. msgA-RSRP-ThresholdSSB for 2-step RA procedure) is included in FeatureCombinationPreambles-r17 which is included in the msgA-ConfigCommon-r17.

On P1:

-	Xiaomi think 1a is correct. Samsung think we should not change from Rel-15, but can consider a change in Rel-17. Vivo think we should confirm the understanding, and think 1a is as current spec. Qualcomm agrees with vivo. LG think its important to have a common understanding but don’t think we need to change the spec but can capture in minutes instead.

Agreements RAN2 understands that if the rsrp-ThresholdSSB (without suffix) is absent in rach-ConfigCommon (without suffix) the UE does not apply any threshold for SSB selection in the 4-step RA procedure. RAN2 will clarify in the spec that for 4-step Case 2 and case 3 (Rel-17): • If the rsrp-ThresholdSSB-r17 is absent in FeatureCombinationPreambles included in rach-ConfigCommon(without suffix) or rach-ConfigCommon-r17, the UE shall apply the corresponding parameter (i.e. rsrp-ThresholdSSB without suffix) included in the RACH-ConfigCommon which includes the FeatureCombinationPreambles. RAN2 understands that if the rsrp-ThresholdSSB-r17 is absent in FeatureCombinationPreambles and the rsrp-ThresholdSSB (without suffix) is absent in the RACH-ConfigCommon which includes the FeatureCombinationPreambles, the UE shall not apply any threshold for SSB selection in the 4-step RA procedure, i.e. NO parameter selection fallback from the rach-ConfigCommon-r17 in AdditionalRACH-Config-r17 to rach-ConfigCommon (without suffix). RAN2 understands that for 2-step Case 4 (Rel-16): • If the msgA-RSRP-ThresholdSSB-r16 is absent in msgA-ConfigCommon-r16 the UE does not apply any threshold for SSB selection in the 2-step RA procedure. RAN2 to clarify in the spec that for 2-step Case 5 (Rel-17): - If the rsrp-ThresholdSSB-r17 (i.e. msgA-RSRP-ThresholdSSB for 2-step RA procedure) is absent in FeatureCombinationPreambles-r17 included in msgA-ConfigCommon-r16 or msgA-ConfigCommon-r17 the UE shall apply the msgA-RSRP-ThresholdSSB-r16 if included in the msgA-ConfigCommon-r16 or msgA-ConfigCommon-r17 including the FeatureCombinationPreambles-r17 RAN2 understands that for 2-step Case 6 (Rel-17): • If the msgA-RSRP-ThresholdSSB-r16 is absent in msgA-ConfigCommon-r17 and rsrp-ThresholdSSB-r17 (i.e. msgA-RSRP-ThresholdSSB for 2-step RA procedure) is absent in FeatureCombinationPreambles-r17 included in the msgA-ConfigCommon-r17 the UE does not apply any threshold for SSB selection in the 2-step RA procedure, i.e. NO parameter selection fallback from the msgA-ConfigCommon-r17 in AdditionalRACH-Config-r17 to msgA-ConfigCommon-r16

[AT130][604][Maint] rsrp-ThresholdSSB handling (Huawei)

Scope:

    - Produce agreeable CRs

Intended outcome:

    - Agreed CR(s) in R2-2504845 and R2-2504846

Deadline:

    - Wednesday 17:00. The intention is to agree over email.

-	Huawei reports that the agreement needs updated (done above already) and the CRs need to be revised to align with this.

R2-2504850	Corrections to rsrp-ThresholdSSB threshold	Huawei

R2-2504851	Corrections to rsrp-ThresholdSSB threshold	Huawei

Both are agreed unseen

NTN – To be handled in Sergio’s session

R2-2504203	Corrections on measurement with (cond)EventD1/D2/T1	Samsung	discussion	Rel-17	NR_NTN_solutions, NR_NTN_enh-Core

R2-2504204	Corrections on measurement with (cond)EventD1/T1	Samsung	CR	Rel-17	38.331	17.12.0	5365	-	F	NR_NTN_solutions

R2-2504205	Corrections on measurement with (cond)EventD1/D2/T1	Samsung	CR	Rel-18	38.331	18.5.1	5366	-	F	NR_NTN_solutions, NR_NTN_enh-Core

Withdrawn/Old revisions

R2-2504001	Discussion on rsrp-ThresholdSSB-r17 in TS 38.331	Huawei, HiSilicon, Qualcomm, MediaTek, ZTE,  Xiaomi, Vivo	discussion	Rel-17	NR_redcap-Core, NR_cov_enh-Core, NR_slice-Core, NR_SmallData_INACTIVE-Core	Withdrawn

#### 6.1.3.2	UE capabilities

UE cap corrections 38306, 38331.

NTN – To be handled in Sergio’s session

R2-2503937	Capability for SMTC adjustment in RRC_IDLE and RRC_INACTIVE	Huawei, HiSilicon, CMCC, Qualcomm Incorporated.	CR	Rel-17	38.306	17.12.0	1281	-	F	NR_NTN_solutions-Core

R2-2503938	Capability for SMTC adjustment in RRC_IDLE and RRC_INACTIVE	Huawei, HiSilicon, CMCC, Qualcomm Incorporated.	CR	Rel-18	38.306	18.5.0	1282	-	A	NR_NTN_solutions-Core

R2-2504315	Clarification on support of DCP in NTN	Qualcomm Inc.	CR	Rel-17	38.306	17.12.0	1294	-	F	NR_NTN_solutions-Core

R2-2504316	Clarification on support of DCP in NTN	Qualcomm Inc.	CR	Rel-18	38.306	18.5.0	1295	-	A	NR_NTN_solutions-Core

#### 6.1.3.3	Other

Including idle and inactive behaviour specified in 38.304 or 36.304.

eDRX operation with emergency

R2-2504025	Discussion on eDRX operation with emergency PDU session	ZTE Corporation	discussion	Rel-17	NR_newRAT-Core, NR_redcap-Core

Proposal 1: 	RAN2 confirm that the RRC_INACTIVE UE does not need to ignore the RAN eDRX (if configured) when the UE has emergency PDU session (no spec change is needed).

Proposal 2: 		RAN2 confirm that if gNB is able to recognize the presence of emergency PDU session, the gNB can also keep the UE in RRC_CONNECTED state for fast emergency callback (this is not forbidden by the endorsed RAN3 CR).

Proposal 3: 	Approve the reply LS to SA2 and RAN3 in [4].

R2-2504266	Discussion on emergency services and eDRX	Nokia, Huawei, HiSilicon	discussion	Rel-17	NR_redcap-Core

Proposal 1:	RAN2 confirms and replies SA2 that the RAN node may release the UE to RRC_IDLE or RRC_INACTIVE state from RRC_CONNECTED when the UE has an emergency PDU session.

Proposal 2:	RAN2 agrees the UE side solution, which does not require any SA2 solution/conclusion: Clarify in TS 38.304 Clause 7.1 that in RRC_IDLE and RRC_INACTIVE states, the UE shall operate in DRX, not in eDRX, when the UE has a PDU session associated with emergency services. We provide a draft TP for Rel-17 TS 38.304 in the Annex.

R2-2504267	(e)DRX usage clarification for UEs with emergency PDU Sessions	Nokia, Huawei, HiSilicon	CR	Rel-17	38.304	17.10.0	0435	-	F	NR_redcap-Core

R2-2504268	(e)DRX usage clarification for UEs with emergency PDU Sessions	Nokia, Huawei, HiSilicon	CR	Rel-18	38.304	18.4.0	0436	-	A	NR_redcap-Core

R2-2504287	eDRX and emergency PDU session	Ericsson	discussion	Rel-17	NR_newRAT-Core, NR_redcap-Core	R2-2502909

Proposal 1	No further clarifications are needed for eDRX and emergency PDU session from RAN2 perspective.

-	Huawei think that RAN3 only discussed INACTIVE, not IDLE. Huawei think that current NWs may release the UE to IDLE and want to tell this to SA2 and RAN3. Huawei think SA2 are waiting for RAN2. ZTE think we already clarified that IDLE is not a problem. Qualcomm agrees with ZTE and think this is not a RAN2 discussion, and think that NW can solve this problem so no need to update (in an NBC-way) the UE behaviour. Xiaomi agrees with Qualcomm. Vivo think that SA2 already captured IDLE and INACTIVE is solved by RAN3. LG agrees with ZTE. MediaTek agrees with Qualcomm. Samsung agrees with QC.

R2-2504026	[draft] Reply LS on emergency call back and paging	ZTE Corporation	LS out	Rel-17	NR_newRAT-Core, NR_redcap-Core	To:SA2, RAN3	Cc:CT1

-	Nokia also want to ask if/who should address the case when the NW is not aware of an emergency PDU session. Ericsson assumes the NW can know this. Qualcomm think we don’t need to discuss SA2 matters in RAN2.

-	Qualcomm wants to clarify that RAN2 specs does not specify gNB behaviour.

-	Huawei want to clarify that there may be networks already out there that releases the UE to IDLE.

-	vivo is OK with this version of the LS.

We will not update our specifications to address this scenario.

The direction of this LS is approvable but we will polish the wording offline.

[AT130][605][Maint] Reply LS on emergency call back and paging (ZTE)

Scope:

    - Produce approvable LS

Intended outcome:

    - Approved LS in R2-2504847

Deadline:

    - Wednesday 17:00. The intention is to agree over email.

    - In time for the Thursday session

R2-2504847	Reply LS on emergency call back and paging	RAN2

Approved

# 7	Rel-18

#### 7.0.2.11	Further enhancement of data collection for SON MDT in NR and EN-DC

(NR_ENDC_SON_MDT_enh2-Core; leading WG: RAN3; REL-18; WID: RP-221825)

IPA SON MDT

R2-2503611	Updating references to TS 28.558 from TS 28.552	Nokia	CR	Rel-18	37.320	18.3.0	0140	1	F	NR_ENDC_SON_MDT_enh2-Core	R2-2502407

R2-2504227	Correction on SON for NPN in TS 38.300	Huawei, HiSilicon, CATT, Samsung, ZTE, CMCC	CR	Rel-18	38.300	18.5.0	0986	1	F	NR_ENDC_SON_MDT_enh2-Core

R2-2504455	Correction on logging intendedSIBs	Samsung	CR	Rel-18	38.331	18.5.1	5322	1	F	NR_ENDC_SON_MDT_enh2-Core	R2-2502844

The 3 above are agreed

Other

R2-2504831	Correction on timeSinceSHR in SHR report	ZTE Corporation, Sanechips, Ericsson	CR	Rel-18	38.331	18.5.1	5304	2	F	NR_ENDC_SON_MDT_enh2-Core	R2-2504076

Updated version of an earlier IPA

Agreed

R2-2503575	Corrections on SPR configuration	CATT, Ericsson	CR	Rel-18	38.331	18.5.1	5339	-	F	NR_ENDC_SON_MDT_enh2-Core

Agreed

R2-2504292	Correction on ra-InformationCommon logging in RLF-Report SHR and SPR	Ericsson	CR	Rel-18	38.331	18.5.1	5369	-	F	NR_ENDC_SON_MDT_enh2-Core

-	Samsung think we should not change 5.7.10.6 and 5.7.10.7. Ericsson agrees

-	Xiaomi wonders if we need the CR since MRO is not supported for RACH-less. Ericsson does not agree. Huawei think that the CR is aligned with the intended behaviour.

First change is agreeable, polish offline.

[AT130][606][SONMDT] ra-InformationCommon logging in RLF-Report (Ericsson)

Scope:

    - Produce agreeable CR based on the first change in R2-2504292

Intended outcome:

    - Agreed CR(s) in R2-2504848

Deadline:

    - Wednesday 17:00. The intention is to agree over email.

R2-2504848	Correction on ra-InformationCommon logging in RLF-Report SHR and SPR	Ericsson	CR	Rel-18	38.331	18.5.1	5369	1	F	NR_ENDC_SON_MDT_enh2-Core

Agreed

R2-2504848	Correction on ra-InformationCommon logging in RLF-Report SHR and SPR	Ericsson	CR	Rel-18	38.331	18.5.1	5369	1	F	NR_ENDC_SON_MDT_enh2-Core

R2-2504293	Corrections on PCell information in SPR	Ericsson	CR	Rel-18	38.331	18.5.1	5370	-	F	NR_ENDC_SON_MDT_enh2-Core

-	Nokia wonders what the change does. Ericsson think current text suggests that pCellId is optional, but its not.

Agreed

R2-2504535	Correction on areaConfiguration	Samsung	CR	Rel-18	38.331	18.5.1	5381	-	F	NR_ENDC_SON_MDT_enh2-Core

Agreed

Old revisions

R2-2504076	Correction on timeSinceSHR in SHR report	ZTE Corporation, Sanechips	CR	Rel-18	38.331	18.5.1	5304	1	F	NR_ENDC_SON_MDT_enh2-Core	R2-2502530

# 8	Rel-19

## 8.10	SON/MDT Ph4

(NR_ENDC_SON_MDT_Ph4-Core; leading WG: RAN3; REL-19; WID: RP-234038)

Time budget: 0.5 TU

Tdoc Limitation: 2 tdocs

### 8.10.1	Organizational

LS, Rapporteur input, including workplan, etc.

Running CRs

R2-2503582	Introduction of SONMDT UE Capabilities	CATT	draftCR	Rel-19	38.331	18.5.1	B	NR_ENDC_SON_MDT_Ph4-Core

R2-2503583	Introduction of SONMDT UE Capabilities	CATT	draftCR	Rel-19	38.306	18.5.0	B	NR_ENDC_SON_MDT_Ph4-Core

R2-2503584	Introduction of SONMDT UE Capabilities	CATT	draftCR	Rel-19	36.306	18.4.0	B	NR_ENDC_SON_MDT_Ph4-Core

To be used as baseline in a post meeting email disc

[Post130][603][SONMDT] Running capability CR (CATT)

Scope:

    - Update the running CR

Intended outcome:

    - Updated running CR to be submitted to next meeting

Deadline:

    - Long

R2-2504234	Running 36.331 CR for R19 SONMDT	Huawei, HiSilicon	draftCR	Rel-19	36.331	18.5.0	B	NR_ENDC_SON_MDT_Ph4-Core

Endorsed and to be updated in a post meeting email disc

[Post130][604][SONMDT] Running LTE RRC CR (Huawei)

Scope:

    - Update the running CR

Intended outcome:

    - Updated running CR to be submitted to next meeting

Deadline:

    - Long

R2-2504235	Open issue list for running 36.331 CR for R19 SONMDT	Huawei, HiSilicon	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1: It is proposed RAN2 to discuss the open issue LTERRC-1, i.e. whether to keep the following condition or not (it has been captured in section 5.6.13a.3 in running 36.331 CR):

3>	if the last RRCReconfiguration message including the reconfigurationWithSync for the SCG was received to enter the PSCell in which the SCG failure was declared:

Proposal 2: It is proposed RAN2 to discuss the open issue LTERRC-2, i.e. when the LTM based recovery fails and the UE selects E-UTRA cell and receives RRCConnectionSetup, timeUntilReconnection need to refer to the first failure and this should be captured in running 36.331 CR.

Proposal 3: It is proposed RAN2 to discuss the open issue LTERRC-3, i.e. whether to have a single container to carry all the fields (PerRAInfoList-r16, PerRAInfoList-v1660 and PerRAInfoList-v1800) or not.

Noted

R2-2504295	Running CR for SONMDT features	Ericsson	draftCR	Rel-19	38.331	18.5.1	B	NR_ENDC_SON_MDT_Ph4-Core

Endorsed and to be updated in a post meeting email disc

R2-2504296	Open Issues for NR RRC SONMDT features	Ericsson	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1. Companies are invited to discuss the Open issue RRC-1 (i.e., the correlation of SHR and SPR and avoiding duplication of the information) in their contribution for the meeting RAN2#130.

Noted

[Post130][605][SONMDT] Running NR RRC CR (Ericsson)

Scope:

    - Update the running CR

Intended outcome:

    - Updated running CR to be submitted to next meeting

Deadline:

    - Long

### 8.10.2	MRO enhancements for Rel-18 mobility features

LTM has 1st priority. CHO with candidate SCGs has 2nd priority

Subsequent CPAC is paused until if/when we get a RAN3 LS on the subject

Early TA and RRC-1

R2-2503633	MRO for LTM and near failure handling for CHO with candidate SCGs	Nokia	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Focus on P2 – P5

Proposal 1: RAN2 agrees that optimization of timing of ETA is a crucial part of MRO for LTM

Proposal 2: RAN2 agrees to log additional information regarding TAs that were unnecessarily acquisitioned.

Proposal 3: RAN2 to discuss adding the following information into RLF report: TA difference between TA provided by network in Cell Switch Command for which the RACH-less LTM failed, and TA received in RAR for recovery/re-establishment.

Proposal 4: RAN2 to discuss signalling that allows the UE to indicate that the use of the UE measurement-based TA is preferred.

Proposal 5: RAN2 to discuss and decide on a correlation mechanism for SHR and SPR generated for the same CHO with candidate SCG execution, for example by introducing a coexistence flag in each report.

Proposal 6: RAN2 to agree that there is a need to distinguish if the T310 started before or after fulfilment of the first execution conditions for CHO with Candidate SCG(s) and an indication about this in SHR and SPR.

On P2:

-	vivo wonders what is the relation between P2 and P3? Nokia think they are separate issues.

-	LG think this is a useful proposal but has concerns, e.g. NW can have this info, and usually early TA happens shortly before LTM switch.

-	Samsung think NW knows this by itself. Offino finds the proposal useful but “unnecessary” is not a good wording. Qualcomm think NW knows this.

On P3:

-	CATT this this is not necessary since the UE may have moved so the info is not useful. Ericsson think the info is useful and it would be possible to make an average TA diff value. Samsung think the UE may go to another cell and the NW can figure this out somehow.

On P4:

-	LG thinks this is an LTM enhancement.

On P5:

-	vivo presents their paper below (R2-2504618).

-	Nokia think its heavy for the NW to do the correction without a flag. Qualcomm think this should be a corner case that both are generated at the same time so its not a big issue. Ericsson think that the UE can indicate there are correlated reports in the availability report instead. LG think its important to specify some correlation information. Samsung think there will be some duplication meaning that the NW can correlate based on the duplicated info. Xiaomi thinks that no need to add correction info and think it’s a corner case. Huawei agrees with vivo and sees that there are several proposals to solve this.

R2-2504618	Discussion on open issue about CHO with candidate SCGs	vivo	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1: For CHO with candidate SCGs, the NW can correlate the SHR and SPR based on the existing source/target cells information and the new paring information (in ChoWithCandidateSCGInfo). Thus no additional correlation information is needed.

Proposal 2: For CHO with candidate SCGs, no need to consider avoiding duplicated information in SHR and SPR.

Agreements - We will not introduce UE reporting regarding TAs that were “unnecessarily” acquisitioned. - We will not add TA difference between TA provided by network in Cell Switch Command for which the RACH-less LTM failed, and TA received in RAR for recovery/re-establishment. - FFS if we add some correlation indication. - We will not add any optimizations to avoid duplicated info in correlated reports

LTERRC-1, LTERRC-2, LTERRC-3 and beam info in RLF report

R2-2504236	Discussion on MRO for mobility	Huawei, HiSilicon	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Focus on P1-P5

Proposal 1: (LTERRC-1) It is proposed to remove the condition below:

3>	if the last RRCReconfiguration message including the reconfigurationWithSync for the SCG was received to enter the PSCell in which the SCG failure was declared:

Proposal 2: (LTERRC-2) When the LTM based recovery fails and the UE selects E-UTRA cell and receives RRCConnectionSetup, timeUntilReconnection need to refer to the first failure.

Proposal 3: (LTERRC-3) It is proposed to have separate containers for all the fields i.e. PerRAInfoList-r16, PerRAInfoList-v1660 and PerRAInfoList-v1800.

Proposal 4: For RACH-less access based on network TA, include the following information in RLF report:

－	Time period between the reception of the PDCCH order and the reception of HO command MAC CE related to the failed LTM cell switch

Proposal 5: It is proposed RAN2 to agree to enhance the RLF report with the beam information contained in the LTM cell switch command.

Proposal 6: It is proposed RAN2 to agree that the legacy MRO for CHO is reused for triggered CHO-only in terms of RLF and SHR report, i.e. no special handling for CHO-only.

On P4:

-	Qualcomm thinks RAN3 are working on this.

On P5:

-	Qualcomm thinks RAN3 are discussing this.

Agreements - Remove the condition:3> if the last RRCReconfiguration message including the reconfigurationWithSync for the SCG was received to enter the PSCell in which the SCG failure was declared. - When the LTM based recovery fails and the UE selects E-UTRA cell and receives RRCConnectionSetup, timeUntilReconnection need to refer to the first failure. - Specify separate containers for all the fields i.e. PerRAInfoList-r16, PerRAInfoList-v1660 and PerRAInfoList-v1800.

RA report for LTM

R2-2504498	Discussion on random access report for LTM	ASUSTeK	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1:	RA reports for LTM shall include the following cases:

-	RACH for Early UL synchronization on a candidate cell.

-	NW-triggered RACH-based LTM towards a target cell.

-	UE-triggered RACH-based LTM towards a selected Cell after failed LTM execution.

Proposal 2:	Discuss whether to reuse current values of raPurpose (e.g., reconfigurationWithSync) or introduce new values of raPurpose for LTM-related RA reports.

Proposal 3:      A single RA report is used to indicate random access information for all early UL synchronization on the same candidate cell.

On P1:

-	vivo think the info is not useful to the NW. Ericsson does not agree with the first bullet of P1, but support the second two bullets. CATT agrees with the entire P1. Nokia think the first bullet is not needed and the second bullet is already covered by the current spec and the third one is also covered. Ofinno wants only the first bullet. Xiaomi does not think P1 is useful.

On P2:

-	Ericsson want a new codepoint. Vivo want to reuse the existing purpose cause we already have HO-type and with this info the NW knows everything. Ericsson think this is the RA report, not about the RLF report. ZTE wonders whats the purpose of this new purpose? Ericsson think that these are dedicated RA resources for early TA acquisition, so the NW need a new purpose indication. Samsung is OK with a new purpose. Xiaomi think we reports RA info common and that should tell if it was for LTM or not. Vivo does not agree that there is dedicated RA resources for LTM.

On P3:

-	 Nokia think this is not relevant since we didn’t agree bullet 1 in P1.

Agreements - Introduce new values of raPurpose for RACH-based LTM-related RA reports.

TA and L1 measurements

R2-2504613	Further considerations on MRO for LTM and CHO with candidate SCG	Ericsson	discussion

Focus on P2- P3

Proposal 1	RAN2 to consider the case of LTM cell switch failure due to outdated/wrong TA calculated by UE.

Proposal 2	The UE logs the TA received from RAR in the target cell in the RLF report.

Proposal 3	UE logs available L1 measurement results in the SHR for the serving cell, the target cell and other LTM candidate cells when the UE is LTM configured.

Proposal 4	UE reports the time gap between the reception of the last configuration of the CHO with candidate SCG configuration and the RLF/HOF.

Proposal 5	RAN2 to agree on the following alternatives to include target PSCell ID in the SHR:

a.	Explicit inclusion of the target PSCell ID in the SHR.

b.	Include a flag in the ChoWithCandidateSCG IE to indicate the target PCell and PSCell pair to which UE executed the CHO with candidate SCG procedure.

On P2:

-	Nokia does not know what the NW can do with this info. Samsung agrees with Nokia.

On P3:

-	vivo think there is no point in optimizing this. Ericsson explains its about the case when the NW configured L1 measurements but anyway a normal L3 Handover was performed. Ofinno supports the proposal.

-	Samsung are OK with the proposal but it only relates to the new L1 measurements.

Agreements - UE logs available L1 measurement results in the SHR for the serving, target and and neighbouring cells when the UE performs L3 handover and has an LTM configuration.

CHO with candidate SCGs with CHO-only

R2-2504399	MRO enhancements for CHO with candidate SCGs	CMCC	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1: RAN2 discusses MRO enhancements for the scenario that network provides a complementary CHO only configuration for CHO with candidate SCGs.

Proposal 2: UE reports the information of fulfilled condition(s), i.e., neither, CHO, or CPAC, when CHO only configuration is received, in RLF report and SHR report.

Proposal 3: UE reports following information in RLF report and SHR report:

－	Identifier of candidate PCell(s) which met the CHO execution conditions when CHO only configuration is received

－	Identifier of candidate PSCell(s) which met the CPAC execution conditions when CHO only configuration is received

－	The time difference between the first fulfilled execution condition and UE receives CHO only configuration

Discussion:

-	vivo wonders what the motivation for this is.

-	Ericsson want to reuse ASN.1 structures as much as possible if we do this.

-	Huawei supports doing enhancements in this area.

-	CATT supports P3.

-	Ericsson think we cannot tie anything to when a config is received, but we can tie things to when the RLF happens. Ericsson want to look at spec impact.

-	After hearing companies discuss, vivo are still not convinced what is the motivation for this.

-	Nokia agrees with Ericsson in that we must understand the actual impact. Nokia think we should have clear proposals in the next meeting.

-	Xiaomi does not want to spend time on this since there is not a lot time left and this seems rather complex.

Agreements - We will look in to if/what to specify for the scenario with CHO with candidate SCG alongside a CHO-only configuration. Proponents should have clear and well-defined proposals next meeting preferably with text proposals.

R2-2503914	Discussion on MRO enhancements for LTM	Lenovo	discussion	Rel-19

R2-2504224	Remaining issues for MRO enhancements	Samsung	discussion

R2-2503932	MRO enhancement for LTM	NEC	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

R2-2504568	Remaining MRO issues for MOB	LG Electronics Inc.	discussion	Rel-19

R2-2504621	Remaining issues for Rel-19 MRO	ZTE Corporation, Sanechips	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

R2-2504270	Discussion on MRO for LTM	Ofinno	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

To be treated in the comeback session (Thursday after coffeebreak):

R2-2503585	MRO Enhancements for Rel-18 Mobility	CATT	discussion	Rel-19

Focus on P2 and P8

Proposal 2: Include TCI state ID from LTM cell switch command in RLF report for RACH-less LTM failure.

Proposal 8: To address the issue of frequent overriding RLF report/SHR for subsequent LTM, recording multiple reports can be considered as a candidate solution.

Noted

On P2:

-	Qualcomm think we already agreed to not include this since NW knows this since its in the LTM command. Samsung agrees that the NW can store this info so no need for the UE to report. CATT think the local DU and the target CU can communicate this so they want the UE to report. Huawei supports the proposal. Huawei think that if we don’t do this then RAN3 must add new signalling. Samsung think the UE should only report what the NW cannot know. Ericsson supports the proposal since for a NW-based solution the DU must be involved which is not good. Qualcomm think RAN3 are already discussing these things.

-	CATT want to have a conditional agreement that if RAN3 solves this we don’t need to add UE reporting, but if they don’t we should have RAN2 signalling. ZTE thinks this makes sense and want to wait to see what happens in RAN3

On P8:

-	LG think the scenario is not valid. Samsung also think the scenario is invalid. Vivo agrees with LG and Samsung.

R2-2504512	Discussion on SON enhancement for R18 mobility features	SHARP Corporation	discussion

Focus on P6 and P7

Proposal 6: RAN2 clarifies the timeSinceCHO-Reconfig in SHR is for the time elapsed between CHO execution and the reception of the last executed CHO configuration for the target cell in CHO with candidate SCG case.

Proposal 7: RAN2 clarifies the timeSinceCPAC-Reconfig in SPR is for the time elapsed between CPAC execution and the reception of the last executed conditional reconfiguration for the target cell in CHO with candidate SCG case.

Noted

-	SHARP is not in the room to present. Ericsson think this is an open issue which needs to be addressed in the running CR and can be handled during the post meeting email discussion where we update the running CR.

R2-2504558	MRO enhancements for mobility	Qualcomm Incorporated	discussion	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1	Enhance MHR to include a “HO-type” (i.e., LTM or L3 HO).

Proposal 2	UE indicates to gNB in RLF Report if RACH-less LTM is attempted or not before RACH-based LTM failure.

Proposal 3	UE indicates to gNB in SHR if RACH-less LTM is attempted or not before RACH-based LTM successful RACH-based LTM recovery.

On P1:

-	Samsung think we shouldn’t include this in the MHI and the NW should know this. ZTE agrees. Qualcomm think the UE does not always know. Ericsson think the UHI solves this problem already.

On P2 and P3:

-	Samsung think this is know for RA-InformationCommon. LG supports P2 and P3. MediaTek think there is no such case as described by P2. Ericsson and ZTE agrees with MediaTek. ZTE think there might be NTN scenarios where this could be relevant.

-	MediaTek says that it is about fast recovery then there might be a point.

Agreements - We will not enhance MHI to include a “HO-type” (i.e., LTM or L3 HO).

R2-2503417	Discussion on Rel-19 SONMDT open issues	Xiaomi	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Focus on P2 and P4

Proposal 2: To align with RAN2#129bis agreement, RAN2 to explicitly capture target PSCell ID (without measurements) in SHR for successful CHO with candidate SCGs in RRC CR. Network can associate this ID with measurements retrieved from neighbour cell measurement results based on implementation.

Proposal 4: For SDT, the maximum value of 172800 in timeSinceSdtExecution IE only indicate 172800 seconds or longer. The FFS “FFS for the case beyond this value and the understanding from UE and network side, as for value beyond this range NW cannot recognize the right SDT configuration” can be removed.

On P2:

-	Ericsson think the PSCell ID is already in SHR but with the measurements, so is the proposal to remove the measurements? Nokia think we are duplicating some info if we do this proposal. Xiaomi clarifies that the proposal is to actually exclude measurements. Samsung does not want to change in the proposed way.

Agreements - The FFS in the running NR RRC CR “FFS for the case beyond this value and the understanding from UE and network side, as for value beyond this range NW cannot recognize the right SDT configuration” is removed.

### 8.10.3	Other

RACH optimization for SDT focus on RSRP and data volume in SON reports, and existing failure causes.

MHI Enhancement for SCG Deactivation/Activation.

SON/MDT for Slicing

SON/MDT for NTN

Slicing

R2-2504237	Discussion on SONMDT for others	Huawei, HiSilicon	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Focus on P2-P4

Proposal 1: It is proposed RAN2 to agree on introducing the distance from UE to serving cell and candidate cell.

Proposal 2: It is proposed RAN2 to agree on introducing the following report type in logged MDT configuration:

-	highest ranked cell or best cell does not support any or all of the intended NSAGs

Proposal 3: It is proposed RAN2 to agree on including the NSAG information received from NAS in logged MDT report.

Proposal 4: It is proposed RAN2 to discuss whether to address the case where the UE fall backs to legacy cell reselection procedure from slice-based cell reselection.

Proposal 5: It is proposed RAN2 to discuss possible solutions, including:

-	addition of a new report type related to P3 in logged MDT configuration

-	addition of a fallback indicator in logged MDT report

Proposal 6: It is proposed RAN2 to adopt the percentage ratio or accumulated time for UE in SCG deactivated state in MHI.

On P2 and P3:

-	Samsung don’t think this is needed since existing info is enough for the NW to deduce this from knowing where the UE is. Nokia does not think the NW knows which NSAG info the UE has or the reason why the UE is in a cell. Qualcomm think the point of the slicing reselection feature is to distribute UEs and there is no point in optimising this as proposed here. Ericsson also does not see the point of this info for the NW. Xiaomi think the enhancement is not needed. CMCC agrees with Huawei and Nokia.  ZTE think the NW can know this.

On P4:

-	Samsung thinks is already known to the NW. Huawei does not agree.

R2-2504443	Discussion on SON/MDT for Slicing and NTN	CMCC	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Focus on P1

Proposal 1: It is proposed to log the NSAG(s) supported by the highest ranked cell or best cell, the NSAG(s) and priority information supported by the UE (received from NAS), which is helpful for NW to optimize the configuration of sliceAllowedCellListNR and/or sliceExcludedCellListNR in SIB16.

Proposal 2: RAN2 to select Option 2 in RAN3 LS to optimize the distance threshold for location-based CHO in NTN for earth moving scenario, i.e., to introduce measured distance from UE to moving reference location of serving cell, and measured distance from UE to moving reference location of candidate cell when RLF/HOF happens in RLF report.

Proposal 3: To inform RAN3 with RAN2 decision in the reply LS.

Agreements - We aim to specify that: If the UE supports slice-based cell reselection does not find any suitable cell in the frequencies corresponding to the highest ranked NSAG, the UE logs the highest ranked NSAG. FFS the need of cell or frequency info.

NTN

R2-2503317	LS on SONMDT for NTN (R3-252382; contact: CMCC)	RAN3	LS in	Rel-19	NR_ENDC_SON_MDT_Ph4-Core	To:RAN2

Moved from 8.10.1

-	Option 1: to introduce the absolute UTC time when RLF/HOF happens, based on which the network computes the distance.

-	Option 2: to introduce measured distance from UE to moving reference location of serving cell, and measured distance from UE to moving reference location of candidate cell when RLF/HOF happens.

Noted

R2-2503418	Discussion on other aspects of Rel-19 SONMDT	Xiaomi	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1: RAN2 to introduce measured distance from UE to moving reference location of serving cell, and measured distance from UE to moving reference location of candidate cell in RLF report when RLF/HOF happens, based on which the network computes the distance.

Proposal 2: RAN2 to define a new CGI-Info-Logging-r19 IE to log trackingAreaList information for the support of MRO for NTN.

Proposal 3: When the UE is in IDLE/INACTIVE and the UE cannot obtain its location, the UE stops MDT logging for NTN.

R2-2504622	Remaining other issues for Rel-19 SON MDT	ZTE Corporation, Sanechips	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Focus on P3-P5

# for slicing

Proposal 1	RA report is to be used to optimize the slice based cell reselection, i.e., when UE does not re-select to the cell corresponding to the NSAG with highest priority.

Proposal 2	LS reply to RAN3 on RAN2 decision.

# for NTN

Proposal 3	RAN2 to adopt option 1 (i.e., absolute UTC time when RLF/HOF happens), and this time information could also be used for time based NTN CHO optimization.

Proposal 4	The additional geographical information is configured as a complimentary to existing areaConfiguration.

Proposal 5	If configured with additional geographical information, the UE logs the MDT data even if the UE cannot obtain its location.

Proposal 6	RAN2 LS RAN3 on the conclusion of both NTN (logged MDT, CHO for NTN) and slicing.

# For MHI

Proposal 7	Use an activated ratio instead of an absolute value to indicate the activation status of an PSCell.

Proposal 8	UE includes traffic characteristics (e.g., average data rate during activation period) in the MHI together with the SCG activation/deactivation information.

On distance vs. time:

-	ZTE prefer option 1 (time). Huawei think most are OK with distance. Ericsson think granularity may be an issue.

On P3 from Xiaomi and P5 from ZTE:

-	Ericsson thinks legacy approach is to continue to log. Nokia want to postpone to check with RAN3.

Agreements - We go with option 2 (distance), unless we find critical issues with granularity. - RAN2 to add logging of the first entry from trackingAreaList for the support of MRO for NTN. - If configured with additional geographical information, FFS if the UE logs the MDT data or not when it cannot obtain its location.

Capabilities

R2-2503581	Views on R19 SONMDT UE capabilities	CATT	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Moved from 8.10.1

Proposal 1: an optional UE capability with signalling is defined for RLF report for MCG LTM.

Proposal 2: an optional UE capability without signalling is defined for SHR for MCG LTM.

Proposal 3: an optional UE capability with signalling is defined for RLF report for CHO with candidate SCG.

Proposal 4: an optional UE capability without signalling is defined for SHR/SPR/SCGFailureInformation for CHO with candidate SCG.

Proposal 5: an optional UE capability without signalling is defined for RLF report for time/location based CHO.

Proposal 6: an optional UE capability with signalling is defined for geographic area scope checking for logged MDT.

Proposal 7: an optional UE capability without signalling is defined for RA-report for SDT.

Proposal 8: an optional UE capability without signalling is defined for MHI Enhancement for SCG Deactivation/Activation.

Proposal 9: an optional UE capability without signalling is defined for MRO for EN-DC SCG failure in LTE specification.

On P2

-	Samsung wants this to have signalling.

On all:

-	Qualcomm

Add an optional UE capability for RLF report for MCG LTM.

Add an optional UE capability for SHR for MCG LTM.

Add an optional UE capability for RLF report for CHO with candidate SCG.

Add an optional UE capability for SHR/SPR/SCGFailureInformation for CHO with candidate SCG.

Add an optional UE capability for RLF report for time/location based CHO.

Add an optional UE capability for geographic area scope checking for logged MDT. This will be an optional capability with signalling.

Add an optional UE capability for RA-report for SDT.

Add an optional UE capability for MHI Enhancement for SCG Deactivation/Activation.

Add an optional UE capability for EN-DC SCG failure in LTE specification.

SDT

R2-2504513	SON enhancements for R18 leftovers	SHARP Corporation	discussion

Proposal 1: UE logs the DL RSRP and UL data volume in both RA-SDT initiation failure case and normal ongoing RA-SDT failure case.

Proposal 2: the elapsed time since the execution of RA-SDT is also included in RA report for successful RA-SDT case.

Proposal 3: UE does not set the scgActiveDuration for a PSCell entry if the activation time UE spent in a PSCell is the same to the legacy timeSpent of the entry for PSCell.

On P1:

-	Ericsson think this is not useful and unclear which RSRP measuring time this refers to. ZTE agrees with Ericsson.

On P2:

-	ZTE thinks that this is not the intention of RAN3. Ericsson and Lenovo agrees.

On P3:

-	ZTE thinks this adds complexity for no good reason. Qualcomm thinks its an optimization.

R2-2503586	MHI Enhancement for SCG Activation and Deactivation	CATT	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

R2-2503634	SON/MDT enhancements for network slicing, MHI and NTN	Nokia	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

R2-2503669	MRO for intra-NTN mobility	China Telecom	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

R2-2503915	Discussion on MRO for intra-NTN mobility	Lenovo	discussion	Rel-19

R2-2504247	Discussion on NTN, slicing and other open issues	Samsung	discussion

R2-2504294	Discussion on SON-MDT enhancements for Slicing and NTN	Ericsson	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

R2-2504559	UE capabilitie for SON and MDT	Qualcomm Incorporated	discussion	NR_ENDC_SON_MDT_Ph4-Core

To be treated in the comeback session (Thursday after coffeebreak):

R2-2504569	NTN Logging for Unchanged PCI Mobility	LG Electronics Inc.	discussion	Rel-19	R2-2502808

Proposal 1: For unchanged PCI satellite mobility, the UE uses the RLF report, including a new cause to indicate a failure of the unchanged PCI satellite mobility if RLF is declared due to this mobility.

Proposal 2: For unchanged PCI satellite mobility, the UE reports the near failure information that out-of-sync indication(s) has been detected during the mobility if the UE detects the out-of-sync indications during the unchanged PCI satellite mobility but successfully complete this mobility.

Proposal 3: To determine a proper near failure of the unchanged PCI satellite mobility, a new logging condition is considered based on reception of a significant number of out-of-sync indications during the mobility.

On P1:

-	Nokia think this is a RAN3 discussion if anything. Qualcomm think we should just follow RAN3 progress. Samsung agrees. ZTE are interested in this topic but agrees with others that this should be discussed in RAN3 if anything. Huawei agrees with former speakers.

Agreements - RAN2 will not work on unchanged PCI satellite mobility unless RAN3 tells us to do so.

# Summary

Comebacks:

None

NBC CRs:

Note these were IPA in Wuhan:

R2-2503887	Slot aggregation configuration with multi-PUSCH	Nokia	CR	Rel-17	38.331	17.12.0	5328	1	F	NR_ext_to_71GHz-Core	R2-2502920

R2-2503888	Slot aggregation configuration with multi-PUSCH	Nokia	CR	Rel-18	38.331	18.5.1	5329	1	A	NR_ext_to_71GHz-Core	R2-2502921

Post meeting email discussions:

[Post130][602][Maint] Spatial Relation Info list extension (ZTE)

Scope:

    - Discuss if/how to update the spec to clarify handling of extension lists.

Intended outcome:

    - Agreeable CRs submitted to the next meeting, if needed

Deadline:

    - Long

[Post130][603][SONMDT] Running capability CR (CATT)

Scope:

    - Update the running CR

Intended outcome:

    - Updated running CR to be submitted to next meeting

Deadline:

    - Long

[Post130][604][SONMDT] Running LTE RRC CR (Huawei)

Scope:

    - Update the running CR

Intended outcome:

    - Updated running CR to be submitted to next meeting

Deadline:

    - Long

[Post130][605][SONMDT] Running NR RRC CR (Ericsson)

Scope:

    - Update the running CR

Intended outcome:

    - Updated running CR to be submitted to next meeting

Deadline:

    - Long