3GPP TSG RAN WG2#130	R2-2504995

St Julian’s, Malta, 19th - 23rd May 2025

3GPP TSG RAN WG1 #121	R1-2504959

St Julian's, Malta, 19th -23rd May, 2025

Title:	Reply LS on CB Msg3 EDT for IoT NTN Ph3

Response to:	R1-2503613/R2-2503175

Release:	Release 19

Work Item:	IoT_NTN_Ph3

Source:	RAN1

To:	RAN2

Contact Person:

Name:	Gilles Charbit

E-mail Address:	gilles.charbit@mediatek.com

# 1	Overall description

RAN1 would like to thank RAN2 for their LS on L1 parameters for on CB-msg3-EDT. RAN1 has considered the questions Q1, Q2, Q3, Q4, Q6, and Q7 on CB ms3 EDT. RAN1 made the following replies:

Q1: For both eMTC and NB-IoT, RAN2 assumes power ramping should be supported for CB-msg3-EDT. RAN2 would like to confirm with RAN1 on whether to have power ramping on this Msg3 (re-)transmission. If RAN1 confirms, please provide the control parameters for power ramping.

Question Q1

Reply to Q1:

RAN1 has not evaluated the potential performance of power ramping for CB-msg3-EDT, and it is likely that there will not be sufficient time to evaluate this topic within the R19 timeframe

- For open loop power control the following UL power control parameters can be reused for CB-msg3-EDT

- p0-UE-NPUSCH-r16 and alpha-r16 for NB-IoT NTN

Q2: For eMTC, RAN2 agrees to have MPDCCH configuration for shared resource configuration and decides to reuse the parameters from PUR MPDCCH configuration (in IE PUR-MPDCCH-Config-r16, as below) as baseline. RAN2 think TDD parameter is not needed and maybe narrowband configuration should be defined as a set, not one single narrow band. RAN2 would like to check with RAN1 on how to define the detail MPDCCH configuration.

Question Q2

PUR-MPDCCH-Config-r16 ::=		SEQUENCE {

mpdcch-FreqHopping-r16			BOOLEAN,

mpdcch-Narrowband-r16			INTEGER (1..maxAvailNarrowBands-r13),

mpdcch-PRB-PairsConfig-r16		SEQUENCE{

numberPRB-Pairs-r16				ENUMERATED {n2, n4, n6, spare1},

resourceBlockAssignment-r16		BIT STRING (SIZE(4))

},

mpdcch-NumRepetition-r16		ENUMERATED {r1, r2, r4, r8, r16, r32, r64, r128, r256},

mpdcch-StartSF-UESS-r16			CHOICE {

fdd								ENUMERATED {v1, v1dot5, v2, v2dot5, v4, v5, v8, v10},

tdd							ENUMERATED {v1, v2, v4, v5, v8, v10, v20, spare1}

},

mpdcch-Offset-PUR-SS-r16	ENUMERATED {zero, oneEighth, oneQuarter,

threeEighth, oneHalf, fiveEighth,

threeQuarter, sevenEighth}

}

Reply to Q2:

From RAN1 perspective, the MPDCCH PUR configuration can be reused, with the following modifications:

- The TDD parameters are not needed.

- The search space will be a common search space (CSS) instead of UE-specific search space (USS).

- There is no consensus in RAN1 on the need to define the set of narrowbands as a set.

- mpdcch-FreqHopping-r16 is not needed.

Q3: For eMTC, RAN2 agrees to have PUSCH configuration for shared resource configuration and decides to reuse the parameters from PUR PUSCH configuration (in IE PUR-PUSCH-Config-r16, as below) as baseline. RAN2 has some questions on some parameters:

- Whether pusch-CyclicShift-r16 and pusch-NB-MaxTBS-r16 are needed

- Whether prb-AllocationInfo should be defined as a “set” format with intention to provide a set of shared frequency-domain resources

Note that:

- CE mode B related parameters will not be included as RAN2 agreed to preclude CE mode B.

- The power related parameters (p0-UE-PUSCH-r16 and alpha-r16) should be updated based on the result of Q1.

RAN2 would like to check with RAN1 on how to define the detail PUSCH configuration.

Question Q3

PUR-Config-r16 ::=		SEQUENCE {

<Skip unrelated Part>

pur-PDSCH-FreqHopping-r16		BOOLEAN,

<Skip unrelated Part>

...,

[[	pur-PDSCH-maxTBS-r17		BOOLEAN						OPTIONAL	-- Need ON

]]

}

Reply to Q3:

From RAN1 perspective:

- pusch-NB-MaxTBS-r16 and pusch-CyclicShift-r16 are not needed to be signaled.

- prb-AllocationInfo should be defined as a “set” format with intention to provide a set of shared frequency-domain resources

- pur-PUSCH-FreqHopping-r16 is not needed

- RAN1 wonders whether RAN2 intends to support multi-PRB allocation or sub-PRB allocation or both

Q4: For eMTC, RAN2 has discussed the PDSCH configuration for shared resource configuration. Some companies think PUR PDSCH configuration is configured according to UE capability, which is not the case for CB-Msg3/Msg4 configuration. So, RAN2 does not know whether the parameters of PUR PDSCH configuration (in pur-PDSCH-FreqHopping and pur-PDSCH-maxTBS within IE PUR-Config-r16) is needed. RAN2 would like to check with RAN1 on whether and how to define the detail PDSCH configuration.

Question Q4:

PUR-PUCCH-Config-r16 ::=			SEQUENCE {

n1PUCCH-AN-r16						INTEGER (0..2047)			OPTIONAL,	-- Need ON

pucch-NumRepetitionCE-Format1-r16	ENUMERATED {n1, n2, n4, n8}	OPTIONAL	-- Need ON

}

Reply to Q4:

From RAN1 perspective:

- pur-PDSCH-FreqHopping and pur-PDSCH-maxTBS are not needed to be signaled.

## Question Q6

Q6: For NB-IoT, RAN2 agrees to reuse the physical layer parameters pur-PhysicalConfig-r16 in PUR-Config-NB as baseline. RAN2 assumes to have below parameters:

- Number of resource units for NPUSCH (as in npusch-NumRUsIndex-r16)

- Number of repetitions for NPUSCH (as in npusch-NumRepetitionsIndex-r16)

- Set of subcarriers (similar to npusch-SubCarrierSetIndex but change it to a “set”), FFS whether subcarriers are provided as a contiguous set.

- MCS configuration for NPUSCH (as in npusch-MCS-r16).

Note that the power parameters (p0-UE-NPUSCH-r16 and alpha-r16) should be updated according to the result of Q1.

RAN2 would like check with RAN1 on how to define the detail /physical layer parameters.

pur-PhysicalConfig-r16				SEQUENCE {

carrierConfig-r16					CarrierConfigDedicated-NB-r13,

npusch-NumRUsIndex-r16			INTEGER (0..7),

npusch-NumRepetitionsIndex-r16		INTEGER (0..7),

npusch-SubCarrierSetIndex-r16		CHOICE {

khz15							INTEGER (0..18),

khz3dot75							INTEGER (0..47)

},

npusch-MCS-r16					CHOICE {

singleTone						INTEGER (0..10),

multiTone						INTEGER (0..13)

},

p0-UE-NPUSCH-r16				INTEGER (-8..7),

alpha-r16						ENUMERATED {al0, al04, al05, al06, al07, al08, al09, al1},

npusch-CyclicShift-r16				ENUMERATED {n0, n6},

npdcch-Config-r16				NPDCCH-ConfigDedicated-NB-r13

}	OPTIONAL,	-- Need ON

Reply to Q6

RAN1 agrees to re-use pur-PhysicalConfig-r16 configuration for CB-msg3-EDT as baseline. RAN1 discussed the following fields in CB-msg3-EDT configuration for NB-IoT NTN:

- The following parameters can be supported:

  - npusch-NumRUsIndex-r16

  - npusch-NumRepetitionsIndex-r16

  - npusch-SubCarrierSetIndex-r16 (but defining this as a set)

  - npusch-MCS-r16

Note: To be confirmed by RAN2 whether to support both singleTone and multitone, or singleTone only for HL parameter npusch-MCS-r16.

## Question Q7

Q7: For NB-IoT, RAN2 agrees to have NPDCCH configuration for shared resource configuration and decides to reuse the parameters from PUR PDCCH configuration (in IE NPDCCH-ConfigDedicated-NB-r13, as below) as baseline. RAN2 would like to check with RAN1 on how to define the detail NPDCCH configuration.

NPDCCH-ConfigDedicated-NB-r13 ::=	SEQUENCE {

npdcch-NumRepetitions-r13			ENUMERATED {r1, r2, r4, r8, r16, r32, r64, r128, r256, r512, r1024, r2048,

spare4, spare3, spare2, spare1},

npdcch-StartSF-USS-r13			ENUMERATED {v1dot5, v2, v4, v8, v16, v32, v48, v64},

npdcch-Offset-USS-r13				ENUMERATED {zero, oneEighth, oneFourth, threeEighth}

}

NPDCCH-ConfigDedicated-NB-v1530 ::=	SEQUENCE {

npdcch-StartSF-USS-v1530			ENUMERATED {v96, v128}

}

Reply to Q7

NPDCCH-ConfigDedicated-NB-r13 IE can be used as baseline for NPDCCH configuration for indication of msg4 on NPDSCH for CB-msg3-EDT. RAN1 assumes this configuration will be provided over broadcast RRC signalling.

# 2	Actions

To RAN2

ACTION:   RAN1 respectfully asks RAN2 to take into account their response to questions Q1, Q2, Q3, Q4, Q6, Q7.

# 3	Dates of next TSG RAN WG1 meetings

TSG-RAN1 Meeting #122	25 - 29 August 2025	Bengaluru, India

TSG-RAN1 Meeting #122bis	13 - 17 October 2025	Prague, CZ