3GPP TSG-RAN WG2 Meeting #131                                                                R2-2505077

Bangalore, India, August 25th – 29th, 2025

Agenda item:	8.20.2

Source:	vivo

Title:	Discussion on SA4 LS regarding RAN simulation Assumptions for ULBC

Document for:	Discussion and Decision

# Introduction

3GPP SA4 has an ongoing feasibility study on Ultra Low Bitrate Speech Codec (FS_ULBC) for GEO voice service. An LS in [1] has been sent to RAN for seeking confirmation on the RAN assumptions. The specific questions regarding RAN2 are shown as below:

# To SA2 and RAN2: ACTION: SA4 kindly asks SA2 and RAN2 to comment on - the different options among UP/CP and IP/Non-IP and the respective overall packet overhead (including RTP/UDP/IP with RoHC, PDCP, RLC and MAC and any potential AS layer optimization if applicable), and if there is any preferred option, and - specifically, whether a packet overhead of 1 byte of MAC header is realistic.

In this contribution, we will provide our analysis on the related questions and give our views.

# Discussion

Q1: Whether a packet overhead of 1 byte of MAC header is realistic

In the TS 36.321, a MAC header consists of one or more MAC PDU sub-headers, and each MAC PDU sub-header corresponds to either a MAC SDU, a MAC CE or padding, as shown in Fig.1. A MAC PDU sub-header consists of the header fields R/F2/E/LCID/(R/R/eLCID)/(F)/(L), with an overhead of 1~4 Bytes. In NB-IoT, eLCID related function is not supported. Thus, the minimum size of MAC PDU sub-header for NB-IoT can be one Byte when L field is absent. However, considering that L field plays an important role in MAC (de)multiplexing function, the L field shall be present in the MAC PDU sub-header except for some special cases (e.g., the last sub-header in the MAC PDU and fixed sized MAC CE), as stated in LTE MAC specification.

Fig.1: Example of MAC PDU in TS 36.321

For GEO voice use case, it is currently unclear whether parallel data transmission of different bearers (DRBs, DRB and SRB) still exists, which should coordinate with SA2. For example, if parallel data transmission (e.g., DRB carrying IMS signalling and DRB carrying voice packet) is allowed, MAC (de)multiplexing function should be used and then multiple MAC sub-headers in the MAC header are needed. In addition, from MAC perspective, there may exist the requirement that MAC SDU and MAC CE (e.g., SPS confirmation MAC Control Element) should be multiplexed. In the context of the aforementioned cases, a1-Byte of MAC header is not realistic. More importantly, whether one Byte of MAC header can be used or not also depends on the down-selected solution (e.g., UP vs CP, IP and non-IP) for supporting GEO voice, which also requires coordination with SA2. Specifically, if IP based solution is selected (e.g., ROHC mechanism is used to reduce protocol overhead), due to the variable ROHC header generated in different ROHC states, the MAC SDU carrying voice packet will have a variable length. In such cases, the L fields shall be present in MAC PDU sub-header to indicate the length of the corresponding MAC SDU. Moreover, due to the talk-silence period transition and codec adjustment, different length of MAC SDU will exist during the ongoing IMS call. At least from these aspects, there are significant limitations to using one Byte of MAC header.

Observation 1: Whether 1 Byte of MAC header can be used for MAC PDU carrying voice packet depends on the down-selected solution for GEO voice support and the need of MAC multiplexing function (e.g., parallel data transmission, talk-silence transition), which should coordinate with SA2.

Q2: The respective overall packet overhead for UP/CP and IP/Non-IP and preferred option

In NB-IOT CP architecture, service data transmission is performed via SRB1bis, with PDCP entity bypassed. RLC AM is the only applicable RLC mode for SRB1bis. AM RLC SN for NB-IoT is 10bits, as specified in TS 36.323.

In NB-IOT UP architecture, service data transmission is performed via DRB mapped on RLC AM or on RLC UM, with PDCP entity used. For NB-IoT, an UM RLC entity uses a 5-bit SN. When the 5-bit SN is used, the length of the fixed part of the UMD PDU header is one Byte (i.e., no RLC concatenation and segmentation). When the 10-bit RLC SN is used, the length of the fixed part of the AMD PDU header is two Bytes. Regarding to the PDCP protocol header, only PDCP data PDU with short PDCP SN (7-bit) is supported for NB-IOT, which has a fixed header size of one Byte.

Observation 2: In legacy NB-IoT, given no RLC concatenation and segmentation, AM protocol header has a fixed size of 2 Bytes, whereas UM RLC protocol header has a fixed size of 1 Byte.

Observation 3: In legacy NB-IoT, PDCP protocol header with a size of 1 Byte (i.e., PDCP SN=7bits) only exists in UP architecture, whereas PDCP entity is bypassed in CP architecture.

For GEO voice service, the overall packet overhead for voice packets will be totally different under different solutions (e.g., UP/CP and IP/Non-IP). Currently, SA2 is working on supporting GEO voice over NB-IOT, and the candidate solutions have been documented in SA2 TR 23.700-19, which can serve as a reference for RAN2’s work.

Below, from RAN perspective, the overall packet overhead for voice packet will be analyzed for each solution. Per the analysis in Q1, there may be significant limitations to using one Byte of MAC header. Thus, it should be noted that MAC PDU sub-header with fixed size of 2 Byte (including L fields) is taken as an example in the following analysis.

- UP IP based solution

For UP IP based solution, the original protocol stack for voice packet is illustrated in Fig.2-1. In the legacy NB-IoT UP architecture, ROHC mechanism between UE and eNB is already supported, except for RTP related ROHC profiles. For GEO voice, NB-IOT ROHC mechanism can be enhanced to support compression and decompression of RTP/UDP/IP protocol header, taking into account low data rate (e.g., 1~3kbps) as specified in SA1 TS 22.261. According to the ROHC specification (e.g., RFC 3095), the RTP/UDP/IP protocol header can be compressed to 3 Bytes in ROHC SO state, as depicted in Fig.1-2.

Fig.2-1 Original protocol header size                                         Fig.2-2 Compressed protocol header size (UE-eNB ROHC)

- UP non-IP based solution

According to the description of solution 3 in SA2 TR 23.700-19, the PDN for voice packet is IP type (e.g., IPv4, IPv6) PDN connection. Then, the original protocol stack for voice packet is the same with UP IP based solution, as illustrated in Fig.3-1. In UP Non-IP based solution, voice packet will use non-IP type transmission. Specifically, the UE sends the voice packet with RTP info (e.g., 1 Byte SN), whereas UDP/IP is removed considering static characteristics. When the PGW receives uplink voice packet with non-IP type transmission, the PGW converts the voice packet with RTP info to voice with full RTP/UDP/IP header. The compressed protocol header size for voice packet is shown in Fig.3-2. It should be noted that the exact length of the reduced RTP header needs further discussion.

Fig.3-1 Original protocol header size                                                          Fig.3-2 Compressed protocol header size

- CP IP based solution

In NB-IOT CP architecture, uplink user data are transmitted in a NAS message, which is transmitted in a UL RRC container. Thus, the original protocol stack for voice packet is illustrated in Fig.4-1. In the legacy NB-IOT, ROHC function between UE and MME is already supported, except for RTP related ROHC profiles. For GEO voice, NB-IOT ROHC mechanism can be enhanced to support compression and decompression of RTP/UDP/IP protocol header. In Rel-19, CT1 has supported NAS header with 7 Bytes for the connected UE. Given this, the compressed protocol header size for voice packet is shown in Fig.3-2. It should be noted that whether further NAS header reduction is up to SA discussion. In TN, UM RLC is usually used for voice packet transmission. In CP NB-IoT, it should further discuss whether UM mode should be introduced for SRB carrying voice packets to reduce RLC protocol overhead. Moreover, RAN2 should also discuss whether further RRC overhead reduction is needed to satisfy the requirement of low data rate (i.e., 1~3kbps) for GEO scenario as specified in SA1 TS 22.261.

Fig.3-1 Original protocol header size                     Fig.3-2 Compressed protocol header size (UE-MME ROHC)

- CP non-IP based solution

According to the description of solution 3 in SA2 TR 23.700-19, the PDN for voice packet is non-IP PDN connection. Then, UDP/IP protocol header is not present. The original protocol stack for voice packet is illustrated in Fig.4-1. Same with UP non-IP based solution, the exact length of reduced RTP header needs further SA discussion. In addition, the same as CP IP based solution, whether further reducing RRC overhead needs RAN2 discussion.

Fig.4-1 Original protocol header size                                                   Fig.4-2 Compressed protocol header size

In summary, the evaluation of overall packet overhead for voice packet needs coordination with SA2, e.g., RPT header reduction. In addition, the evaluation of overall packet overhead for voice packet has a strong relationship with down-selection between CP and UP solutions.

Observation 4: The evaluation of overall packet overhead for voice packet needs coordination with SA2, considering solution down-selection and the potential RTP header reduction.

Based on the analysis of Q1 and Q2 above, the answers to both questions largely depend on the selected solution for supporting voice over NB-IoT via GEO. At this stage, RAN2 is unable to provide a definitive response regarding which option (i.e., CP/UP and IP/Non-IP) is the preferred solution without in-depth discussion. Considering that RAN2 will begin discussions on down select between CP and UP solutions for voice support over NB-IoT-NTN in the upcoming RAN2 meetings, with coordination with SA2, it is suggested that RAN2 postpone the response to SA4’s LS until RAN2 gets progress on down-selection between UP and CP solution.

Proposal 1: RAN2 postpones the response to SA4 LS until progress is made on solutions for GEO voice support.

To advance the discussion on solutions for GEO voice support, it is suggested that an LS is send to SA2 to clarify the details of candidate solutions, especially the non-IP-based solution. Since there is no differentiation between the IP and Non-IP data types in the AS, RAN2 may not fully understand the protocol stack of non-IP data. RAN2 can ask SA2 to confirm whether UDP/IP protocol header is not transmitted via Uu interface at all. Additionally, the question regarding the details of reduced RTP header should also be involved in the LS. These questions include: the exact size of the reduce RTP header, the detailed content in reduced RTP header, and whether any RAN impact exists. Please refer to the content of LS in our companion contribution R2-2506125.

Proposal 2: RAN2 agrees to send an LS to SA2 for clarifying the details of non-IP based solution.

# Conclusion

In this contribution, we have discussed the SA4 LS on RAN simulation assumptions for ULBC. And we make the following observations and proposals:

Observation 1: Whether 1 Byte of MAC header can be used for MAC PDU carrying voice packet depends on the down-selected solution for GEO voice support and the need of MAC multiplexing function (e.g., parallel data transmission, talk-silence transition), which should coordinate with SA2.

Observation 2: In legacy NB-IoT, given no RLC concatenation and segmentation, AM protocol header has a fixed size of 2 Bytes, whereas UM RLC protocol header has a fixed size of 1 Byte.

Observation 3: In legacy NB-IoT, PDCP protocol header with a size of 1 Byte (i.e., PDCP SN=7bits) is only exists in UP architecture, whereas PDCP entity is bypassed in CP architecture.

Observation 4: The evaluation of overall packet overhead for voice packet needs coordination with SA2, considering solution down-selection and the potential RTP header reduction.

Proposal 1: RAN2 postpones the response to SA4 LS until progress is made on solutions for GEO voice support.

Proposal 2: RAN2 agrees to send an LS to SA2 for clarifying the details of non-IP based solution.

# Reference

- R2-2505068, LS on the RAN simulation assumptions for ULBC (SA4)