3GPP TSG-RAN WG2 Meeting #131                                                                R2-2505078

Bangalore, India, August 25th – 29th, 2025

Agenda item:	8.8.2

Source:	vivo

Title:	Remaining Issues on SMTC Enhancements for NTN

Document for:	Discussion and Decision

# Introduction

In RAN2#130 [1], the issue of DL coverage enhancement was discussed and the following agreements were made.

Agreements: - The maximum configured SMTCs per frequency for idle/inactive UEs is 6 (can come back if we find and issue) - To support SMTC enhancements for DL CE UE, introduce a new SMTC list instead of extending legacy smtc4list (can consider signalling optimizations for the new list to refer to the content of smtc4list to avoid signalling duplications) - We introduce a location-based SMTC selection procedure where each SMTC can be associated with a reference location of the intended neighbor cells that need to be measured by the UE. FFS if also an SSB-index based SMTC selection is supported - No enhancement for cell selection/reselection for DL CE is pursued (can re-discuss this if we will finally introduce a new barring scheme) - Working Assumption: We introduce a mechanism to assist the NW to configure the SMTCs in connected mode, according to one of the following 2 options: Alt 1: UE provide the closest N reference locations/neighbor cells to network. Alt 2: UE reports the selected SMTCs from configured SMTC set to network. We continue in the next meeting on whether to confirm the WA (and go for either Alt1 or Alt2) or whether we don’t introduce any enhancement (i.e. not confirm the WA)

In the contribution, we will continue to analyse the above open issues and give our views correspondingly.

# Discussion

For SSB-based SMTC selection:

In RAN2#130, RAN2 agreed to introduce location-based SMTC selection procedure where each SMTC can be associated with a reference location of the intended neighbor cells that need to be measured by the UE. Moreover, RAN2 also confirmed that up to 6 SMTCs can be configured, where each SMTC can cover a surrounding neighbour cell. Based on this, location-based SMTC selection can already cover all scenarios including scattered and clusted deployment.

Figure 1: Location-based SMTC

Besides, introducing SSB-based SMTC selection may lead to additional spec impatcs, e.g., how to configure association of SSB and SMTC, UE behaviour on how to select the SMTC based on SSB, UE behaviour upon there is coexistence of SSB-based SMTC and location-based SMTC. Considerding that R19 is to be frozen and location-based SMTC can also work well, there is no need to further introduce SSB-based SMTC selection.

Observation 1: Location-based SMTC selection can work well for all deployment scenarios, i.e., scattered and clusted scenario.

Observation 2: Supporting SSB-based SMTC selection may lead to additional spec impacts, e.g., how to configure the SSB-based SMTC.

Proposal 1: Not support the SSB-based SMTC selection in R19.

For SMTC enhancement in CONNECTED:

According to the spec [2], UE can report its coarse UE location to NW upon measurement reporting based on measurement configuration or provide its coarse UE location to NW via UEInformationResponse upon NW's request. As descripted in the Figure 1, along with the movement of UE, when UE moves towards the cell#3, at some time the event D1/D2 will be triggered, then UE can report its coarse UE location to the network correspondingly. Moreover, based on NW implementation, NW can also request the UE to report its coarse UE location via the UE information request/response procedure.

### 5.5.5 Measurement reporting #### 5.5.5.1 General ……Skip unrelated part…… 1> if the coarseLocationRequest is set to true in the corresponding reportConfig for this measId: 2> include coarseLocationInfo, if available; ……Skip unrelated part…… #### 5.7.10.3 Reception of the UEInformationRequest message ……Skip unrelated part…… 1> if the coarseLocationRequest is set to true: 2> include coarseLocationInfo, if available; ……Skip unrelated part……

Figure 2: Coarse UE location report

From our perspective, based on coarse location and deployment of the beam hopping, NW can decide how to configure each UE’s SMTC configurations correspondingly. So there is no need for any other enhancement in RRC CONNECTED to address the issue that the SSBs of surrounding cells may be transmitted at different times. Based on this, we think there is no need to confirm the working assumption in RAN2#130.

Observation 3: If the coarse UE location is needed, the legacy mechanism, i.e., UE measurement report or the UE information request/response procedure, can be used to acquire the location.

Observation 4: NW can decide the SMTC configuration for a specific UE based on the reported UE coarse location and knowledge of the beam hopping pattern.

Proposal 2: Do not confim the WA on SMTCs in CONNECTED mode, i.e., NW configures the corresponding SMTCs based on the reported coarse location.

# Conclusion

In this contribution, we have further discussed the DL coverage aspect. And we make the following observations and proposals:

For SSB-based SMTC selection:

Observation 1: Location-based SMTC selection can work well for all deployment scenarios, i.e., scattered and clusted scenario.

Observation 2: Supporting SSB-based SMTC selection may lead to additional spec impacts, e.g., how to configure the SSB-based SMTC.

Proposal 1: Not support the SSB-based SMTC selection in R19.

For SMTC enhancement in CONNECTED:

Observation 3: If the coarse UE location is needed, the legacy mechanism, i.e., UE measurement report or the UE information request/response procedure, can be used to acquire the location.

Observation 4: NW can decide the SMTC configuration for a specific UE based on the reported UE coarse location and knowledge of the beam hopping pattern.

Proposal 2: Do not confim the WA on SMTCs in CONNECTED mode, i.e., NW configures the corresponding SMTCs based on the reported coarse location.

# Reference

- Draft_RAN2_130_Meeting_Report_v1.

- TS38.331, NR; Radio Resource Control (RRC); Protocol specification.