3GPP TSG-RAN WG2 Meeting #131                                                                R2-2505081

Bangalore, India, August 25th – 29th, 2025

Agenda item:	8.9.2

Source:	vivo

Title:	Remaining Issues on S&F Operation

Document for:	Discussion and Decision

# Introduction

In RAN2#130 [1], supporting of the S&F was discussed and the following agreements were made.

Agreements: - When the S&F operation indication is present, the agreed time information in SIB31 (i.e., t-ModeSwitching) indicates when the normal mode will start; otherwise, the agreed time information in SIB31 (i.e., t-ModeSwitching) indicates when the S&F mode will start. - In a S&F network deployment which also exhibits discontinuous coverage, existing mechanisms to handle discontinuous coverage can be leveraged (e.g. satellite assistance information, UE not needing to perform idle mode tasks when the UE determines that is out of coverage, etc.). There is no need to modify existing discontinuous coverage features due to the addition of S&F Satellite operation. FFS if we clarify in discontinuous coverage procedure in idle mode that the UE also takes into account the information about NAS configured S&F monitoring list. - Clarify in Stage 2 from Rel-18 (so in general, not only for S&F) that the values used in “satelliteId-r17” are expected to correspond to the values used in “satelliteId-r18” in other SIBs (e.g. the value used to identify the serving satellite in SIB33, SIB32 and SIB31 should be the same). Come back in the next meeting with an actual Rel-18 CR for this. FFS if in Rel-19 we need to say anything about the correspondence between the “satelliteId” IE and “S&F Monitoring List”. - Working Assumption: In the neighbour cell list we introduce an indication whether the cell operates in S&F mode or not (FFS if we also include the transition time). This WA can only be confirmed if we converge on the corresponding UE behaviour.

According to the RRC open issue list, there is one open issue related with S&F.

Proposal 3: (RRC-4) RAN2 to discuss whether the information of current working mode should be forwarded to upper layers after SI acquisition.

In the contribution, we will continue to analyse the above open issues and give our views correspondingly.

# Discussion

For S&F indication of neighbour cell:

For S&F capable UE, it can both work in cell operating in S&F mode and cell operating in regeneration payload mode. Upon cell reselection, no need to restrict the cell type that the S&F UE can reselect to. While for S&F not capable UE, since the UE does not support the cell operating in S&F mode, it is straight forward to try best to avoid the S&F not capable UE operate in cell operating in S&F mode.

Proposal 1: S&F capable UE can reselect to cell operating in regeneration payload or cell operating in S&F mode.

Corresponding to the UE behaviour of the S&F capable UE, no use case of the S&F indication for S&F capable UE. Additionally, the S&F operation mode is changing along with the satellite movement. If S&F indication is included in the neighbour cell, it may lead to the frequent changing of the SIB carrying the cell reselection information of the neighbour cell. And there is risk that the acquisition of the system information by UE may be not timely.

Proposal 2: No additional S&F indication is needed to be added in the neighbour cell list.

For interaction between AS and NAS on S&F information:

During RAN2 discussion, it is agreed that the S&F transmission time information can be forwarded to upper layers. According to the running CR [2], the AS only indicate the time information upon received within the SIB31 to the upper layer.

5.2.2.39 Actions upon reception of SystemInformationBlockType31 Upon receiving SystemInformationBlockType31 (SystemInformationBlockType31-NB), the UE shall: - start or restart timer T317 with the duration ul-SyncValidityDuration from the subframe indicated by epochTime. - forward the t-ModeSwitching to upper layers, if present.

However, there is transimition time information from S&F mode to regeneration payload mode and from regeneration payload mode to S&F mode. Since AS layer only forward the time information to upper layer without the information on the cell operation mode or the transition direction of the cell operation mode, the upper layer may still cannot properly use the information. Thus, it is proposed that the AS layer also forward the cell operation mode or the transition direction of the cell operation mode information to upper layer.

Proposal 3 (RRC-4): AS layer additionally forward the cell operation mode or the transition direction of the cell operation mode information to upper layer.

The following is an example TP.

5.2.2.39 Actions upon reception of SystemInformationBlockType31 Upon receiving SystemInformationBlockType31 (SystemInformationBlockType31-NB), the UE shall: - start or restart timer T317 with the duration ul-SyncValidityDuration from the subframe indicated by epochTime. - forward the t-ModeSwitching to upper layers, if present. - Indicate the operation mode associated with the t-ModeSwitching to upper layers.

# Conclusion

In this contribution, we have further discussed the S&F operation aspect. And we make the following observations and proposals:

For S&F indication of neighbour cell:

Proposal 1: S&F capable UE can reselect to cell operating in regeneration payload or cell operating in S&F mode.

Proposal 2: No additional S&F indication is needed to be added in the neighbour cell list.

For interaction between AS and NAS on S&F information:

Proposal 3 (RRC-4): AS layer additionally forward the cell operation mode or the transition direction of the cell operation mode information to upper layer.

# Reference

- Draft_RAN2_130_Meeting_Report_v1.

- https://www.3gpp.org/ftp/Email_Discussions/RAN2/%5BRAN2%23130%5D/%5BPost130%5D%5B306%5D%5BR19%20IoT%20NTN%5D%20RRC%20CR%20(Huawei)

- TS36.331, EUTRAN; Radio Resource Control (RRC); Protocol specification.