3GPP TSG RAN WG2#131	R2-2505098

Bengaluru, India, 25th - 29thAugust 2025

Agenda Item:	8.6.4

Source:	Lekha Wireless Solutions

Title:	Discussion on Conditional Intra CU LTM

Document for:	Discussion, Decision

Introduction

At last meeting, we discussed Conditional intra CU LTM and made the following agreements [1]:

- Agreements on Conditional intra CU LTM #RAN2 130: - 1.(MAC-9) If there is at least one beam associated with the configured uplink grant with RSRP above configured RSRP threshold, the UE selects a beam among the beams and CG-based RACH-less CLTM is performed. If the UE finds out multiple beams above threshold, it is up to UE implementation to select which one. If there is no beam associated with the configured uplink grant with RSRP above configured RSRP threshold, RACH-based CLTM is performed. - 2. For L3 based CLTM, the UE performs RACH-based CLTM if there is no configured grant. - 3. Upon CLTM execution, if TA is valid (i.e. TAT timer is running) at the first available CG occasion, and there is CG resource configured, RACH-less CLTM will be performed. - 4. During CLTM is ongoing, after the first transmission, if TAT timer expires while RACH-less LTM is ongoing, fallback to RACH-based CLTM. - 5. In case CG resource is only associated with SSB, if the beam meeting the execution condition is CSI-RS, the UE determines whether a CG resource is valid based on the SSB associated with the CSI-RS. - 6. RAN2 will not do anything for the separate TCI mode optimization.

This contribution discussed different aspect to support conditional intra CU mobility design.

Discussion on Conditional intra CU LTM

  - 2.1 MAC-14: When CLTM candidate TAT timer is running, an (Enhanced) LTM Cell switch Command MAC CE is received

Observation:

UE holds one or more CLTM candidate configurations with stored parameters (candidate beams/SSBs/CSI‑RS, configured grant / RA mapping, stored Timing Advance per candidate) and associated ltm-Candidate-TimeAlignmentTimer (and ltm-Candidate-TimeAlignmentTimerTAG2 if two TAGs).

Network may send:

- LTM Candidate Timing Advance Command MAC CE (2-octet eLCID) to store/apply/refresh TA for a specific candidate and start/restart candidate TAT; and/or

- (Enhanced) LTM Cell Switch Command MAC CE to instruct immediate or scheduled execution, possibly carrying contention-free RA resources, preamble index, SSB/CSI-RS index, Msg1 repetition number, and Timing Advance Command (TAC) or FFF (use measured TA).

- Ambiguity arises if a Cell Switch MAC CE arrives while candidate TAT is running (possibly due to recent TA MAC CE). The UE must decide how to use stored candidate TA, apply MAC CE TA, choose RA procedure, and maintain state consistency.

Key Challenges:

- Which TA to use (stored candidate TA vs TAC in the Cell Switch MAC CE vs measured TA when TAC = FFF)?

- Should candidate TAT be stopped, restarted, or merged?

- How to choose preamble/SSB ((contention-free resources and Msg1 repetition) when MAC CE provides CF-RA resources vs when only candidate config has them?

- Avoiding double RA attempts, partial state updates, or executing with stale beam/TA.

- Option 1 (Baseline)

- Step 1 (TA precedence): If the (Enhanced) LTM Cell Switch Command MAC CE contains a Timing Advance Command (TAC) != FFF, the UE SHALL apply the TAC (overwrite stored candidate TA for that candidate) and start/restart the ltm-Candidate-TimeAlignmentTimer for that candidate (and TAG-specific timer if TI indicates a TAG). The UE SHALL stop the candidate TAT if the MAC CE indicates immediate execution.

- Step 2 (TAC = FFF): If TAC = FFF and a stored candidate TA exists (from LTM Candidate TA MAC CE) and the stored TA is recent (candidate TAT running), the UE SHALL apply the stored candidate TA and start/restart the timeAlignmentTimer. If stored TA not present, and TAC = FFF, the UE SHALL apply the measured TA if available per clause 5.18.35.

- Step 3 (RA resources): If MAC CE explicitly provides contention‑free RA resources (preamble & SSB/CSI‑RS) → UE SHALL set RA_TYPE = 4-stepRA, set PREAMBLE_INDEX as signalled, apply Msg1 repetition number if non‑zero, and proceed with contention‑free RA. Msg3 repetition not applicable.

- Step 4 (no CF-RA in MAC CE): If MAC CE does not include CF‑RA, but UE has CF‑RA associated with the candidate in rach-ConfigDedicated (and SSB/CSI‑RS quality > threshold) → use candidate mapping (select SSB/CSI‑RS and preamble as per rach-ConfigDedicated).

- Step 5 (atomic state change): When executing MAC CE (immediate), the UE SHALL atomically clear candidate execution flags/timers after storing/applying any TA per step 1/2, to avoid double attempts.

- Rationale: The mentioned steps gives explicit precedence to an explicit TAC in MAC CE (network intent), but preserves utility of stored candidate TA when MAC CE only signals FFF. It also keeps RA mapping deterministic.

- Option 2 (Enhanced robustness)

- Prior to using the MAC CE signalled SSB/CSI‑RS, the UE SHALL check the measured SS‑RSRP/CSI‑RS power for the signalled beam. If it is below rsrp-ThresholdSSB/rsrp-ThresholdCSI-RS, the UE SHALL:

- If candidate mapping has a valid beam/preamble above threshold, use candidate beam with MAC CE RA resources (merge); or

- If no valid candidate mapping, send immediate UL status report (MAC CE status) indicating inability to apply MAC CE and preserve candidate TAT.

- Rationale: Avoids switching using stale/weak beam indicated by network.

Proposal 1 – Baseline
	UE shall give precedence to TAC ≠ FFF in MAC CE, applying it and restarting TAT. If TAC = FFF, UE shall use stored candidate TA if valid, else measured TA. For RA, UE shall use CF-RA from MAC CE or fallback to rach-ConfigDedicated mapping; candidate execution flags/timers shall be cleared atomically after TA handling.

Proposal 2 – Enhanced Robustness
	Before using MAC CE-signalled beam, UE checks SS-RSRP/CSI-RS level; if below threshold, use a valid candidate beam/preamble if available, otherwise send immediate UL status to indicate inability and keep candidate TAT.

- MAC Procedure for processing (Enhanced) LTM Cell Switch Command MAC CE when CLTM candidate TAT is running:

- The MAC entity shall, for each CLTM candidate cell:

- 1> When the CLTM candidate ltm-Candidate-TimeAlignmentTimer (or ltm-Candidate-TimeAlignmentTimerTAG2, if applicable) is running, and an (Enhanced) LTM Cell Switch Command MAC CE as described in clause 6.1.3.4y is received:

- 2> If contention-free Random Access (RA) resources are explicitly provided in the (Enhanced) LTM Cell Switch Command MAC CE:

- 3> If the SS-RSRP of the SSB indicated in the MAC CE is above rsrp-ThresholdSSB:

- 4> Set the PREAMBLE_INDEX to the RA Preamble Index signalled in the MAC CE;

- 4> Select the SSB signalled in the MAC CE;

- 4> If a non-zero Msg1 repetition number is indicated, apply the repetition for the current RA procedure.

- 3> Else:

- 4> Discard the indicated SSB for RA-less access and fallback to alternative beam/resource selection as per clause 5.2.1x.

- 2> Else if contention-free RA resources are not explicitly provided:

- 3> If contention-free RA resources are available via rach-ConfigDedicated associated with SSBs or CSI-RSs and an eligible beam/reference signal meets the configured RSRP threshold:

- 4> Select the beam/reference signal and set the PREAMBLE_INDEX accordingly.

- 1> If the Timing Advance Command field in the (Enhanced) LTM Cell Switch Command MAC CE is not set to 'FFF':

- 2> Apply the Timing Advance Command for the PTAG as specified in clause 6.1.3.75;

- 2> Start or restart the timeAlignmentTimer for the PTAG.

- 1> Else if the Timing Advance Command field is set to 'FFF' and the UE has a valid measured TA value as per clause 5.18.35:

- 2> Apply the measured TA for the PTAG;

- 2> Start or restart the timeAlignmentTimer for the PTA

  - 2.2 MAC-26: How to consider the TAG ID when determining whether to set the PREAMBLE_POWER_RAMPING_COUNTER to 1 for an (C)LTM candidate cell which is configured with two TAGs in RACH-less (C)LTM

Observation:

In RACH-less Conditional (C)LTM, when evaluating a candidate cell, the UE determines whether to set the PREAMBLE_POWER_RAMPING_COUNTER to 1. For cells configured with multiple Timing Advance Groups (TAGs), current specifications (Rel-18 and Rel-19) do not consider the TAG ID in this evaluation. This means that if a candidate cell is configured with two TAGs, the reset condition is evaluated without distinguishing between TAG IDs. The rapporteur has identified that this issue exists for both Rel-19 RACH-less (C)LTM and Rel-18 LTM [2].

Problem:

Without TAG ID consideration, power ramping may be unnecessarily restarted or skipped, leading to:

- Incorrect initial power setting for the target TAG in the candidate cell.

- Potential access failures or excessive access delays in scenarios with multiple TAGs per cell.

- Inconsistent behavior between UEs and between RACH-less and RACH-based mobility procedures

- Proposal 3: When determining whether to set the PREAMBLE_POWER_RAMPING_COUNTER to 1 for a (C)LTM candidate cell configured with multiple TAGs, the UE shall perform the evaluation separately for each TAG ID associated with that candidate cell.

- Proposal 4: Upon RA trigger from MAC CE, select TAG from TAG_ID if present, else from PTAG. If TAG changes from last RA attempt, set counter to 1; else maintain as per existing rules. Legacy single-counter UEs treat it as TAG-scoped and initialise to 1 on TAG change.

- Proposal 5: The last RA attempt TAG is the TAG used when UE previously initiated an RA procedure (successful or unsuccessful) towards this candidate cell.

- MAC Procedure: TAG-aware initialisation of PREAMBLE_POWER_RAMPING_COUNTER for (C)LTM candidate cells configured with multiple TAGs

- For each (C)LTM candidate cell that is configured with multiple TAGs:

- 1> When a MAC CE (for example, an (Enhanced) LTM Cell Switch Command MAC CE or an LTM Candidate Timing Advance Command MAC CE) triggers a Random Access (RA) procedure or indicates contention-free RA resources for that candidate cell:

- 2> Determine the TAG to be used for the RA procedure as follows:

- 3> If the MAC CE explicitly indicates a TAG (TAG_ID field present), use the indicated TAG; otherwise use the PTAG associated with the candidate cell per clause X.Y.Z.

- 2> Maintain a separate PREAMBLE_POWER_RAMPING_COUNTER per TAG (logical or physical).

- 2> If the determined TAG is different from the TAG that was used for the last RA attempt associated with this candidate cell:

- 3> Set the PREAMBLE_POWER_RAMPING_COUNTER for the determined TAG to 1.

- 2> Else:

- 3> Maintain the PREAMBLE_POWER_RAMPING_COUNTER value for the determined TAG according to existing rules.

- 2> If the UE implementation only maintains a single PREAMBLE_POWER_RAMPING_COUNTER value (legacy behaviour):

- 3> Treat that single counter as logically TAG-scoped. On TAG change, initialise the counter to 1 for the new TAG and preserve the counter for the old TAG in UE internal storage if possible.

  - 2.3 MAC-28: During CLTM is ongoing, after the first transmission, if TAT timer expires while RACH-less LTM is ongoing, if UE based TA is valid, whether/how to fallback.

Observation:

In general, Conditional LTM (CLTM) allows a UE to prepare for a cell switch based on predefined conditions without the full RRC signaling delay at the time of execution. When the target cell supports RACH-less LTM, the UE can skip the random access procedure and use an existing Timing Advance (TA) value—either received from the network or estimated locally. The TAT (Timing Advance Timer), defined in 3GPP TS 38.321 and 38.213, controls the validity of TA. Once TAT expires, the UE is expected to treat the TA as invalid, which typically forces RA to re-synchronize uplink timing. The rapporteur has mentioned that this issue for Rel-19 RACH-less (C)LTM [3].

Issue-During a RACH-less CLTM execution:

After the first transmission in the target cell, the TAT timer may expire (e.g., due to long UE inactivity or timer misalignment).

If TAT expires while the CLTM procedure is still ongoing, there is ambiguity:

- Standard behavior would consider TA invalid and trigger RA.

- But in RACH-less CLTM, RA was intentionally skipped to reduce latency.

If the UE has locally derived TA validity (e.g., from downlink time tracking), the specs are unclear whether the UE:

- Should fallback to RA immediately upon TAT expiry, or

- Can continue transmissions using UE-derived TA.

This uncertainty can cause:

- Uplink decoding failures if the TA is wrong but RA was skipped.

- Unnecessary RA fallback if the TA was still good, defeating the low-latency purpose of RACH-less CLTM.

- Potential Solutions

- Option 1 – Immediate RA Fallback on TAT Expiry

- On TAT expiry during CLTM, trigger RA (contention-free if possible).

- Guarantees sync but may add latency and signaling.

- Option 2 – UE-Based TA Validation Before Fallback

- Allow UE to check TA validity (e.g., via DL timing tracking loop).

- If TA is still valid within tolerance (per 38.133), continue RACH-less operation.

- Only fallback to RA if validation fails.

- Requires minor spec clarification that UE can maintain TA validity check independent of TAT.

- Option 3 – Extend TAT for RACH-less CLTM

- When CLTM is ongoing and RACH-less execution is configured, network can configure TAT offset extension (via RRC).

- Reduces chance of expiry during ongoing procedure.

- Proposal 6: Upon TAT expiry during RACH-less CLTM execution, the UE shall initiate a random access procedure (preferably contention-free) to re-establish uplink timing, ensuring sync reliability.

- Proposal 7: Upon TAT expiry during RACH-less CLTM, the UE may continue transmissions if local TA validation (e.g., via downlink timing tracking) confirms TA remains within allowed tolerance; otherwise, trigger RA.

- Proposal 8: When RACH-less CLTM is configured, the network may configure an extended TAT value or offset to avoid expiry during the procedure, minimizing unnecessary RA fallback.

  - 2.4 FFS for introducing separate conditional events (CondEventLTM3/4/5) for CLTM evaluation to avoid simultaneous triggering with L1 MR

Observation:

As per TS 38.321 and the draft additions for CLTM measurement, the network may configure an RRC_CONNECTED UE to perform L1 beam-level measurements for LTM candidate cell(s) and to report results via event-triggered L1 measurement configuration.

The measurement configuration (LTM-CSI-ReportConfig + LTM-CSI-ResourceConfig) can specify:

- Measurement type: SS/PBCH block(s) or CSI-RS resources

- Trigger quantity: L1-RSRP

- Event type: eventLTM2, eventLTM3, eventLTM4, eventLTM5

- Time-to-trigger (TTT), reportOnLeave, periodic reporting, etc.

The UE’s MAC entity evaluates the triggering conditions for these events based on lower layer L1-RSRP measurements. Once TTT is satisfied, the UE maintains variables such as:

- MR_LIST – active reporting entries for a given ltm-CSI-ReportConfigId

- BEAM_TRIGGERED_LIST – beams that triggered the event for that config

- MR_SENT_COUNTER – number of reports sent

If an event is met for an applicable beam (serving or candidate cell depending on event type), the UE triggers a measurement report per 5.x.3 [2] rules.

In the current 3GPP specifications (Rel-18/Rel-19 drafts), Conditional LTM (CLTM) evaluation at the UE side can be based on L1 measurement events (e.g., EventLTM3/4/5). These events are similar in principle to L1-based measurement reporting triggers.

When a configured condition is satisfied, both:

- Layer 1 Measurement Report (L1 MR)

- CLTM execution

can be triggered at the same instant, because both are using the same underlying L1 measurement event as a reference.

This creates two main issues:

- Simultaneous triggering:

- If the configured L1 event threshold is met, the UE may send an L1 MR and simultaneously execute a CLTM.

- This results in redundant signalling — the MR does not provide additional benefit when CLTM has already been executed.

- Failure handling ambiguity:

- If CLTM execution fails (e.g., due to SN gap issues or fast recovery procedures), there is no independent trigger to quickly send updated measurements to the network.

- Because MR triggering is coupled to the same event as CLTM, the UE might not send an MR immediately after failure if the event condition is still “already satisfied” and TTT not restarted.

Configuration inflexibility: Operators cannot set different thresholds, hysteresis, or filtering for CLTM and MR separately, even though they serve different purposes — CLTM for mobility action, MR for network situational awareness.

Proposal 9: Introduce separate conditional events dedicated to CLTM evaluation: CondEventLTM3, CondEventLTM4, CondEventLTM5.

Proposal 10: Use the same event definition but introduce a rule in 38.321: If an event triggers both L1 MR and CLTM execution at the same time, CLTM evaluation takes precedence and MR triggering is deferred until after CLTM outcome.

  - 2.5 Coexistence between CLTM and (e)RedCap, CovEnh?

Observation: ·

- (e)RedCap: Reduced Capability UEs (including enhanced RedCap) with simplified processing and power constraints, often targeting use cases like industrial sensors, wearables, etc [2].

- CovEnh: Coverage Enhancement procedures, typically using repetitions and relaxed performance metrics to ensure robust communication at the cell edge or for deep indoor UEs.

- C-LTM (Conditional L1/L2 Triggered Mobility): Enables fast mobility based on pre-configured conditions (e.g., measurements) without requiring RACH in certain scenarios.

·  RAN2 previously agreed that inter-CU LTM in Rel-19 can coexist with both CovEnh and (e)RedCap. The concern arises because CLTM involves tight timing, beam selection, and pre-configured CG — which might be affected by RedCap/CovEnh characteristics.

2.5.1 Coexistence Between Rel-19 CLTM and (e)RedCap

Rationale:

- RedCap supports LTM in Rel-17/18 with constraints (e.g., reduced measurements, simplified beam handling).

- CLTM reuses core LTM procedures, with added optimization from conditional triggering.

- Conditional execution based on simplified triggers can benefit RedCap, as it avoids the need for full RACH.

Why Feasible:

- (e)RedCap UEs can still report L1/L3 events.

- Beam selection strategies (e.g., simplified or pre-indicated beams) can be tailored to RedCap constraints.

Proposal 11: CLTM can be applied to (e)RedCap UEs with minimal tailoring, e.g., using fewer measurement events, relaxed filtering, and pre-indicated beams for CG.

2.5.2 Coexistence Between Rel-19 CLTM and CovEnh

Rationale:

- CovEnh procedures already support LTM through repeated SS/PBCH and PDCCH decoding.

- Conditional mobility can reduce failed handovers in coverage-challenged areas.

Why Feasible:

- CLTM could benefit from CovEnh's robustness, e.g., using repeated SS-RSRP measurements for triggering.

- Conditional triggering allows gNB to predefine handover attempts only when quality is acceptable, reducing RACH overload.

Minor Considerations:

- CG resource availability for CovEnh UEs must match their repetition capabilities.

- CG scheduling must respect CovEnh timing (e.g., extended TTIs or repetitions).Coexistence Between Conditional LTM and Rel-19 Inter-CU LTM

Proposal 12:The LTM procedure's flexibility allows adaptation to RedCap’s reduced complexity and CovEnh's repetition-oriented structure. Conditional triggering reduces control overhead, aligns with power/performance limitations, and offers robust handover in weak coverage scenarios.

Proposal 13: Supporting coexistence between Rel-19 Conditional LTM and both (e)RedCap and CovEnh.

- 3 Conclusion

In this contribution, the following proposals are made:

  - MAC-14: When CLTM candidate TAT timer is running, an (Enhanced) LTM Cell switch Command MAC CE is received:

Proposal 1 – Baseline
	UE shall give precedence to TAC ≠ FFF in MAC CE, applying it and restarting TAT. If TAC = FFF, UE shall use stored candidate TA if valid, else measured TA. For RA, UE shall use CF-RA from MAC CE or fallback to rach-ConfigDedicated mapping; candidate execution flags/timers shall be cleared atomically after TA handling.

Proposal 2 – Enhanced Robustness
	Before using MAC CE-signalled beam, UE checks SS-RSRP/CSI-RS level; if below threshold, use a valid candidate beam/preamble if available, otherwise send immediate UL status to indicate inability and keep candidate TAT.

  - MAC-26: How to consider the TAG ID when determining whether to set the PREAMBLE_POWER_RAMPING_COUNTER to 1 for an (C)LTM candidate cell which is configured with two TAGs in RACH-less (C)LTM:

- Proposal 3: When determining whether to set the PREAMBLE_POWER_RAMPING_COUNTER to 1 for a (C)LTM candidate cell configured with multiple TAGs, the UE shall perform the evaluation separately for each TAG ID associated with that candidate cell.

- Proposal 4: Upon RA trigger from MAC CE, select TAG from TAG_ID if present, else from PTAG. If TAG changes from last RA attempt, set counter to 1; else maintain as per existing rules. Legacy single-counter UEs treat it as TAG-scoped and initialise to 1 on TAG change.

- Proposal 5: The last RA attempt TAG is the TAG used when UE previously initiated an RA procedure (successful or unsuccessful) towards this candidate cell.

  - MAC-28: During CLTM is ongoing, after the first transmission, if TAT timer expires while RACH-less LTM is ongoing, if UE based TA is valid, whether/how to fallback:

- Proposal 6: Upon TAT expiry during RACH-less CLTM execution, the UE shall initiate a random access procedure (preferably contention-free) to re-establish uplink timing, ensuring sync reliability.

- Proposal 7: Upon TAT expiry during RACH-less CLTM, the UE may continue transmissions if local TA validation (e.g., via downlink timing tracking) confirms TA remains within allowed tolerance; otherwise, trigger RA.

- Proposal 8: When RACH-less CLTM is configured, the network may configure an extended TAT value or offset to avoid expiry during the procedure, minimizing unnecessary RA fallback.

  - Separate conditional events (CondEventLTM3/4/5) for CLTM evaluation to avoid simultaneous triggering with L1 MR:

Proposal 9: Introduce separate conditional events dedicated to CLTM evaluation: CondEventLTM3, CondEventLTM4, CondEventLTM5.

- Proposal 10: Use the same event definition but introduce a rule in 38.321: If an event triggers both L1 MR and CLTM execution at the same time, CLTM evaluation takes precedence and MR triggering is deferred until after CLTM outcome.

  - Coexistence between CLTM and (e)RedCap, CovEnh:

Proposal 11: CLTM can be applied to (e)RedCap UEs with minimal tailoring, e.g., using fewer measurement events, relaxed filtering, and pre-indicated beams for CG.

Proposal 12:The LTM procedure's flexibility allows adaptation to RedCap’s reduced complexity and CovEnh's repetition-oriented structure. Conditional triggering reduces control overhead, aligns with power/performance limitations, and offers robust handover in weak coverage scenarios.

Proposal 13: Supporting coexistence between Rel-19 Conditional LTM and both (e)RedCap and CovEnh.

- 4 References

- R2-2504671 “Report from session on R18 SL, R18/19 MOB and R19 NES”, Vice Chairman (Samsung)

- TS 38.321, NR; Medium Access Control (MAC) protocol specification, Release 18 V 18.6.0

- R2-2505398 “Discussion summary and list of MAC open issue for Mob Ph4”, agenda 8.6.1, RAN#131,2025