3GPP TSG-RAN WG2 Meeting #131	R2-2505115

Bangalore, India  Aug 25th – 29th , 2025

Source:	NTT DOCOMO, INC.

Title:	Discussions on data collection for UE-sided model

Agenda Item:	8.3.2.3

Document for:	Discussion and Decisions

- Introduction

During the RAN#108 meeting, the new WID on “Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR”[1] was approved. The WID describes the objective of the work on UE data collection as below,

- For both RRM measurement prediction and measurement event prediction, specify signalling and protocol aspects to enable LCM functionality management for UE sided model including [RAN2]:

- UE capability request and response procedure

- Applicability report for both full and partial configuration approach (considering further progress for AI/ML PHY)

- Inference configuration

- Performance monitoring performed in UE or network side, based on which network can manage UE sided functionality

- UE sided data collection, for which AI/ML PHY request/configuration framework is taken as baseline

Note 3: Data transfer and model delivery for UE sided model are out of scope of this WID. And model related choices can be up to UE’s implementation.

This paper discusses the training data collection for UE-sided model.

- Data collection configuration

At the RAN2 #129bis and #130 meeting, the baseline framework has been agreed for UE-side data collection [2][3],

Agreement at #129bis meeting

- Study UE side data collection configuration, taking AI ML PHY relevant procedure as baseline.   Postpone UE side data collection transfer discussion until further progress is made in AI ML PHY in R20.

Agreement at #130 meeting

- For UE sided data collection – use AI/ML PHY request/configuration framework as baseline.  FFS enhancements/or differences

Figure 2: UE data collection procedure for AI4PHY

Figure 1 shows the general procedure for UE-side data collection specified in the AI4PHY session. The main principle could be reused for AI4Mob RRM measurement prediction and event prediction, while some detailed configurations need adaptation and/or reconsideration. Below is the mapping:

AI4PHY agreements that can be reused for AI4Mob:

The candidate data collection configuration is used to check the preference and/or willingness of the UE on specific requirement on the collected data. This is also useful for RRM and event prediction with UE-sided model since UE may be interested in certain radio features of dataset, e.g., cell edge. Therefore, it is meaningful to keep this candidate configuration for UE to choose.

Since the measurement for UE data collection occupies the downlink transmission resources, NW should make the final decision on whether certain resources can be scheduled by considering all the UE status in the cell, such as radio condition and resource utilization. Moreover, the request of data collection could be sent by UE server, which makes the NW-initiated UE data collection a straightforward way from the air interface perspective. To ensure the consistency between training and inference, the associated ID is also beneficial to be included in the data collection configuration for AI4Mob.

Proposal 1: Following procedure from AI4PHY is supported for UE data collection for AI4Mob:

- The UE may be configured with candidate data collection configurations which UE doesn’t need to measure.

- The UE responds the preferred/unpreferred configurations from the candidate data collection configurations if configured.

- The network can decide when to start/stop the data collection and send the configuration with or without UE preference. The associated ID is optionally included.

- The (candidate) data collection configuration and corresponding response should be under RRM framework and details are designed in the WI phase.

Items that require adaptation and/or reconsideration for AI4Mob:

For AI4PHY, UE is allowed to initiate request for data collection under NW configuration. However, the collected data will finally be transferred to UE server for model training. The entity initiating the data collection is different from the entity collecting the data, which makes the procedure complicated and ambiguous. Therefore, to make the specification simpler and clearer, it is better to restrict the data collection request only from one single entity, i.e., UE server.

Proposal 2: RAN2 to discuss whether UE is allowed to initiate request for data collection (e.g., start/stop indication) for AI4Mob.

- Conclusion

In this contribution, we have the following proposals,

Proposal 1: Following procedure from AI4PHY is supported for UE data collection for AI4Mob:

- The UE may be configured with candidate data collection configurations which UE doesn’t need to measure.

- The UE responds with the preferred/unpreferred configurations from the candidate data collection configurations if configured.

- The network can decide when to start/stop the data collection and send the configuration with or without UE preference. The associated ID is optionally included.

- The (candidate) data collection configuration and corresponding response should be under RRM framework and details are designed in the WI phase.

Proposal 2: RAN2 to discuss whether UE is allowed to initiate request for data collection (e.g., start/stop indication) for AI4Mob.

References

- Moderator (CATT), RP-251864, “New WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR,” Jun. 2025.

- RAN2 Chair (InterDigital), “RAN2 #129bis Chair Notes”, Apr, 2025.

- RAN2 Chair (InterDigital), “RAN2 #130 Chair Notes”, May. 2025.