3GPP TSG-RAN WG2 Meeting #131	R2-2505121

Bangalore, India  Aug 25th – 29th , 2025

Agenda Item:	8.2.2

Source: 	Xiaomi

Title:  	Remaining open issues on A-IOT paging procedure

Document for:	Discussion and Decision

# 1	Introduction

In the email discussion [POST130][027][AIoT] MAC Running CR (Huawei), A-IoT MAC CR Rapporteur listed open issues for Paging, Random access and data transmission.

In this contribution, we discuss the following paging related open issues:

Issue 1: Mismatch between indicated total AOs and the real number of AOs (raised in CR review)

Issue 2: Issue 1-5: Paging content for CFRA (Q#4)

# 2	Discussion

## 2.1 Mismatch between indicated total AOs and the real number of AOs

The issue was raised during CR review as

Xiaomi: Current total number of Access Ocassions is defined in power of 2 which cannot always match the real total number of access Ocassions as it is counted based number of access trigger message * X* Y and the number of Y could be 3, 5 6, or 7. Therefore some of access occasions of the last trigger message will never be used. This also contradicts agreement “RAN2 assumes that device randomly selects among FDMA occasions as the baseline” The procedure should enable use of all AOs. QC: By using exponential way, 4 bits, the total number of AOs is 1 2 4 8, … However, Frequency Resource Indication Y could be 1 to 8, especially 3 5 6 7. Since RAN2 has agreed device will randomly select the access occasion, it seems some values of total of AOs can not be expressed. Rapporteur: I do agree the analysis the number of AO may not exactly match the AOs provided by reader in the end, but this is the consequence of agreeing 4bits. And it looks acceptable that less than 8 AOes is wasted. My understanding is that the potential waste is the AOs triggered by the last trigger message, which is less than 8 AOs. It looks acceptable if we want to save the bits of AO number indication, which is the main reason why companies prefer 4bits than 14bits (liner way) during last meeting discussion.

In our understanding, more than 8 AOs may be wasted. For instance, assuming NSFS=7 and X=2, and the number of signalled AOs 32, then 10 AOs are wasted. If the number of signalled AOs is 16, 12 AOs are wasted. Using X=1 would reduce the waste but increase the number of access triggers from 2 to 4, and 1 to 2 respectively. Adjusting NSFS=7 could also reduce the waste. But both parameters should rather be adjusted based e.g. on channel conditions/reader capabilities, not to solve issues introduced by RAN2 signalling limitations. We also think that a small number of AOs would be quite a common case. In general, as far as possible, the number of AOs should match the number of expected devices participating in the CBRA procedure, which could lead to the optimal slotted ALOHA operating point (i.e., a success rate of 0.37). The initial round may already expect a small number of devices (based on received assistance information), and if not the case, the subsequent paging rounds would anyway expect smaller and smaller number of devices. The figure below represents the impact on the number of collected AOs in the second example.

Figure 1 - Impact of #AOs mismatch on #Collected RN16s

To address the issue, the simple way is to revert the agreements, i.e., the number of access occasions is introduced as linear value instead of exponential way.

Agreements from RAN2 130

- Issue (1-4) For number of access occasions introduce exponential way, 4 bits, value range FFS

Proposal 1a: The number of access occasions is introduced as linear value instead of exponential way.

If companies do not want to revert the agreements, another simple way to address this issue would be to update the AO selection procedure. For instance, the device could adjust the number of AOs to the smallest larger or equal multiple of X*NSFS before performing the AO selection, i.e., adjust the number of AOs to add the wasted resources back, e.g., for the examples shown above, the device:

- should add 10 wasted resources back if number of signalled AOs is 32, i.e., the number of AOs is changed to 42;

- should add 12 wasted resources back if number of signalled AOs is 16, i.e., the number of AOs is changed to 28.

Proposal 1b: If proposal 1a is not acceptable, the device should adjust the number of AOs to the smallest larger or equal multiple of X*NSFS before performing the AO selection

## 2.2 Issue 1-5: Paging content for CFRA (Q#4)

| Issue 1-5: Paging content for CFRA | As baseline, the transaction ID is absent in Paging message for CFRA. FFS on the need for the transaction ID for command case. - For CFRA, as a baseline the fields related to the transaction ID, indication of paging ID present/absent and number of access occasions are absent. FFS on the need for the transaction ID for command case. - Status in running CR: the CR is implemented assuming no transaction ID for CFRA, and no issue is identified. | Companies are invited to input views for Q#4 |
| --- | --- | --- |

The issue has been discussed in the email discussion. Most companies agreed that transaction ID is not needed for CFRA. To our understanding, the issue raised in the meeting was how to release AS ID for CFRA since the ID is configured in the first command Msg. Can AS ID release solution, e.g., paging with transaction ID be used for the release of AS ID for CFRA case? We do not see the problem considering there is not interleaving between CFRA and CBRA procedure. Therefore the device shall release AS ID assigned during CFRA upon receiving Paging, no matter whether transaction ID is contained or not.

But there is no additional specification impact since it is same as current MAC CR as below:

1> if the Access Type field in the A-IoT Paging message indicates CBRA: 2> if the device has no stored Transaction ID; or 2> if the value of the Transaction ID field is different from the stored Transaction ID; or 2> if the value of the Transaction ID field is the same as the stored Transaction ID, and the previous procedure was determined as failed for this Transaction ID as specified in clause 5.5: 3> release the stored AS ID if any; 1> else (i.e., the Access Type field in the A-IoT Paging message indicates CFA): 2> release the stored AS ID if any;

Proposal 2: The device shall release AS ID assigned during CFRA upon receiving Paging, no matter whether transaction ID is contained or not. No specification impacts.

# 3	Conclusion

In this contribution, we discussed the paging related open issues and have following proposals.

Proposal 1a: The number of access occasions is introduced as linear value instead of exponential way.

Proposal 1b: If proposal 1a is not acceptable, the device should adjust the number of AOs to the smallest larger or equal multiple of X*NSFS before performing the AO selection

Proposal 2: The device shall release AS ID assigned during CFRA upon receiving Paging, no matter whether transaction ID is contained or not. No specification impacts.

# Reference

[1] R2-2505521 Email discussion report: [POST130][027][AIoT] MAC Running CR (Huawei)