3GPP TSG-RAN WG2 Meeting #131	R2-2505139

# Bangalore, India, 25 – 29 Augugst 2025

Agenda item:		8.7.5

Source:	Xiaomi

Title:	RLC AM retransmission enhancements

Document for:		Discussion and Decision

# Introduction

In RAN2#130 meeting, following was agreed regarding RLC retransmission enhancements [1]:

Agreements on RLC enhancements - (RLC-8) We just keep the current specifications for polling triggering, i.e. no need to specify that polling shall only be triggered once per RLC SDU when its remaining time falls below a specified threshold, unless an issue is identified with this - (RLC-9) No additional conditions are needed for the polling enhancement. - RAN2 agrees that autonomous retransmissions should trigger an increment of the RETX_COUNT. - (PDCP-2) The UE shall trigger remaining-time-based RLC retransmission and polling in PDCP SDU level, no matter pdu-SetDiscard is configured or not. No specification change is needed. - Autonomous retransmission and polling is triggered only based on discardTimer, i.e. not based on discardTimerForLowImprotance - The remaining time thresholds (for both autonomous retransmission and polling) are configured per PDCP entity. - (RLC-6) There is no specification impact foreseen for RLF triggering due to RLC maximum retransmission. - Working assumption: (RLC-11) No need to address window stalling issue with polling retransmission (TBC next meeting) - Assumption from companies is that this should not happen due to SR triggering from Rx side, but it needs to be checked whether this will always work - Define an (optional) per-UE capability with signalling for the Rx-side aspect, where an outdated SDU is abandoned based on a new RLC timer and the abandoned SDUs are positively acknowledged in an RLC status report. - Define an (optional) per-UE capability with signalling for the Tx-side aspect, where the Tx side stops transmissions for an outdated SDU based on an indication from the PDCP. FFS A UE supporting this feature shall also indicate the support of Rx-side aspect.

In this contribution, we discus open issues of RLC AM retransmission enhancements.

# Discussion

## Avoid unnecessary RLC retransmissions

Open issue RLC-10

In “[POST130][508][XR] RLC running CR and open issues (vivo)” [3], open issue RLC-10 is as follows: Whether/How to introduce any limitations or associations for the configuration of stopReTxObsoleteSDU and DL t-RxDiscard. The intention is to avoid Tx window stall, and the related proposal during email discussion is that if DL stopReTxDiscardedSDU is supported by gNB, DL t-RxDiscard should be configured; and if UL stopReTxDiscardedSDU is configured, UL timer based Rx discard should be activated in NW.

This issue is mainly up to network implementation. Taking DL as an example. Only if UE indicates it supports the Rx side operation and gNB supports the Tx side operation, gNB may configure UE with t-RxDiscard for some RLC bearers. This is similar to many other operations which require collaboration between UE and gNB. There is no need to specify such network implementation details.

Proposal 1: (RLC-10) It is up to network implementation on whether to configure stopReTxDiscardedSDU / t-RxDiscard and related network behavior.

Open issue UE capability-04

A related issue is the UE capability as in “[POST130][509][XR] UE capabilities CRs (Xiaomi)” [2]. Open issue UE capability-04 is: Whether a UE supporting txRLC-StopReTxDiscardedSDU-r19 shall also indicate support of rxRLC-Discard-r19. As analyzed above, UE Rx side operation (capability rxRLC-Discard-r19) is linked to gNB Tx side operation, and UE Tx side operation (capability txRLC-StopReTxDiscardedSDU-r19) is linked to gNB Rx side operation. Configuration wise, UE Rx side operation (t-RxDiscard) and Tx side operation (stopReTxDiscardedSDU) can be configured independently. Capability wise, UE Rx side capability (rxRLC-Discard-r19) and Tx side capability (txRLC-StopReTxDiscardedSDU-r19) can be independent, i.e. we can conclude that the feature txRLC-StopReTxDiscardedSDU-r19 does not have any pre-requisite (including rxRLC-Discard-r19).

Proposal 2: (UE capability-04) The feature txRLC-StopReTxDiscardedSDU-r19 does not have any pre-requisite (including rxRLC-Discard-r19).

Open issue RLC-11

In “[POST130][508][XR] RLC running CR and open issues (vivo)” [3], open issue RLC-11 is as follows: FFS how to handle the case “if there are only SDUs buffered whose transmissions have been stopped due to discard indication from PDCP, there is no SDU to retransmit the poll with”.

In RAN2#130 meeting, following was agreed:

- Working assumption: (RLC-11) No need to address window stalling issue with polling retransmission (TBC next meeting)

- Assumption from companies is that this should not happen due to SR triggering from Rx side, but it needs to be checked whether this will always work

The motivation of RLC-11 is to resolve the potential window stalling due to Tx side discarding. However, RLC Tx side discarding (e.g. stopReTxDiscardedSDU for UE UL) is only enabled if receiver side enhancement is supported at gNB. As discussed in RAN2#130 meeting, for Tx side discard, Rx side eventually will send RLC status report, which moves Tx side window and avoid window stalling issue. Therefore the working assumption in RAN2#130 meeting can be confirmed.

Proposal 3: (RLC-11) RAN2 to confirm the working assumption that “No need to address window stalling issue with polling retransmission”.

Open issue RLC-14

In “[POST130][508][XR] RLC running CR and open issues (vivo)” [3], open issue RLC-14 is as follows: If stopReTxDiscardedSDU is configured, when indicated from upper layer to discard a particular RLC SDU, whether the transmitting side of an AM RLC entity shall discard the corresponding RLC SDU or RLC SDU segment(s) if the RLC SDU or the RLC SDU segment(s) thereof has been submitted to lower layers.

In Rel-18, when a RLC PDU is submitted to MAC layer, corresponding RLC SDU (and PDCP SDU) is not discarded, as specified in TS 38.322 clause 5.4 (copied below).

When indicated from upper layer (e.g. PDCP) to discard a particular RLC SDU, the transmitting side of an AM RLC entity or the transmitting UM RLC entity shall discard the indicated RLC SDU, if neither the RLC SDU nor a segment thereof has been submitted to the lower layers. The transmitting side of an AM RLC entity shall not introduce an RLC SN gap when discarding an RLC SDU.

In Rel-19, if configured, RLC retransmissions are not performed for the discarded RLC SDU (segment), as from RLC running CR [4]:

If stopReTxDiscardedSDU is configured, when indicated from upper layer to discard a particular RLC SDU   (see TS 38.323 [4]), the transmitting side of an AM RLC entity shall not consider the corresponding RLC SDU or RLC SDU segment(s) for transmission or retransmission if the RLC SDU or the RLC SDU segment(s) thereof has been submitted to lower layers.

Our understanding is that if the RLC SDU or the RLC SDU segment(s) has been submitted to lower layers, they should be still discarded when stopReTxDiscardedSDU is configured. Otherwise, there is no difference between Rel-19 and Rel-18 behavior (i.e. there is no need to configure stopReTxDiscardedSDU at all if RLC SDU or the RLC SDU segment(s) has been submitted to lower layers are not discarded).

Proposal 4: (RLC-14) RAN2 to confirm that (as captured in RLC running CR) if stopReTxDiscardedSDU is configured, when indicated from upper layer to discard a particular RLC SDU, the transmitting side of an AM RLC entity shall discard the corresponding RLC SDU or RLC SDU segment(s) if the RLC SDU or the RLC SDU segment(s) thereof has been submitted to lower layers. Modification to clause 5.4 is needed to reflect this agreement.

## Impact of discard on PDCP SN gap report

Open issue PDCP-2

In “[POST130][507][XR] PDCP running CR and open issues (LGE)” [6], open issue PDCP-2 is as follows: Whether the PDCP triggers a PDCP SN gap report when at least one byte for the discarded PDCP SDU(s) have been submitted by any RLC entity to lower layers.

The issue is related to the triggering of PDCP SN gap report, as defined in TS 38.323 clause 5.16.1, as copied below:

For UM DRBs and AM DRBs configured by upper layers to send a PDCP SN gap report in the uplink (sn-GapReport in TS 38.331 [3]), the transmitting PDCP entity shall trigger a PDCP SN gap report when:

-	PDCP SDU(s) are discarded as specified in clause 5.3; and

-	there is at least one stored PDCP SDU(s) which is associated with a COUNT value larger than the COUNT value associated to the discarded PDCP SDU(s); and

-	the discarded PDCP SDU(s) have not been submitted by any RLC entity to lower layers.

As discussed in clause 2.1 above, in Rel-18, when a RLC PDU is submitted to MAC layer, corresponding RLC SDU (and PDCP SDU) is not discarded, and RLC AM is lossless, i.e. RLC retransmission is always performed until RLC PDU is successfully received. Therefore in Rel-18, a discarded PDCP SDU can be either completely submitted to MAC, or not submitted to MAC at all. The highlighted condition above assumes that if a PDCP SDU has been submitted to MAC, RLC AM can ensure the PDCP SDU is eventually successfully received, therefore a discard indication to the receiver side is not needed.

In Rel-19, if configured, RLC retransmissions are not performed for the discarded RLC SDU (segment). Therefore in Rel-19, it can happen that a discarded PDCP SDU is partially submitted to MAC layer. The expiry of timer t-RxDiscard means that the corresponding RLC SDU will not be received by RLC layer or PDCP layer. Consequently, it is necessary to handle this case.

In [7], it is proposed to modify the triggering condition to “at least one byte for the discarded PDCP SDU(s) have not been submitted by any RLC entity to lower layers”. This can cover both the Rel-18 case (no submission to MAC) and Rel-19 case (partial submission to MAC).

Proposal 5: (PDCP-2) Transmitting PDCP entity triggers a PDCP SN gap report when at least one byte for the discarded PDCP SDU(s) have not been submitted by any RLC entity to lower layers.

Open issue RLC-12

In “[POST130][508][XR] RLC running CR and open issues (vivo)” [3], open issue RLC-12 is as follows: FFS how to handle PDCP SN gap report handling during UE mobility. The issue is that before mobility, the UE sends SN gap report to the source gNB, and such gap report might not be successfully received by the source gNB. A handover happens and consequently, the target gNB does not have the SN gap report and has to wait for the expiry of t-Reordering, which delays the delivery of packets to upper layers. The issue was discussed in Rel-18, and the agreement was: “RAN2 can discuss in the context of Rel-19 whether this problem is a critical problem to solve due to RLC enhancements. If it is a problem further discussions are needed on the alternatives.”

Although in Rel-19, there can be more triggering for PDCP SN gap report compared to Rel-18 (as discussed above), it is still not a critical problem to solve. The issue only happens when a PDCP SN gap report is not successfully received by the source gNB when handover happens. This is a rare and corner case and enhancement is not necessary.

Proposal 6: (RLC-12) Enhancements for PDCP SN gap reporting during UE mobility is not needed.

# Conclusion

In this contribution, we discuss open issues of RLC AM retransmission enhancements, and propose the following:

Avoid unnecessary RLC retransmissions

Proposal 1: (RLC-10) It is up to network implementation on whether to configure stopReTxDiscardedSDU / t-RxDiscard and related network behavior.

Proposal 2: (UE capability-04) The feature txRLC-StopReTxDiscardedSDU-r19 does not have any pre-requisite (including rxRLC-Discard-r19).

Proposal 3: (RLC-11) RAN2 to confirm the working assumption that “No need to address window stalling issue with polling retransmission”.

Proposal 4: (RLC-14) RAN2 to confirm that (as captured in RLC running CR) if stopReTxDiscardedSDU is configured, when indicated from upper layer to discard a particular RLC SDU, the transmitting side of an AM RLC entity shall discard the corresponding RLC SDU or RLC SDU segment(s) if the RLC SDU or the RLC SDU segment(s) thereof has been submitted to lower layers. Modification to clause 5.4 is needed to reflect this agreement.

Impact of discard on PDCP SN gap report

Proposal 5: (PDCP-2) Transmitting PDCP entity triggers a PDCP SN gap report when at least one byte for the discarded PDCP SDU(s) have not been submitted by any RLC entity to lower layers.

Proposal 6: (RLC-12) Enhancements for PDCP SN gap reporting during UE mobility is not needed.

# References

[1] R2-2504675, Session chair (Huawei), “Report from session on R19 XR and LTE Broadcast”

[2] R2-2505138, Xiaomi, “Open issues of Rel-19 XR UE capabilities”

[3] R2-2505403, vivo, “Discussion summary and list of RLC open issue for R19 XR”

[4] R2-2505402, vivo, “RLC CR for R19 XR”

[5] R2-2503788, Huawei, HiSilicon, “Summary of [POST129bis][503][XR] RRC running CR (Huawei)”

[6] R2-2505439, LG Electronics Inc. (Rapporteur), “Summary of [POST130][507][XR] PDCP running CR and open issues (LGE)”

[7] R2-2503507, Samsung, “Discussion on RLC Enhancements for XR”