3GPP TSG-RAN WG2 Meeting #131	R2-2505140

# Bangalore, India, 25 – 29 Augugst 2025

Agenda item:		8.7.6

Source:	Xiaomi

Title:	XR rate control

Document for:		Discussion and Decision

# Introduction

In RAN2#130 meeting, following was agreed on XR rate control [1]:

Agreements on XR rate control - (MAC-05) XR rate control MAC CE can comprise multiple QoS flows (which may belong e.g. to different LCHs) unless it becomes very complicated to specify. - We rule out explicit signalling of PDU session ID + QFI - We will down-select between, considering the max number of flows we want to be able to indicate and trying to minimize overhead: - Explicit DRB ID + QFI (FFS if QFI is explicit or implicit) - Implicit, e.g. index or mapping - (RRC-1) The granularity of bit rate query prohibit timer is QoS flow. - FFS The value of the prohibit timer is the same for all flows - (MAC-08, MAC-10) Legacy Recommended bit rate query procedure (i.e. triggering, multiplexing, cancellation, prohibit timer) is used as baseline for bit rate query, with the change of logical channel to QoS flow. - (MAC-12) The index 0 in the new bit rate table does not have a special meaning, i.e., indicate 0 bit rate or make it reserved bit. - (MAC-12) The available bit rate query is not supported and the bit rate query from the UE always includes a desired bit rate. - No optimization is needed to address DC case for XR rate control MAC CE - We capture in stage-2 that XR rate indication is for application and not for MAC entity. - RAN2 will not specify DL rate control and DL rate control query

In this contribution, we discus open issues of XR rate control.

# Discussion

## (MAC-3, MAC-4) UL Rate Control MAC CE design

In RAN2#130 meeting, following was agreed regarding the MAC CE design:

  - (MAC-05) XR rate control MAC CE can comprise multiple QoS flows (which may belong e.g. to different LCHs) unless it becomes very complicated to specify.

  - We rule out explicit signalling of PDU session ID + QFI

  - We will down-select between, considering the max number of flows we want to be able to indicate and trying to minimize overhead:

    - Explicit DRB ID + QFI (FFS if QFI is explicit or implicit)

    - Implicit, e.g. index or mapping

In “[POST130][505][XR] MAC running CR and open issues (Qualcomm)” [2], the following open issues are related to UL Rate Control MAC CE design.

[MAC-3]	Design of identifier for QoS flows indicated in the UL Rate Control MAC CE.

[MAC-4]	Format of the UL Rate Control MAC CE.

The main difference between explicit ID vs. implicit index/mapping is the MAC CE overhead. With index/mapping approach, a bitmap can be used to indicate the QoS flows subject to rate control in the MAC CE. For explicit ID approach, 19 bits (or 3 octets) are needed (5 bit DRB ID + 6 bit QFI + 8 bit Bit Rate) for one QoS flow, and 3k octets for k QoS flows. For bitmap approach, suppose n octets are used for the bitmap, then n + k octets are needed to indicate k QoS flows. The condition that bitmap approach has less than or equal number of bits compared to explicit ID approach is n + k ≤ 3k, i.e. k ≥ n / 2. Table 1 below shows the overhead comparison between explicit ID and bitmap approach. It can be seen that if maximum number of QoS flows to be supported is less than or equal to 32, bitmap approach is almost always better than explicit ID approach. If maximum number of QoS flows to be supported is 64, bitmap approach is only worse when number of QoS flows is less than 4.

Table 1: Overhead comparison

| n | Max # of QoS flows | Condition that bitmap approach is better or equal to explicit ID approach (k is the # of QoS flows indicated in MAC CE) |
| --- | --- | --- |
| 1 | 8 | k ≥ 1 |
| 2 | 16 | k ≥ 1 |
| 4 | 32 | k ≥ 2 |
| 8 | 64 | k ≥ 4 |

From above signalling overhead comparison, it is proposed to consider a bitmap approach, where each bit corresponds to the QoS flow subject to rate control, and the mapping between bit and QoS flow is configured by RRC.

Proposal 1: (MAC-3) QoS flow is indicated with a bitmap, with each bit corresponding to QoS flow subject to rate control, and the mapping between bit and QoS flow is configured by RRC.

With above proposal, the MAC CE design includes a bitmap indicating QoS flows subject to the rate control, and one or multiple Bit Rate fields corresponding to the indicated QoS flows in the bitmap.

Proposal 2: (MAC-4) Rate Control MAC CE includes the following fields: one bitmap for QoS flows, one or multiple Bit Rate fields corresponding to the indicated QoS flows in the bitmap.

Rate Control MAC CE format is shown in Figure 1 below, assuming maximum 8 QoS flows can be controlled by the MAC CE.

Figure 1: UL Rate Control MAC CE format

## (MAC-5) Whether to supported truncated query

In email discussion “[POST130][505][XR] MAC running CR and open issues (Qualcomm)” [2], open issue MAC-5 is as follows:

Discuss UE behavior if available UL-SCH resources cannot accommodate all the pending bit rate queries in a single UL Rate Control MAC CE, e.g. whether the UE can transmit some of the queries or it must wait until it can send all pending queries together.

Legacy Recommended bit rate MAC CE is per logical channel, and MAC CE content contains only two bytes. The procedure to send the Recommended bit rate query is to transmit the MAC CE if there is space available for the MAC CE after LCP, as copied below:

1>	for each Recommended bit rate query that the Recommended Bit Rate procedure determines has been triggered and not cancelled:

2>	if bitRateQueryProhibitTimer for the logical channel and the direction of this Recommended bit rate query is configured, and it is not running; and

2>	if the MAC entity has UL resources allocated for new transmission and the allocated UL resources can accommodate a Recommended bit rate MAC CE plus its subheader as a result of LCP as defined in clause 5.4.3.1:

3>	instruct the Multiplexing and Assembly procedure to generate the Recommended bit rate MAC CE for the logical channel and the direction of this Recommended bit rate query;

Rel-19 UL Rate Control MAC CE can contain multiple queries, and the procedure to send the MAC CE in MAC CR [3] is as follows:

When UL-SCH resources are available for a new transmission, the MAC entity shall:

1> for each QoS flow with a pending bit rate query:

2> if bitRateQueryProhibitTimer for the QoS flow is configured but not running:

3> include the QoS flow and its preferred bit rate in the MAC entity’s list of bit rate queries;

1>	if there is at least one entry in the MAC entity’s list of bit rate queries and the UL-SCH resources can accommodate the UL Rate Control MAC CE plus its subheader as a result of logical channel prioritization:

2> instruct the Multiplexing and Assembly procedure to generate the UL Rate Control MAC CE;

The logical channel priority of MAC CE of UL bit rate query is lower than that of UL data, as from MAC CR [3]:

Logical channels shall be prioritised in accordance with the following order (highest priority listed first):

-	MAC CE for C-RNTI, or data from UL-CCCH;

…

-	data from any Logical Channel, except data from UL-CCCH;

-	MAC CE for Recommended bit rate query, or MAC CE for UL bit rate query;

-	MAC CE for BSR included for padding;

Given that triggering of bit rate query is up to UE implementation and the low logical channel priority of UL bit rate query, there is no issue for UE to wait for UL resource to send all pending queries together. MAC CR [3] can be interpreted in this way and no specification change is needed.

Proposal 3: (MAC-5) Truncated rate query is not supported, i.e. UE transmits UL Rate Control MAC CE when the UL-SCH resources can accommodate the MAC CE containing all pending bit rate queries plus its subheader as a result of LCP. No specification change is needed.

## (UE capability-05) Maximum number of QoS flows subject to rate control

In “[POST130][509][XR] UE capabilities CRs (Xiaomi)” [4], open issue UE capability-05 is: Whether maximum number of QoS flows subject to rate control should be defined as UE AS capability and the value range of the capability (if to be defined).

Since UE needs to maintain the internal communication between AS layer and application layer about the rate control, maximum number of QoS flows subject to rate control should be a UE capability to cater for different UE implementations. One question is whether such capability should be AS UE capability or NAS UE capability. It is preferred that AS UE capability is defined based on following reasoning:

- There is already an AS UE capability ul-RateControl-r19 defined, and it is straightforward that maximum number of QoS flows subject to rate control is considered as a component of ul-RateControl-r19.

- Since gNB is sending Rate Control MAC CEs to the UE, gNB should be aware of the number of QoS flows subject to rate control to avoid exceeding UE’s capability. If a NAS capability is defined, mechanism should be defined to let gNB know such NAS capability. This increases the specification complexity.

From above discussion, it is preferred to define AS capability. Existing UE capability ul-RateControl-r19 can be reused to indicate the maximum number of QoS flows subject to rate control, e.g. as below:

ul-RateControl-r19       INTEGER (1..64)     OPTIONAL

The value range of the capability can be 1 to the maximum number of QoS flows controlled by the MAC CE, which is expected to be decided in discussion of MAC CE design.

Proposal 4: (UE capability-05) Maximum number of QoS flows subject to rate control is defined as component of UE capability ul-RateControl-r19, and the value range is 1 to the maximum number of QoS flows controlled by the MAC CE.

# Conclusion

In this contribution, we discuss open issues of XR rate control, and propose the following:

Proposal 1: (MAC-3) QoS flow is indicated with a bitmap, with each bit corresponding to QoS flow subject to rate control, and the mapping between bit and QoS flow is configured by RRC.

Proposal 2: (MAC-4) Rate Control MAC CE includes the following fields: one bitmap for QoS flows, one or multiple Bit Rate fields corresponding to the indicated QoS flows in the bitmap.

Proposal 3: (MAC-5) Truncated rate query is not supported, i.e. UE transmits UL Rate Control MAC CE when the UL-SCH resources can accommodate the MAC CE containing all pending bit rate queries plus its subheader as a result of LCP.

Proposal 4: (UE capability-05) Maximum number of QoS flows subject to rate control is defined as component of UE capability ul-RateControl-r19, and the value range is 1 to the maximum number of QoS flows controlled by the MAC CE.

# References

[1] R2-2504675, Session chair (Huawei), “Report from session on R19 XR and LTE Broadcast”

[2] R2-2505070, Qualcomm Incorporated, “List of open issues in MAC”

[3] R2-2505069, Qualcomm Incorporated, “Introduction to R19 XR enhancements”

[4] R2-2505138, Xiaomi, “Open issues of Rel-19 XR UE capabilities”