3GPP TSG RAN WG2 Meeting #131	R2-2505144

Bangalore, India, Aug 25 - 29, 2025

Title:	Final aspects on loT NTN TDD mode

Source: 	Iridium, CCL

Document for:	Discussion and Decision

Agenda Item:	8.17

# Introduction

The IoT NTN TDD mode work item was approved in RAN#105 [2] and later revised in RAN#106 [1] following the conclusion of the RAN1 study phase. The revised WID outlines the following objectives for RAN2:

- Specify a new NB-IoT TDD NTN mode based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures, including: - Definition, configuration (if needed) and signaling (if needed) of the periodic pattern, necessary adaptation (if needed) and associated UE procedures [RAN1, RAN2] - Support a pattern with a period of 9 radio frames for the target MSS allocated band, where D=U=8 with a fixed guard period. RAN1 to consider whether there is a need for a mechanism to achieve an adjustable guard period for the purpose of allowing deployment with the TDD frame structure of the legacy system operating in the target MSS allocated band. Other necessary impacts on higher layers [RAN2]

For NB-IoT frame structure type-1 to be utilized on the same frequency in both uplink and downlink, separate spectrum time is allocated for each, separated by a time gap. The period during which uplink operations are allowed is referred to here as the U subframes, and the period during which downlink operations are allowed is referred to as D subframes (Figure 1). In RAN#106 it is agreed that the periodicity of the active uplink and downlink times are fixed for NGSO satellite system operating in L-band (1616 – 1626.5 MHz) at 90ms with N=9. There is no RF transmission or reception during inactive times, resulting in discontinuous downlink transmissions. It is also agreed that duration of uplink and downlink active time i.e. D=U=8ms and the gap from the end of downlink active time to the start of next uplink active time is fixed for MSS spectrum spanning 1616.0 to 1626.5 MHz.

Figure 1 IoT NTN TDD mode with HD-FDD frame structure

The “non-D” are subframes not contained within the set of D=8 usable consecutive downlink subframes in the IoT NTN TDD structure. Similarly, the “non-U” are subframes not contained within the set of U=8 usable consecutive uplink subframes in the IoT NTN TDD frame structure.

In this contribution, we outline our views on the remaining open issues related to IoT NTN TDD mode, as well as the applicability of new Rel-19 IoT-NTN features for IoT NTN TDD mode.

# Discussion

## Remaining Open Issues

In this section, we present our views on remaining open issues for IoT NTN TDD.

### Paging

The following relevant agreements are made in RAN2#129bis [4]:

Agreement - Regarding paging occasion determination, legacy NB-IoT PO determination mechanism is used. When the determined paging subframe is not a valid downlink subframe, the Paging monitoring is postponed to the nearest valid downlink subframe.

The following relevant agreements are made in RAN2#130:

Agreement - The remaining paging repetitions falling on the invalid DL SFNs are postponed to the next valid DL SFNs. - It is up to network to configure the gap between two POs (i.e., parameter NB) to be sufficiently long such that it includes enough number of valid DL subframes for NumRepetitionPaging-r13 (no spec impact)

UE paging is possible only in certain frames and subframes. The frames where paging is allowed are called Paging Frames (PF), and the subframes within these paging frames are referred to as Paging Occasions (PO). Each UE is assigned to one of these paging frames based on its IMSI value, so it only needs to monitor one paging occasion per DRX cycle.

A paging message is transmitted in consecutive NB-IoT downlink subframes.  As per RAN1#120 agreement, in IoT NTN TDD mode the definition of NB-IoT downlink subframes also excludes those subframes which are falling into downlink inactive time (non-D subframes). This agreement also results in postponement of any paging opportunities that fall into downlink inactive time (non-D subframes) to the next downlink active time (D subframes).

One of the remaining issues from RAN 2#130 is to determine whether RAN2 should introduce a new DRX cycle value aligned with the 90ms IoT NTN TDD mode periodicity. Introducing DRX cycle lengths in multiples of 90ms can help avoid postponement of paging occasions for some UEs. However, this approach does not increase network paging capacity or offer additional scheduling flexibility. Similar paging capacity can be achieved by configuring the DRX cycle using the existing parameter set and applying the already agreed NPDCCH and NPDSCH postponement rules.

Even if we introduce the DRX cycle length in multiples of 90ms, due to the way PF selection works, the selected SFN for paging for many UEs may still fall within downlink inactive periods, requiring the use of the agreed postponement rules as shown in Figure 2.

Figure 2 Example DRX cycle of 2520ms (multiple of 90ms)

Proposal 1: There is no benefit in introducing a new idle mode DRX cycle length for IoT NTN TDD mode.

The start subframe that can be used for paging occasions are restricted by the number of POs per frame, and the only allowed subframes are subframe #0, subframe #4, subframe #5, and subframe #9. The wording of the agreement made in RAN1#130 “The remaining paging repetitions falling on the invalid DL SFNs are postponed to the next valid DL SFNs” could be interpreted to mean that only the PF is postponed, while the PO can still be calculated within the postponed PF, based on the reference table provided below. (36.304 §7.2).

To avoid confusion caused by the above interpretation, we would like to clarify that distributing the start of the PO across the D=8 subframes does not lead to an increase in paging capacity or provide significant scheduling flexibility.

In the case of paging, the network is unaware of the coverage class of a UE. Therefore, NPDCCH/NPDSCH is transmitted with many repetitions to ensure that UEs in poor coverage areas can receive the paging message. Assuming 4 repetitions are used for NPDCCH and NPDSCH transmissions for paging with a gap of 4 subframes, one paging message transmission (NPDCCH + NPDSCH) will always span across more than one active downlink opportunity.

Observation 1: There is no performance benefit in distributing the start paging subframes within D=8 subframes.

Proposal 2: There is no specification impact for RAN2 (36.304) related to paging scheduling.

### SIB1 Scheduling

One of the other remaining open issues from RAN2#130 is whether there is a need to update the SIB1 scheduling mechanism in the existing specifications when operating in IoT NTN TDD mode.

In RAN1#120bis [3] following agreement is made which concludes that network configuration can support broadcasting SIB1 subframes in every alternate downlink opportunity and it meets the link budget and latency requirements. The scheduling configuration of SIB1 remains the responsibility of the network operator, and no changes to the existing SIB1 scheduling mechanism are required.

Agreement SIB1-NB is dropped in non-D NB-IoT subframes - NOTE 1: “non-D NB-IoT subframes” are subframes not contained within the set of D=8 usable consecutive downlink subframes in the TDD structure. - NOTE 2: RAN1 observes that some combinations of {TBS, number of repetitions, additional SIB1 repetitions}, and depending on aspects such as minimum elevation angle or antenna gain, may not meet the link budget. It is up to the network to properly configure such values to ensure the link budget is met. No specification impact.

Proposal 3: There is no specification change required in the existing SIB1 scheduling mechanism.

### Neighbor Cell Information

One of the remaining open issues from RAN2#130 is whether to introduce a new list of neighbor cells operating in IoT NTN TDD mode for measurements and cell reselection.

For cell reselection purposes, neighbor cell information is broadcast in SIB4-NB for intra-frequency neighbor cells and in SIB 5-NB for inter-frequency neighbor cells. In the case of terrestrial NB-IoT, the same neighbor cell list is used for both FDD and TDD modes. IoT NTN TDD mode can also re-use the existing neighbor cell lists in SIB4-NB and SIB5-NB.

In NTN, SIB33-NB broadcasts satellite assistance information for neighbor cells, including the k-mac value. Since IoT NTN TDD uses an extended k-mac value range, SIB33-NB should also be updated to include the extended k-mac value for neighbor cells. The following relevant agreement is made in RAN2#129bis [4]:

Agreement - For IoT NTN TDD mode, support k-Mac with a value range up to 1023 ms (add corresponding a restriction in the field description) - Can further discuss extending this proposal to IoT NTN and NR NTN in the next meetings

Proposal 4: No specification changes are required for providing neighbor cell information for cell reselection.

Proposal 5: SIB33-NB should be updated to provide extended k-mac value up to 1023ms for the neighbor cells

## Applicability of IoT-NTN Rel-19 features for IoT NTN TDD

This section evaluates the applicability of new Rel-19 features being discussed under IoT-NTN phase 3 WI for IoT NTN TDD mode.

### CB-Msg3 and CB-Msg4

In Rel-19 IoT-NTN WI, enhancement to early data transfer procedure is in progress to improve the network uplink capacity. This feature is commonly known as Contention Based Msg-3(CB-Msg3) and Contention Based Msg-4 (CB-Msg4) transmission. In the existing RACH based EDT procedure, UE first sends NPRACH preamble (msg1), receives RAR with timing advance and uplink grant (msg2) and then transmits user data in NPUSCH (msg3). In the case of NTN since it is mandatory for UE to have GNSS and using ephemeris data, UEs are expected to pre-compensate timing errors before any uplink transmission. This makes timing correction based on msg-1 and msg-2 somewhat redundant. So, with this feature if a UE is uplink time synchronized, it can directly transmit msg-3 in the uplink without going through the RACH procedure.

Uplink resources should be reserved for CB-Msg3 transmissions, and the allocated resources are broadcasted in SIB(s). A periodically occurring time window is defined during which a UE can transmit msg-3 with one or more configured repetition values. The definition of configuration parameters is still under discussion in RAN2.

Like NPRACH, multiple UEs may attempt to transmit msg-3 within the same time window. To address this, Diversity ALOHA (DSA) is introduced, allowing a UE to transmit multiple copies of msg-3 randomly in both the time and frequency domains within the configured window. Once a UE transmits CB-msg3-EDT, it will monitor for msg-4 within a new time window, following the same concept as RAR monitoring. A new MAC PDU is introduced, enabling the network to include contention resolution identity, timing advance, and/or user data in msg-4.

This feature can also contribute to improved network uplink capacity in IoT NTN TDD mode. All specification changes agreed upon under IoT-NTN Phase-3 WI so far do not introduce any additional complexity that would hinder its operation in IoT NTN TDD mode.

Proposal 6: There is no specification impact for Rel-19 IoT-NTN CB-Msg3 and CB-Msg4 features to work in IoT NTN TDD mode.

### Orthogonal Cover Codes (OCC)

In Rel-19 IoT-NTN WI, another feature is being introduced to improve uplink capacity by using Orthogonal Cover Codes (OCC).  This feature is primarily driven by RAN1.

In NTN with large cell sizes, UL resources per UE are limited. Due to the transmit power constraints of UEs, large repetitions are required in the uplink, which significantly reduces uplink capacity. To address this, Release 19 introduces a new feature in IoT-NTN that allows two UEs to be multiplexed on the same UL resources—both in time and frequency—using OCC. On the network side, each UE can be uniquely identified by the orthogonal nature of OCC applied to the DMRS.

This feature is only applicable to NPUSCH Format 1 transmissions (i.e., not for HARQ ACKs) and is supported only in connected mode. The specific OCC code to be used is dynamically signaled to the UE by overriding existing fields in DCI. OCC is not applicable for NPRACH transmissions.

The application of OCC differs between 3.75 kHz and 15 kHz subcarrier spacing (SCS). In the case of 15 kHz SCS, slot level scheme is used where slots are spread before applying the OCC. So, when OCC is applied, for a given UE NPUSCH transmission, it will take twice the time required compared to the case when OCC is not applied.  Even with OCC enabled, the number of effective NPUSCH repetitions required to meet the target BLER remains unchanged in some scenarios. As a result, the network can accommodate a greater number of UEs within the same time window.

In IoT NTN TDD mode, NPUSCH transmissions overlapping with uplink inactive times (non-U subframes) are postponed to the next uplink active time (U subframes). The unit of postponement is a subframe whereas OCC spreading applies at the slot level. Thus, orthogonality of OCC code can be maintained even for IoT NTN TDD mode.

This feature can also contribute to improved network uplink capacity in IoT NTN TDD mode. All specification changes agreed upon under IoT-NTN Phase-3 WI so far do not introduce any additional complexity that would hinder its operation in IoT NTN TDD mode.

Proposal 7: The Rel-19 IoT-NTN OCC feature is compatible with IoT NTN TDD without any specification impact.

### Public Warning System (PWS)

In Rel-19 IoT-NTN Phase-3, the Public Warning System (PWS) feature is introduced for NB-IoT NTN covering all PWS services including ETWS and CMAS. Similar to LTE, warnings are broadcasted via system information, and new SIBs have been introduced: SIB10-NB, SIB11-NB and SIB12-NB. Like LTE, a UE can be paged for PWS which triggers UE to acquire corresponding PWS SIB. It is understood that for NB-IoT a PWS requirement may not be met when UE is in a power saving mode like PSM/e-DRX.

This PWS feature will also work with IoT NTN TDD mode without requiring any additional specification changes, as it leverages the existing paging and SI scheduling mechanisms.

Proposal 8: The Rel-19 IoT-NTN PWS feature is compatible with IoT NTN TDD mode without any specification impact.

# Conclusion

In this contribution, we discussed remaining open issues and applicability of new Rel-19 IoT-NTN features discussed under IoT-NTN phase-3 WI for IoT NTN TDD mode. Our observations and proposals are listed below:

Observation 1: There is no performance benefit in distributing the start paging subframes within D=8 subframes.

Proposal 1: There is no benefit in introducing a new idle mode DRX cycle length for IoT NTN TDD mode.

Proposal 2: There is no specification impact for RAN2 (36.304) related to paging scheduling.

Proposal 3: There is no specification change required in the existing SIB1 scheduling mechanism.

Proposal 4: No specification changes are required for providing neighbor cell information for cell reselection.

Proposal 5: SIB33-NB should be updated to provide extended k-mac value up to 1023ms for the neighbor cells

Proposal 6: There is no specification impact for Rel-19 IoT-NTN CB-Msg3 and CB-Msg4 features to work in IoT NTN TDD mode.

Proposal 7: The Rel-19 IoT-NTN OCC feature is compatible with IoT NTN TDD without any specification impact.

Proposal 8: The Rel-19 IoT-NTN PWS feature is compatible with IoT NTN TDD mode without any specification impact.

# References

- RP-243293, Revised WID on introduction of IoT-NTN TDD mode, RAN#106, 2024-12, Madrid, ES.

- RP-242415, “New WID on introduction of IoT-NTN TDD mode”, RAN #105, Melbourne, Australia, 2024-09

- R1-2503201, “Final Meeting Report”, 3GPP TSG RAN WG1 Meeting #120bis, Wuhan, China, April 07-11, 2025

- R2-2503302, “Final Meeting Report”, 3GPP TSG RAN WG2 Meeting #129bis, Wuhan, China, April 07-11, 2025