3GPP TSG RAN WG2 #131	R2-2505146

Bengaluru, Karnataka, India, August 25th - 29th, 2025

Source:	Panasonic

Title:	Store & Forward: Remaining neighbour cell issues

Agenda Item:	8.9.2  Support of Store & Forward

Document for:	Discussion, decision

Introduction

Store & Forward operation of satellites is seen as a key enhancement of IoT-NTN in order to facilitate cost-effective deployment of IoT-NTN to allow for delay-tolerant, non-real-time IoT-NTN services [1].

- Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore: - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2, RAN4] - At least specify necessary enhancements e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3] Note: Strive to minimise UE impact. Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required.

Furthermore, the NTN sessions of the last RAN2 meeting #130 in St. Julian’s resulted in the following S&F-related results (see [3]):

St. Julian’s agreements (meeting #130):

- When the S&F operation indication is present, the agreed time information in SIB31 (i.e., t-ModeSwitching) indicates when the normal mode will start; otherwise, the agreed time information in SIB31 (i.e., t-ModeSwitching) indicates when the S&F mode will start.

- In a S&F network deployment which also exhibits discontinuous coverage, existing mechanisms to handle discontinuous coverage can be leveraged (e.g. satellite assistance information, UE not needing to perform idle mode tasks when the UE determines that is out of coverage, etc.). There is no need to modify existing discontinuous coverage features due to the addition of S&F Satellite operation. FFS if we clarify in discontinuous coverage procedure in idle mode that the UE also takes into account the information about NAS configured S&F monitoring list.

- Clarify in Stage 2 from Rel-18 (so in general, not only for S&F) that the values used in “satelliteId-r17” are expected to correspond to the values used in “satelliteId-r18” in other SIBs (e.g. the value used to identify the serving satellite in SIB33, SIB32 and SIB31 should be the same). Come back in the next meeting with an actual Rel.18 CR for this. FFS if in Rel.19 we need to say anything about the correspondence between the “satelliteId” IE and “S&F Monitoring List”.

Working Assumption:

- In the neighbour cell list we introduce an indication whether the cell operates in S&F mode or not (FFS if we also include the transition time). This WA can only be confirmed if we converge on the corresponding UE behaviour.

The aim of this document is to propose solutions for the remaining open S&F issues.

Discussion on remaining S&F-related neighbour cell issues

# 2.1 Frame conditions

The presence of a second tuner could make a-priori signalling for neighbouring cells superfluous. On the other hand, operating a second tuner for longer time periods consumes precious energy – what might not be in the best interest of the user.

Anyway, RAN2 came to consensus during the recent Malta meeting regarding this Working Assumption:
In the neighbour cell list we introduce an indication whether the cell operates in S&F mode or not (FFS if we also include the transition time). This WA can only be confirmed if we converge on the corresponding UE behaviour.

2.2	Models and real world scenarios

As an example, a LEO EMC situation is outlined in this clause – in steps. A MEO EMC scenario would look similar.

First of all, a simplified view on a single orbit situation is presented. On the ground five UEs and two gateways are located. Up in the atmosphere there is a constellation subset of four satellites A, B, C and D shown occupying the same orbit. The light blue and light rose triangles underneath the satellites are illustrating their illumination zone, the yellow triangles emitted by the gateway show the zone within which satellites are connected to the gateway (capture range). A blue triangle indicates Normal Mode (NM) and a rose triangle is for Store & Forward Mode (SFM).
Since satellites are moving, they are losing connection to the gateway when leaving its capture range and they are getting connected to the related gateway once entering its capture range, i.e. these are the switches between Normal Mode and Store & Forward Mode.

Figure 1 is just supposed to illustrate the movement of the satellites and the corresponding mode changes. In addition, figure 1 also indicates the mode changes for the UEs on the ground.

  - Figure 1: LEO/MEO satellites (EMC case) moving along UEs on the ground and
partly changing between Normal Mode (NM) and Store & Forward Mode (SFM)

  - Figure 2 below provides an impression of a Walker-Delta constellation as applied to e.g. the Starlink LEO mega-constellation.

  - Figure 2: Walker-Delta constellation as deployed for Starlink LEO network (source Wikipedia)

  - Figure 3 below introduces the reality of the Starlink mega-constellation with a snapshot onto Europe taken these days. Interesting is the apparently irregular distribution of satellites. Since we don’t see the zones of illumination of each satellite on the ground, we can’t derive from the aforementioned irregularity of satellite positions a similarly irregular illumination of areas on the Earth’s surface. Starlink claims uninterrupted coverage of the shown area.

  - Note the at least eight satellites are always in sight of our lab’s location (see list in the bottom left corner). Even satellites in 600 km are received with sufficient fieldstrength.

  - Figure 3: Real world Starlink satellite distribution over central Europe on 29th July 2025, 12:11:04 UTC (source https://satellitemap.space/)

  - 2.3 Converting gained findings into a signalling proposal

  - Taking clauses 2.1 and 2.2 as a basis, figure 4 below is an example for different UEs being connected to a LEO EMC network. It is an idealisation of what has been introduced with previous clauses, figure 3 in particular.

  - Figure 4 outlines a single shell of a constellation with two triples of parallel orbits that are crossing one another with a 90° angle. Whereas the multiple coloured circular areas of same size illustrate the satellites’ illumination zones on the ground, the larger red circle identifies the capture range of a gateway on the ground. Assuming vertical satellite beams only, the sketched constellation section is connected to the postulation that once the centre of an illumination area reaches the capture range of the gateway, the related satellite gets connected to the gateway.

## Figure 4: Idealised LEO constellation (EMC) with four UEs and one gateway

## As can be seen here, different situations might occur for the different UEs:

- UE1 could see an NM cell for a pretty short time period, i.e. as long as the centre of the cell the UE is connected to is within the capture range of the gateway

  - Is the case for the light blue sequence of cells

  - Is not the case for the light yellow sequence of cells, i.e. being connected to those means permanent SFM

- UE2 will never see an NM cell, it always sees an SFM cell

- UE3 should be switching to the neighbour cell after that one has entered NM, i.e. when its centre has reached the capture range of the gateway

  - In that case UE3 would always see an NM cell

- UE4 is confronted with a dynamic situation, i.e. it sometimes sees a cell in NM and sometimes the same cell in SFM – depending on the position of the corresponding cell centre

  - If the cell centre is outside of the capture range of the gateway, the UE sees an SFM cell

  - If the cell centre is inside the capture range of the gateway, the UE sees an NM cell

  - In certain cases it doesn’t matter which neighbouring cell the UE reselects, but in other cases it really matters. Not mattering is equivalent to the mode of operation remains the same or a mode change from NM to SFM can’t be prevented anyway. Mattering means there are alternative cells, of which at least one provides NM operation and at least one other can’t provide NM.

  - Observation 1:	Different realistic cases can be drawn up illustrating advantages for UEs that have a priori information available about the mode of operation of neigh-bouring cells as well as about the point of time the mode the related neigh-bouring cell operates in changes.

  - The above is reason enough – from our point of view – for adopting both,
a) the mode the neighbouring cell currently operates in (cellMode) as well as
b) the point of time it is going to switch to the opposite mode (t-ModeSwitchingNeigh).

  - We are hence suggesting sticking to the Working Assumption regarding the indication of the mode of the neighbouring cells following the related changes. And we also support the indication of those points of time the mode of operation of each neighbouring cell changes. This is because it helps preventing switching where not necessary to cells operating in SFM or will be operating in SFM.

  - Proposal 1:	Introduce an indication of the mode the neighbouring satellite/cell currently operates in - satelliteModeNeigh - to UEs in order to influence the decision for switching to the most suitable neighbouring cell.

  - Proposal 2:	The point of time the cell is going to switch to the opposite mode is indicated by a new field t-ModeSwitchingNeigh.

  - As can be derived from the figures above as well, the situation of the UE depends widely on its geographical position relative to the gateway on the ground. Should it be positioned closely to the gateway, it will always “see” NM, far away it always “sees” SFM, only in an area at the edge of the gateway geographical capture range things are ambiguous and one can benefit from the proposed signalling.

  - 2.4 Signalling in SIB32

  - Signalling should be connected to the indication of frequencies of other cells as realised with SIB32 for the LTE-based IoT-NTN case (see [2]). Therefore, we suggest to add a new carrierFreqList-v19 entry into the SIB32 coding – as shown below.

  - Observation 2:	SIB32 carries the indication of frequencies of other cells. That SIB might hence be the best place for indicating the two new fields in question.

  - Proposal 3:	Indicate the two new fields indicating the mode the related neighbour satellite is operating in as well as the point of time the next mode switch is going to take place in SIB32 by means of a new carrierFreqList-v19 loop.

  - Figure 5: Complete SIB32 with proposed insertion of satelliteModeNeigh and t-ModeSwitchingNeigh

  - Observation 3:	It seems useful aligning the definitions of the two new fields to those agreed already for the corresponding fields describing serving satellite attributes.

  - Proposal 4:	Definitions of the two new parameters should be these:

  - satelliteModeNeigh
Indicates the S&F mode the neighbour cell identified by satelliteId-r17 is currently operating in. “0” indicates Normal Mode, whereas “1” indicates Store & Forward Mode (SFM).

  - t-ModeSwitchingNeigh
Indicates the next point of time the satellite identified by satelliteId-r17 changes from its current mode of operation to the other mode, i.e. from Normal Mode to Store & Forward Mode in case the current mode is Normal Mode or from Store & Forward Mode to Normal Mode in case the current mode is Store & Forward Mode.
The form of indication shall be absolute UTC time.

Conclusions

In this document, we discussed the remaining open issues in conjunction with Store and Forward satellite operation mode.

The following observations and proposals were made:

  - Observation 1:	Different realistic cases can be drawn up illustrating advantages for UEs that have a priori information available about the mode of operation of neigh-bouring cells as well as about the point of time the mode the related neigh-bouring cell operates in changes.

  - Observation 2:SIB32 carries the indication of frequencies of other cells. That SIB might hence be the best place for indicating the two new fields in question.

  - Observation 3:	It seems useful aligning the definitions of the two new fields to those agreed already for the corresponding fields describing serving satellite attributes.

  - Proposal 1:	Introduce an indication of the mode the neighbouring satellite/cell currently operates in - satelliteModeNeigh - to UEs in order to influence the decision for switching to the most suitable neighbouring cell.

  - Proposal 2:	The point of time the cell is going to switch to the opposite mode is indicated by a new field t-ModeSwitchingNeigh.

  - Proposal 3:	Indicate the two new fields indicating the mode the related neighbour satellite is operating in as well as the point of time the next mode switch is going to take place in SIB32 by means of a new carrierFreqList-v19 loop.

  - Proposal 4:	Definitions of the two new parameters should be these:

  - satelliteModeNeigh
Indicates the S&F mode the neighbour cell identified by satelliteId-r17 is currently operating in. “0” indicates Normal Mode, whereas “1” indicates Store & Forward Mode (SFM).

  - t-ModeSwitchingNeigh
Indicates the next point of time the satellite identified by satelliteId-r17 changes from its current mode of operation to the other mode, i.e. from Normal Mode to Store & Forward Mode in case the current mode is Normal Mode or from Store & Forward Mode to Normal Mode in case the current mode is Store & Forward Mode.
The form of indication shall be absolute UTC time.

References

[1]	RP-240776 Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3

[2]	3GPP TS 36.331 V18.6.0 (2025-06)				3rd Generation Partnership Project; Technical
                    	Specification Group Radio Access Network;
                    	Evolved Universal Terrestrial Radio Access (E-
                    	UTRA); Radio Resource Control (RRC);
                    	Protocol specification (Release 18)

[3]	R2-130 NR-NTN-IoT-NTN (Sergio)_ EOM