3GPP TSG-RAN WG2 Meeting #131	                                          R2-2505153

Bengaluru, India, 25 – 29, Aug, 2025

Source: 	Xiaomi

Title:  	Discussion on data collection

Agenda Item:	8.3.2.3

Document for:	Discussion and Decision

Introduction

In this discussion, we discuss potential configuration and procedure of data collection for UE sided model in AI mobility.

Discussion

In AI air, the NW would provide candidate data collection configuration. UE can request the data collection from the candidate configurations. The motivation is to avoid UE to request data collection configuration which NW can’t support. For example, UE may request more beams than NW can support. However, in AI mobility, we understand the collected data is L3 measurement result. It’s not clear whether it’s possible that UE requested data collection configuration which NW can’t support. We shall first which data collection configuration can be requested by UE.

The RS for RRM can be SSB or CSI-RS. During the study, the evaluation is mainly based on SSB. It’s not clear whether CSI-RS can be supported in AI mobility. If CSI-RS is supported, the AI model may be different for SSB and CSI-RS. UE should be allowed to request the RS type for RRM.

Inter-frequency prediction is also supported in AI mobility. UE may need to collect the measurement result on inter-frequency. Depending on the capability, UE may require measurement gap to perform inter-frequency measurement. The measurement gap configuration should be provided by serving cell, since UE can’t receive the downlink transmission from serving cell within measurement gap. Therefore, UE shall be allowed request the preferred measurement gap for data collection. The measurement gap configuration can be indicated by explicit gap pattern explicitly or by the cell/frequency to be measured implicitly.

In legacy measurement, NW would provide SMTC to UE for measurement. The SMTC can be configured per frequency or per cell. During data collection, UE should be allowed to request the SMTC per cell or per frequency to perform measurement. UE can indicate the cell/frequency to be measured.

Proposal 1: RAN2 discuss the requested data collection configuration including,

- RS type, i.e. SSB or CSI-RS

- measurement gap configuration

- Cell/frequency to be measured

Based on above discussion, the requested data collection configurations are basic measurement configuration. Seems NW should be capable to provide any requested configuration. Therefore, unless we can identify the case that requested configuration can’t be supported by NW, NW doesn’t need to provide the candidate data collection configuration. It can be left to UE implementation to decide the requested data collection configuration. NW can decide whether to provide the requested configuration by implementation.

Conclusions

According to the analysis given above, we have the following observations and proposals:

Proposal 1: RAN2 discuss the requested data collection configuration including,

- RS type, i.e. SSB or CSI-RS

- measurement gap configuration

- Cell/frequency to be measured

Reference

[1]