3GPP TSG-RAN WG2 Meeting #131	                                          R2-2505154

Bengaluru, India, 25 – 29, Aug, 2025

Source: 	Xiaomi

Title:  	Discussion on NW sided model

Agenda Item:	8.3.3

Document for:	Discussion and Decision

Introduction

In this discussion, we discuss the scope of NW sided model and the NW sided model data collection.

Discussion

In last meeting, following scope is agreed for further study,

|  | UE sided model | Network sided model |
| --- | --- | --- |
| Measurement event prediction | Yes, | No (up to NW implementation) |
| Temporal domain case A | Yes, all sub cases | Yes |
| Temporal domain case B | Yes, all sub cases | Yes |
| Frequency domain | Yes, all sub cases | Yes |
| Spatial domain | Pending | Pending |
| L3 beam level measurement prediction | Yes, | Yes |

  - Temporal domain case B

For scenario 2, the measurement reduction is achieved by skipped measurement in temporal domain. Although SID [2] indicated both UE and NW sided model are in the scope of RRM prediction, we understand only UE sided model is useful in scenario 2. This is mainly because that, for NW side model inference, network would still require UE to measure and report same number of measurement sampling as model input, at least for model monitoring purpose. Therefore, measurement reduction cannot be achieved by using NW side model.

Observation 1: NW-side model in temporal domain case B can’t reduce UE measurement

In last meeting, some companies commented NW sided model can be used to reduce the SSB signalling. However, we believe this is not feasible. Unlike BM, SSB for RRM is cell specific rather than UE specific. As legacy UEs would be there anyway, reducing SSB would impact legacy UE’s measurement accuracy.  Therefore, we believe it’s impossible for NW to reduce SSB signalling by NW side model in temporal domain case B.

Observation 2: SSB can’t be reduced by NW-sided model in temporal domain case B, due to impact to legacy UEs.

Proposal 1: NW sided model is not supported in temporal domain case B.

  - Input to NW side model

Three sub use cases are supported for RRM prediction according to TR[1] as following,

Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated

Sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s)

Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s)

For L3 beam level measurement prediction, following three sub-cases are supported according to TR [1],

Sub-use case 4: L1 filtered beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 beam-level measurement result is generated

Sub-use case 5: L3 beam-level measurement result(s) is predicted based on actual L3 beam-level measurement result(s)

Sub-use case 6: L3 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s)

The inference input of sub use case 1, 3, 4 and 6 are L1 beam measurement results. Note the L1 beam for RRM is configured by CSI-RS-ResourceConfigMobility, which is different from the beam for beam management. Current CSI report framework doesn’t support reporting L1 beam measurement result for RRM purpose. Furthermore, RRM prediction may require neighbor cell’s L1 beam measurement result as model input. Current CSI report framework only support reporting serving cell’s L1 beam measurement result.

It’s up to UE how to measure the RRM L1 beam within the required latency, i.e. by what periodicity filtering. NW side model may require L1 beam measurement with fixed periodicity as input. It’s new requirement for UE measurement, which is impossible to apply from UE point of view. Furthermore, it’s up to UE how to perform the L1 filtering. UEs may apply different L1 filtering method. NW is not aware which L1 filtering is applied by UE. The inference accuracy may be poor without such information.

As discussed above, new report signalling is required to report L1 beam measurement result. New requirement on UE measurement is needed to align with the model input interval. UE may also need to report how L1 filtering is done. The spec impact is huge to support L1 beam as model input, i.e. sub-case 1 or 3, for NW sided model.

Observation 3: Huge spec impact is foreseen to support L1 beam as model input for NW sided model, i.e. sub case 1, 3, 4 and 6, including

- new signalling to report L1 beam measurement result of serving and neighbour cell for RRM, as current L1 beam for RRM is configured by CSI-RS-ResourceConfigMobility, rather than using CSI framework

- new requirement on UE measurement with potential RAN4 impact

- new signalling to report how L1 filtering is done, which is now completely up to UE implementation

Therefore, we propose to only support sub-case 2, i.e. L3 cell level measurement results as model input, is supported for NW sided model in RRM prediction.

Proposal 2: L1 beam measurement result as model input, i.e. sub case 1, 3, 4 and 6, is not supported for NW sided model in RRM prediction and L3 beam prediction.

In NW sided model, the inference is done at NW. The model input is real measurement results, which has to be reported by UE. Current spec has already supported periodical and event triggered measurement report of L3 cell level measurement result and L3 level beam measurement result. NW can configure the report periodicity, which matches the interval of model inputs. Therefore, legacy measurement report can be reused to report model input of NW sided model.

Proposal 3: Legacy measurement report procedure can be reused for UE to report L3 cell level or L3 beam level measurement result as model input for NW sided model.

It’s up to NW how to perform inference and use the inference results, which has no impact on the spec.

Proposal 4: There is no specification impact associated to NW-side model inference. It’s up to NW how to use the inference results.

  - NW side data collection

In AI air, the data logging can be triggered in following ways,

- Radio Event, RSRP of serving cell becomes higher or lower (A1/A2)

- Periodic

These triggers can be reused. In addition, new radio event shall can be introduced. In RRM prediction 3 and 4, the target goal is to improve mobility performance. Handover is expected to be occur at cell edge rather than cell centre. Therefore, the data logged in cell centre may not be useful for NW model in RRM prediction 3 and 4. To reduce the unnecessary data logging, new radio event, e.g. A3, A5 alike event, can be introduced.

Proposal 5: New radio event, e.g. A3, A5 alike event, can be introduced to control data logging in AI mobility.

In AI air, a minimum AS layer memory size is assumed for data logging. And the memory is shared across all use cases of three use cases. It’s not clear whether additional/separate memory is needed for AI mobility data logging. We understand the AI mobility data can also share the same memory. Because it’s more efficient and can avoid the fragmentation. However, in future, more AI use cases are expected. Larger memory size may be required to log more data from many AI use cases. For forward compatibility, UE may support larger memory size and shall be allowed to report the supported memory size.

Proposal 6: AS layer memory for logged data is shared across all AI use cases, including AI mobility, AI air and future AI use cases.

Proposal 7: UE can indicate the supported AS layer memory size for data logging.

In AI air, it’s agreed NW can indicate UE to release or retain the logged data during handover. After AI mobility is introduced, NW may configure UE to perform data collection for AI air and AI mobility at the same time. During the handover, NW may have different logged data handling per use cases, since the AI model may be trained in different ways. For example, the data for AI mobility can be retained and the data for AI air can be released. Therefore, the logged data retain indication during handover should be indicated per use case.

Proposal 8: NW indicate whether to retain logged data during handover per use case.

If NW would release the data during handover, NW may request UE to report the logged data before handover. As in proposal 8, we support NW to indicate whether to retain logged data during handover per use case. NW should be allowed to request UE to report logged data of specific use case. UE can still retain the logged data of the other use cases.

Proposal 9: NW can request UE to report logged data of specific use case.

Conclusions

According to the analysis given above, we have the following observations and proposals:

Temporal domain case B

Observation 1: NW-side model in temporal domain case B can’t reduce UE measurement

Observation 2: SSB can’t be reduced by NW-sided model in temporal domain case B, due to impact to legacy UEs.

Proposal 1: NW sided model is not supported in temporal domain case B.

Input to NW side model

Observation 3: Huge spec impact is foreseen to support L1 beam as model input for NW sided model, i.e. sub case 1, 3, 4 and 6, including

- new signalling to report L1 beam measurement result of serving and neighbour cell for RRM, as current L1 beam for RRM is configured by CSI-RS-ResourceConfigMobility, rather than using CSI framework

- new requirement on UE measurement with potential RAN4 impact

- new signalling to report how L1 filtering is done, which is now completely up to UE implementation

Proposal 2: L1 beam measurement result as model input, i.e. sub case 1, 3, 4 and 6, is not supported for NW sided model in RRM prediction and L3 beam prediction.

Proposal 3: Legacy measurement report procedure can be reused for UE to report L3 cell level or L3 beam level measurement result as model input for NW sided model.

Proposal 4: There is no specification impact associated to NW-side model inference. It’s up to NW how to use the inference results.

NW side data collection

Proposal 5: New radio event, e.g. A3, A5 alike event, can be introduced to control data logging in AI mobility.

Proposal 6: AS layer memory for logged data is shared across all AI use cases, including AI mobility, AI air and future AI use cases.

Proposal 7: UE can indicate the supported AS layer memory size for data logging.

Proposal 8: NW indicate whether to retain logged data during handover per use case.

Proposal 9: NW can request UE to report logged data of specific use case.

Reference

[1] 38.744